# Natural Resources Wales permitting decisions

# Variation

We have decided to issue the variation for Carew Quarry water discharge activity permit operated by A&C Aggregates Ltd.

The application number is PAN-026220

The permit number is BP0236901

The discharge is for an additional new direct discharge point to Mill Pond lagoon from Carew Quarry of surface water consisting of mineral extraction dewatering and rainfall related site run off. This new discharge point is in addition to the existing permitted groundwater discharge via soakaway, that has connectivity to a natural Adit which emerges in Mill Pond lagoon.

The new surface water discharge point will be controlled and operated in accordance with the agreed operational technique cited in Table S1.1 and Table S1.2 of this permit.

The combined total volume discharged from the quarry via the groundwater discharge (activity reference A1) and the surface water discharge (activity reference A2) shall not exceed 10,000 cubic meters per day.

Abstraction of waters associated with this activity is controlled via the Water resources consent WA/061/0006/0023.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | There were no responses to web publicising. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | WFD stage 2 scoping has been completed and the activity/project is considered as having no risk of causing deterioration or preventing any water body or WFD Protected Area from reaching its objectives and taking account of the advice from technical officers - is considered compliant with the WFD Regulations 2017. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat. A full assessment of the application and its potential to affect the site/species/habitat has been carried out as part of the permitting process. We consider that the application will affect the features of the site/species/habitat. Operational controls/emission limits have been placed on the permit to mitigate against these effects. The following will apply “limit the discharge from the outfall pipe to periods when the mudflats are submerged, i.e. when the water level in the Mill Pond is at, or above, the level of lower of the two overflows, 3.1mAOD.” and as detailed in Table S1.2 operating techniques and Table S3.1 limits within the issued permit. We have not formally consulted on the application. The decision was taken in accordance with our guidance. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | Appropriate assessment: In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. (As documented in section 4 of OGN 200 form 1, and section 5 if applicable) | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment or similar methodology supplied by the operator and reviewed by ourselves, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The measures proposed by the operator have been assessed against current TGN 7.01. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Updating permit conditions during consolidation. | We have updated previous permit conditions to those in the new generic permit template as part of permit consolidation. The new conditions have the same meaning as those in the previous permit(s). The operator has agreed that the new conditions are acceptable. | ü |
| Incorporating the application | We have specified that the applicant must operate the permit in accordance with descriptions in the application, including all additional information received as part of the determination process. These are specified in the Operating Techniques table in the permit. | ü |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - Maximum daily discharge volume - See condition 2.3 - Maximum rate of discharge -115 litres per second - 15-minute instantaneous or averaged flow - No limit set. Record as l/s - Suspended solids (measured after drying at 105oC) - 50 mg/l - pH - 6 to 9 - Visible oil or grease - No significant trace present The maximum flow rate is controlled to ensure that the historic permitted volume is not exceeded via the use of both discharge points. All existing permit limits have been carried forward from the existing permit and applied to the new permitted discharge point. They have been assessed current WFD guidance and policy, Habitats Directive and EPR 2016 TGN 7.01. | ü |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. These monitoring requirements have been imposed in order to comply with section 2.4 Discharge Flow Measurements of TGN 7.01. We made these decisions in accordance with How to comply with your environmental permit Additional guidance for: Water Discharge and Groundwater (from point source) Activity Permits (EPR 7.01) | ü |
| Reporting | We have specified reporting in the permit. The reporting frequency request is considered appropriate regulation and continues the historic requirements of the permit. We made these decisions in accordance with How to comply with your environmental permit Additional guidance for: Water Discharge and Groundwater (from point source) Activity Permits (EPR 7.01) | ü |
| Considerations of foul sewer | We agree with the operator’s justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a groundwater and stand alone water discharge activity permit under EPR2016. | ü |