| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-030780 |
| Permit Reference Number: | EPR/AB3891CX |
| Permit Holder: | Mr Daniel James and Mrs Carys James |
| Site Address: | Castell Malgwyn Farm, Llechryd, Cardigan, Pembrokeshire, SA43 2QB |
| Grid Reference: | Farm location: SN 22107 43065 Fields: CASTELL MALG 9 SN 23710 42800 CASTELL MALG 11 (A) SN 22824 42796 CASTELL MALG 11 (B) SN 22693 42954 CASTELL MALG 12 SN 22910 42550 CASTELL MALG 15 SN 23630 42510 CASTELL MALG 16 SN 23450 42460 CASTELL MALG 17 SN 23280 42500 CASTELL MALG 18 SN 22930 42360 CM 4968 SN 23500 42680 CM 0961 SN 23090 42610 Storage: 1 SN 22043 43082 2 SN 22827 42616 3 SN 22136 43107 4 SN 22855 42607 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: pembsdistrictteam@cyfoethnaturiolcymru.gov.uk NLO: Malcolm.Dines@cyfoethnaturiolcymru.gov.uk |
| Date due: | 12/11/2025 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Fundamental checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- DMS/PaLS |
| Correct fee? | Y | Transaction number: 2149773 Date: 28/10/2025 Ref on MyNRW: BS/281025/34 STEPSIDE AGRICULTUEPDEPSTEPS0150FP 28/10/25 1146500000001654128931 Amount: £2,093.00 |
| Agricultural Benefit Statement | Y |  |
| Waste Analysis Report | Y |  |
| Soil Analysis Report | Y |  |
| Location Map | Y |  |
| Environmental Impact Assessment- Agricultural Regulations Evidence | Y |  |
| Risk assessment (if applicable) | Y |  |
| Landowner/occupier consent | Y |  |
| If confidentiality box has been ticked, no evidence provided | n/a |  |
| Declaration authorisation | n/a |  |

| Saving documents- by determining permitting officer | Y N |
| --- | --- |
| Save embedded application documents from application form | Y |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | See comments at bottom of decision document. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-026593 expired: 01/10/2025 |
| - Which risk band does the activity fall within? High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | Site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y |  |
| Flood risk | Y | Field Malg 9 encroaches on a flood plain from the River Teifi SAC/SSSI. There is a 20 meter no spread buffer zone that largely removes this area of high risk flooding. |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields Malg 2, Malg 18, CM 0961, Malg 17 and Malg 16 7 -12 degrees (Yellow)- fields Malg 9, CM 4968 and Malg 15 12 degrees plus (Red)- field Malg 11 |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | Spreading non stackable wastes |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Waste is high in RAN, but storage location are over 200 meters away from all SSSI and SACS. |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 19 09 02 - Al, Fe, Mn 02 05 02 – Fats and oils OMP |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | See comments in request for information section of the decision document and/or request for information response |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y | See comments in request for information section of the decision document and/or request for information response |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | Y |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y | See comments in request for information section of the decision document and/or request for information response |
| If the cropping is any of the following, check if that the available Nitrogen applied is less than the below amount: - Field beans, Peas- 0 kg/ha - Sugar beet- 120 kg/ha - Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes- 150 kg/ha - Spring-sown wheat, Winter barley- 180 kg/ha - Autumn or early winter sown wheat- 220 kg/ha *Regulation 10 The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021 | Y |  |
| Nutrient assessments for P- If land is above target index (3+ for arable, grassland, and forage crops. 4+ for vegetables) has a net reduction in P levels across the crop rotation been demonstrated? | Y | See comments in request for information section of the decision document and/or request for information response |
| Nutrient assessments for P- if the above has been agreed for a previous deployment, is it still correct for this this application? | n/a |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | N | See previous deployment: PAN-026593 |
| - The nutrients supplied by the material? | N |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant Environment team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site (project) within 500 meters of a European Site (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Yes - there is no conceivable impact pathway between the project and any European site features, due to the location of the project. |
| HRA is not required (Stage 1 of OGN200 assessment- Determining the need for a Habitats Regulations Assessment): HRA is not required (no FORM 1 has been completed) because there is no conceivable impact pathway between the project and any European site features, due to the nature or location of the project. Afon Teifi SAC Special features (management unit 1553) - Sea lamprey - Atlantic salmon - European otter Conclusion The spreading fields are located within 500 m of the Afon Teifi SAC. The closest field to the Afon Teifi SAC is field Malg 11 and is located 108 meters. Due to the proximity of the closest field to the SAC no impact pathways has been established where the deployment complies with SR201no4. In addition to this the deployment will operate with the following spreading techniques as outlined in the submitted agricultural benefit statement and site specific risk assessment: - Areas of highly sloping land in this deployment have implemented no spread buffer zones, as well as no spread buffer zones for areas of high risk of flooding. - All proposed waste streams are non-stackable liquids therefore dust is not a potential impact pathway. - Wastes high in RAN (VOLAC Sensient & DCWW Bontgoch) are not being spread in nitrogen closed periods. Storage area associated with this deployment are also further than 200 meters the SAC reducing risk of nitrogen volatilisation risks. - The submitted site plan includes the appropriate no spread buffer zones around water courses to mitigate the risk of nutrients and PTEs entering nearby water course / SAC. These no spread buffer zones comply with the conditions set out in SR2010n04. - Wastes will be spread via a low trajectory dribble bar that applies waste directly to the crop/grass minimising surface runoff. - EWC 19 09 02 waste streams that contain high aluminium flocculates have no land applications on fields that are < pH 6.0. For ferric flocculated sludges, no applications on fields that are < pH 5.5 to comply with TGN 8.01. - Spillages: all spillages will be reported immediately to NRW. - Regular servicing of all machinery is conducted and spreading equipment is annually calibrated. To prevent waste being held in faulty machinery replacement spreading equipment will be available. - Spreading machinery will travel over the field in a direction which will most easily allow the machinery to turn within the boundaries of the field. Any spreading equipment will be turned off prior to turning at the end of each run. - Machinery turns will be routed to avoid rutting and wheel slip. The turns will not be executed on any buffer strips. - There will be sufficient trained staff available to ensure that the operation continues throughout operational hours (i.e. There will be sufficient cover for illness, holiday etc.). - Weather conditions will be monitored prior to spreading with wind speed and direction assessed. - A site-specific risk assessment has been provided by the operator to take into account any potential issues and provided appropriate mitigation measures. |  |

Comments

Consultation responses: - No comments needing action

MyMap/location and slope assessments: MyMap/location assessment See Mymap assessment on the DMS. Slope assessment Field number: Malg 11 Slope of the land: 12o+ Soil type: Medium clay Wastes types applied: non stackable liquid Cropping type: Mazie Method of application: Dribble bar Weather conditions before during and after spreading: in compliance with the weather restriction outlined in CoGAP. Conclusions (is run off risk high?): The runoff risk has been determined as low due to the highly sloping land being removed from the deployment from no spread buffer zones. Field number: Malg 9, CM 4968 and Malg 15 Slope of the land: 7-12o Soil type: Medium clay Wastes types applied: non stackable liquid Cropping type: Mazie & Grass Method of application: Dribble bar Weather conditions before during and after spreading: in compliance with the weather restriction outlined in CoGAP. Conclusions (is run off risk high?): The runoff risk has been determined as low due to the waste being applied via low trajectory dribble bar that applies the waste directly to the crop and grass. The moderately sloping land is largely contained to the spreading fields. In addition The maximum application rate for each field over 50t/ha will be split into multiple applications and will not exceed 50t/ha in any one application to a field lowering surface runoff risk. Fields Malg 2, Malg 18, CM 0961, Malg 17 and Malg 16 are located on flat to gently sloping land therefore the runoff risk has been determined as low requiring no action.

CPH details: CPH number provided LPD1: 55/502/0059 My map record – All fields - 55/502/0059 MESSRS MORRIS The CPH numbers provided in the LPD1 form match the Mymap record no action needed.

Protection of habitats and ecosystems See MyMap report on the DMS.

Agricultural benefit calculations: Malg 9, 12, 15, 16,17 and 18 have a soil index value of either 2 or 3 (at or above optimum for phosphorus, grass and barely). . While the waste streams in this deployment are all adding varying amounts of phosphate to the soils at a maintenance dressing. The applicant has justified applying phosphorus to these fields by outlining the crop rotation the fields with the aim of bringing down phosphorus levels . Fields Malg 11, CM 4969 and CM 0961 have a soil index value of 1 for phosphorus (below optimum levels). The deployment is applying additional phosphate in these fields to build up the phosphorus levels in the soil so that the soil index value are closer to optimum levels. The increases in this deployment comply with he recommended levels for the grass and crops set out in RB209 section 3. Fields Malg 11a ,11b, Malg 12 & CM 0961 have a magnesium indices of 3 where the wastes under this deployment are adding various amounts of magnesium to the soil. The applicant has justified this by stating ‘The magnesium applied by the wastes is low and is less than is likely to be removed by the crops. Grass is not responsive to magnesium however herbage levels should be maintained to prevent ‘Grass Staggers’ in lactating animals. Potassium applications can reduce magnesium uptake resulting in ‘Staggers’. The addition of magnesium will help maintain levels over a rotation period. The receiving soils have magnesium indexes of 2 or 3. Application of liquid sludges at the proposed application rates with potassium also being applied in the sludges is very unlikely to reduce potassium availability which can be seen in soils where the magnesium index is 5 or above.’ The above justification has been accepted and complies with TGN 8.01 on magnesium justification. EWC 19 09 02 wastes containing high amounts of Al and Fe flocculants are not being spread on field with p.H of 5.5 (Fe) and 6 for Al to comply with TGN 8.01.

Request for information: 05/12/2025 - Update the ABS so that it is clear how it is intended to reduce the phosphorus levels in the soil over the whole crop rotation for fields Malg 9, Malg 15, Malg 16, Malg 17 & Malg 18. The applicant has explained how phosphate levels will be monitored and the proposed crop type for the fields listed for 2027. - Update the ABS so that includes justification why the increase of magnesium is appropriate while also explaining the risks in the ABS and SSRA. The applicant has updated the SSRA and ABS for the justification needed.

Other information to note: The applicant has provided an odour management plan for this deployment as 3 of the wastes are dairy wastes (EWC 02 05 02) where TGN 8.01 states an odour management plan is likely to be needed. After review of nearby local residents NRW agree an odour management plan is required. The odour management plan has been assessed in line with the H4 odour management plan guidance and complies with it. If spreading occurs as stated in the odour management plan the risk of odour is negligible and manageable for the nearby local residents/ sensitive receptors. No further action was required.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 12/12/2025 |
| ## Effective date | 12/12/2025 |
| ## Permitting Officer Date | Owen Clee 12/12/2025 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 12/12/2025 |

| Time recording |  |
| --- | --- |
| Admin/consultations (Permit support time) | 45 |
| Determination (waste permitting team time) | 1,825 |
| Total (to be added to PALS) | 1,870 |