Part A

| Permit Number | VP3935AT |
| --- | --- |
| Name of Operator | Enovert South Ltd |
| Location of Installation | Tir John Landfill, Wern Terrace, Off Fabian Way, Swansea, SA1 8PA |
| Time and date of the detection | October 2025 |
| (b) Notification requirements for the breach of a limit (within 24 hours unless otherwise agreed) |  |
| Emission point reference/ source | Phase 2 LEW Leachate Wells Leachate Head above Cell Base >1m See Table 1 Below The October monitoring had to be undertaken in early November, and a further set will be undertaken later in November. |
| Parameter(s) |  |
| Limit |  |
| Measured value and uncertainty |  |
| Date and time of monitoring |  |
| Measures taken, or intended to be taken, to stop the emission | Leachate remains contained within the engineered containment system, so no emission from the site is occurring. |

Part B to be supplied as soon as practicable

| Any more accurate information on the matters for notification under Part A. | Leachate removal to sewer is ongoing. A Leachate Action Plan was submitted to NRW on 30th August 2025 which provides a wealth of detail about our investigations and planned further actions. |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident | Landfill capping works are now fully completed across the site, which reduce rainfall infiltration and leachate generation rates. We may experience new fluctuations across leachate levels as the site settles. It is unlikely that all the dipped levels are representative of the actual leachate levels in the waste mass. It is likely that as the waste ages and compresses, the pore spaces are reduced, and liquid is effectively squeezed out into the wells. Pumping, capping and drainage methods have impacted average leachate heads across the site, which appear to be following an overall continuous downward trend year on year. |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission | The elevated level recorded in wells LEW15a are due to well or pump clogging issues, due to the presence of the solidifying jelly-like substance. Having commissioned Ayesa to undertake some work on the jelly it is clear that a single method is not going to resolve the issue for the widespread impact however we have subsequently installed a macerator pump down LEW14 which is currently proving successful in clearing the liquid below in that well. Unfortunately the size limits its use in some of the other affected wells and alternative pumps are being reviewed. |
| The dates of any unauthorised emissions from the installation in the preceding 24 months. | See previous notifications. |
| Name* | Michelle Preston |
| Post | Environmental Data Analyst |
| Date | 12/11/2025 |

* authorised to sign on behalf of Enovert South Ltd

| Sample Point | Date | Leachate Head (m) | Comment |
| --- | --- | --- | --- |
| LEW15A | 04/11/2025 | 13.26 |  |
| LEW16 | 04/11/2025 | 1.86 |  |
| LEW20 | 04/11/2025 | 3.34 |  |
| LEW21 | 04/11/2025 | 3.47 |  |
| LEW23 | 04/11/2025 | 3.13 |  |
| LEW25 | 04/11/2025 | 4.18 |  |
| LEW26 | 04/11/2025 | 1.41 |  |
| LEW27 | 04/11/2025 | 4.76 |  |
| LEW28 | 04/11/2025 | 8.45 |  |
| LEW29 | 04/11/2025 | 7.52 |  |
| LEW32 | 04/11/2025 | 3.09 |  |
| LEW33 | 04/11/2025 | 2.09 |  |

Table 1: Breach of Compliance Limit for Leachate Boreholes Phase 2 area October 2025