Permit Number: 	EPR/BW9999IG			Operator: 		Kronospan Limited

Facility:		Chirk Particleboard Factory		Form Number: 	 Air1 / 04/06/2025

# Reporting of emissions to air for the period from 01/07/2025 to 30/09/2025

| Emission Point | Substance / Parameter | Emission # Limit Value | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Formaldehyde | 5 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each | 0.4 | CEN / TS 17638:2021 | 20/05/2025 ¦ 10:58 - 11:28 ¦ 11:41 - 12:11 ¦ 12:20 - 12:50 | 0.07 |
|  | Total volatile organic compounds | 30 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each | 1.3 | EN 12619 | 20/05/2025 ¦ 10:58 - 11:28 ¦ 11:41 - 12:11 ¦ 12:20 - 12:50 | 0.08 |
| A5, A6, A16, A17 A18 | No Annual testing this Quarter |  |  |  |  |  |  |
| A19, A20 | No quarterly testing, Turbines not operational for more than 28 days in Q3 |  |  |  |  |  |  |
| A21 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 280 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) | 231.3 | BS EN 14792 | 05/09/2025 ¦ 09:33 - 10:03 ¦ 10:03 - 10:33 ¦ 10:33 - 11:03 | 14.30 |
|  | Carbon Monoxide | No Limit set |  | 381.7 | BS EN 15058 | 05/09/2025 ¦ 09:33 - 10:03 ¦ 10:03 - 10:33 ¦ 10:33 - 11:03 | 24.2 |
| A22 | No quarterly testing. non operational for more than 28 days |  |  |  |  |  |  |
| A23 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 280 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) | 239.3 | BS EN 14792 | 05/09/2025 ¦ 11:06 - 11:36 ¦ 11:36 - 12:06 ¦ 12:06 - 12:36 | 15.20 |
|  | Carbon Monoxide | No Limit set |  | 739.3 | BS EN 15058 | 05/09/2025 ¦ 11:06 - 11:36 ¦ 11:36 - 12:06 ¦ 12:06 - 12:36 | 47.90 |
| A24, A25 | Not installed |  |  |  |  |  |  |
| A26 (K7) | Oxides of nitrogen (NO and NO2 expressed as NO2) | 250 mg/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) | 1032.0 | BS EN 14792 | 24/09/2025 ¦ 09:32 - 10:02 ¦ 10:09 - 10:39 ¦ 10:47 - 11:17 | 91.6 |
|  | Carbon Monoxide | 150 mg/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) | 13.3 | BS EN 15058 | 24/09/2025 ¦ 09:32 - 10:02 ¦ 10:09 - 10:39 ¦ 10:47 - 11:17 | 1.2 |
|  | Particulate Matter | 30 mg/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) | 89.7 | BS EN 13284-1 | 24/09/2025 ¦ 09:32 - 10:02 ¦ 10:09 - 10:39 ¦ 10:47 - 11:17 | 8.8 |
|  | Sulphur Dioxide | 200 mg/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) | 76.7 | BS EN 14791 | 24/09/2025 ¦ 09:32 - 10:02 ¦ 10:09 - 10:39 ¦ 10:47 - 11:17 | 9.0 |
|  | Hydrogen fluoride | Bi-annual testing to be completed in Q4 |  |  |  |  |  |
|  | Total Organic Carbon (TOC) |  |  |  |  |  |  |
|  | Hydrogen Chloride |  |  |  |  |  |  |
|  | Ammonia (NH3) |  |  |  |  |  |  |
|  | Nitrous Oxide (N2O) |  |  |  |  |  |  |
|  | Cd & Tl and their compounds (total) |  |  |  |  |  |  |
|  | Sb, As, Pb, Cr, Co, Cu, Mn, Ni & V and their compounds (total) |  |  |  |  |  |  |
|  | Mercury and its compounds |  |  |  |  |  |  |
|  | Dioxins / furans (I-TEQ) |  |  |  |  |  |  |
|  | Dioxin-like PCBs (WHO-TEQ Humans / Mammals, Fish, Birds) |  |  |  |  |  |  |
|  | Dioxins / furans (WHO-TEQ Humans / Mammals, Fish, Birds) |  |  |  |  |  |  |
|  | Specific individual poly-cyclic aromatic hydrocarbons (PAHs) as specified in Schedule 6 | Annual testing to be completed in Q4 |  |  |  |  |  |
|  | Exhaust gas temperature | No limit set | Periodic (average value of three consecutive measurements of at least 30 minutes each) | 297.00 | Traceable to national standards | 24/09/2025 ¦ 09:32 - 10:02 ¦ 10:09 - 10:39 ¦ 10:47 - 11:17 | N/A |
|  | Exhaust gas pressure |  |  | Not tested | Traceable to national standards |  | N/A |
|  | Exhaust gas flow |  |  | 251378 (actual) 44735 (ref) | EN 16911-2 |  | 115814 / 2061 |
|  | Exhaust gas oxygen content |  |  | 15.10 | EN 14181 and EN 17255 |  | 0.39 |
|  | Exhaust gas water vapour content |  |  | 7.30 | EN 14181 and EN 17255 |  | 0.22 |
| A27 (K8) | Hydrogen Fluoride | 1 mg/m3 | Average of three consecutive measurements of at least 30 minutes each | 0.11 | CEN TS 17340 | 26/08/2025 ¦ 12:02 - 13:02 | 0.013 |
|  | Cadmium & thallium and their compounds (total) | 0.03 mg/m3 | Average of three consecutive measurements of at least 30 minutes each | 0.0290 | BS EN 14385 | 26/08/2025 ¦ 10:06 - 10:54 | 0.006 |
|  | Mercury and its compounds | 0.03 mg/m3 | Average of three consecutive measurements of at least 30 minutes each | 0.01 | EN 13211 | 26/08/2025 ¦ 11:06 - 11:54 | 0.001 |
|  | Antimony, Arsenic, Lead, Chromium, Cobalt, Copper, Manganese, Nickel & Vanadium and their compounds (total) | 0.45 mg/m3 | Average of three consecutive measurements of at least 30 minutes each | 0.61 | BS EN 14385 | 26/08/2025 ¦ 10:06 - 10:54 | 0.14 |
|  | Dioxins and Furans (I-TEQ) | 0.09 ng/m3 | Periodic over minimum 6 hours, maximum 8 hour period | 0.063 | Relevant parts of EN 1948 | 27/08/2025 ¦ 08:55 - 14:55 | 0.02 |
|  | Dioxin-like PCBs (WHO-TEQ Humans / Mammals, Fish, Birds) | No limit set | Periodic over minimum 6 hours, maximum 8 hour period | Not tested this Quarter. |  |  |  |
|  | Dioxins / furans (WHO-TEQ Humans / Mammals, Fish, Birds) |  |  | 0.059 | BS EN 1948 Parts 1, 2 and 3 | 27/08/2025 ¦ 08:55 - 14:55 | 0.02 |
|  | Specific individual poly-cyclic aromatic hydrocarbons (PAHs) as specified in Schedule 6 |  |  | Annual testing Due to Q4 |  |  |  |
| A28 | Particulate Matter | 15 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 3.1 | BS EN 13284-1 | 25/09/2025 ¦ 11:29 - 12:08 ¦ 12:17 - 12:56 ¦ 13:02 - 13:41 | 1.2 |
|  | Total Volatile Organic Compounds | 30 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 5.5 | BS EN 12619 | 25/09/2025 ¦ 11:30 - 12:00 ¦ 12:20 - 12:50 ¦ 13:05 - 13:35 | 0.4 |
|  | Formaldehyde | 5 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 2.7 | CEN / TS 17638:2021 | 25/09/2025 ¦ 11:29 - 12:08 ¦ 12:17 - 12:56 ¦ 13:02 - 13:41 | 0.5 |
| A29 (cyclone 3) | Particulate Matter | 20 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 7.0 | BS EN 13284-1 | 01/09/2025 ¦ 12:41 - 13:12 ¦ 13:15 - 13:46 ¦ 13:49 - 14:20 | 1.3 |
|  | Total Volatile Organic Compounds | 120 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 76.8 | BS EN 12619 | 01/09/2025 ¦ 11:00 - 11:30¦ 11:30 - 12:00 ¦ 12:00 - 12:30 | 1.3 |
|  | Formaldehyde | 15 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 16.5 | CEN / TS 17638:2021 | 01/09/2025 ¦ 12:41 - 13:12 ¦ 13:15 - 13:46 ¦ 13:49 - 14:20 | 3.2 |
|  | Oxides of nitrogen (NO & NO2 expressed as NO2) | 100 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 90.0 | BS EN 14792 | 01/09/2025 ¦ 11:00 - 11:30¦ 11:30 - 12:00 ¦ 12:00 - 12:30 | 2.6 |
|  | Carbon Monoxide | 200 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each | 25.9 | BS EN 15058 | 01/09/2025 ¦ 11:00 - 11:30¦ 11:30 - 12:00 ¦ 12:00 - 12:30 | 0.92 |
|  | Hydrogen chloride | 30 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each | 0.20 | BS EN 1911 | 01/09/2025 ¦ 12:41 - 13:12 ¦ 13:15 - 13:46 ¦ 13:49 - 14:20 | 0.16 |
|  | Hydrogen fluoride | 1 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each | 0.05 | CEN TS 17340:2020 | 01/09/2025 ¦ 12:41 - 13:12 ¦ 13:15 - 13:46 ¦ 13:49 - 14:20 | 0.005 |
|  | Mercury | No limit Set | Average value of three consecutive measurements of at least 30 minutes each | 0.00080 | EN 13211 | 01/09/2025 ¦ 14:25 - 14:56 ¦ 15:00 - 15:30 ¦ 15:35 - 16:06 | 0.00012 |
|  | Metals (Antimony, Arsenic, Cadmium, Cobalt, Chromium, Copper, Manganese, Nickel, Lead, Thallium & Vanadium) | No limit Set | Average value of three consecutive measurements of at least 30 minutes each | 0.16 | EN 14385 | 02/09/2025 ¦ 09:40 - 10:11 ¦ 10:16 - 10:47 ¦ 10:52 - 11:23 | 0.03 |
|  | Polychlorinated dibenzo-dioxins and -furans | No limit Set | Average value of three consecutive measurements of at least 30 minutes each | 0.011 | EN 1948 parts 1, 2 and 3 | 01/09/2025 ¦ 10:58 - 11:29 ¦ 11:31 - 12:02 ¦ 12:04 - 12:35 | 0.0016 |
|  | Ammonia | No limit Set | Average value of three consecutive measurements of at least 30 minutes each | 1.2 | EN ISO 21877 | 02/09/2025 ¦ 09:40 - 10:10 ¦ 10:16 - 10:46 ¦ 10:52 - 11:22 | 0.11 |
| A30 Cyclone 2 | Particulate Matter | 20 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 1.5 | BS EN 13284-1 | 02/09/2025 ¦ 13:46 - 14:16 ¦ 14:21 - 14:51 ¦ 14:56 - 15:26 | 1.3 |
|  | Total Volatile Organic Compounds | 120 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 39.0 | BS EN 12619 | 02/09/2025 ¦ 12:01 - 12:31 ¦ 12:31 - 13:01 ¦ 13:01 - 13:31 | 2.6 |
|  | Formaldehyde | 15 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 27.4 | CEN / TS 17638:2021 | 02/09/2025 ¦ 13:46 - 14:16 ¦ 14:21 - 14:51 ¦ 14:56 - 15:26 | 5.40 |
|  | Carbon Monoxide | 200 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each | 7.9 | BS EN 15058 | 02/09/2025 ¦ 12:01 - 12:31 ¦ 12:31 - 13:01 ¦ 13:01 - 13:31 | 0.23 |
|  | Oxides of nitrogen (NO & NO2 expressed as NO2) | 100 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 5.9 | BS EN 14792 | 02/09/2025 ¦ 12:01 - 12:31 ¦ 12:31 - 13:01 ¦ 13:31 - 14:01 | 0.16 |
|  | Hydrogen Chloride | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.08 | BS EN 1911 | 02/09/2025 ¦ 13:46 - 14:16 ¦ 14:21 - 14:51 ¦ 14:56 - 15:26 | 0.0069 |
|  | Hydrogen Fluoride | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.20 | CEN TS 17340:2020 | 02/09/2025 ¦ 13:46 - 14:16 ¦ 14:21 - 14:51 ¦ 14:56 - 15:26 | 0.021 |
|  | Mercury | No limit set | Average value of three consecutive measurements of at least 30 minutes each | < 0.00054 | EN 13211 | 03/09/2025 ¦ 11:14 - 11:44 ¦ 11:49 - 12:19 ¦ 12:25 - 12:55 | 0.000083 |
|  | Metals (Antimony, Arsenic, Cadmium, Cobalt, Chromium, Copper, Manganese, Nickel, Lead, Thallium & Vanadium) | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.124 | EN14385 | 03/09/2025 ¦ 09:29 - 09:59 ¦ 10:04 - 10:34 ¦ 10:39 - 11:09 | 0.022 |
|  | Polychlorinated dibenzo-dioxins and -furans | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.02 | EN 1948 parts 1, 2 and 3 | 02/09/2025 ¦ 12:01 - 12:31 ¦ 12:36 - 13:06 ¦ 13:11 - 13:41 | 0.0034 |
|  | Ammonia | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.26 | EN ISO 21877 | 03/09/2025 ¦ 09:39 - 10:09 ¦ 10:14 - 10:44 ¦ 10:46 - 11:16 | 0.023 |
| A31 | Not in use this Quarter |  |  |  |  |  |  |
| A32 | Particulate Matter | 20 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 0.3 | BS EN 13284-1 | 28/08/2025 ¦ 12:26 - 13:27 ¦ 13:34 - 14:35 ¦ 14:42 - 15:43 | 0.70 |
|  | Total Volatile Organic Compounds | 200 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 130.7 | BS EN 12619 | 28/08/2025 ¦ 12:26 - 13:27 ¦ 13:34 - 14:35 ¦ 14:42 - 15:43 | 31.4 |
|  | Formaldehyde | *110 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 18.2 | CEN / TS 17638:2021 | 28/08/2025 ¦ 12:26 - 13:27 ¦ 13:34 - 14:35 ¦ 14:42 - 15:43 | 4.9 |
|  | Oxides of nitrogen (NO & NO2 expressed as NO2) | 200 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each | 165.0 | BS EN 14792 | 29/08/2025 ¦ 01:30 - 02:00 ¦ 01:00 - 01:30 ¦ 00:30 - 01:00 | 38.1 |
|  | Carbon Monoxide | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 5.4 | BS EN 15058 | 29/08/2025 ¦ 01:30 - 02:00 ¦ 01:00 - 01:30 ¦ 00:30 - 01:00 | 1.3 |
|  | Hydrogen Chloride | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.199 | BS EN 1911 | 28/08/2025 ¦ 12:26 - 13:27 ¦ 13:34 - 14:35 ¦ 14:42 - 15:43 | 0.70 |
|  | Hydrogen Fluoride | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.035 | CEN TS 17340:2020 | 28/08/2025 ¦ 12:26 - 13:27 ¦ 13:34 - 14:35 ¦ 14:42 - 15:43 | 0.068 |
|  | Mercury | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.0021 | EN13211 | 28/08/2025 ¦ 15:48 - 16:49 29/08/2025 ¦ 08:30 - 09:31 ¦ 15:48 - 16:49 | 0.00055 |
|  | Metals (Antimony, Arsenic, Cobalt, Chromium, Copper, Manganese, Nickel, Lead & Vanadium) | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.087 | EN 14385 | 29/08/2025 ¦ 11:49 - 12:50 ¦ 12:55 - 13:56 ¦ 14:03 - 15:04 | 0.026 |
|  | Cadmium & thallium and their compounds (total) | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.0021 | EN 14385 | 29/08/2025 ¦ 11:49 - 12:50 ¦ 12:55 - 13:56 ¦ 14:03 - 15:04 | 0.001 |
|  | Polychlorinated dibenzo-dioxins and -furans | No limit set | Average value of three consecutive measurements of at least 30 minutes each | 0.025 | EN 1948 parts 1, 2 and 3 | 28/08/2025 ¦ 09:55 - 10:56 ¦ 11:01 - 12:02 29/08/2025 ¦ 10:43 - 11:44 | 0.0089 |
| B01 - B31 | Testing/Method still to TBC |  |  |  |  |  |  |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

*1 raised limit of 15mg/Nm3 applies for this quarter, Please see 2025 08 25 Q3 A32 WESP 21 Formaldehyde A and B Signed.pdf

Signed ……Victoria Smith………………………………………….  		Date…25/11/2025..

(Authorised to sign as representative of Operator)