PAN-031163  My Map Report:

Application maps

Map key does not give buffer distances

Deployment Boundaries and storage

Storage, lagoon size is incorrect and not clearly identified on application map, this significantly reduces the spreadable area of the feild Storage is permitted under a separate SR2010No17 and clarified in application but should not be stated as storage for this application but should be a waste source, permitted boundary shown below.

Detailed river network 10m

Watercourse at boundary of 3 and within 1, buffered on application map

Watercourse within 15, buffered on application map

Pond W of 13, appears to be buffered on application map

Watercourse within 7, appears to be buffered on application map

Multiple watercourses at including Nant Melai 17-21 which appear to be buffered, although the NW boundary of 18 does not appear to be buffered as well as other boundaries. See comments re map key above

Watercourse within 17 not clearly shown or buffered

Groundwater source protection zones

none

Any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies within 50m.

Well W of and Spring within 7, shown on application map but do not appear to sufficiently buffered, see comment re map key.

Well within 14 shown on application map but does not appear to sufficiently buffered, see comment re map key.

None at other feilds

Any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies within 250m.

Issue NW of 1, shown on application map

Issue E of 18, shown on application map

None at other feilds

Within Groundwater Safeguard Zone

none

European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m

SSSI Coedydd Derw Elwy, NE of 21. Shown on application map and identified in SSRA.

SSRA states SSSI Coedydd Derw Elwy is 980m from spreading area but as below shows it is approx. 456m which is within the 500m range. At 980m it would not be considered.

Other protected sites within 500m:

- Protected Species-

Multiple within range including bat, otter, eel and salmon. Nil risk from the activity.

- Protected Habitats-

Lowland dry acid grassland and Lowland calcareous grassland within range. Nil risk

- Scheduled Ancient Monument- none

- Local Nature Reserve- none

- National Nature Reserve- none

- Ancient Woodlands-

Multiple areas of ancient woodland in range, shown on application maps but some are adjacent to fields and buffer width is not stated.

- Local Wildlife Sites (SINC)-

The following SINC are not identified in the application.

Pont Ty'n-y-cyll Ponds 3, Pont Ty'n-y-cyll Ponds 1, Pont Ty'n-y-cyll Ponds 2, Dyffryn Elwy (just in range) N of 21

Hafod y Gog and Nant Melai, adjacent to 7

Moel Unben, Pen-y-Coed, E of 17

Coed Hendrellwyn-y-Maen, NW of 1

Plas Buchan/ Coed Maenllwyd, E of 20

Coed Tan-y-Graig, W of 19

- National Park- none

- Nitrate Vulnerable Zone (NVZ)- ALL Wales

- Groundwater Safeguard Zone- none

Flood risk

Risks shown in application maps match NRW mapping system

Public rights of way

Footpath S of 3 not on application map

Footpath within 7 not on application map

Footpath within 18 & 19 not on application map

None at other feilds

Distance from Main Farm Address

Adjacent

Slope Map

See comments in DD, no assessment of slopes is given, fields 12 and 15 are steeply sloping to watercourses, run off risk is high.

END