# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Margam Substation operated by Laing O’Rourke Delivery Limited.

The application number is PAN-030261.

The permit number is EPR/DB3690ZS.

This permit regulates a temporary discharge of abstracted groundwater as a result of dewatering in order to construct a new substation, including an associated cable route. This is in order to deliver new 33kv supplies to TATA steel UK establishing a connection point with the existing steelworks as a connection point for transmission network reinforcement.

The dewatering discharge will be passed from excavated low points to a balancing pond for controlled surface water releases. This discharge from the balancing pond will then be pumped by sump pumps to a siltbuster unit where it will be treated. Within the siltbuster water will enter the MT30 mix tank whereby coagulant and flocculant are added proportionally to the measured flow rate. The Coagulant to be used is PAC: Poly-aluminium Chloride and the Flocculant to be used is Aquatreat 2084, an anionic polymer. The discharge then passes into the HB150 settlement tank where solids settle out. Following treatment, the discharge exits via a pipe to Upper Mother Ditch at a maximum rate of 30 litres per second.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | The application was publicised on our website in accordance with our guidance from 7 October – 3 November 2025. No responses were received. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | WFD stage 2 scoping has been completed and the activity/project is considered as having no risk of causing deterioration or preventing any water body or WFD Protected Area from reaching its objectives and taking account of the advice from technical officers - is considered compliant with the WFD Regulations 2017. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the sites has been carried out as part of the permitting process. We consider that the application will not affect the features of the sites. Formal consultation has been carried out with the Neath Port Talbot Council local Biodiversity Team as the discharge is within a local Wildlife Site: Junction 38 Wetland Complex Local Wildlife Site. No response was received. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. There is no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project (As documented in section 2.1 of OGN 200 form 1). | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment , all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN 7.01 “How to Comply with Your Environmental Permit” and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - Suspended solids (measured after drying at 105oC) – 60 mg/l - Total Aluminium at Al – 1,000 µg/l - pH between 6 and 9 The suspended solids limit has been set in line with current Water Quality Standards set out in the TGN 7.01 “How to Comply with Your Environmental Permit” The total Aluminium limit has been set as the MAC. The pH limit has been set in order to safeguard the receiving environment | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |