| TASK / JOB | Washing Down Post Dredging Campaign – TSHD (Valid for Bluefin, Orca and Marlin) |  |  |
| --- | --- | --- | --- |
| ASSESSED BY | Jac Williams | REFERENCE NUMBER | UKD/BIO/001 |
| DATE OF ASSESSMENT | 23rd January 2026 | REVIEW DUE DATE | 23rd January 2027 |

Indicate Areas of Ship “at risk” of Carrying INNS due to Dredging or Marine Works

Hopper Area Hopper Area Dredge pipes including pipework, pump and launder chute Dredge pipes including pipework, pump and launder chute Dragheads and associated deck area Dragheads and associated deck area

| Job Step | Risks Identified | Method Statement |
| --- | --- | --- |
| On completion of final dredge | Transfer of INNS outside of dredge area. | - Continue to flush system whilst recovering dredge pipes so that clean water is drawn through the system. - Recover and secure pipes if required during transit to deposit. |
| Prior to Washing Down |  | - Ensure ship is either within the dredge or deposit area, at no time during washing down is the ship to navigate out of these areas. - Inspect any dredge mud if accessible for flora and fauna, photograph and forward to Bio-Security Manager for identification and recording. |
| Dredge pipes including pipework, pump and launder chute | INNS within inner dredge system. | - Previous dredge cargo to be deposited, pumping clean water on top, flushing pump and system from intakes to launder chute. - If conditions allow, bottom doors to be closed and surplus water to be pumped out leaving a dry hold. |
| Hopper area | INNS within dredge arising left within hopper, particularly in door wells are around hopper infrastructure | - After cargo deposited above, inspect hopper from walkways to identify any areas containing dredge debris, pay particular attention to hopper infrastructure and weir top (spot lamp may be required at night). - Using monitors and/or hoses, direct jets to remove debris. - Pump from hopper adding additional water if needed to allow good suction from hopper. - Once hopper empty, inspect and repeat if necessary. |
| Dragheads and associated deck area | INNS on dragheads particularly on blades or other extremities. | - Use hose to wash deck area around draghead stow location. - Use hose to wash draghead of any remaining debris. - Inspect and repeat if necessary. |
| Post wash down | N/A | - Fill hopper with clean water if required as ballast. - Record in logbook |