| Date of Plan / Last Review | 23rd January 2026 |
| --- | --- |
| Date of Next Review | 23rd January 2027 |

| Fleet Composition |  |  |  |  |
| --- | --- | --- | --- | --- |
| UKD Bluefin |  | Type: Trailing Suction Hopper Dredger IMO Number: 9143427 LOA: 98m Breadth: 18m Draft: 5.97m Hopper Cap: 3900m3 |  |  |
| UKD Orca |  | Type: Trailing Suction Hopper Dredger IMO Number: 9491355 LOA: 78m Breadth: 15.85m Draft: 5.953m Hopper Cap: 2373m3 |  |  |
| UKD Marlin |  | Type: Trailing Suction Hopper Dredger IMO Number: 9064176 LOA: 84.6m Breadth: 16m Draft: 5.909m Hopper Cap: 2968m3 |  |  |
| Cherry Sand |  | Type: Hopper Grab Dredger IMO Number: 6811059 LOA: 62.5m Breadth: 12.04m Draft: 54.02m Hopper Cap: 765m3 |  |  |
| UKD Sealion |  | Type: Multi-cat Workboat with Plough IMO Number: 9267314 LOA: 25.97m Breadth: 10m Draft: 2.24m Hopper Cap: N/A |  |  |
| UKD Seahorse |  | Type: Multi-cat Workboat with Plough IMO Number: 9066655 LOA: 25.97m Breadth: 10m Draft: 2.24m Hopper Cap: N/A |  |  |
| UKD Resolution |  | Type: Coded Survey Vessel (Kept out of water) LOA: 5.80m Breadth: Draft: 0.60m Hopper Cap: N/A |  |  |
| UKD Seadragon | Type: Plough and water injection IMO Number: 1020370 LOA: 27.05m Breadth: 11.6m Draft: 2.55m Hopper Cap: N/A |  |  |  |

| On-board Biosecurity Responsibilities |  |  |
| --- | --- | --- |
| Master | Overall Responsibility for Biosecurity on-board. Accommodation, Food & Water Inspections. |  |
| Mate | Deck Maintenance, Potable Water Hygiene, Ballast Water Management. |  |
| Chief Engineer | Machinery and Plant Maintenance. |  |
| Head Office Responsibilities |  |  |
| General Manager | Assigning Biosecurity Functions, Signing Off Biosecurity Plans. |  |
| Safety & Compliance Manager | Responsibilities as Biosecurity Manager |  |
| Technical Manager | Control of Maintenance Budgets, Dry Dock Management. |  |
| Interested Parties | Contact Details | Specific Interest / Knowledge |
| ABP Group Compliance | atinline@abports.co.uk | Advice and guidance on INNS |
| ABP Clients | Various | Prevention of vessel as a pathway for INNS |
| Third Part Clients | Various | Prevention of vessel as a pathway for INNS |
| Port State | MCA in UK, otherwise various | Compliance with relevant legislation |
| MMO | info@marinemanagement.org.uk | Control of Marine INNS / Licensing |
| NRW | enquiries@naturalresourceswales.gov.uk | Control of Marine INNS / Licensing |
| GB NNS Secretariat | nnss@apha.gsi.gov.uk | Recording of INNS |

| Bio-Security Risk Assessment |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Pathway | Untreated Risk | Legislation | Preventative Measures | Responsibility | Residual Risk |
| Ballast Water (Bluefin, Marlin, Cherry Sand, Sealion & Seahorse) | Discharge directly to Sea | Medium | BWMC and associated national legislation | Follow approved ballast water management plan. | Master, Mate | Low |
| Ballast Water (Orca) | Discharged to Sea via Treatment plant | Low | BWMC and associated national legislation | No further measures required |  | Low |
| Ballast Sediment | Discharged during docking | Low | BWMC and associated national legislation | Follow approved ballast water management plan. | Master, Technical Manager | Low |
| Clean Hopper Water Carried as Ballast | Discharge directly to Sea | Low |  | Limiting amount carried. Before each new dredging campaign/and or when moving to a different region, once clear of harbour and outside shallow waters, conduct full exchange of hopper water so that at least 95% volume exchange is achieved. If unable to empty hopper due to weather, 95% volumetric exchange can be achieved by pumping through three times the volume of the hopper in accordance with the ballast water Management convention. Pump water out prior to arrival at destination port. | Master, Mate | Low |
| Mud Residues from Dredging Operations | Discharge directly to sea | High |  | Clean hopper, dredge equipment and plough prior to sailing from port before each new dredging campaign/and or when moving to a different region. Photograph and report any fauna & flora dredged up. Identify, record and report if INNS discovered. | Master, Bio-security Manager | Low |
| Fresh (Potable) Water | Discharge directly to Sea | Low | International Health Regulations 2005 | Application of chlorine, monitoring levels in accordance with Integrated Management System 3.1.18 – Fresh Water Management. | Mate | Low |
| Biofouling of Ships Hull | In water | Medium | MS (Bio-Fouling Systems) Reg 2009 | Use of certified anti-foul paint system, maintenance and reapplication every 2 years (which is over and above Class requirement of 3 yearly) | Technical Manager | Low |
| Biofouling of seawater intakes and strum boxes | Discharged with waste, thrown overboard | Low |  | Copper anodes in sea-boxes or in sea-box flush systems prevents build-up of biofouling. Any biofouling removed from filters is disposed of or incinerated. | Chief Engineer | Low |
| Biofouling of Survey Boat | In water | Low |  | Boat is usually kept out of water on the trailer. When kept in water for any period, hull is inspected and if necessary, washed down before removal from site. | Surveyor | Low |
| Pests / Rodents carried in Food Stores | Gangway, mooring ropes | Low | MS (Crew Accommodation) Regs 1997 International Health Regs 2005 | Masters weekly inspection of accommodation & food storage areas Ship Sanitation Inspection (6 monthly) | Master Port Health | Low |