# Supporting Information for Marine Licence Application

# NRW Grab Sampling Survey programme 2019-2021

Contents

Introduction	3

Monitoring Survey Summary	5

MPA monitoring (mainly Habitats Directive Surveys)	5

Ad hoc MPA surveys	5

Annex I Large Shallow Inlet and Bay monitoring outside of SACs	6

Milford Haven Waterway Environmental Surveillance Group Sampling	10

Chemistry assessment:	10

Sediment and Invertebrates assessment:	10

Marine Protected Area reference monitoring	12

Water Framework Directive Surveys	14

Water Framework Directive Investigation Monitoring	14

Skomer Marine Conservation Zone Monitoring	16

Clean Seas Environment Monitoring Programme (CSEMP)	18

NRW and Bangor University Collaborative Survey of sediment dynamics of Caernarfon Bay and Menai Strait and Conwy Bay SAC	18

INNS investigations at Disposal Grounds	20

Sampling within protected areas	25

Environmental Mitigation	27

HRA Assessment	27

Designated Sites	27

Candidate Sites	28

WFD Assessment	28

Volume of sediment	29

Worst Case Scenario	29

Scenario 1; Milford Haven Inner:	29

Scenario 2; Three Rivers:	29

Conclusion:	29

Appendix 1 – Copy of Sensitive Habitats and Marine Licence relating to previous licence RML1556	30

Bristol Channel Inner North	33

Milford Haven Inner WFD Water Body survey	35

Milford Haven inlet SSSI and Habitats Directive Body survey	36

Appendix 2. Indicative sample point locations	37

Appendix 3. A list of Transitional WFD waterbodies within SACs	57

Appendix 4. A list of Coastal WFD waterbodies within SACs	58

# Introduction

Marine licence RML1556 expired on Dec 12th, 2018.  RML1556 authorised NRW’s ongoing programme of benthic sampling and therefore this application is being made for the next 3 years of the survey programme (2019-2021).  The activities are likely to be similar, but not identical, to those authorised under RML1556. Due to organisational re-design and possible changing monitoring priorities there may be some changes to NRW’s current planned sediment sampling programme (Figure 1). Therefore, we have had to build in flexibility with this licence application and the total number of grab samples taken is expected to be significantly lower than the maximum total number provided in this document. Numbers quoted in this document include successful samples (not failed samples; for example, where the grab does not close properly, or a sediment sample is not deemed large enough to accept).

The proposed marine benthic grab sampling will take place at various offshore sediment habitats, bays, coasts and estuaries around Wales.  The application covers multiple surveys between 2019-2021, mainly to be undertaken to meet statutory needs. All sampling will improve knowledge and be used to assess marine habitat health to better understand, manage and conserve marine habitats in Wales.

The surveys can be grouped into the following categories:

- Marine Protected Area monitoring (mainly Habitats Directive [currently planned standard monitoring and ad hoc/incident response])

- Annex 1 Large Shallow Inlet and Bay monitoring outside of SACs

- Marine Protected Area reference sampling

- Water Framework Directive (standard monitoring and investigation)

- NRW and Milford Haven Waterway Environmental Surveillance Group collaborative sampling

- Skomer Marine Conservation Zone monitoring

- Invasive Non-Native Species (INNS) investigations at marine sediment disposal grounds

- Clean Seas Environmental Monitoring Programme (CSEMP)

- NRW and Bangor University Collaborative Survey

Each grab sample has a maximum area of 0.1m2 and the approximate maximum volume of 15 litres (although rarely would reach 14 litres and usually wouldn’t reach 10 litres).  The volume attained depends on the sediment type.

The Marine Licensing Team were contacted regarding advertising requirements as this application does not meet the requirements for a Band 1 licence.  The Marine Licensing team confirmed advertisement of a public notice would not be required (query reference Q18176).

Figure 1 A map of currently planned samples around Wales. Circles and squares = grab sample indicative positions. Red = Annex I Large Shallow Inlet and Bay surveys; Blue = WFD surveys, Green = MPA monitoring (mainly Special Area of Conservation), Pink = MCZ samples, Orange = Collaborative WFD and Habitats Directive Surveys and black squares = Planned Grab Samples in collaboration with Milford Haven Waterway Environmental Surveillance Group. Designated SACs shown in light blue and WFD waterbodies indicated by black lines.

# Monitoring Survey Summary

In this section the purpose and nature of survey work is summarised. However, samples primarily taken for one purpose are often used to inform/report for numerous directives and NRW ensure that surveys are undertaken in a way which enables as much use of the data as possible to add value to our time at sea and laboratory analysis time and power to our assessments. The sample numbers are provided in summary tables below (Table 1, Table 2 & Table 3). There may be some double counting, for example Conway bay SAC monitoring includes almost all samples taken for Conway Bay WFD sampling.

## MPA monitoring (mainly Habitats Directive Surveys)

Surveys are planned to be undertaken in Special Areas of Conservation in order to assess the condition of designated Annex I habitats (Large Shallow Inlets and bays, Sandbanks and Estuaries) (see Figure 1 for indicative planned sample positions) in order to assess the condition of Annex I sediment habitats such as Sandbanks, Estuaries and Large Shallow Inlets and Bays. Planned numbers are summarised in Table 1 and currently planned positions are presented in appendix II and Figure 1 however, positions and timings are indicative and may change if priorities/conditions change but will remain within the SAC area within 2019-2021 timeframe. A small number of samples lie outside of the SAC where the feature (e.g. sandbank) intersects with the SAC boundary or historic sample sites lie just outside the SAC boundary (this includes Bias Bank, Pembrokeshire Marine SAC Survey [15 samples] and in Conwy Bay [6 samples], Menai Strait and Conwy Bay SAC Survey – see appendix II) Habitats Directive surveys are currently planned at the following Special Areas of Conservation:

- Pen Lleyn a’r Sarnau SAC

- Carmarthen Bay and Estuaries SAC

- Pembrokeshire Marine SAC

- Menai Strait and Conwy Bay SAC

- Cardigan Bay SAC

Table 1 A summary of the number of samples currently planned in various SACs to undertake MPA (Habitats Directive) feature condition monitoring

| Special Area of Conservation | 2019 | 2020 | 2021 | Total |
| --- | --- | --- | --- | --- |
| Cardigan Bay SAC |  | 30 |  | 30 |
| Carmarthen Bay and Estuaries SAC | 76 | 92 | 190 | 358 |
| Lleyn Peninsula and the Sarnau SAC | 40 | 40 | 121 | 201 |
| Menai Strait and Conwy Bay SAC | 163 | 40 | 40 | 243 |
| Pembrokeshire Marine SAC | 118 | 102 | 160 | 380 |
| Total | 409 | 316 | 523 | 1,248 |

## Ad hoc MPA surveys

There may be a requirement for unplanned surveys to be undertaken in response to an incident or if monitoring priorities change. Ad Hoc surveys (for incident response, or to investigate an issue not currently planned) could be carried out within all SACs outlined below – a maximum of 100 additional samples per site per year.  It is extremely unlikely that all these ad hoc samples will be taken (for comparison in the 3 years sampling programme authorised by Marine Licence RML1556 approximately 200 of a possible 2,100 Ad hoc samples were taken across all the SACs in Wales).

- Pen Lleyn a’r Sarnau SAC

- Carmarthen Bay and Estuaries SAC

- Pembrokeshire Marine SAC

- Menai Strait and Conwy Bay SAC

- Cardigan Bay SAC

- Severn Estuary (Wales) SAC

- The Dee Estuary (Wales) SAC

## Annex I Large Shallow Inlet and Bay monitoring outside of SACs

Surveys are planned to be undertaken at Large Shallow Inlets and Bays outside of the SAC network. This allows comparison with protected sites and appraisal of this habitat for EU Article 17 Habitats Directive reporting. The number of samples to be taken are summarised in Table 2 and in Figure 2, Figure 3 & Figure 4. These surveys are planned at the following Large Shallow Inlets and Bays:

- Caernarfon Bay (Figure 2)

- Swansea Bay (Figure 3)

- Fishguard Bay (Figure 4)

Currently planned target positions are presented in Appendix II and in Figure 2, Figure 3 & Figure 4. Planned numbers are summarised in Table 2 and currently planned positions are presented in appendix II and Figure 2, Figure 3 & Figure 4 however, positions and timings are indicative and may change if priorities/conditions change but will remain within the areas defined within 2019-2021 timeframe. At Swansea bay, a number of samples are outside the Large Shallow Inlets and Bay Feature, this is so that the data can also be usefully used for WFD assessment for the Swansea Bay Waterbody; therefore sampling will be undertaken within the area encompassed by the Large Shallow Inlet and Bay and the Swansea Bay WFD waterbody 2019-2021 (Figure 4).

Table 2 A summary of the number of samples planned to be taken at Large Shallow Inlet and Bays outside of SACs in Wales

| Annex I Bay | 2019 | 2020 | 2021 | Total |
| --- | --- | --- | --- | --- |
| Caernarfon Bay | 40 | 64 | 40 | 144 |
| Fishguard Bay | 40 | 58 | 40 | 138 |
| Swansea Bay | 40 | 70 | 40 | 150 |
| Total | 120 | 192 | 120 | 432 |

Figure 2 Caernarfon Bay Large Shallow Inlet and Bay Feature with indicative currently planned sample positions.; Red = Bay Large Shallow Inlet and Bay Feature monitoring, Blue = WFD surveys. Light green polygon = Large Shallow Inlet and Bay Feature extent.

Figure 3 Fishguard Bay Large Shallow Inlet and Bay Feature with indicative currently planned sample positions. Red = Bay Large Shallow Inlet and Bay Feature monitoring. Light green polygon = Large Shallow Inlet and Bay Feature extent.

Figure 4 Swansea Bay Large Shallow Inlet and Bay Feature (light green polygon) and Swansea Bay WFD waterbody (black line border) with indicative currently planned sample positions; Red = Bay Large Shallow Inlet and Bay Feature monitoring, Blue = WFD surveys, Green = MPA monitoring (mainly Special Area of Conservation) and Orange = Collaborative WFD and Habitats Directive Surveys. Light green polygon = Large Shallow Inlet and Bay Feature extent.

## Milford Haven Waterway Environmental Surveillance Group Sampling

NRW funds and has a collaborative relationship with the Milford Haven Waterway Environmental Surveillance Group (MHWESG). The MHWESG is a group of organisations (including NRW) who aim to provide high quality environmental data to enable members of the group and other authorities and industry working in and adjacent to Milford Haven Waterway to contribute to the maintenance and enhancement of the rich and diverse marine environment in the waterway.

Every three years the group sample around Milford Haven for sediment chemistry, sediment dwelling invertebrates and sediment, the indicative positions of samples are presented in Figure 5 and appendix II.

### Chemistry assessment:

12 stations are planned to be grab sampled to monitor contaminants (chemistry), 5 replicate samples are taken at 3 of these stations and 1 replicate sample is taken at the remaining 9 stations. In total 24 samples are planned to be taken in 2021 but timings may change (Figure 5).

### Sediment and Invertebrates assessment:

5 replicate grab samples are planned to be taken at all stations for biological macrobenthic sampling, plus 1 grab for PSA analysis = 6 grabs per station. In total, 48 grab samples are planned to be taken in 2021 but timings may change (Figure 5).

As with surveys detailed above the positions and timings presented are indicative and will depend on priorities and conditions but the remain within the Milford Haven Waterway.

Figure 5 A chart showing indicative sample positions of the sampling to be undertaken by Milford Haven Waterway Environmental Surveillance Group in collaboration with NRW (black squares indicate approximate position of planned sample locations).

## Marine Protected Area reference monitoring

As part of changing priorities of sampling (currently under review by NRW), there may be a need to sample outside of MPAs to compare with protected areas. These samples will be outside protected sites (Figure 1) within 12 nautical miles of the welsh coast (Figure 6). This could be up to 300 samples per year across wales, this area overlaps with all WFD waterbodies outside of SACs (Figure 1).  This work is currently not planned but we would like it included in the licence if possible in case this becomes a priority following a current review of biodiversity monitoring (Inshore Monitoring Options project), the total actual numbers attained is likely to be significantly lower than that requested annually.

Figure 6 National Welsh nautical limits (see legend) along with indicative planned monitoring stations

## Water Framework Directive Surveys

Grab sampling is required to assess the benthic invertebrate element of the Water Framework Directive Assessment, this will contribute to status assessments along with other ecological and physical elements. WFD surveys are planned at the following waterbodies (Figure 7). Planned number of samples are summarised in Table 3. The planned target positions and timings presented in Figure 1, Figure 7 and appendix II are indicative and may change with changing priorities and conditions on the day but the samples will be taken within stated WFD waterbodies in 2019-2021.

- Holyhead Bay

- Conwy Estuary

- Conwy Bay (joint habitats directive survey)

- Teifi

- Carmarthen Bay (joint habitats directive survey)

- Cardigan Bay North

- Cardigan Bay Central

- Milford Haven Inner

- Milford Haven Outer

- Caernarfon Bay South

Table 3 A summary of samples planned for WFD assessments 2019-2021

| WFD Waterbody | 2019 | 2020 | 2021 | Total |
| --- | --- | --- | --- | --- |
| Coastal (total) | 70 | 52 | 120 | 242 |
| Caernarfon Bay South |  |  | 58 | 58 |
| Cardigan Bay Central |  |  | 20 | 20 |
| Cardigan Bay North |  |  | 22 | 22 |
| Carmarthen Bay |  | 52 |  | 52 |
| Conwy Bay | 40 |  |  | 40 |
| Holyhead Bay | 20 |  |  | 20 |
| Milford Haven Outer |  |  | 20 | 20 |
| Transitional (total) | 20 | 24 | 50 | 94 |
| MILFORD HAVEN INNER |  |  | 50 | 50 |
| TEIFI ESTUARY |  | 24 |  | 24 |
| Conwy Estuary | 20 |  |  | 20 |
| Total | 90 | 76 | 170 | 336 |

## Water Framework Directive Investigation Monitoring

Ad Hoc samples for investigation of WFD failures / incidents may be required.  A maximum of 100 samples per waterbody, with a maximum of 400 samples overall is expected to be sufficient in include in this licence. It is very unlikely that all these potential samples would be taken.

Figure 7 Welsh Coastal and Transitional WFD waterbodies with indicative currently planned sample positions; Red = Large Shallow Inlets and Bays outside of SACs, Blue = WFD surveys, Pink = MCZ monitoring stations, Green = MPA monitoring (mainly Special Area of Conservation) and Orange = Collaborative WFD and Habitats Directive Surveys.

## Skomer Marine Conservation Zone Monitoring

Sediment infauna and epifaunal communities are recognized as management features of the Skomer Marine Conservation Zone. The sediments studied at Skomer also include several of the priority sediment habitats identified in Section 7 of the Environment (Wales) Act 2016. Surveys are required every 3 years to assess the condition of local habitats.  3 samples are planned to be collected at 12 locations (36 samples in total). See Figure 8 and appendix II for a summary of sampling locations. The planned target positions and timings presented in figure Figure 8 and appendix II are indicative and may change with changing priorities and conditions on the day but sampling will remain within the MCZ boundary between 2019-2021.

Figure 8 Skomer Marine Conservation Zone with indicative currently planned sample positions.

## Clean Seas Environment Monitoring Programme (CSEMP)

The CSEMP programme provides a coordinated approach to environmental monitoring in the UK's coastal and estuarine areas. There are approximately 1000 stations monitored to determine long-term trends around the UK coastline. Contaminants are measured in waters, sediments and biota to assess their distribution and fate in the environment. Biological effects are also measured to determine the response of organisms to contaminants. The programme fulfils the UK's commitment to European directives including its mandatory monitoring requirements under the Oslo and Paris Convention (OSPAR) Joint Assessment Monitoring Programme (JAMP). There are two sites in Wales (In the Dee and Milford Haven) which are monitored using grab sampling. This is an important long-term monitoring programme which informs various statutory assessments (Figure 1). Up to 20 samples are taken at each CSEMP station annually for biota, sediment and chemistry (Metals and Organic compounds), usually 15 samples are taken.

## NRW and Bangor University Collaborative Survey of sediment dynamics of Caernarfon Bay and Menai Strait and Conwy Bay SAC

Sampling is required to better understand the sediment grain size and composition of offshore sediments within Caernarfon Bay and Menai Strait and Conwy Bay SAC. This will help aid our understanding of the offshore habitat matrix and potential for coastal process movement and dynamics. The data will feed into the below listed NRW work areas.

NRW work areas: SAC monitoring (Annex 1 habitats; sandbanks), Sands of LIFE (Newborough and Aberffraw), Newborough coastal Management (onshore-offshore sediment movement), Flood Risk Management (Project - Dinas Dinlle).

Sands of LIFE project

To understand the complex movement of sediment from offshore sub-tidal banks, through the inter-tidal, up on to the supra-tidal coastal environment and in conjunction with existing and planned sand dune remediation works.

Max number of samples: 100 in the area shown in Figure 9.

Water Framework Directive and Habitats Directive:

The area overlaps with 3 WFD waterbodies (Caernarfon Bay North, Caernarfon Bay South & Menai Strait) and 1 SAC (Menai Strait and Conwy Bay).

Equipment:

Equipment which may be used: 0.1m2 day grab, Hamon grab, shipek grab and small van veen grab.

Maximum capacity: 15 litres for day grab and Hamon grab, shipek (2 litres) and small Van Veen grab (0.5 litres).

Figure 9 Caernarfon Bay NRW/Bangor University sampling area; with indicative currently planned sample positions; Blue = WFD surveys, Green = MPA monitoring (mainly Special Area of Conservation) and Orange = Collaborative WFD and Habitats Directive Surveys.

## INNS investigations at Disposal Grounds

NRW are assessing the potential of various marine sediment disposal grounds around Wales to harbour invasive non-native species. NRW may want to undertake sampling around these grounds to assess habitat suitability and presence of invasive non-native species. Intersecting WFD waterbodies and SACs are presented in Table 4. Sample stations are not decided but up to 50 (0.1m2 / 15 litre maximum grab) samples may be taken at each of these disposal grounds (Table 4, Figure 10, Figure 11, Figure 12 & Figure 13):

- Holyhead North (Figure 10)

- Milford Haven Three (Figure 11)

- Milford Haven Two (Figure 11)

- Swansea Bay Outer (Figure 12)

- Newport (Figure 13)

- Cardiff Grounds (Figure 13)

- Merkur Buoy (Figure 13)

Table 4. Disposal sites with corresponding Special Area of Conservation or WFD waterbody

| Disposal Ground | Special Area of Conservation | WFD waterbody |
| --- | --- | --- |
| Holyhead North | North Anglesey Marine (candidate) | - |
| Milford Haven Two | - | - |
| Milford Haven Three | West Wales Marine (candidate) | - |
| Swansea Bay Outer | - | Bristol Channel Outer North (Coastal) |
| Newport | Severn Estuary SAC (designated) | Severn Estuary Lower (Transitional) |
| Cardiff Grounds | Severn Estuary SAC (designated) | - |
| Merkur Buoy | - | Bristol Channel Inner North (Coastal) |

Figure 10 The location of Holyhead North licenced disposal ground

Figure 11 The location of the licenced disposal grounds south west of Milford Haven Waterway

Figure 12 The location of Swansea Bay licenced disposal ground

Figure 13 The location of various licenced disposal grounds in Severn Estuary/Bristol Channel.

# Sampling within protected areas

Information below can be used to inform Habitat Regulations and Water Framework Assessments required for this licence. Some of the various surveys described above overlap spatially and samples are planned to be used for multiple drivers to maintain high efficiency. Rather than give numbers of samples per survey we provide below the numbers of planned samples with indicative sample points divided up by SAC and by WFD waterbodies (which themselves overlap).  This information should provide a good indication of the scale of the activity for HRA /WFD Assessment purposes. However actual numbers of samples and timing may change with changes in monitoring priorities (as described above).

Table 5 Maximum number of samples within welsh SACs

| SAC | 2019 | 2020 | 2021 |
| --- | --- | --- | --- |
| Planned | 418 | 394 | 669 |
| Lleyn Peninsula and the Sarnau | 40 | 42 | 165 |
| Menai Strait and Conwy Bay | 157 | 40 | 40 |
| Cardigan Bay | - | 54 | 16 |
| Carmarthen Bay and Estuaries | 76 | 92 | 190 |
| Dee Estuary | 20 | 20 | 20 |
| Pembrokeshire Marine | 111 | 146 | 310 |
| Menai Strait and Conwy Bay | 14 | - | - |
| Adhoc | 700 | 700 | 700 |
| Lleyn Peninsula and the Sarnau | 100 | 100 | 100 |
| Cardigan Bay | 100 | 100 | 100 |
| Carmarthen Bay and Estuaries | 100 | 100 | 100 |
| Dee Estuary | 100 | 100 | 100 |
| Pembrokeshire Marine | 100 | 100 | 100 |
| Menai Strait and Conwy Bay | 100 | 100 | 100 |
| Severn Estuary | 100 | 100 | 100 |
| Project | 600 |  |  |
| Disposal (Severn Estuary) | 100 |  |  |
| Bangor Uni (Menai Strait and Conwy Bay) | 100 |  |  |
| WFD Investigations (unknown SAC) | 400 |  |  |
| Grand Total | 4,181 |  |  |

The number of samples potentially planned in the (currently candidate) porpoise cSACS are presented in a separate table as they overlap with other SACs in the table above.

Table 6 Maximum number of possible samples within various SACs under various scenarios

| Candidate Porpoise SAC | 2019 | 2020 | 2021 |
| --- | --- | --- | --- |
| Bristol Channel Approaches |  |  |  |
| No Target - adhoc | 100 | 100 | 100 |
| Planned | 43 | 92 | 40 |
| North Anglesey Marine |  |  |  |
| No Target - disposal | 50 |  |  |
| Target | 20 | - | - |
| West Wales Marine |  |  |  |
| No Target - adhoc (SAC) | 300 | 300 | 300 |
| No Target - disposal | 50 |  |  |
| Planned | 146 | 238 | 187 |
| Potentially All |  |  |  |
| WFD investigations | 400 |  |  |
| Grand Total | 2,460 |  |  |

Table 7 summarises the number of samples planned to be taken over time by water body with associated estimated max volume (m3).

Table 7 Maximum number of possible samples within various WFD waterbodies with associated volume

|  | Number of samples |  |  | Max possible volume (m3) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| WFD Waterbodies | 2019 | 2020 | 2021 | 2019 | 2020 | 2021 |
| SAC adhoc |  |  |  |  |  |  |
| SAC ad hoc (100 per SAC)1 | 700 | 700 | 700 | 10.50 | 10.50 | 10.50 |
| Project (Bangor Uni) |  |  |  |  |  |  |
| Menai Strait, Caernarfon Bay North, Caernarfon Bay South | 100 |  |  | 1.50 |  |  |
| Project (disposal sites) |  |  |  |  |  |  |
| Bristol Channel Inner North | 50 |  |  | 0.75 |  |  |
| Severn LOWER | 100 |  |  | 1.50 |  |  |
| Unplanned - WFD investigations |  |  |  |  |  |  |
| All WFD waterbodies2 | 400 |  |  | 6.00 |  |  |
| MPA reference samples3 | 300 | 300 | 300 | 4.50 | 4.50 | 4.50 |
| Planned (including MCZ, WFD, SAC and MHWESG sampling) |  |  |  |  |  |  |
| Anglesey North | 68 | 20 | 20 | 1.02 | 0.30 | 0.30 |
| Burry Inlet Outer | - | - | 6 | - | - | 0.09 |
| Caernarfon Bay North | 10 | 20 | 10 | 0.15 | 0.30 | 0.15 |
| Caernarfon Bay South | 20 | 24 | 90 | 0.30 | 0.36 | 1.35 |
| Cardigan Bay Central | - | - | 20 | - | - | 0.30 |
| Cardigan Bay North | 10 | 10 | 38 | 0.15 | 0.15 | 0.57 |
| Cardigan Bay South | 40 | 58 | 40 | 0.60 | 0.87 | 0.60 |
| Carmarthen Bay | 55 | 92 | 40 | 0.83 | 1.38 | 0.60 |
| Conwy Estuary | 20 | - | - | 0.30 | - | - |
| Conwy Bay | 60 | 20 | 20 | 0.90 | 0.30 | 0.30 |
| DEE (N. WALES) | 20 | 20 | 20 | 0.30 | 0.30 | 0.30 |
| Holyhead Bay | 20 | - | - | 0.30 | - | - |
| MILFORD HAVEN INNER | 20 | 20 | 172 | 0.30 | 0.30 | 2.58 |
| Milford Haven Outer | - | - | 92 | - | - | 1.38 |
| Pembrokeshire South | 58 | 126 | 40 | 0.87 | 1.89 | 0.60 |
| Swansea Bay | 40 | 70 | 40 | 0.60 | 1.05 | 0.60 |
| TEIFI ESTUARY | - | 24 | - | - | 0.36 | - |
| Tremadog Bay | 30 | 30 | 60 | 0.45 | 0.45 | 0.90 |
| TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | - | - | 144 | - | - | 2.16 |

1 SACs overlap with many WFD waterbodies, potentially up to 100 samples taken as part of annual ad hoc SAC monitoring could be taken in a WFD waterbody. See Appendix 3 & 4 to see which WFD waterbodies overlap with SACs (with associated area).

2 A maximum of 100 samples will be taken to investigate WFD failures in one waterbody over the three-year period.

3 A maximum of 300 are planned to be undertaken outside of MPAs these could be taken within WFD waterbodies.

Table 8 A summary of all samples included in the licence 2019-2021

| Samples | No. of. Samples |
| --- | --- |
| 2019 Planned samples | 585 |
| 2020 Planned samples | 584 |
| 2021 Planned samples | 901 |
| HD Ad Hoc Samples | 2,100 |
| HD Reference Samples | 900 |
| INNS disposal site samples | 350 |
| WFD Investigation Samples | 400 |
| Bangor university/NRW samples | 100 |
| Maximum TOTAL | 5,920 |

The planned samples numbers are indicative based on current monitoring plans (barring operational difficulties).  The other sample numbers are maximums, and it is extremely unlikely that all will be taken but are included to allow flexibility to respond to events over the 3 years. Timing (year) of surveys may be subject to change.

# Environmental Mitigation

Damage to the following habitats must be avoided wherever possible

- Sabellaria

- Seagrass

- Modiolus

- Maerl

We have attached a document (see Appendix 2) “Sensitive Habitats and Marine Licence” produced following the conditions of the previous licence (RML1556) which outlines why a blanket ban on sampling in these areas may conflict with our statutory duties. We would welcome discussion on this topic and can provide similar up to date information if required.  Please note the specific details within the document are relevant to the previous sample programme but illustrate the issue.

# HRA Assessment

## Designated Sites

We note that the HRA undertaken for the previous 3 years survey programme (RML1556 – HRA attached to the application) concluded no likely significant effect (alone or in combination) for the following sites:

- Liverpool Bay SPA

- Menai Straits and Conwy Bay SAC

- Pen Llyn a’r Sarnau SAC

- Cardigan Bay SAC

- Pembrokeshire Marine SAC

- Carmarthen Bay & Estuaries SAC

- Carmarthen Bay SPA

- Bury Inlet Ramsar

- Dee Estuary SAC

- Dee Estuary SPA

- Dee Estuary Ramsar Site

- Castlemartin Coast SPA

- Lavan Sands, Conway Bay SPA

- Stockholm & Skomer SPA

While this application is not identical to the previous one, the nature of the activity is very similar.  We therefore consider that there will be no likely significant effect (alone or in-combination) for the above sites for the same reasons as detailed in the HRA for RML1556.

## Candidate Sites

Since the previous application was determined new harbour porpoise candidate SACs have been notified. We provide information to inform the HRA for these sites below:

This Information covers all 3 porpoise cSACs as the features (and DRAFT conservation objectives) are the same: - Bristol Channel Approaches. - North Anglesey Marine. - West Wales Marine.

| Feature | Relevant Conservation objectives | Potential Impact Pathway |
| --- | --- | --- |
| Harbour Porpoise (Phocoena phocaena) | To avoid deterioration of the habitats of the harbour porpoise or significant disturbance to the harbour porpoise, thus ensuring that the integrity of the site is maintained, and the site makes an appropriate contribution to maintaining Favourable Conservation Status (FCS) for the UK harbour porpoise. To ensure for harbour porpoise that, subject to natural change, the following attributes are maintained or restored in the long term: - The species is a viable component of the site. - There is no significant disturbance of the species. - The supporting habitats and processes relevant to harbour porpoises and their prey are maintained. | While there is a theoretical impact pathway due to boat traffic while undertaking the surveys the intermittent nature and small scale of the activity (spatially and temporally) mean that there is no likely significant effect, either alone or in combination with other projects. |

# WFD Assessment

The surveys described above overlap with many WFD waterbodies (Table 7). We note from appendix 2 of OGN 72 that benthic grab surveys with a total volume of less than 4 cubic metres (with a density of no more than 50 samples in any one hectare) can be screened out from further assessment.

This application (though it encompasses a total maximum possible volume of more than 4 cubic metres) is made up of multiple discrete surveys carried out over a 3-year period.

## Volume of sediment

With current planned sampling the annual volume of sediment to be removed will be under the 4 cubic metres limit (assuming worst case sample size of 15 litres, 4 cubic metres = 266.6 samples).  It is extremely unlikely that the density of the surveys will be anywhere close to 50 samples per hectare (indicative sample locations illustrate this – and Ad Hoc surveys would be extremely unlikely to reach this density).

## Worst Case Scenario

To examine the various surveys in-combination with each other we can look at the worst-case scenario (highest number of samples) using current plans (Table 7):

### Scenario 1; Milford Haven Inner:

The Milford Haven Inner Waterbody has 172 planned survey samples in 2021, if all potential 100 Ad Hoc HD samples for Pembrokeshire Marine SAC, and all potential 100 ad Hoc WFD samples for the WFD waterbody also happened to be taken within the water body that would be a total of 372 samples, this equates to a maximum volume (assuming max volume is 15l) of 5.58m3. This is a vast overestimate of the likely volume. The most recent log from this waterbody (whilst sampling target locations provided in this document) showed that the average sample volume attained was 6.4 litres. If the average volume attained on this survey is used to calculate the volume of sediment to be sampled for this worst-case scenario then the total volume sampled (372 x 6.4/1000) is estimated to be 6.4m3 which is comfortably under 4m3.

### Scenario 2; Three Rivers:

The TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARYWFD Waterbody has 144 planned survey samples in 2021, if all potential 100 Ad Hoc HD samples for Carmarthen Bay and Estuaries SAC, and all potential 100 ad Hoc WFD samples for the WFD waterbody also happened to be taken within the water body that would be a total of 344 samples, this equates to a maximum volume (assuming max volume is 15l) of 5.16m3. This is a vast overestimate of the likely volume. The most recent log from this waterbody (2018) (whilst sampling target locations provided in this document) showed that the average sample volume attained was slightly under 2.3 litres of sediment. If the average volume attained on this survey is used to make estimates (344 x 2.3/1000), then the total volume sampled would be 0.79m3 which is significantly under the 4m3.

Even in the extremely unlikely scenario that all those ad hoc samples were taken in that same waterbody the total volume would be very likely to be less than 4 cubic metres annually.

The MPA reference samples would likely be spread out over multiple areas and not be in areas which are already sampled (which are generally in MPAs) therefore are very unlikely to be anywhere close to 4m3.

## Conclusion:

We consider that there is no risk from the proposed sampling programmes of causing any WFD waterbody to deteriorate or to prevent any waterbody from achieving its objectives. Furthermore, more importantly, this sampling is undertaken to aid assessment WFD waterbody status and therefore is necessary monitoring for the Water Framework Directive.

# Appendix 1 – Copy of Sensitive Habitats and Marine Licence relating to previous licence RML1556

(note: figure and table references have been updated to avoid confusion with above)

NRW have applied for a marine licence from the NRW licence department as required by the Marine and Coastal Access Act, 2009.

As it stands there is a condition in the NRW licence stipulates that:

8.6 The Licence Holder must ensure that no grab samples are taken where sensitive habitat are known to occur, these include maerl beds, Modiolus modiolus (Horse mussel) beds, Sabellaria spinsulosa reef, Sabellaria alveolata reef and Zostera (eelgrass) beds.

8.7 If sensitive habitats are encounter outside of their known limit, the Licence Holder must ensure no further sampling takes place within these areas.

In some cases we will be required to sample within these habitats in order to assess habitat feature condition. Sampling within these areas is needed to ascertain SAC habitat condition. All information gained from sampling is used to maintain or enhance sensitive protected habitats.

Below are some specific examples of where condition 8.6 & 8.7 will cause problems:

1.	(8.6) In some cases, we need to sample ‘sensitive’ habitats in order to assess condition. For example, several historic Milford Haven sampling locations are in areas of Zostera noltei (eel grass). The data gained from this sampling aids assessment of health and condition of beds and can be used to enhance or maintain beds.

2.	(8.6) Much of our sampling forms part of long term monitoring of fixed locations looking at trends in community change – having to relocate due to a community change (e.g. through expansion of Zostera beds or colonisation by Sabellaria spp.) would confound the sampling design and our ability to draw reliable conclusions from the already established monitoring programme.

3.	(8.7) Where we encounter the habitat outside known established areas the number of samples of the habitat present gives an indication of both extent and certainty of presence (i.e. confidence), this information will be used to assess condition and for future casework advice (this is then used to protect sensitive habitats). For example, we may want to continue sampling an area where we find sensitive habitats (such as Sabellaria spp.) in order to assess the habitat extent and then report the area covered so we can use this in casework advice.

The current planned 2015 sample station locations have been mapped alongside available sensitive habitats GIS layers (summarised in Table 9) to assess where planned sample locations and sensitive habitats co-occur (Figure 14). Where known sampling is positioned within known areas of sensitive habitat the sampling rationale is explained and will be discussed with conservation officers prior to sampling or planned sample stations are relocated. This should be viewed as a short term solution to the limitations set by conditions in our 2015 licence to ensure that the 2015 survey program is not compromised. In the longer term (2016 onwards), it is anticipated that the monitoring team, licencing and others discuss a more appropriate less time consuming solution.

Table 9 A summary of data used to assess the impact of NRW Habitats Directive, MCZ and WFD and partner organisation MHWESG sampling.

| Sensitive Habitat | Data used |  |
| --- | --- | --- |
| Modiolus reef | Polygons generated from ground-truthed acoustic data, from CCW/NRW and casework data. Point data are Modiolus records from CCW and partner organisation monitoring (source: Marine recorder) | M:\GIS_Users\marine\BAP_OSPAR\Current\Modiolus beds\CCW_horse_mussel_polygon_v2 M:\GIS_Users\marine\BAP_OSPAR\Current\Modiolus beds\CCW_Modiolus_bed_point_v2 |
| Sabellaria reef | S. spinulosa Reef polygon were generated from (un-groundtruthed) predicted S. spinsulosa based on Habmap predicted habitats. Actual point data does exist but the distribution is limited to one point off North Penllyn. S. alveolata reef Polygon and point were created using Intertidal phase 1 survey data, Intertidal SAC monitoring and data in marine recorder. The point data reflects the polygon layer, but with additional subtidal points within the Bristol Channel | M:\GIS_Users\marine\BAP_OSPAR\Current\Sabellaria spinulosa reef\NRW_Sabellaria_spinulosa_point_v2 M:\GIS_Users\marine\BAP_OSPAR\Current\Honeycomb worm reefs\CCW_honeycomb_worm_point_v1 M:\GIS_Users\marine\BAP_OSPAR\Current\Honeycomb worm reefs\CCW_honeycomb_worm_polygon_v4 |
| Maerl bed | The polygon was generated from using the Voronoi technique based on point data from diver, drop down camera and commercial data (the only bed is found within Milford Haven) | M:\GIS_Users\marine\BAP_OSPAR\Current\Maerl\CCW_TotalMaerl_polygon |
| Seagrass bed | The Seagrass polygons were generated from Intertidal phase 1 data, Intertidal WFD monitoring, SAC monitoring (intertidal and subtidal) and other targeted surveys by Seasearch and NRW staff. Some mapped records are derviced from survey point data and aerial photo intertpretation. These records tend to have a lower confidence than those that have been mapped specifically. | M:\GIS_Users\marine\BAP_OSPAR\Current\Seagrass\CCW_seagrass_polygon_v2 |

Planned sample locations and sensitive habitats have been mapped using NRW BAP and OSPAR layers. Using this information, it has highlighted that planned sample station/s from three surveys are within habitats identified as sensitive (Figure 14). Sample stations from the Bristol Channel Inner North survey (WFD; Sabellaria Reef), Milford Haven Outer WFD survey (WFD; maerl) and Milford Haven Inlet survey (Habitats Directive; seagrass) are within or near identified sensitive habitats. More detail is given for each of these surveys below.

Figure 14 A map of NRW and MHWESG samples alongside sensitive habitats as defined by NRW Marine Licence, 2015.

### Bristol Channel Inner North

There are 7 sites which have been sampled previously, where Sabellaria alveolata reef has been identified using BAP and OSPAR GIS layers (Table 9). In marine recorder these samples have been described as ‘Sabellaria alveolata on variable salinity sublittoral mixed sediment’ biotope. Planned station 3, 5, 7 and 12 have significant numbers of S. alveolata, whereas other sites are unlikely to be reef as there are low numbers of S. alveolata individuals and habitats have not been described as reef in the habitat descriptions held in Marine Recorder (Table 10). The position of planned stations (Figure 15) which are likely to be reef can be re-located (see Figure 16). The primary objective of this survey is to assess the Bristol Channel Inner North WFD water body. Therefore samples where reef is likely can be moved which will reduce the chance of the Sabellaria reef.

Where Sabellaria reef is likely, planned sample stations can be re-located, for WFD surveys, the primary objective is to undertake a WFD water body assessment. The statistical tool used to ascertain ecological status (WFD) can’t properly process samples of Sabellaria reef therefore not sampling this habitat is not an issue for this survey. Therefore the current condition in the licence can be adhered to during the Bristol Channel Inner North survey (and other WFD surveys) without compromising the objectives of the survey. However, where Sabellaria is sampled by accident it may be taken for analysis (as already sampled) or notes taken. This can then to be used for Habitats Directive Article 17 map work and casework advice. This will help protect Sabellaria spp. reef in the future.

Long term solution

Table 10 A summary of Sabellaria records close to NRW planned sampling stations from marine recorder database.

| Survey | 1988 STPG Severn Estuary sublittoral survey | 1988 STPG Severn Estuary sublittoral survey | 1988 STPG Severn Estuary sublittoral survey | 1988 STPG Severn Estuary sublittoral survey | 1988 STPG Severn Estuary sublittoral survey | 1988 STPG Severn Estuary sublittoral survey | 1988 STPG Severn Estuary sublittoral survey |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NRW Planned sample Sample Code | BCIN 3 | BCIN 5 | BCIN 27 | BCIN 4 | BCIN 7 | BCIN 15 | BCIN 12 |
| MR Sample Ref | 460.029.001 | 460.063.001 | 460.064.001 | 460.069.001 | 460.333.001 | 460.444.001 | 460.525.001 |
| Habitat | sand with Sabellaria alveolata | 460.063.001 | well sorted sand | 460.069.001 | sand with Sabellaria alveolata | 460.444.001 | sandy gravel with Sabellaria alveolata |
| Sabellaria alveolata | 176 | 410 | 2 | 1 | 54 | 1 | 163 |
| Sabellaria spinulosa | 1 |  |  |  |  |  |  |
| Reef notes | Likely reef | Likely reef | Unlikely reef | Unlikely reef | Likely reef | Unlikely reef | Likely reef |

Figure 15 Planned sample stations with BAP and OSPAR habitat map information.

Figure 16 Planned sample stations with BAP and OSPAR habitat map information with new positions for 4 stations (3, 5, 7 & 12) moved to reduce the risk of sampling Sabellaria reef.

### Milford Haven Inner WFD Water Body survey

Figure 17 Planned sample stations with BAP and OSPAR habitat map information.

One sample station (5) is on the fringe of a maerl bed (Figure 17). At the planned location the amount of live maerl is low and the maerl is at risk due to reduced light & increased siltation. Eventual loss of live maerl from here will act as an early warning for the main bed. The local conservation officer will be consulted before sampling is undertaken.

### Milford Haven inlet SSSI and Habitats Directive Body survey

Figure 18 Planned sample stations with sea grass BAP and OSPAR habitat map information.

One sample station (Station 7) is within and four sample stations (Station 6, 5, 9 & 12) are close to identified intertidal sea grass beds (Figure 18). The Milford Haven Inlet survey is designed to assess change in the health of benthic communities over time in order to inform SSSI and SAC condition assessment and inform SAC site management. At considerable cost these locations have been sampled twice over a 6 year period, in order to keep a valid time series of data the same locations should be sampled again in 2015. The local conservation officer will be consulted before sampling is undertaken.

# Appendix 2. Indicative sample point locations

NB* Please note these locations and timings are indicative may change with changing priorities or conditions on the day

| Main driver | Habitat | Location | Sample Code | Easting | Northing | Grid Ref | 19 | 20 | 21 | Waterbody Name | SAC Designated |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Habs | Sand Bank | Devil's ridge | DVR_12 | 218770 | 320148 | SH1877020148 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Devil's ridge | DVR_13 | 219181 | 320448 | SH1918120448 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Devil's ridge | DVR_14 | 219907 | 320816 | SH1990720816 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Devil's ridge | DVR_15 | 220410 | 321210 | SH2041021210 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Devil's ridge | DVR_16 | 220930 | 321296 | SH2093021296 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Tripods | TRP_07 | 212953 | 328384 | SH1295328384 |  |  | 3 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Tripods | TRP_08 | 213400 | 328390 | SH1340028390 |  |  | 3 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Tripods | TRP_09 | 213852 | 328362 | SH1385228362 |  |  | 3 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Tripods | TRP_10 | 213109 | 328790 | SH1310928790 |  |  | 3 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Tripods | TRP_11 | 212855 | 328868 | SH1285528868 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Bastram Shoal | BAS_50 | 211160 | 316456 | SH1116016456 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Bastram Shoal | BAS_51 | 211762 | 316609 | SH1176216609 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Bastram Shoal | BAS_52 | 212488 | 316802 | SH1248816802 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Bastram Shoal | BAS_53 | 213025 | 317062 | SH1302517062 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Bastram Shoal | BAS_54 | 213144 | 317248 | SH1314417248 |  |  | 3 | Outside WFD Waterbody | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_01 | 237489 | 326931 | SH3748926931 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_02 | 237240 | 328506 | SH3724028506 | 10 | 10 | 10 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_03 | 239347 | 316265 | SH3934716265 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_04 | 236104 | 318184 | SH3610418184 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_05 | 246467 | 333982 | SH4646733982 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_06 | 235202 | 326001 | SH3520226001 | 10 | 10 | 10 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_07 | 237489 | 331756 | SH3748931756 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_10 | 241233 | 326056 | SH4123326056 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_12 | 251616 | 336406 | SH5161636406 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_13 | 252789 | 334368 | SH5278934368 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_14 | 252872 | 330831 | SH5287230831 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_15 | 256220 | 330511 | SH5622030511 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_16 | 250870 | 324339 | SH5087024339 | 10 | 10 | 10 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_17 | 247548 | 326278 | SH4754826278 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_18 | 246982 | 330264 | SH4698230264 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_19 | 234672 | 329784 | SH3467229784 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_20 | 241348 | 332149 | SH4134832149 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_21 | 244039 | 330184 | SH4403930184 | 10 | 10 | 10 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_22 | 245057 | 320315 | SH4505720315 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_23 | 246929 | 331437 | SH4692931437 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_24 | 246939 | 330290 | SH4693930290 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Tremadog Bay | TRM_25 | 245669 | 329350 | SH4566929350 |  |  | 2 | Tremadog Bay | Lleyn Peninsula and the Sarnau |
| Habs | Sand Bank | Helwick | HELW27 | 236617 | 184039 | SS3661784039 | 3 |  |  | Outside WFD Waterbody | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW28 | 236346 | 183634 | SS3634683634 | 3 |  |  | Outside WFD Waterbody | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW29 | 236361 | 183060 | SS3636183060 | 3 |  |  | Outside WFD Waterbody | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW30 | 236358 | 182943 | SS3635882943 | 3 |  |  | Outside WFD Waterbody | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW31 | 236327 | 182501 | SS3632782501 | 3 |  |  | Outside WFD Waterbody | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW32 | 236301 | 182648 | SS3630182648 | 3 |  |  | Outside WFD Waterbody | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW37 | 238230 | 182085 | SS3823082085 | 3 |  |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW33 | 244907 | 184095 | SS4490784095 | 3 |  |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW34 | 244949 | 183527 | SS4494983527 | 3 |  |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW35 | 244890 | 183220 | SS4489083220 | 3 |  |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW36 | 244744 | 182689 | SS4474482689 | 3 |  |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Helwick | HELW38 | 233784 | 183560 | SS3378483560 | 3 |  |  | Outside WFD Waterbody | Carmarthen Bay and Estuaries |
| Habs | Sand Bank | Knoll Sand bank | KNOL1 | 172780 | 206701 | SM7278006701 | 3 |  |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Sand Bank | Knoll Sand bank | KNOL2 | 172816 | 206642 | SM7281606642 | 3 |  |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Sand Bank | Knoll Sand bank | KNOL3 | 172723 | 206769 | SM7272306769 | 3 |  |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Sand Bank | Knoll Sand bank | KNOL4 | 172901 | 206549 | SM7290106549 | 3 |  |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Sand Bank | Knoll Sand bank | KNOL5 | 173055 | 206374 | SM7305506374 | 3 |  |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Sand Bank | Knoll Sand bank | KNOL6 | 173376 | 206534 | SM7337606534 | 3 |  |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Sand Bank | Turbot Bank | TURB39 | 182378 | 195741 | SR8237895741 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Turbot Bank | TURB40 | 182700 | 195848 | SR8270095848 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Turbot Bank | TURB41 | 183155 | 196031 | SR8315596031 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Turbot Bank | TURB42 | 183374 | 196467 | SR8337496467 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Turbot Bank | TURB48 | 183189 | 196792 | SR8318996792 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Turbot Bank | TURB49 | 184349 | 195441 | SR8434995441 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Bais Bank South | BAIS22 | 166991 | 232057 | SM6699132057 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Bais Bank South | BAIS23 | 167414 | 231774 | SM6741431774 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Bais Bank South | BAIS24 | 167452 | 231584 | SM6745231584 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Bais Bank South | BAIS25 | 167734 | 231359 | SM6773431359 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Bais Bank South | BAIS26 | 167873 | 230943 | SM6787330943 | 3 |  |  | Outside WFD Waterbody | Pembrokeshire Marine |
| Habs | Sand Bank | Bais Bank North | BAIS22N | 169034 | 234208 | SM6903434208 | 3 |  |  | Outside WFD Waterbody | Outside SAC |
| Habs | Sand Bank | Bais Bank North | BAIS23N | 169465 | 233675 | SM6946533675 | 3 |  |  | Outside WFD Waterbody | Outside SAC |
| Habs | Sand Bank | Bais Bank North | BAIS24N | 169656 | 233528 | SM6965633528 | 3 |  |  | Outside WFD Waterbody | Outside SAC |
| Habs | Sand Bank | Bais Bank North | BAIS25N | 169758 | 233458 | SM6975833458 | 3 |  |  | Outside WFD Waterbody | Outside SAC |
| Habs | Sand Bank | Bais Bank North | BAIS26N | 169932 | 233299 | SM6993233299 | 3 |  |  | Outside WFD Waterbody | Outside SAC |
| Habs | Estuary | Three Rivers Inlets | Taf 1 | 229005 | 213828 | SN2900513828 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Taf 2 | 229564 | 213175 | SN2956413175 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Taf 3 | 230356 | 212936 | SN3035612936 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Taf 4 | 231319 | 212281 | SN3131912281 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Taf 5 | 231525 | 211461 | SN3152511461 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Taf 6 | 230462 | 210595 | SN3046210595 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Taf 7 | 230891 | 210458 | SN3089110458 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Taf 8 | 231648 | 209642 | SN3164809642 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Taf 9 | 233008 | 208962 | SN3300808962 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 10 | 235059 | 208861 | SN3505908861 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 11 | 236188 | 209971 | SN3618809971 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 12 | 236360 | 211145 | SN3636011145 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 13 | 236830 | 211809 | SN3683011809 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 14 | 237225 | 212898 | SN3722512898 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 15 | 238666 | 213709 | SN3866613709 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 16 | 239869 | 216576 | SN3986916576 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 17 | 240075 | 215300 | SN4007515300 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | Tywi 18 | 239864 | 214238 | SN3986414238 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | GWN 19 | 239325 | 205707 | SN3932505707 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | GWN 20 | 238807 | 206302 | SN3880706302 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | GWN 21 | 238057 | 206794 | SN3805706794 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | GWN 22 | 239062 | 206617 | SN3906206617 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | GWN 23 | 237522 | 206466 | SN3752206466 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | 3R 24 | 234095 | 207090 | SN3409507090 |  |  | 6 | Burry Inlet Outer | Carmarthen Bay and Estuaries |
| Habs | Estuary | Three Rivers Inlets | 3R 25 | 235809 | 206867 | SN3580906867 |  |  | 6 | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | Carmarthen Bay and Estuaries |
| Habs | Estuary | Milford Haven Inlets | SH Upp 1 | 186064 | 208546 | SM8606408546 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | SHLow 2 | 185656 | 207939 | SM8565607939 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Angle H 3 | 186877 | 202887 | SM8687702887 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Angle Bay 4 | 187786 | 202726 | SM8778602726 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Pemb Upp 5 | 195918 | 201922 | SM9591801922 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Pemb Low 6 | 194974 | 202206 | SM9497402206 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Cosh Upp 7 | 199158 | 203596 | SM9915803596 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Low Cosh 8 | 198371 | 203873 | SM9837103873 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Carew Low 9 | 202504 | 205435 | SN0250405435 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Cres Upp 11 | 204340 | 206533 | SN0434006533 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Cres Low 12 | 202639 | 206432 | SN0263906432 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | W Cled Low 13 | 198270 | 212118 | SM9827012118 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | W Cled Upp 14 | 197097 | 214002 | SM9709714002 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | E Cled Low 15 | 201398 | 212182 | SN0139812182 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | E Cled Low 16 | 204312 | 214017 | SN0431214017 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Gann1 G1 | 181312 | 206389 | SM8131206389 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Gann2 G2 | 181429 | 206740 | SM8142906740 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| Habs | Estuary | Milford Haven Inlets | Gann3 G3 | 181707 | 206594 | SM8170706594 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_1 | 195733 | 240049 | SM9573340049 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_2 | 196643 | 240635 | SM9664340635 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_3 | 197505 | 240810 | SM9750540810 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_4 | 198918 | 240783 | SM9891840783 | 10 | 10 | 10 | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_5 | 196575 | 240084 | SM9657540084 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_6 | 199058 | 240875 | SM9905840875 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_7 | 197913 | 240074 | SM9791340074 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_8 | 196592 | 239371 | SM9659239371 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_9 | 197895 | 239324 | SM9789539324 | 10 | 10 | 10 | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_10 | 195726 | 238814 | SM9572638814 | 10 | 10 | 10 | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_11 | 195888 | 238211 | SM9588838211 | 10 | 10 | 10 | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_12 | 197214 | 238543 | SM9721438543 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Fishguard Bay | FISHG_15 | 198863 | 239951 | SM9886339951 |  | 2 |  | Cardigan Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_1 | 263278 | 188884 | SS6327888884 | 10 | 10 | 10 | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_2 | 266248 | 187496 | SS6624887496 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_3 | 267885 | 189842 | SS6788589842 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_4 | 267885 | 187853 | SS6788587853 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_5 | 267957 | 190279 | SS6795790279 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_6 | 264787 | 189595 | SS6478789595 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_7 | 263803 | 187641 | SS6380387641 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_8 | 265579 | 188926 | SS6557988926 | 10 | 10 | 10 | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_9 | 272580 | 190274 | SS7258090274 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_10 | 273081 | 188090 | SS7308188090 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_11 | 268870 | 190836 | SS6887090836 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_12 | 268921 | 189915 | SS6892189915 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_13 | 269909 | 188265 | SS6990988265 | 10 | 10 | 10 | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_14 | 270950 | 190144 | SS7095090144 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_15 | 271695 | 189087 | SS7169589087 | 10 | 10 | 10 | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_16 | 270562 | 186118 | SS7056286118 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_17 | 268137 | 186161 | SS6813786161 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_18 | 273490 | 185067 | SS7349085067 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Swansea Bay | SWAN_19 | 275771 | 182937 | SS7577182937 |  | 2 |  | Swansea Bay | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_1 | 239280 | 361992 | SH3928061992 | 10 | 10 | 10 | Caernarfon Bay North | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_2 | 240832 | 361471 | SH4083261471 |  | 2 |  | Caernarfon Bay North | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_4 | 238263 | 360637 | SH3826360637 |  | 2 |  | Caernarfon Bay North | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_5 | 240341 | 357819 | SH4034157819 |  | 2 |  | Caernarfon Bay North | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_6 | 238946 | 357409 | SH3894657409 | 10 | 10 | 10 | Outside WFD Waterbody | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_8 | 241276 | 353524 | SH4127653524 | 10 | 10 | 10 | Caernarfon Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_9 | 238211 | 352878 | SH3821152878 |  | 2 |  | Outside WFD Waterbody | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_12 | 234950 | 347550 | SH3495047550 | 10 | 10 | 10 | Caernarfon Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_13 | 231147 | 348006 | SH3114748006 |  | 2 |  | Outside WFD Waterbody | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_18 | 239407 | 360290 | SH3940760290 |  | 2 |  | Caernarfon Bay North | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_19 | 241302 | 359327 | SH4130259327 |  | 2 |  | Caernarfon Bay North | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_20 | 237016 | 357399 | SH3701657399 |  | 2 |  | Outside WFD Waterbody | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_21 | 234999 | 355372 | SH3499955372 |  | 2 |  | Outside WFD Waterbody | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_22 | 233810 | 351600 | SH3381051600 |  | 2 |  | Outside WFD Waterbody | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_23 | 228782 | 342151 | SH2878242151 |  | 2 |  | Caernarfon Bay South | Outside SAC |
| Art 17 | Non-SAC Bay | Caernafon Bay | CAERN_24 | 228285 | 341382 | SH2828541382 |  | 2 |  | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| Habs | Bay | Red Wharf Bay | RWB001 | 257899 | 382662 | SH5789982662 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB002 | 255770 | 382159 | SH5577082159 | 10 | 10 | 10 | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB003 | 257120 | 383749 | SH5712083749 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB004 | 254860 | 382353 | SH5486082353 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB005 | 252695 | 387037 | SH5269587037 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB006 | 255255 | 383229 | SH5525583229 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB010 | 252673 | 385521 | SH5267385521 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB011 | 253734 | 383802 | SH5373483802 | 10 | 10 | 10 | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB012 | 260025 | 382837 | SH6002582837 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB013 | 259318 | 383754 | SH5931883754 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB014 | 257814 | 385695 | SH5781485695 | 2 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB015 | 256367 | 384428 | SH5636784428 | 2 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB016 | 253702 | 384525 | SH5370284525 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB017 | 254215 | 386361 | SH5421586361 | 2 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB018 | 256702 | 382507 | SH5670282507 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB019 | 253628 | 385668 | SH5362885668 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB020 | 255186 | 383904 | SH5518683904 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Bay | Red Wharf Bay | RWB022 | 252665 | 384172 | SH5266584172 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Red Wharf Bay Sand bank | RWBFFR001 | 255099 | 384611 | SH5509984611 | 3 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Red Wharf Bay Sand bank | RWBFFR002 | 255398 | 384972 | SH5539884972 | 3 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Red Wharf Bay Sand bank | RWBFFR003 | 255488 | 385202 | SH5548885202 | 3 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Red Wharf Bay Sand bank | RWBFFR004 | 256014 | 385213 | SH5601485213 | 3 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Red Wharf Bay Sand bank | RWBFFR005 | 256169 | 385775 | SH5616985775 | 3 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC001 | 267902 | 381237 | SH6790281237 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC002 | 268484 | 380236 | SH6848480236 | 10 | 10 | 10 | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC003 | 269418 | 378544 | SH6941878544 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC004 | 269238 | 381805 | SH6923881805 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC005 | 271024 | 378404 | SH7102478404 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC006 | 269934 | 382679 | SH6993482679 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC007 | 271786 | 379701 | SH7178679701 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC008 | 271250 | 381948 | SH7125081948 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC009 | 273768 | 382217 | SH7376882217 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC010 | 273615 | 383324 | SH7361583324 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON004 | 270126 | 377737 | SH7012677737 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON009 | 271811 | 382294 | SH7181182294 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON013 | 272770 | 382804 | SH7277082804 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON014 | 272725 | 382522 | SH7272582522 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON015 | 273238 | 382546 | SH7323882546 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON016 | 273047 | 382188 | SH7304782188 | 10 | 10 | 10 | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON017 | 272537 | 382124 | SH7253782124 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON018 | 272732 | 381687 | SH7273281687 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON019 | 273295 | 381806 | SH7329581806 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON020 | 273568 | 382203 | SH7356882203 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON021 | 272952 | 383540 | SH7295283540 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON022 | 273351 | 385340 | SH7335185340 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON023 | 271045 | 383565 | SH7104583565 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON025 | 268630 | 382911 | SH6863082911 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON026 | 267786 | 384291 | SH6778684291 | 2 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON029 | 273726 | 386446 | SH7372686446 | 2 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CON030 | 271340 | 388178 | SH7134088178 | 2 |  |  | Outside WFD Waterbody | Outside SAC |
| Habs/wfd | Bay | Conwy Bay | CON031 | 268442 | 387499 | SH6844287499 | 2 |  |  | Outside WFD Waterbody | Outside SAC |
| Habs/wfd | Bay | Conwy Bay | CON033 | 274656 | 387531 | SH7465687531 | 2 |  |  | Outside WFD Waterbody | Outside SAC |
| Habs/wfd | Bay | Conwy Bay | CWC15c | 270315 | 379800 | SH7031579800 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs/wfd | Bay | Conwy Bay | CWC20c | 271189 | 380678 | SH7118980678 | 2 |  |  | Conwy Bay | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Conwy Bay Sand Bank | CONFFC001 | 269114 | 385602 | SH6911485602 | 3 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Conwy Bay Sand Bank | CONFFC002 | 271391 | 386246 | SH7139186246 | 3 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Conwy Bay Sand Bank | CONFFC003 | 271118 | 385717 | SH7111885717 | 3 |  |  | Outside WFD Waterbody | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Conwy Bay Sand Bank | CONFFC004 | 271053 | 385178 | SH7105385178 | 3 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Conwy Bay Sand Bank | CONFFC005 | 271107 | 384807 | SH7110784807 | 3 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs | Sand Bank | Conwy Bay Sand Bank | CONFFC006 | 270937 | 384217 | SH7093784217 | 3 |  |  | Anglesey North | Menai Strait and Conwy Bay |
| Habs/WFD | Bay | Carmarthen Bay | CMB001 | 231253 | 195943 | SS3125395943 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB002 | 232928 | 198702 | SS3292898702 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB003 | 236293 | 190560 | SS3629390560 | 10 | 10 | 10 | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB004 | 234164 | 195793 | SS3416495793 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB005 | 236270 | 196307 | SS3627096307 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB006 | 234805 | 197369 | SS3480597369 | 10 | 10 | 10 | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB007 | 236786 | 198616 | SS3678698616 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB008 | 234647 | 199794 | SS3464799794 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB009 | 234857 | 201661 | SN3485701661 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB010 | 233167 | 200533 | SN3316700533 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB011 | 227920 | 201876 | SN2792001876 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB012 | 226178 | 202873 | SN2617802873 | 10 | 10 | 10 | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB013 | 224505 | 204024 | SN2450504024 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB014 | 223624 | 205619 | SN2362405619 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB015 | 221772 | 205148 | SN2177205148 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB016 | 221125 | 202137 | SN2112502137 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB017 | 224199 | 200616 | SN2419900616 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB018 | 217884 | 206196 | SN1788406196 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB019 | 216906 | 204671 | SN1690604671 | 10 | 10 | 10 | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB020 | 218826 | 205262 | SN1882605262 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB021 | 218079 | 202196 | SN1807902196 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB022 | 218651 | 200791 | SN1865100791 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB023 | 219270 | 199340 | SS1927099340 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB024 | 226092 | 198998 | SS2609298998 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB025 | 215292 | 200983 | SN1529200983 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB026 | 215775 | 199555 | SS1577599555 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB027 | 215623 | 197655 | SS1562397655 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB028 | 217581 | 198391 | SS1758198391 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB029 | 227825 | 195968 | SS2782595968 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs/WFD | Bay | Carmarthen Bay | CMB030 | 229562 | 198922 | SS2956298922 |  | 2 |  | Carmarthen Bay | Carmarthen Bay and Estuaries |
| Habs | Bay | St Brides Bay | SBB009 | 181268 | 220277 | SM8126820277 | 10 | 10 | 10 | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB016 | 179518 | 216586 | SM7951816586 | 10 | 10 | 10 | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB001 | 183310 | 222258 | SM8331022258 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB002 | 184821 | 219615 | SM8482119615 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB003 | 185268 | 216832 | SM8526816832 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB004 | 185312 | 214302 | SM8531214302 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB005 | 183991 | 213268 | SM8399113268 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB006 | 184691 | 215546 | SM8469115546 | 10 | 10 | 10 | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB007 | 183751 | 219592 | SM8375119592 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB008 | 181311 | 222279 | SM8131122279 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB010 | 182659 | 217336 | SM8265917336 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB011 | 182488 | 214235 | SM8248814235 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB012 | 181622 | 213235 | SM8162213235 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB014 | 180172 | 211350 | SM8017211350 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB015 | 179311 | 214318 | SM7931114318 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB017 | 179304 | 220315 | SM7930420315 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB018 | 177984 | 223036 | SM7798423036 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB019 | 175358 | 223194 | SM7535823194 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB020 | 176213 | 222269 | SM7621322269 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB021 | 176439 | 221306 | SM7643921306 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB022 | 177547 | 218756 | SM7754718756 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB025 | 177275 | 210277 | SM7727510277 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB026 | 174593 | 212176 | SM7459312176 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB027 | 174869 | 214192 | SM7486914192 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB028 | 173610 | 214495 | SM7361014495 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB029 | 174362 | 215716 | SM7436215716 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB031 | 175302 | 220272 | SM7530220272 | 10 | 10 | 10 | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB032 | 173192 | 221196 | SM7319221196 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Bay | St Brides Bay | SBB039 | 176811 | 211882 | SM7681111882 |  | 2 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB17 | 230708 | 260228 | SN3070860228 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB18 | 230307 | 260765 | SN3030760765 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB19 | 229908 | 260755 | SN2990860755 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB20 | 229912 | 261081 | SN2991261081 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB21 | 229287 | 261479 | SN2928761479 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB55 | 234773 | 268379 | SN3477368379 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB56 | 235885 | 268153 | SN3588568153 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB57 | 236831 | 267965 | SN3683167965 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB58 | 237540 | 267762 | SN3754067762 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| Habs | Sand Bank | Cardigan Bay Sand Bank | CBSB59 | 238336 | 267599 | SN3833667599 |  | 3 |  | Outside WFD Waterbody | Cardigan Bay |
| MCZ | MCZ monitoring | Skomer | 01A | 173194 | 208556 | SM7319408556 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 02A | 173264 | 208955 | SM7326408955 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 04A | 173781 | 209572 | SM7378109572 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 06A | 173037 | 210151 | SM7303710151 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 07A | 175624 | 209648 | SM7562409648 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 09A | 176522 | 209688 | SM7652209688 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 10A | 177760 | 209720 | SM7776009720 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 11A | 176096 | 209303 | SM7609609303 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 12A | 177004 | 209327 | SM7700409327 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | 17A | 174294 | 209354 | SM7429409354 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | LPT | 176671 | 209364 | SM7667109364 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| MCZ | MCZ monitoring | Skomer | NNK | 174158 | 209311 | SM7415809311 |  | 3 |  | Pembrokeshire South | Pembrokeshire Marine |
| Habs/WFD | CSEMP | Milford Haven CSEMP | Milford Haven CSEMP | 198428 | 204270 | SM9842804270 | 20 | 20 | 20 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| Habs/WFD | CSEMP | The Dee CSEMP | The Dee | 315179 | 383013 | SJ1517983013 | 20 | 20 | 20 | DEE (N. WALES) | Dee Estuary |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_01 | 183092 | 205320 | SM8309205320 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_02 | 185623 | 204551 | SM8562304551 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_03 | 186165 | 203912 | SM8616503912 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_04 | 187051 | 204260 | SM8705104260 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_05 | 186708 | 204728 | SM8670804728 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_06 | 187734 | 203952 | SM8773403952 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_07 | 189138 | 204418 | SM8913804418 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_08 | 189322 | 205037 | SM8932205037 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_09 | 191859 | 204447 | SM9185904447 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Coastal water body | Milford Haven Outer Waterbody | MHC_10 | 185922 | 206072 | SM8592206072 |  |  | 2 | Milford Haven Outer | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_01 | 200256 | 211429 | SN0025611429 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_02 | 200481 | 211136 | SN0048111136 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_03 | 200459 | 210821 | SN0045910821 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_04 | 200439 | 210698 | SN0043910698 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_05 | 200217 | 210222 | SN0021710222 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_06 | 199976 | 210140 | SM9997610140 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_07 | 200002 | 209748 | SN0000209748 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_08 | 199905 | 209185 | SM9990509185 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_09 | 197710 | 205055 | SM9771005055 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_10 | 200282 | 208266 | SN0028208266 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_11 | 200807 | 207670 | SN0080707670 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_12 | 200675 | 207144 | SN0067507144 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_13 | 200676 | 206259 | SN0067606259 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_14 | 200390 | 206096 | SN0039006096 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_15 | 200340 | 205870 | SN0034005870 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_16 | 200226 | 205280 | SN0022605280 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_17 | 197186 | 205142 | SM9718605142 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_18 | 194868 | 204017 | SM9486804017 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_19 | 194377 | 203997 | SM9437703997 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_20 | 194972 | 204432 | SM9497204432 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_21 | 198425 | 204226 | SM9842504226 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_22 | 194596 | 203767 | SM9459603767 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_23 | 200555 | 206414 | SN0055506414 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_24 | 195218 | 204245 | SM9521804245 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Transitional water body | Milford Haven Inner Waterbody | MHT_25 | 195490 | 204681 | SM9549004681 |  |  | 2 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC001 | 224849 | 253628 | SN2484953628 |  |  | 2 | Cardigan Bay Central | Cardigan Bay |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC002 | 226448 | 252459 | SN2644852459 |  |  | 2 | Cardigan Bay Central | Cardigan Bay |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC003 | 229091 | 253320 | SN2909153320 |  |  | 2 | Cardigan Bay Central | Cardigan Bay |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC004 | 229888 | 255198 | SN2988855198 |  |  | 2 | Cardigan Bay Central | Cardigan Bay |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC006 | 234912 | 259636 | SN3491259636 |  |  | 2 | Cardigan Bay Central | Cardigan Bay |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC007 | 241376 | 262258 | SN4137662258 |  |  | 2 | Cardigan Bay Central | Cardigan Bay |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC008 | 218909 | 252042 | SN1890952042 |  |  | 2 | Cardigan Bay Central | Cardigan Bay |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC008 | 242773 | 263308 | SN4277363308 |  |  | 2 | Cardigan Bay Central | Cardigan Bay |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC009 | 248131 | 265914 | SN4813165914 |  |  | 2 | Cardigan Bay Central | Outside SAC |
| WFD | Coastal water body | Cardigan Bay Central Waterbody | CBC010 | 253178 | 273288 | SN5317873288 |  |  | 2 | Cardigan Bay Central | Outside SAC |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN001 | 258621 | 288507 | SN5862188507 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN002 | 258450 | 291201 | SN5845091201 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN003 | 258125 | 294897 | SN5812594897 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN004 | 255348 | 295131 | SN5534895131 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN005 | 253148 | 297945 | SN5314897945 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN006 | 253690 | 306367 | SH5369006367 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN007 | 257854 | 314923 | SH5785414923 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN008 | 248222 | 309408 | SH4822209408 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN008 | 248189 | 309374 | SH4818909374 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN009 | 244276 | 308365 | SH4427608365 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Cardigan Bay North Waterbody | CBN010 | 250351 | 312984 | SH5035112984 |  |  | 2 | Cardigan Bay North | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Teifi Waterbody | TEF_11B | 214002 | 250296 | SN1400250296 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_12B | 214247 | 250403 | SN1424750403 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_13B | 214409 | 250126 | SN1440950126 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_14B | 214588 | 249953 | SN1458849953 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_15B | 214740 | 249785 | SN1474049785 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_16B | 214657 | 249534 | SN1465749534 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_17B | 214970 | 249595 | SN1497049595 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_18B | 215352 | 249756 | SN1535249756 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_19B | 215626 | 249934 | SN1562649934 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_20B | 215368 | 250063 | SN1536850063 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_21B | 214573 | 250380 | SN1457350380 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Teifi Waterbody | TEF_22B | 214824 | 250608 | SN1482450608 |  | 2 |  | TEIFI ESTUARY | Cardigan Bay |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB1 | 226753 | 384504 | SH2675384504 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB2 | 225159 | 383949 | SH2515983949 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB3 | 226243 | 383274 | SH2624383274 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB4 | 226346 | 384021 | SH2634684021 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB5 | 226064 | 384292 | SH2606484292 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB6 | 226757 | 383463 | SH2675783463 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB7 | 226635 | 383970 | SH2663583970 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB8 | 227156 | 382434 | SH2715682434 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB9 | 227776 | 383132 | SH2777683132 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Holyhead Bay WFD waterbody | HHB10 | 226286 | 382378 | SH2628682378 | 2 |  |  | Holyhead Bay | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS009 | 213781 | 328312 | SH1378128312 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS012 | 213168 | 328076 | SH1316828076 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS016 | 213889 | 329127 | SH1388929127 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS023 | 228791 | 342144 | SH2879142144 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS024 | 216133 | 333254 | SH1613333254 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS028 | 216533 | 333901 | SH1653333901 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS037 | 219497 | 336438 | SH1949736438 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS039 | 220234 | 337750 | SH2023437750 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS040 | 221397 | 338419 | SH2139738419 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS042 | 226584 | 343582 | SH2658443582 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS047 | 220980 | 339071 | SH2098039071 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS060 | 231901 | 344949 | SH3190144949 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS065 | 233773 | 346893 | SH3377346893 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS068 | 236035 | 348185 | SH3603548185 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS070 | 237512 | 348187 | SH3751248187 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS072 | 236391 | 348832 | SH3639148832 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS076 | 238258 | 349482 | SH3825849482 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS077 | 239024 | 349452 | SH3902449452 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS080 | 240163 | 350122 | SH4016350122 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS081 | 239044 | 350784 | SH3904450784 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS082 | 239777 | 350781 | SH3977750781 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS083 | 240521 | 350780 | SH4052150780 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS088 | 241285 | 352071 | SH4128552071 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS097 | 240533 | 354675 | SH4053354675 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS100 | 241660 | 355324 | SH4166055324 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS105 | 240495 | 355959 | SH4049555959 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS111 | 242024 | 357287 | SH4202457287 |  |  | 2 | Caernarfon Bay South | Outside SAC |
| WFD | Coastal water body | Caernarfon Bay South | CFS121 | 212975 | 327651 | SH1297527651 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Coastal water body | Caernarfon Bay South | CFS124 | 228277 | 341376 | SH2827741376 |  |  | 2 | Caernarfon Bay South | Lleyn Peninsula and the Sarnau |
| WFD | Transitional water body | Conwy Estuary | CWT006 | 278406 | 377861 | SH7840677861 | 2 |  |  | Conwy Estuary | Outside SAC |
| WFD | Transitional water body | Conwy Estuary | CWT013 | 278072 | 378282 | SH7807278282 | 2 |  |  | Conwy Estuary | Outside SAC |
| WFD | Transitional water body | Conwy Estuary | CWT034 | 277790 | 378953 | SH7779078953 | 2 |  |  | Conwy Estuary | Outside SAC |
| WFD | Transitional water body | Conwy Estuary | CWT035 | 275087 | 382651 | SH7508782651 | 2 |  |  | Conwy Estuary | Menai Strait and Conwy Bay |
| WFD | Transitional water body | Conwy Estuary | CWT038 | 275637 | 379147 | SH7563779147 | 2 |  |  | Conwy Estuary | Menai Strait and Conwy Bay |
| WFD | Transitional water body | Conwy Estuary | CWT043 | 276522 | 379784 | SH7652279784 | 2 |  |  | Conwy Estuary | Menai Strait and Conwy Bay |
| WFD | Transitional water body | Conwy Estuary | CWT044 | 274877 | 379076 | SH7487779076 | 2 |  |  | Conwy Estuary | Menai Strait and Conwy Bay |
| WFD | Transitional water body | Conwy Estuary | CWT050 | 275525 | 380655 | SH7552580655 | 2 |  |  | Conwy Estuary | Menai Strait and Conwy Bay |
| WFD | Transitional water body | Conwy Estuary | CWT052 | 275752 | 382425 | SH7575282425 | 2 |  |  | Conwy Estuary | Menai Strait and Conwy Bay |
| WFD | Transitional water body | Conwy Estuary | CWT066 | 275249 | 381482 | SH7524981482 | 2 |  |  | Conwy Estuary | Menai Strait and Conwy Bay |
| MHWESG |  | Milford Haven Waterway | LH8 | 183601 | 204394 | SM8360104394 |  |  | 6 | Pembrokeshire South | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | LH2 | 185653 | 204551 | SM8565304551 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | MH10 | 194795 | 203855 | SM9479503855 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | MH6 | 189050 | 204408 | SM8905004408 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | MH4 | 191808 | 204442 | SM9180804442 |  |  | 6 | Milford Haven Outer | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | UH6 | 198447 | 204320 | SM9844704320 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | UH3 | 200500 | 205797 | SN0050005797 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | 14 | 200066 | 209085 | SN0006609085 |  |  | 6 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | St 5 | 194794 | 203851 | SM9479403851 |  |  | 5 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | St 3 | 189050 | 204412 | SM8905004412 |  |  | 5 | Milford Haven Outer | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | St 4 | 191808 | 204442 | SM9180804442 |  |  | 5 | Milford Haven Outer | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | St 6 macr | 198446 | 204316 | SM9844604316 |  |  | 1 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | AB3 | 188482 | 202698 | SM8848202698 |  |  | 1 | Milford Haven Outer | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | AB4 | 188921 | 202423 | SM8892102423 |  |  | 1 | Milford Haven Outer | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | PR1 | 194663 | 202320 | SM9466302320 |  |  | 1 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | PR2 | 196551 | 201942 | SM9655101942 |  |  | 1 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | PR3 | 194456 | 202607 | SM9445602607 |  |  | 1 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | CP1 | 198365 | 203729 | SM9836503729 |  |  | 1 | MILFORD HAVEN INNER | Pembrokeshire Marine |
| MHWESG |  | Milford Haven Waterway | CP2 | 198368 | 204086 | SM9836804086 |  |  | 1 | MILFORD HAVEN INNER | Pembrokeshire Marine |

# Appendix 3. A list of Transitional WFD waterbodies within SACs

A list of WFD tractional waterbodies within SACs in Wales

| SAC | WFD Waterbody | Total (km2) |
| --- | --- | --- |
| Cardigan Bay / Bae Ceredigion | TEIFI ESTUARY | 4.49 |
|  | Outside Transitional Waterbody | 954.17 |
| Carmarthen Bay and Estuaries / Bae Caerfyrddin ac Aberoedd | BURRY INLET INNER | 10.92 |
|  | TYWI & TAF & GWENDRAETH - THREE RIVERS ESTUARY | 21.27 |
|  | Outside Transitional Waterbody | 628.89 |
| Dee Estuary / Aber Dyfrdwy (Wales) | DEE (N. WALES) | 60.50 |
|  | Outside Transitional Waterbody | 14.73 |
| Kenfig / Cynffig | OGMORE ESTUARY | 0.06 |
|  | Outside Transitional Waterbody | 11.85 |
| Pembrokeshire Marine / Sir Benfro Forol | MILFORD HAVEN INNER | 19.96 |
|  | SOLFACH ESTUARY | 0.12 |
|  | Outside Transitional Waterbody | 1360.59 |
| Pen Llyn a'r Sarnau / Lleyn Peninsula and the Sarnau | ARTRO | 0.64 |
|  | DWYFOR | 0.00 |
|  | DYFI & LERI | 14.16 |
|  | DYSYNNI | 0.03 |
|  | GLASLYN | 15.46 |
|  | MAWDDACH | 9.25 |
|  | Outside Transitional Waterbody | 1420.80 |
| Severn Estuary (Wales) | SEVERN LOWER | 247.19 |
|  | USK | 0.85 |
|  | WYE | 0.13 |
|  | Outside Transitional Waterbody | 19.54 |
| Y Fenai a Bae Conwy / Menai Strait and Conwy Bay | CONWY | 3.52 |
|  | FORYD BAY | 2.33 |
|  | SEIONT | 0.05 |
|  | Outside Transitional Waterbody | 259.12 |

# Appendix 4. A list of Coastal WFD waterbodies within SACs

A list of WFD Coastal Waterbodies within SACs in Wales

| SAC | WFD Waterbody | Total (km2) |
| --- | --- | --- |
| Bae Cemlyn / Cemlyn Bay | Cemlyn Lagoon | 0.17 |
|  | The Skerries | 0.11 |
|  | Outside Coastal Waterbody | 0.15 |
| Cardigan Bay / Bae Ceredigion | Cardigan Bay Central | 79.07 |
|  | Cardigan Bay South | 21.90 |
|  | Outside Coastal Waterbody | 857.69 |
| Carmarthen Bay and Estuaries / Bae Caerfyrddin ac Aberoedd | Bristol Channel Outer North | 0.62 |
|  | Burry Inlet Outer | 76.35 |
|  | Carmarthen Bay | 474.40 |
|  | Outside Coastal Waterbody | 109.72 |
| Dee Estuary / Aber Dyfrdwy (England) | Mersey Mouth | 22.77 |
|  | North Wales | 0.03 |
|  | Outside Coastal Waterbody | 60.03 |
| Dee Estuary / Aber Dyfrdwy (Wales) | North Wales | 4.48 |
|  | Outside Coastal Waterbody | 70.75 |
| Glannau Mon: Cors heli / Anglesey Coast: Saltmarsh | Caernarfon Bay North | 1.59 |
|  | Menai Strait | 2.67 |
|  | Outside Coastal Waterbody | 6.32 |
| Kenfig / Cynffig | Bristol Channel Outer North | 1.17 |
|  | Swansea Bay | 1.53 |
|  | Outside Coastal Waterbody | 9.21 |
| Limestone Coast of South West Wales / Arfordir Calchfaen De Orllewin Cymru | Bristol Channel Outer North | 0.44 |
|  | Carmarthen Bay | 1.43 |
|  | Pembrokeshire South | 0.10 |
|  | Outside Coastal Waterbody | 13.98 |
| Pembrokeshire Marine / Sir Benfro Forol | Cardigan Bay South | 32.36 |
|  | Grassholm Island & The Smalls | 51.51 |
|  | Milford Haven Outer | 35.03 |
|  | Pembrokeshire South | 412.03 |
|  | Pickleridge Lagoon | 0.05 |
|  | Outside Coastal Waterbody | 849.68 |
| Pen Llyn a'r Sarnau / Lleyn Peninsula and the Sarnau | Caernarfon Bay South | 56.41 |
|  | Cardigan Bay North | 683.56 |
|  | Tremadog Bay | 237.76 |
|  | Outside Coastal Waterbody | 482.63 |
| Severn Estuary (Wales) | Bristol Channel Inner North | 15.56 |
|  | Outside Coastal Waterbody | 252.14 |
| Y Fenai a Bae Conwy / Menai Strait and Conwy Bay | Anglesey North | 64.45 |
|  | Conwy Bay | 48.45 |
|  | Menai Strait | 62.93 |
|  | North Wales | 15.75 |
|  | Outside Coastal Waterbody | 73.44 |