Water Framework Directive assessment: scoping for activities in estuarine and coastal waters

Use this template to record the findings of the scoping stage of your Water Framework Directive (WFD) assessment for an activity in an estuary or coastal water.

If your activity will:

- take place in or affect more than one water body, complete a template for each water body

- include several different activities or stages as part of a larger project, complete a template for each activity as part of your overall WFD assessment

The WFD assessment guidance for estuarine and coastal waters will help you complete the table.

| Your activity | Description, notes or more information |
| --- | --- |
| Applicant name | Gwynedd Council Maritime Department |
| Application reference number (where applicable) |  |
| Name of activity | Coastal Infrastructure: Construction or modification of Coastal flood and erosion risk management schemes. |
| Brief description of activity | Modification to a crib groyne structure on the southern side of the harbour entrance at Pwllheli. The work will involve removing steelwork, sort and grade stones and reconstruct the new groyne by utilising the existing stones where applicable and using larger stones so that the structure can withstand the wave climate that it is exposed to. |
| Location of activity (central point XY coordinates or national grid reference) | Pwllheli Harbour Entrance (southern side) Grid Referance: 238899, 334532 |
| Footprint of activity (ha) | Approx. 3,000m2 (0.3ha) |
| Timings of activity (including start and finish dates) | Start: May / June 2019 (TBC) Finish: TBC |
| Extent of activity (for example size, scale frequency, expected volumes of output or discharge) |  |
| Use or release of chemicals (state which ones) | No chemicals used. However, machinery will be operated, and there is an inherent risk of accidental spillages to the marine environment. Chemicals include oils (engine, hydraulic), coolants etc. |

| Water body1 | Description, notes or more information |
| --- | --- |
| WFD water body name | Tremadog Bay |
| Water body ID | GB651009350000 |
| River basin district name | Western Wales River Basin |
| Water body type (estuarine or coastal) | Coastal |
| Water body total area (ha) | 239km2 (23,900ha) |
| Overall water body status (2015) | Good |
| Ecological status | Good |
| Chemical status | Good |
| Target water body status and deadline | Not Designated |
| Hydromorphology status of water body | Not Designated A/HMWB |
| Heavily modified water body and for what use | No |
| Higher sensitivity habitats present | None within close proximity of the proposed work |
| Lower sensitivity habitats present | Intertidal soft sediment, gravel and cobbles, rockyshores (as per Magicmap data) |
| Phytoplankton status | Not known |
| History of harmful algae | None |
| WFD protected areas within 2km | Pen Llyn a’r Sarnau SAC (directly adjacent) |

1 Water body information can be found in the Environment Agency’s catchment data explorer and the water body summary table. Magic maps provide additional information on habitats and protected areas. Links to these information sources can be found in the WFD assessment guidance for estuarine and coastal waters.

Specific risk information

Consider the potential risks of your activity to each of these receptors: hydromorphology, biology (habitats and fish), water quality and protected areas. Also consider invasive non-native species (INNS).

Section 1: Hydromorphology

Consider if hydromorphology is at risk from your activity.

Use the water body summary table to find out the hydromorphology status of the water body, if it is classed as heavily modified and for what use.

| Consider if your activity: | Yes | No | Hydromorphology risk issue(s) |
| --- | --- | --- | --- |
| Could impact on the hydromorphology (for example morphology or tidal patterns) of a water body at high status | Requires impact assessment | Impact assessment not required | The scheme involves constructing a groyne structure that would replace an existing groyne structure. In terms of effect on hydromorphology, It is likely that the new proposed groyne would change dynamics locally (in and around the harbour entrance) as it would reduce the amount of sediment accumulating at the harbour entrance. |
| Could significantly impact the hydromorphology of any water body | Requires impact assessment | Impact assessment not required | No significant impact on hydromorphology of the waterbody is anticipated since the proposal is to replace a groyne structure, which will serve the same function as the existing. The proposed groyne will be marginally longer (10m), however, this increase is not considered to significantly alter hydro morphological process within the water body. |
| Is in a water body that is heavily modified for the same use as your activity |  | Impact assessment not required | The scheme involves constructing a groyne structure that would replace an existing groyne structure. |

Record the findings for hydromorphology and go to section 2: biology.

Section 2: Biology

Habitats

Consider if habitats are at risk from your activity.

Use the water body summary table and Magic maps, or other sources of information if available, to find the location and size of these habitats.

| Higher sensitivity habitats 2 | Lower sensitivity habitats 3 |
| --- | --- |
| chalk reef | cobbles, gravel and shingle |
| clam, cockle and oyster beds | intertidal soft sediments like sand and mud |
| intertidal seagrass | rocky shore |
| maerl | subtidal boulder fields |
| mussel beds, including blue and horse mussel | subtidal rocky reef |
| polychaete reef | subtidal soft sediments like sand and mud |
| saltmarsh |  |
| subtidal kelp beds |  |
| subtidal seagrass |  |

2 Higher sensitivity habitats have a low resistance to, and recovery rate, from human pressures.

3 Lower sensitivity habitats have a medium to high resistance to, and recovery rate from, human pressures.

| Consider if the footprint4 of your activity is: | Yes | No | Biology habitats risk issue(s) |
| --- | --- | --- | --- |
| 0.5km2 or larger | Yes to one or more – requires impact assessment | No to all – impact assessment not required | No - The total area of the proposed work is not likely to exceed 0.3ha. |
| 1% or more of the water body’s area |  |  | No – The area of the work constitute 0.0012% of the water body area. |
| Within 500m of any higher sensitivity habitat |  |  | Yes – Mussel beds have been identified have been identified approximately 400m south of the proposed work. Risk to this habitat would be mitigated through good working practices and phasing the works with suitable low tides. |
| 1% or more of any lower sensitivity habitat |  |  | No – the proposed work would encroach into intertidal soft sediment habitat and cobble, gravel and shingles habitat, but the footprint of the work is not considered to exceed 1% of the total habitat area within the water body area. |

4 Note that a footprint may also be a temperature or sediment plume. For dredging activity, a footprint is 1.5 times the dredge area.

Fish

Consider if fish are at risk from your activity, but only if your activity is in an estuary or could affect fish in or entering an estuary.

| Consider if your activity: | Yes | No | Biology fish risk issue(s) |
| --- | --- | --- | --- |
| Is in an estuary and could affect fish in the estuary, outside the estuary but could delay or prevent fish entering it or could affect fish migrating through the estuary | Continue with questions | Go to next section | The proposed work does not directly affect an estuary, but would be carried out within close proximity to the harbour entrance where two rivers, Afon Erch and Afon Rhyd Hir, meets before discharging into Tremadog Bay. Construction related noise could potentially affect fish migrating through the estuary. |
| Could impact on normal fish behaviour like movement, migration or spawning (for example creating a physical barrier, noise, chemical change or a change in depth or flow) | Requires impact assessment | Impact assessment not required | Construction activities are likely to generate some degree of noise, in particular when placing the stone bolulders in place, therefore, noise could potentially affect fish migrating through the estuary. The proposed work would not impact spawning sites, create a physical barrier nor lead to change in depth or flow of the water course/ body. |
| Could cause entrainment or impingement of fish | Requires impact assessment | Impact assessment not required | No – the work predominantly would be carried out above tide level and would therefore not cause entrainment or impingement of fish. |

Record the findings for biology habitats and fish and go to section 3: water quality.

Section 3: Water quality

Consider if water quality is at risk from your activity.

Use the water body summary table to find information on phytoplankton status and harmful algae.

| Consider if your activity: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| Could affect water clarity, temperature, salinity, oxygen levels, nutrients or microbial patterns continuously for longer than a spring neap tidal cycle (about 14 days) | Requires impact assessment | Impact assessment not required | No – the proposed work would predominantly be carried out above tide level, and as such would not directly impact water clarity, temperature, oxygen levels, nutrients or microbial patters of the water body, especially not for continuous periods longer than 14 days. There may be some discolouration locally through the placement of stones and through ground disturbance, but this would be minimal, and would clear within 1 tidal cycle. |
| Is in a water body with a phytoplankton status of moderate, poor or bad | Requires impact assessment | Impact assessment not required | No – From available information, the waterbody does not have a phytoplankton status of moderate, poor or bad. |
| Is in a water body with a history of harmful algae | Requires impact assessment | Impact assessment not required | No – As far as we are aware, the waterbody does not have a history of harmful algae. |

Consider if water quality is at risk from your activity through the use, release or disturbance of chemicals.

| If your activity uses or releases chemicals (for example through sediment disturbance or building works) consider if: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| The chemicals are on the Environmental Quality Standards Directive (EQSD) list | Requires impact assessment | Impact assessment not required | No – The activity does not use or release chemicals listed on the EQSD list. |
| It disturbs sediment with contaminants above Cefas Action Level 1 | Requires impact assessment | Impact assessment not required | No – there is no reason to believe that the sediment within and around the area of the works have concentrations of contaminants greater than Cefas Level 1. The material collected from in and around the area of works following past dredging campaigns have been stockpiled at the landward end of the proposed new groyne; this material has recently been tested and have returned levels significantly below the Cefas AL 1 thresholds. There is no reason to believe that the work would disturb and mobilise such contaminants, since the large proportion of the work would be carried out above tide level. |

| If your activity has a mixing zone (like a discharge pipeline or outfall) consider if: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| The chemicals released are on the Environmental Quality Standards Directive (EQSD) list | Requires impact assessment5 | Impact assessment not required | No – the proposal does not use the chemicals listed on the EQSD list. |

5 Carry out your impact assessment using the Environment Agency’s surface water pollution risk assessment guidance, part of Environmental Permitting Regulations guidance.

Record the findings for water quality go on to section 4: WFD protected areas.

Section 4: WFD protected areas

Consider if WFD protected areas are at risk from your activity. These include:

| - special areas of conservation (SAC) | - bathing waters |
| --- | --- |
| - special protection areas (SPA) | - nutrient sensitive areas |
| - shellfish waters |  |

Use Magic maps to find information on the location of protected areas in your water body (and adjacent water bodies) within 2km of your activity.

| Consider if your activity is: | Yes | No | Protected areas risk issue(s) |
| --- | --- | --- | --- |
| Within 2km of any WFD protected area6 | Requires impact assessment | Impact assessment not required | The proposed groyne would encroach approximately 15-20m into the Pen Llyn a’r Sarnau SAC. Would need to look further into how the proposed work would impact habitat features of the site. The west end beach in Pwllheli is designated bathing water. Mitigation measures would be required to ensure that construction activities does not adversely affect the bathing water status. |

6 Note that a regulator can extend the 2km boundary if your activity has an especially high environmental risk.

Record the findings for WFD protected areas and go to section 5: invasive non-native species.

Section 5: Invasive non-native species (INNS)

Consider if there is a risk your activity could introduce or spread INNS.

Risks of introducing or spreading INNS include:

- materials or equipment that have come from, had use in or travelled through other water bodies

- activities that help spread existing INNS, either within the immediate water body or other water bodies

| Consider if your activity could: | Yes | No | INNS risk issue(s) |
| --- | --- | --- | --- |
| Introduce or spread INNS | Requires impact assessment | Impact assessment not required | The NRW coastal risk assessment shows that Tremadog Bay is at risk from the following INNS: - Slipper Limpet - Leathery Sea Squirt - Colonial Tunicate Existing stones will be used within the scheme. However, additional stones would be required, which would be sourced from a quarry (location not known). The risk of introducing INNS by this means is negligible, since it is unlikely that the stones would have been in contact with the marine environment before reaching site. However, plant machinery will be used, and at this stage it cannot be ruled out that the plant is free from INNS, albeit the probability is low. The risk can be reduced easily by contractors ensuring that plant machinery is clean before arrival at site. |

Record the findings for INNS and go to the summary section.

Summary

Summarise the results of scoping here.

| Receptor | Potential risk to receptor? | Note the risk issue(s) for impact assessment |
| --- | --- | --- |
| Hydromorphology | Yes | Changes in sediment transportation along fromtage |
| Biology: habitats | Yes | RAMs could easily be implemented to prevent risk to |
| Biology: fish | Yes | Noise could potentially deter fish from migrating beyond the harbour entrance during periods when boulders are placed, in particular at the most seaward extent of the proposed groyne. |
| Water quality | Yes | RAMs could easily be implemented to prevent risk to Water Quality. |
| Protected areas | YES | Pen Llyn a’r Sarnau SAC is located immediately adjacent to the proposed work. |
| Invasive non-native species | YES | RAMs could easily be implemented to prevent risk of introducing INNS. |

If you haven’t identified any receptors at risk during scoping, you don’t need to continue to the impact assessment stage and your WFD assessment is complete.

If you’ve identified one or more receptors at risk during scoping, you should continue to the impact assessment stage.

Include your scoping results in the WFD assessment document you send to your activity’s regulator as part of your application for permission to carry out the activity.