| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-031796 |
| Permit Reference Number: | EPR/AB3891CX |
| Permit Holder: | Mr Daniel James and Mrs Carys James |
| Site Address: | Hafod Farm, Gwbert, Cardigan, Ceredigion, SA43 1PU |
| Grid Reference: | Farm location: SN 18094 50342 Fields: 4-B SN 17789 49959 5-B SN 17844 50084 6-B SN 17604 50225 7-B SN 17468 49980 8-B SN 17883 50437 9-B SN 18008 50535 8222-B SN 17826 50229 7233-B SN 17727 50338 5641-B SN 17563 50418 4956-B SN 17493 50566 HAFOD B/FRON 2849 SN 17301 50496 HAFOD B/FRON 3836 SN 17366 50337 BOLAFRON 4718 SN 17470 50166 BOLAFRON 3107 SN 17319 50053 BOLAFRON 6609 SN 17653 50093 Storage: 1 SN 17505 49867 2 SN 17940 50095 3 SN 17956 50061 4 SN 18220 50263 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Malcolm.Dines@cyfoethnaturiolcymru.gov.uk |
| Date due: | 04/03/2026 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Fundamental checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- DMS/PaLS |
| Correct fee? | Y | See permit compliance multiplier above and deployment fee- Charging Scheme Transaction reference: 2152432 Date: 11/02/2026 Ref: BS/11022026/15 STEPSIDE AGRICULTUEPDEPSTEPS0164 Amount: £1,780.00 |
| Agricultural Benefit Statement | Y |  |
| Waste Analysis Report | Y | All within 12 months from the date submitted to NRW. |
| Soil Analysis Report | Y |  |
| Location Map | Y |  |
| Environmental Impact Assessment- Agricultural Regulations Evidence | Y |  |
| Risk assessment (if applicable) | n/a | Only if in high risk location |
| Landowner/occupier consent | Y |  |
| If confidentiality box has been ticked, no evidence provided | n/a | Only if confidentiality requested |
| Declaration authorisation | n/a | Only if not signed by a relevant person |

| Saving documents- by determining permitting officer | Y N |
| --- | --- |
| Save embedded application documents from application form | Y |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | CPH provided in LPD1: 55/226/0027 See comments at the bottom of the decision document |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-028923 Expires: 14/05/2026 However, the deployment was withdrawn by the applicant on 16/02/2026. So, no current active deployment for these fields. |
| Which risk band does the activity fall within? - Medium risk (1) deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | Benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| location of all storage sites for the waste to be spread | Y |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | n/a |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species/Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | n/a |  |
| Flood risk | n/a | No flood risk for the fields in this deployment. |
| What is the slope of the fields? | 0-7 degrees (transparent)- All fields |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y | See comments at bottom of the decision document. |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | No field heaps proposed. |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Waste not high in readily available nitrogen |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 02 05 03 – OMP has been provided see comments at the bottom of the decision document. 19 09 02 – Fe, Al, S, Mn S |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | Maintenance dressing only |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y | See comments at the bottom of the decision document |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y | See comments at the bottom of the decision document |
| If the cropping is any of the following, check if that the available Nitrogen applied is less than the below amount: - Field beans, Peas- 0 kg/ha - Sugar beet- 120 kg/ha - Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes- 150 kg/ha - Spring-sown wheat, Winter barley- 180 kg/ha - Autumn or early winter sown wheat- 220 kg/ha *Regulation 10 The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021 | n/a |  |
| Nutrient assessments for P- If land is above target index (3+ for arable, grassland, and forage crops. 4+ for vegetables) has a net reduction in P levels across the crop rotation been demonstrated? | Y | See comments at the bottom of the decision document |
| Nutrient assessments for P- if the above has been agreed for a previous deployment, is it still correct for this this application? | n/a | Not agreed with previous deployment. |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | N | See previous deployment: PAN-028923 |
| - The nutrients supplied by the material? | N |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant Environment team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site (project) within 500 meters of a European Site (SPA, pSPA, SAC or cSAC) or a Ramsar site? | No- there is no conceivable impact pathway between the project and any European site features, due to the location of the project. |

Comments

Consultation responses: Phosphate additions from the wastes under this deployment were raised in regard to soil index values (SIV) being at or above optimum levels (3 & 2) while the wastes proposed to spread maintenance dressing of phosphates only. The applicant has provided justification spreading phosphate to these fields in the submitted agricultural benefit statement. This includes applying phosphate below crop offtake which is approximately -13 kg/ha for fields with a SIV of 3. As a result the applicant has demonstrated that a net reduction of phosphate will take place in these fields over a 12 month period to gradually reducing the level of phosphate in the soil. The wastes are also providing other benefits to the soil such as nitrogen and sulphur that demonstrate agricultural benefit. The ABS also commits to reviewing the fields under a new soil analysis in the near future. As a result NRW has accepted this justification of spreading these wastes to the fields.

MyMap/location and slope assessments: MyMap/location assessment For details of the MyMap assessment see the DMS. Slope assessment Field number: All fields Slope of the land: 0-7o Soil type: Medium soils Wastes types applied: Non stackable sludge. Cropping type: Grass Method of application: Dribble bar Weather conditions before during and after spreading: In compliance with the weather restrictions outlined in CoGAP. Conclusions (is run off risk high?): The wastes will be applied to the fields via dribble bar which is an appropriate spreading method for the waste types (non-stackable sludge). In addition to this The maximum application rate for each field over 50t/ha will be split into multiple applications and will not exceed 50t/ha in any one application to a field. Due to this the surface runoff risk has been determined as low.

CPH details: CPH provided in LPD1: 55/226/0027 My Map record: Fields: 3107, 7B, 4B, 2849, 3836, 4718, 6609, 5B, 6B, 8222B, 4956B, 5641B, 7233B = 55/226/0026 ELIZIBETH RACHEL JAMES Fields: 8B & 9B = 55/226/0027 MRS JC DAVIES AND SONS The county parish holding numbers provided in the LPD1 form match the Mymap records no further action required.

Protection of habitats and ecosystems For details of the MyMap assessment see the DMS.

Agricultural benefit calculations: Phosphate Additions The fields in this deployment (5B, 6B, 7B, 9B, 8222, 7233 and 5641) have a soil index value (SIV of 2) at optimum levels. Fields 4B, 8B, 4956, 2849, 3836, 4718, 3107 & 6609 have a SIV of 3 above optimum levels of 2. The applicant has provided justification spreading phosphate to these fields in the submitted agricultural benefit statement. This includes applying phosphate below crop offtake which is approximately -13 kg/ha for fields with a SIV of 3. As a result the applicant has demonstrated that a net reduction of phosphate will take place in these fields over a 12 month period to gradually reducing the level of phosphate in the soil. The wastes are also providing other benefits to the soil such as nitrogen and sulphur that demonstrate agricultural benefit. The ABS also commits to reviewing the fields under a new soil analysis in the near future. As a result NRW has accepted this justification of spreading these wastes to the fields. Potassium additions The soil index values for potassium for the fields 4B, 5B, 6B, 8B, 9B, 8222, 7233, 5671 & 4956 are above optimum levels at 3. The offtake from the grass is significantly higher than the applied potassium additions of the waste meaning the overall level of potassium in the soil is being significantly reduced in a 12 month period. Due to this no further action is needed. Magnesium additions The soil index values (SIV) for magnesium for all the fields in this deployment are above optimum levels at 3. However the wastes proposed to be applied to the fields are relatively low compared to the main nutrients of these waste streams i.e. nitrogen and sulphur. The magnesium is likely to be removed from offtake. Due to this and the wider benefits of the waste, no action was needed.

Other: Above ground storage tank The above ground storage is constructed of glass fused to steel Perma store no further action required. Odour risk This deployment application contains an odour management plan due to the proposed deployment spreading EWC 02 05 02 (dairy wastes) TGN 8.01 states this waste stream is likely to cause odour issues and an OMP should be provided. From reviewing the nearby sensitive odour receptor NRW agrees an odour management plan was required. The odour management plan has been assessed as part of this application and NRW conclude odour risk from EWC 02 05 02 (dairy wastes) can be effectively managed/ mitigated if spreading and storage prodecures as wrote in the OMP to the nearby sensitive receptors. For further details see the MyMap assessment on the DMS.

Request for information/Return/Refusal reasons:

Request for information: A request for information was not required to be send for this deployment application as the submitted application contained all relevant/correct information to assess.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 19/03/2026 |
| ## Effective date | 19/03/2026 |
| ## Permitting Officer Date | Owen Clee 19/03/2026 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 19/03/2026 |

| Time recording |  |
| --- | --- |
| Admin/consultations (Permit support time) | 50 |
| Determination (waste permitting team time) | 850 |
| Total (to be added to PALS) | 900 |