# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Groundwater Treatment remediation system serving Enterprise Autos operated by

The application number is PAN-029991

The permit number is [insert permit reference]

The discharge is for Groundwater Treatment remediation system serving Enterprise Autos. The discharge of treated effluent to ground water via an injection borehole.

The effluent will comprise treated water, a byproduct of spill response and ground remediation works.100m3 a day will be discharged.

The treated water will be processed through an Oil Water and Silt Separator and Carbon Filtration to separate the contamination from the water byproduct. Treated water will be reinjected hydraulically.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to consultation & web publicising | The web publicising, consultation responses taken into account in the decision. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| The facility |  |  |  |
| The regulated facility | ###### The extent/nature of the facilities taking place at the site required clarification. The regulated facility is a standalone groundwater activity in accordance with EPR RGN 2 Understanding the meaning of regulated facility. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | The discharge activity screens out from needing further assessment under WFD as it is not in a water body at high status or high status for morphology, and it is listed in Annex D as a low risk activity plus there is no in combination impact. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment or similar methodology supplied by the operator and reviewed by ourselves, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. Operating techniques have been reviewed with groundwater remediation strategy and in consultation with Natural Resources Wales ground water specialists are deemed satisfactory. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Incorporating the application | We have specified that the applicant must operate the permit in accordance with descriptions in the application, including all additional information received as part of the determination process. These are specified in the Operating Techniques table in the permit. | ü |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. Emission limits have been set in accordance with NRW geoscience team in line with ground water directives. All limits are set as maximums: Parameter; Limit (incl. unit). Maximum daily discharge volume; 100 m3/day. Visible oil or grease; No significant trace present. Aliphatic C5-C6; 3 µg/L. Aliphatic C6-C8; 3 µg/L. Aliphatic C8-C10; 15 µg/L. Aliphatic C10-C12; 3 µg/L. Aromatic C5-C7; 3 µg/L. Aromatic C7-C8; 15 µg/L. Aromatic C8-C10; 15 µg/L. Aromatic C10-C12; 15 µg/L. Aromatic C12-C16; 15 µg/L. Aromatic C16-C21; 30 µg/L. Aromatic C21-C35; 3 µg/L. 1,2,4 Trimethylbenzene; 0.03 µg/L. Acenaphthene; 0.03 µg/L. Anthracene; 0.03 µg/L. Benzene; 1 µg/L. Benzo[a]pyrene; 0.01 µg/L. Benzo[b]fluoranthene; 0.03 µg/L. Benzo[ghi]perylene; 0.03 µg/L. Benzo[k]fluoranthene; 0.03 µg/L. Ethylbenzene; 3 µg/L. Indeno[1,2,3-cd]pyrene; 0.03 µg/L. Toluene; 3 µg/L. Xylenes total; 3 µg/L. | ü |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. These monitoring requirements have been imposed in order to satisfy the ground water framework requirements. We made these decisions in accordance with the ground water framework requirements. | ü |
| Reporting | We have specified reporting in the permit. To ensure the discharge is operating in line with required remediation targets. We made these decisions in accordance with ground water framework. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a groundwater water discharge activity permit under EPR2016. | ü |