[image: image1.emf]
SITE CONDITION REPORT 

Based on SCR guide for applicants v2.0 August 2008 

(Rebranded to NRW October 2014)
COMPLETE SECTIONS 1-3 AND SUBMIT WITH APPLICATION   

DURING THE LIFE OF THE PERMIT: MAINTAIN SECTIONS 4-7

AT SURRENDER: ADD NEW DOC REFERENCE IN 1.0; COMPLETE SECTIONS 8-10; & SUBMIT WITH YOUR SURRENDER APPLICATION.

	1.0 SITE DETAILS


	

	Name of the applicant


	Ambipar Site Services Ltd (ASSL) 

	Activity address


	Hafod Recycling facility, Hafod Road, Johnstown, Wrexham LL14 6HF

	National grid reference
	SJ3117 4640


	Document reference and dates for Site Condition Report at permit application and surrender


	ASSL/SCR/0A & ASSL/SSR/0A
Permit Ref No. EPR/NB3339RH
 (formerly EPR/WP3990VL)


	Document references for site plans (including location and boundaries)


	Plans submitted to NRW at permit application stage and on NRW file:

Site Location Plan, Permit Boundary Plan, Site Layout and Drainage Plan


Note:

In Part A of the application form you must give us details of the site’s location and provide us with a site plan. We need a detailed site plan (or plans) showing:

· Site location, the area covered by the site condition report, and the location and nature of the activities and/or waste facilities on the site.

· Locations of receptors, sources of emissions/releases, and monitoring points.

· Site drainage.

· Site surfacing.
If this information is not shown on the site plan required by Part A of the application form then you should submit the additional plan or plans with this site condition report. 

	2.0 Condition of the land at permit issue



	Environmental setting including:

· geology

· hydrogeology

· surface waters


	Bedrock Geology: Pennine Middle Coal Coal Measures Formation
Drift: Devensian Diamicton Till 
Not within a Source Protection Zone

No surface waters within the site and site is not susceptible to excess surface water.


	Pollution history including:

· pollution incidents that may have affected land

· historical land-uses and associated contaminants 

· any visual/olfactory evidence of existing contamination

· evidence of damage to pollution prevention measures 


	Not aware of any historic pollution incidents
Waste recycling facility
No evidence
No evidence

	Evidence of historic contamination, for example, historical site investigation, assessment, remediation and verification reports (where available)


	Not aware of any historic contamination.

No reports undertaken

	Baseline soil and groundwater reference data


	N/A

	Supporting information
	· Source information identifying environmental setting and pollution incidents

· Historical Ordnance Survey plans

· Site reconnaissance

· Historical investigation / assessment / remediation / verification reports
· Baseline soil and groundwater reference data


	3.0 Permitted activities



	Permitted activities 


	The recycling centre will allow the acceptance, storage and limited treatment of household, commercial and industrial wastes to aid the recycling, recovery and re-use of waste sourced from the local area or bulking up for onward movement for recovery or disposal. The operations include manual and mechanical sorting of wastes and includes sorting and processing of plastic, metals and wood which reduce the need to use virgin materials. The storage of hazardous wastes such as OILS destined for recovery, solvents destined for recovery or disposal, contaminated rags and contaminated packaging, waste carbon and bonded asbestos is also permitted under the current permit conditions. During sorting, all recyclable materials will be segregated for further recycling elsewhere. Wastes which cannot be recycled will be directed for off-site disposal at an appropriate authorised facility.   

	Non-permitted activities undertaken


	From 2024 part of the site has been operated as a vehicle maintenance facility to service and repair ASSL fleet of vehicles. Refer to Drawing No 4105/1277/03 – Site Layout and Fire Plan

	Document references for:

· plan showing activity layout; and

· environmental risk assessment.


	Drawing No. 4105/1277/01 – Site Location Map




Drawing No. 4105/1277/02 – Permit Boundary Plan




Drawing No. 4105/1277/03 – Site Layout and Fire Plan




Drawing No. 4105/1277/04 – Receptors Plan


Note:

In Part B of the application form you must tell us about the activities that you will undertake at the site. You must also give us an environmental risk assessment.  This risk assessment must be based on our guidance (Environmental Risk Assessment - EPR H1) or use an equivalent approach.

It is essential that you identify in your environmental risk assessment all the substances used and produced that could pollute the soil or groundwater if there were an accident, or if measures to protect land fail. 

These include substances that would be classified as ‘dangerous’ under the Control of Major Accident Hazards (COMAH) regulations and also raw materials, fuels, intermediates, products, wastes and effluents. 

If your submitted environmental risk assessment does not adequately address the risks to soil and groundwater we may need to request further information from you or even refuse your permit application.

	4.0 Changes to the activity



	Have there been any changes to the activity boundary?


	If yes, provide a plan showing the changes to the activity boundary. 
No

	Have there been any changes to the permitted activities?


	If yes, provide a description of the changes to the permitted activities
No 

	Have any ‘dangerous substances’ not identified in the Application Site Condition Report been used or produced as a result of the permitted activities?


	If yes, list of them 

New request to add a selection of EWC 13 Codes

	Checklist of supporting information
	· Plan showing any changes to the boundary (where relevant)

· Description of the changes to the permitted activities (where relevant)

· List of ‘dangerous substances’ used/produced by the permitted activities that were not identified in the Application Site Condition Report  (where relevant)


	5.0  Measures taken to protect land



	Use records that you collected during the life of the permit to summarise whether pollution prevention measures worked. If you can’t, you need to collect land and/or groundwater data to assess whether the land has deteriorated.

.


	Checklist of supporting information
	· Site benefitted from impermeable concrete hardstanding. The containment systems at the waste transfer station consist of waste storage and containment systems in the form of bunded storage areas, external yard areas, associated drainage systems with penstock type isolator valves, external storage bays, skips and cages and waste oil storage tank. 

· Any CAR Inspection records and summary of findings of inspections for all pollution prevention measures held on NRW public register from when the permit was issued.



	6.0 Pollution incidents that may have had an impact on land, and their remediation



	Summarise any pollution incidents that may have damaged the land. Describe how you investigated and remedied each one. If you can’t, you need to collect land and /or groundwater reference data to assess whether the land has deteriorated while you’ve been there.

N/A          
No evidence of any pollution incidents. The site consisted of a concrete slab with sealed drainage.


	Checklist of supporting information
	· Records of pollution incidents that may have impacted on land

· Records of their investigation and remediation 


	7.0 Soil gas and water quality monitoring (where undertaken)



	Provide details of any soil gas and/or water monitoring you did. Include a summary of the findings. Say whether it shows that the land deteriorated as a result of the permitted activities. If it did, outline how you investigated and remedied this.


	Checklist of supporting information
	· Description of soil gas and/or water monitoring undertaken

· Monitoring results (including graphs)


	8.0 Decommissioning and removal of pollution risk



	Describe how the site was decommissioned. Demonstrate that all sources of pollution risk have been removed. Describe whether the decommissioning had any impact on the land. Outline how you investigated and remedied this.



	Checklist of supporting information
	· Site closure plan

· List of potential sources of pollution risk

· Investigation and remediation reports (where relevant)


	9.0 Reference data and remediation (where relevant)



	Say whether you had to collect land and/or groundwater data. Or say that you didn’t need to because the information from sections 3, 4, 5 and 6 of the Surrender Site Condition Report shows that the land has not deteriorated.
If you did collect land and/or groundwater reference data, summarise what this entailed, and what your data found. Say whether the data shows that the condition of the land has deteriorated, or whether the land at the site is in a “satisfactory state”. If it isn’t, summarise what you did to remedy this. Confirm that the land is now in a “satisfactory state” at surrender.



	Checklist of supporting information
	· Land and/or groundwater data collected at application (if collected)

· Land and/or groundwater data collected at surrender (where needed)

· Assessment of satisfactory state

· Remediation and verification reports (where undertaken)

	10.0 Statement of site condition



	Using the information from sections 3 to 7, give a statement about the condition of the land at the site. This should confirm that:

· the permitted activities have stopped

· decommissioning is complete, and the pollution risk has been removed

· the land is in a satisfactory condition.




  V2.0 4 August 2008


