OGN 200 Form 1B

# Record of a Habitats Regulations Assessment

All staff using this form should be familiar with OGN200 and should have had appropriate training in HRA.

A separate version of this form is available with guidance notes on how to complete it. You should refer to those notes when using this form.

| Plan or project name, brief description or application reference number | PAN-032194 – Renewal of abstraction licence number 20/56/11/0025, authorising abstraction from a lake known as un-named lake at Bulmore, for the purpose of filling an off-stream storage reservoir for spray irrigation and direct spray irrigation. |
| --- | --- |

| HRA iteration/version | 1 |
| --- | --- |

# 1	Plan or Project Details

| Information about the plan or project |  |  |
| --- | --- | --- |
| 1 | Date application received | 18/03/2026 |
| 2 | Applicant details | Celtic Manor Resort Limited |
| 3 | NRW team responsible for carrying out, or requiring to be carried out, the plan or project, and name of lead officer | N/a |
| 4 | Activity/ies proposed | - Licence number 20/56/11/0025 is expiring on 31/03/2026 and the applicant, The Celtic Manor Resort Limited, is applying to renew the licence on the same terms The renewed licence will expire on 31 March 2038. Licence details below: Abstraction details; New Proposal (as per previous licence). Location of abstraction; Bulmore, Newport. Source of supply; Inland water (lake) known as un-named lake. Point of abstraction (NGR); ST 36330 91700. Purposes of abstraction; Filling an existing off-stream storage reservoir for subsequent spray irrigation. Spray irrigation.. Period of abstraction; For filling of reservoir: From 01 November to 31 March inclusive. For Spray irrigation: From 01 April to 31 October inclusive.. 4.1: Quantities and rates :; For filling of reservoir: 35 l/s, 126 m3/hr, 1,400 m3/day, 15,000 m3/year For Spray irrigation: 35 l/s, 126 m3/hr, 1,400 m3/day, 84,000 m3/year. Means of abstraction; For filling of reservoir: 2 transfer pumps. For Spray irrigation: 4 irrigation pumps. Measurement of abstraction; Meters. |
| 5 | Relevant legislation or statutory basis | - Water Environment (Water Framework Directive) (England and Wales) Regulations 2017 - Environment Act 1995 - Water Resources Act 1991 as amended by the Water Act 2003 - Water Resources (Abstraction and Impounding) Regulations 2006 - Article 3 of the Natural Resources Body for Wales (Establishment) Order 2012 - The Natural Resources Body for Wales (Functions) Order 2013 - The Conservation of Habitats and Species Regulations 2017 - Planning Consent (to cover construction aspects detailed above) - Ordinary Watercourse Consent (all intakes, pipeline crossings and outfall/discharge are on ordinary watercourse) |
| 6 | Location | - Unnamed lake at Bulmore, Newport - ST 36330 91700 |
| 7 | Plan or project documents, including any application documents | Application Form WRI (dated 18/03/2026, available on the Public Register) |
| 8 | Environmental Statement | N/a |
| 9 | Pre-application correspondence | Renewal reminder letter (dated 22/07/2025, available on the Public Register) |
| 10 | NRW team responsible for preparing this HRA report, and lead officer | Hannah Pearce, Water Resources Permitting |
| 11 | Team or person responsible for approving the plan or project (competent authority role) | N/a |

# 2.	Determining the need for a Habitats Regulations Assessment

| 2.1 Is there any possibility that the plan or project could negatively affect any European sites? | Yes - Severn Estuary (Wales) SAC - River Usk / Afon Wysg SAC |
| --- | --- |
| 2.2 Is the whole of the plan or project directly and only connected with or necessary to the management of one or more European sites, for the purposes of conserving the habitats or species for which the European site(s) is/are designated? | No |
| 2.3 Is there a possibility that the plan or project could affect any other feature of the European site(s) concerned, or of another European site, in a way that would undermine that feature’s conservation objectives? | Yes |

# 3.	Considering the likelihood of a significant effect (LSE)

## 3.1	Renewal of a project authorisation on the same or more restrictive terms as an extant authorisation

| Is this a renewal of an extant authorisation which complies with NRW approved criteria for ruling out significant effects of renewals (see Part 2 of OGN200) without conducting a project-specific LSE test? | No |
| --- | --- |

## 3.2	Likelihood of significant effects (LSE) test

### 3.2.1 Which European sites might be affected by the plan or project?

| (a) | Based on the plan or project specification, or information provided in the application, it is considered that these European sites have features which could be negatively affected by the plan or project | - The Severn Estuary / Môr Hafren SAC - The Severn Estuary / Môr Hafren Ramsar The above sites were identified during the screening exercise for this renewal application and are located 15km downstream of the point of abstraction. There is no documentation available to understand how these sites and their features were considered during the original determination of the licence. Based on the designated features and advice received from the Marine Area and Advice Team, it was considered necessary to consider these sites as part of this assessment. - River Usk / Afon Wysg SAC The above site was identified during the screening exercise for this renewal application and is located 300m downstream of the point of abstraction. There is no documentation available to understand how this SAC and its features were considered during the original determination of the licence. Based on the designated features and proximity to the point of abstraction it was considered necessary to consider this site as part of this assessment. |
| --- | --- | --- |
| (b) | The potential for the plan or project to negatively affect these European sites was also initially considered, but can be ruled out without further consideration | - The Severn Estuary SPA No conceivable impact on the above European site, by virtue of the scale or location or nature of the plan or project - this site was identified during the screening exercise for this renewal application, approximately 15km downstream of the point of abstraction. This site was initially considered; however, it is designated for nternationally important populations of migratory bird species, internationally important populations of wintering bird species and assemblage of nationally important populations of waterfowl Therefore, given the nature of the designated features and their distance downstream from the proposal, it can be immediately ruled out. |

### 3.2.2 Screening for likelihood of significant effect

| Table 3.2.2 Screening assessment |  |  |  |
| --- | --- | --- | --- |
| Designated site feature | Relevant conservation objectives | Screening conclusion – SCREEN OUT’, ‘SCREEN IN’ or ‘IN COMB’ | Explanation |
| European site name: The Severn Estuary / Môr Hafren SAC |  |  |  |
| Estuaries | The feature will be considered to be in favourable condition when, subject to natural processes, each of the following conditions are met - the total extent of the estuary is maintained; - the characteristic physical form (tidal prism/cross sectional area) and flow (tidal regime) of the estuary is maintained; - the characteristic range and relative proportions of sediment sizes and sediment budget within the site is maintained; - the extent, variety and spatial distribution of estuarine habitat communities within the site is maintained; - the extent, variety, spatial distribution and community composition of hard substrate habitats and their notable communities is maintained; - the abundance of the notable estuarine species assemblages is maintained or increased; - the physico-chemical characteristics of the water column support the ecological objectives described above; - Toxic contaminants in water column and sediment are below levels which would pose a risk to the ecological objectives described above. - Airborne nutrient and contaminant loads are below levels which would pose a risk to the ecological objectives described above | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. The abstraction is from a lake which is thought to be disconnected from the surface water flows of the River Usk (offline), therefore there is no impact pathway to this feature of the SAC and potential impacts can be ruled out. |
| Subtidal sandbanks | The feature will be considered to be in favourable condition when, subject to natural processes, each of the following conditions are met: - the total extent of the subtidal sandbanks within the site is maintained; - the extent and distribution of the individual subtidal sandbank communities within the site is maintained; - the community composition of the subtidal sandbank feature within the site is maintained - the variety and distribution of sediment types across the subtidal sandbank feature is maintained; - the gross morphology (depth, distribution and profile) of the subtidal sandbank feature within the site is maintained. | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. The abstraction is from a lake which is thought to be disconnected from the surface water flows of the River Usk (offline), therefore there is no impact pathway to this feature of the SAC and potential impacts can be ruled out. |
| Intertidal Mud and Sand | The feature will be considered to be in favourable condition when, subject to natural processes, each of the following conditions are met: - The total extent of the mudflats and sandflats feature is maintained; - the variety and extent of individual mudflats and sandflats communities within the site is maintained; - the distribution of individual mudflats and sandflats communities within the site is maintained; - the community composition of the mudflats and sandflats feature within the site is maintained; - the topography of the intertidal flats and the morphology (dynamic processes of sediment movement and channel migration across the flats) are maintained. | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. The abstraction is from a lake which is thought to be disconnected from the surface water flows of the River Usk (offline), therefore there is no impact pathway to this feature of the SAC and potential impacts can be ruled out. |
| Atlantic salt meadow / salt marshes | The feature will be considered to be in favourable condition when, subject to natural processes, each of the following conditions are met: - the total extent of Atlantic salt meadow and associated transitional vegetation communities within the site is maintained; - the extent and distribution of the individual Atlantic salt meadow and associated transitional vegetation communities within the site is maintained; - the zonation of Atlantic salt meadow vegetation communities and their associated transitions to other estuary habitats is maintained; - the relative abundance of the typical species of the Atlantic salt meadow and associated transitional vegetation communities is maintained; - the abundance of the notable species of the Atlantic salt meadow and associated transitional vegetation communities is maintained. - the structural variation of the salt marsh sward (resulting from grazing) is maintained within limits sufficient to satisfy the requirements of conditions iv and v above and the requirements of the Ramsar and SPA features - the characteristic stepped morphology of the salt marshes and associated creeks, pills, drainage ditches and pans, and the estuarine processes that enable their development, is maintained. - Any areas of Spartina anglica salt marsh (SM6) are capable of developing naturally into other saltmarsh communities. | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. The abstraction is from a lake which is thought to be disconnected from the surface water flows of the River Usk (offline), therefore there is no impact pathway to this feature of the SAC and potential impacts can be ruled out. |
| Reefs | The feature will be considered to be in favourable condition when, subject to natural processes, each of the following conditions are met: - the total extent and distribution of Sabellaria reef is maintained; - the community composition of the Sabellaria reef is maintained; - the full range of different age structures of Sabellaria reef are present; - the physical and ecological processes necessary to support Sabellaria reef are maintained. |  |  |
| River Lamprey | The feature will be considered to be in favourable condition when, subject to natural processes, each of the following conditions are met: - the migratory passage of both adult and juvenile river lamprey through the Severn Estuary between the Bristol Channel and any of their spawning rivers is not obstructed or impeded by physical barriers, changes in flows, or poor water quality; - the size of the river/sea lamprey population in the Severn Estuary and the rivers which drain into it, is at least maintained and is at a level that is sustainable in the long term; - the abundance of prey species forming the river/sea lamprey’s food resource within the estuary, is maintained. - Toxic contaminants in the water column and sediment are below levels which would pose a risk to the ecological objectives described above. | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. Although these mobile species are likely to migrate upstream via the River Usk, the abstraction is from lake which is not in connectivity with the nearby unnamed tributary of the River Usk and, as a result, will not impact these designated features. During determination of the original licence, no conditions were included to avoid impacts to fish (e.g. screening) and no concerns have been raised regarding entrainment or entrapment of fish at the point of abstraction, therefore potential impacts can be ruled out. |
| Sea Lamprey |  |  |  |
| Twaite shad | The feature will be considered to be in favourable condition when, subject to natural processes, each of the following conditions are met: - the migratory passage of both adult and juvenile twaite shad through the Severn Estuary between the Bristol Channel and their spawning rivers is not obstructed or impeded by physical barriers, changes in flows or poor water quality; - the size of the twaite shad population within the Severn Estuary and the rivers draining into it is at least maintained and is at a level that is sustainable in the long term. - the abundance of prey species forming the twaite shad’s food resource within the estuary, in particular at the salt wedge , is maintained. - Toxic contaminants in the water column and sediment are below levels which would pose a risk to the ecological objectives described above. | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. Although these mobile species are likely to migrate upstream via the River Usk, the abstraction is from lake which is not in connectivity with the nearby unnamed tributary of the River Usk and, as a result, will not impact these designated features. During determination of the original licence, no conditions were included to avoid impacts to fish (e.g. screening) and no concerns have been raised regarding entrainment or entrapment of fish at the point of abstraction, therefore potential impacts can be ruled out. |
| European site name: The Severn Estuary / Môr Hafren Ramsar |  |  |  |
| Estuaries | The conservation objective for the “estuaries” feature of the Severn Estuary Ramsar Site is to maintain the feature in favourable condition, as defined by the conservation objective for the SAC “estuaries” feature” (refer to row 1 of The Severn Estuary / Môr Hafren SAC above), in so far as these objectives are applicable to the area designated as Ramsar Site | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. The abstraction is from a lake which is thought to be disconnected from the surface water flows of the River Usk (offline), therefore there is no impact pathway to this feature of the SAC and potential impacts can be ruled out. |
| Assemblage of migratory fish species Including: - Sea lamprey River lamprey Twaite shad - Allis shad - Salmon - Sea trout - Eel | The feature will be considered to be in favourable condition when, subject to natural processes, each of the following conditions are met: - the migratory passage of both adults and juveniles of the assemblage of migratory fish species through the Severn Estuary between the Bristol Channel and any of their spawning rivers is not obstructed or impeded by physical barriers, changes in flows, or poor water quality; - the size of the populations of the assemblage species in the Severn Estuary and the rivers which drain into it, is at least maintained and is at a level that is sustainable in the long term; - the abundance of prey species forming the principle food resources for the assemblage species within the estuary, is maintained. - Toxic contaminants in the water column and sediment are below levels which would pose a risk to the ecological objectives described above. | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. Although these mobile species are likely to migrate upstream via the River Usk, the abstraction is from lake which is not in connectivity with the nearby unnamed tributary of the River Usk and, as a result, will not impact these designated features. During determination of the original licence, no conditions were included to avoid impacts to fish (e.g. screening) and no concerns have been raised regarding entrainment or entrapment of fish at the point of abstraction, therefore potential impacts can be ruled out. |
| Internationally important populations of waterfowl: Bewick’s swan | The conservation objective for the “Bewick’s swan” feature of the Severn Estuary Ramsar Site is to maintain the feature in favourable condition, as defined by the conservation objective for the SPA “Bewick’s swan ” feature | SCREEN OUT | The point of abstraction is located 15km upstream and outside of the site boundary. The abstraction is from a lake which is thought to be disconnected from the surface water flows of the River Usk (offline), therefore there is no impact pathway to this feature of the SAC and potential impacts can be ruled out. |
| Internationally important populations of waterfowl: European white-fronted goose | The conservation objective for the “European white-fronted goose” feature of the Severn Estuary Ramsar Site is to maintain the feature in favourable condition, as defined by the conservation objective for the SPA “wintering European white-fronted goose” feature | SCREEN OUT | As above |
| Internationally important populations of waterfowl: dunlin | The conservation objective for the “dunlin” feature of the Severn Estuary Ramsar Site is to maintain the feature in favourable condition, as defined by the conservation objective for the SPA “wintering dunlin ” feature ( | SCREEN OUT | As above |
| Internationally important populations of waterfowl: redshank | The conservation objective for the “redshank” feature of the Severn Estuary Ramsar Site is to maintain the feature in favourable condition, as defined by the conservation objective for the SPA “wintering redshank” feature | SCREEN OUT | As above |
| Internationally important populations of waterfowl: shelduck | The conservation objective for the “shelduck” feature of the Severn Estuary Ramsar Site is to maintain the feature in favourable condition, as defined by the conservation objective for the SPA “wintering shelduck” feature | SCREEN OUT | As above |
| Internationally important populations of waterfowl : gadwall | The conservation objective for the “gadwall” feature of the Severn Estuary Ramsar Site is to maintain the feature in favourable condition, as defined by the conservation objective for the SPA “wintering gadwall” feature | SCREEN OUT | As above |
| Internationally important assemblage of waterfowl | The conservation objective for the “internationally important assemblage of waterfowl” feature of the Severn Estuary Ramsar Site is to maintain the feature in favourable condition, as defined by the conservation objective for the SPA “internationally important assemblage of waterfowl” feature | SCREEN OUT | As above |
| European site name: Afon Wysg / River Usk SAC |  |  |  |
| Sea lamprey | The vision for these features is for them to be in a favourable conservation status, where all of the following conditions are satisfied: - The Conservation Objective for the watercourse must be met; - The population of the feature in the SAC is stable or increasing over the long term - The natural range of the feature in the SAC is neither being reduced nor is likely to be reduced for the foreseeable future. - There is, and will probably continue to be, a sufficiently large habitat to maintain the population in the SAC on a long-term basis. | SCREEN OUT | The point of abstraction is located 300m away from the site boundary. Although these mobile species may be able to migrate upstream to the nearby unnamed tributary of the River Usk, the abstraction is from lake which is not in connectivity with nearby watercourse and, as a result, will not impact these designated features. During determination of the original licence, no conditions were included to avoid impacts to fish (e.g. screening) and no concerns have been raised regarding entrainment or entrapment of fish at the point of abstraction, therefore potential impacts can be ruled out. |
| Brook lamprey |  |  |  |
| River lamprey |  |  |  |
| Twaite shad |  |  |  |
| Allis shad |  |  |  |
| Atlantic salmon |  |  |  |
| Bullhead |  |  |  |
| European otter | The vision for this feature is for it to be in a favourable conservation status, where all of the following conditions are satisfied: - The population of otters in the SAC is stable or increasing over the long term and reflects the natural carrying capacity of the habitat within the SAC, as determined by natural levels of prey abundance and associated territorial behaviour; - The natural range of otters in the SAC is neither being reduced nor is likely to be reduced for the foreseeable future; - The safe movement and dispersal of individuals around the SAC is facilitated by the provision, where necessary, of suitable riparian habitat, and underpasses, ledges, fencing etc at road bridges and other artificial barriers. | IN COMB’ | Otter are a mobile species and survey information shows that otters are widely distributed in the Usk catchment. The point of abstraction is located 300m away from the site boundary. It should be noted that no conditions were included on the original licence to avoid impacts to otter therefore, any of the impacts identified outside of the SAC boundary which could impact on the range, supporting habitat and species, and thus population, restoration and recovery of otter, can be considered a de minimis (small scale) impact, i.e., there is an impact pathway in principle but significant effects from the proposal when considered alone can be ruled out. |
| Watercourses of plain to montane levels with the Ranunculion fluitantis and Callitricho-Batrachion vegetation | The vision for this feature is for it to be in a favourable conservation status, where all of the following conditions are satisfied: - The Conservation Objective for the watercourse must be met; - The natural range of the plant communities represented within this feature should be stable or increasing in the SAC; - The area covered by the feature within its natural range in the SAC should be stable or increasing; - More information on the typical species expected to be found with each management unit in the SAC is required. | SCREEN OUT | The point of abstraction is located 300m away from the site boundary. Abstraction is from lake which is not in connectivity with nearby watercourse and, as a result, will not impact these designated features. |

| TABLE 3.2.3 Screening decision of the plan or project ‘alone’ |  |
| --- | --- |
| (a) If the screening conclusion for all features for all sites in Table 3.2.2 is ‘SCREEN OUT’ | The plan or project is not likely to have a significant effect on any European site, and no further consideration under the Habitats Regulations is required in order to determine the approval/application. |
| (b) If the conclusion for any features in Table 3.2.2 is ‘SCREEN IN’ | The plan or project is likely to have a significant effect on one or more European sites and therefore an appropriate assessment is required. |
| (c) If there are no features in Table 3.2.2 that are ‘SCREEN IN’ and any features that are ‘IN COMB’ | The plan or project is not likely to have a significant effect on any European sites when considered alone, but the possibility of significant effects in combination with other plans and projects needs to be considered. |

# 5.	In combination assessment

| Table 5.1 Identification and assessment of in combination effects |  |  |  |  |
| --- | --- | --- | --- | --- |
| European site name: |  |  |  |  |
| ‘INCOMB’ impacts from screening Table 3.2.2 and/or residual effect from appropriate assessment in section 4 | European site feature(s) concerned | Other plans or projects with effects that might interact with the effects of the plan or project to render its effects significant/adverse (if any) | Nature of the in combination effect (if any) and assessment in view of conservation objectives | Can significant or adverse in combination effects be ruled out for all features? |
| Disturbance during ongoing maintenance requirements associated with the intake. | Otter | Application for new licences*: PAN-027616 PAN-032759 & PAN-032760 Renewal applications: PAN-030960 PAN-031382 PAN-031471 PAN-031545 PAN-032048 PAN-032194 *although classed as applications for new licences, these abstractions are already in place and operational, new licences are required due to administrative issues. | All the applications being determined (including the applications for new licences) are to facilitate existing and operational abstractions therefore any structures required to abstract are already constructed/in place so there is no risk of an in-combination effect due to disturbance caused by construction associated with these abstractions. There is an impact pathway for the on-going maintenance of these schemes, however, effects are considered to be small scale. The risk that all or multiple sites requiring maintenance at the same time is low and will have no appreciable effect on the feature. There are no valid applications awaiting determination within an immediate distance to the proposal. The proposals on the catchment are either non-consumptive or form part of the catchments baseline conditions. No concerns have been raised during the lifetime of the previous abstraction licences regarding potential impacts on to this European site, or during the renewal’s consultation process, therefore, no in combination effect considered in view of the conservation objectives. | YES |
| - If the right hand column is ‘YES’ for all rows |  | The plan or project, when considered in combination with other plans and projects, is either not likely to have a significant effect on, or will not adversely affect the integrity of, any European site. |  |  |
| - If any rows in the right hand column are ‘NO’ |  | The plan or project is likely to have a significant effect on, or an adverse effect on, European site/s in combination with other plans or projects. |  |  |

# 6.	Conclusion

| HRA is not required because there is no conceivable impact on any European sites. (As documented in section 2.1) |  |
| --- | --- |
| HRA is not required because the whole of the plan or project is directly connected with or necessary to the management of one or more European sites, for the purposes of conserving the habitats or species for which the site(s) is/are designated, and the plan or project is not likely to have a significant effect on any other European sites. (As documented in section 2.2 and 2.3) |  |
| This project is a renewal of a current permission which complies with NRW agreed criteria for ruling out likely significant effects of a renewal without conducting a project-specific LSE test. Therefore, it is considered not likely to have a significant effect on any European sites, either alone or in-combination with other plans or projects. (As documented in section 3.1 of this form) |  |
| The plan or project has been screened for likelihood of significant effects and is considered not likely to have a significant effect on any European sites. (As documented in section 3.2 of this form, and section 5 if applicable) | X |
| In light of the conclusions of an appropriate assessment it has been established that the plan or project will not adversely affect the integrity of any European sites, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans or projects. (As documented in section 4 of this form, and section 5 if applicable) |  |
| In light of the conclusions of the appropriate assessment, it has not been ascertained that the plan or project will not adversely affect the integrity of any European sites, as documented in section 4 of this form, and section 5 if applicable. Approval for the plan or project cannot be given unless either: - the plan or project specification, and/or the terms under which it might be approved, are modified so as to remove the risk of adverse effects, and a revised HRA is prepared, or - the plan or project (not being an SSSI consent*) satisfies the requirements for a derogation and a Derogation Notice is prepared and submitted for consideration by the appropriate authority, normally Welsh Ministers (*SSSI consents cannot be given as derogations) |  |
| Signed: Hannah Pearce Name: Hannah Pearce Position: Senior Permitting Officer, Water Resources Permitting Date: 11/06/2026 |  |
| Was this HRA conclusion an escalated decision? YES or NO | No |

# 7.	Consultation with the ANCB and how sections 2, 3, 4 and 5 of this HRA report (as applicable) take into account that advice.

| Relevant section of the HRA report | Correspondence and/or meetings with the ANCB | Description of how the comments from the ANCB have been taken into account |
| --- | --- | --- |
| 2 | Environment Team | Confirmed that the unnamed lake ( point of abstraction) does not appear to be connected to the nearby unnamed tributary of the River Usk. This information has been used to assess potential impacts on mobile species of both the Severn Estuary SAC & Ramsar, and the Rive Usk SAC. |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |