| # Method Statement Name: Dock Edge Earthworks |  |  |  |  |  |  |  |  |  |  |  |  | # Method Statement Number: 010 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contract: Sa1 Phase 7 |  |  |  |  |  |  |  |  |  | Contract Number: 4004 |  |  |  |  |  |  |
| Originator Name: Nathan Hunt |  |  |  |  |  |  |  |  |  | Approval Name: Barry Louth |  |  |  |  |  |  |
| Signature: |  |  |  |  |  | Date: 14.05.18 |  |  |  | Signature: Date: |  |  |  |  |  |  |
| KEY: |  | L = Likelihood |  |  |  |  | R = Risk (Likelihood x Severity) |  |  |  |  |  |  |  |  |  |
| RISK RANKING: |  | 3= Likely 2= Possible 1= Unlikely |  | S = Severity |  |  | 1 to 4 = Acceptable – Use basic control measures (PPE, supervision, tool box talks) 5 to 6 = Significant Risks, further control measures to be applied to reduce results to Acceptable 7 to 9 = Critical Risks – Not Acceptable |  |  |  |  |  |  |  |  |  |
| Ref No | Task/Element |  | 3= Fatality 2= Major Injury 1= Minor Injury Potential Hazards |  | Population at Risk |  |  | Risk Rating |  |  |  | Control Measures Specified |  | Revised Risk Ranking |  |  |
|  |  |  |  |  |  |  |  | L | S |  | R |  |  | L | S | R |
| 1 | Excavation |  | Moving plant and machinery |  | Employees Public |  |  | 2 | 3 |  | 6 | All plant to be operated by trained and competent personnel only. Excavation to be in accordance with HSG 47 Trained banksman to be in attendance while plant is working. Correct PPE to be worn. Barriers and signage to be used as agreed and in accordance with Chapter 8 |  | 1 | 3 | 3 |
| 2 | Working at heights |  | Falling into deep water |  | Employees |  |  | 2 | 3 |  | 6 | All operatives will be clipped on to safety line throughout the works. Only operatives involved with the activity will be within the work area Life buoy to be installed within work area, all operatives briefed through method statement and toolbox talks of emergency procedure Operatives to wear life jackets at all times when working No Lone working will be permitted |  | 1 | 3 | 3 |
| 3 | Working at heights |  | Plant falling into deep water |  | Employees |  |  | 2 | 3 |  | 6 | Stop blocks to be placed on dock edge before plant enter the works area. Excavator will only move perpendicular to the dock edge and the dumper will remain outside of the excavation area |  | 1 | 3 | 3 |
| 4 | Working near water |  | Falling into deep water |  | Employees |  |  | 2 | 3 |  | 6 | All operatives will be clipped on to safety line throughout the works. Only operatives involved with the activity will be within the work area Life buoy to be installed within work area, all operatives briefed through method statement and toolbox talks of emergency procedure Operatives to wear life jackets at all times when working No Lone working will be permitted Life jackets to be worn by any operative entering this area. |  | 1 | 3 | 3 |
| 5 | Working near water |  | Plant Falling into deep water |  | Employees |  |  | 2 | 3 |  | 6 | Stop blocks to be installed on existing dock edge before plant enter the work area. Excavator will only travel perpendicular to the dock edge Competent banksman to bank movement of all plant |  | 1 | 3 | 3 |
| 6 | Lifting operations |  | Materials falling from height |  | Employees |  |  | 2 | 3 |  | 6 | All materials will be lifted by a competent and experienced operator. Load will be secured by a competent person Plant and lifting equipment will be inspected prior to use and will have certificates indicating that they are safe to use Exclusion zone to be established. All operatives not involved with the activity will not be within this area Safe working loads of all lifting equipment will be established before being used Competent banksman will be used during all lifting operations |  | 1 | 2 | 2 |
| 7 | Manual Handling |  | Muscle injury from lifting heavy objects/Repetitive actions |  | Employees |  |  | 2 | 3 |  | 6 | Operatives to assess load physical capability prior to lift. Utilise mechanical lifting and carrying aids where possible. Team lifts to be employed where necessary. Operatives to be trained in kinetic method of lifting. Ensure good housekeeping standards i.e. site kept tidy/waste build-up minimized. x Operative to wear PPE against substance or material being carried as required by the COSHH assessment |  | 1 | 2 | 6 |
| 8 | Poor House Keeping |  | Trips and falls |  | Employees |  |  | 2 | 3 |  | 6 | All working areas to be kept free of debris where possible with surplus tools and materials removed. Only those involved in the working operations to be within the area. Provide suitable fencing, signing and guarding to the working area as necessary. |  | 1 | 3 | 3 |
| 9 | Vibrating Poker |  | HAV (Hand Arm Vibration) |  | Employees |  |  | 2 | 3 |  | 6 | Exposure not to exceed manufacturer’s recommendation amounts. Record to be kept on site of all workers vibration exposure. Where possible the job shall be shared between the workers. |  | 1 | 2 | 2 |
| 10 | Concrete |  | Workers coming into contact with concrete. Can be skin contact, in eyes etc. |  | Employees |  |  | 2 | 3 |  | 6 | All persons working with the concrete shall wear safety glasses, gloves and rubber boots to help reduce the exposure to concrete. Eye wash station to be established in welfare unit. |  | 1 | 3 | 3 |
| 11 | Moving Plant |  | Workers being struck by moving plant/machinery |  | Employees |  |  | 3 | 3 |  | 9 | Only persons to be in the works area are those involved with the works. Certified banksman will be directing machine driver and concrete lorry driver. All other workers in the area are to do as instructed by the banksman. Works area to be clearly defined with banksman directing traffic in and out of area |  | 1 | 2 | 2 |