# Form S1

RELEASES TO WATER (SEWER)

REPORTING OF MONITORING DATA

SUMMARY FOR THE SIX MONTH PERIOD ENDING June 2019

Operator	Tradebe Limited

Location	Bridgend Waste Management Centre, Factory Lane, Pencoed, Bridgend, CF35 5BQ

Permit Number	NP3233XX

Release Point: Final Effluent Discharge Point S1

| Table 1.S1: Release to water(Sewer) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter |  | Result(mg/l) Batch 1 | Batch 2 | Batch 3 | Batch 4 | Batch 5 |
| Suspended solids (Limit = 1700mg/l) |  | Result: 87 Date: 11/01/2019 | Result: 154 Date: 20/02/2019 | Result: 78 Date: 25/03/2019 | Result: 511 Date: 08/04/2019 | Result: 78 Date: 21/06/2019 |
| Oil & Grease (limit – 100mg/l |  | Result: <1 Date: 11/01/2019 | Result: <1 Date: 20/02/2019 | Result: <1 Date: 25/03/2019 | Result: <1 Date: 08/04/2019 | Result: <1 Date: 21/06/2019 |
| Total Sulphate (Limit = 1000 mg.l) |  | Result: 43.63 Date: 11/01/2019 | Result: 191 Date: 20/02/2019 | Result: 142.3 Date: 25/03/2019 | Result: 472 Date: 08/04/2019 | Result: 145 Date: 21/06/2019 |
| Total NH3-N (Limit = 500mg/l) |  | Result: 1207 Date: 11/01/2019 | Result: 1088 Date: 20/02/2019 | Result: 1394 Date: 25/03/2019 | Result: 1097 Date: 08/04/2019 | Result: 1462 Date: 21/06/2019 |
| Total Chromium (Limit = 2mg/l) |  | Result: 0.20 Date: 11/01/2019 | Result: 0.27 Date: 20/02/2019 | Result: 0.3 Date: 25/03/2019 | Result: 0.23 Date: 08/04/2019 | Result: 0.32 Date: 21/06/2019 |
| Total Copper (Limit = 2 mg/l) |  | Result: 0.09 Date: 11/01/2019 | Result: 0.06 Date: 20/02/2019 | Result: 0.06 Date: 25/03/2019 | Result: 0.08 Date: 08/04/2019 | Result: 0.06 Date: 21/06/2019 |
| Total Lead (Limit = 2 mg/l) |  | Result: 0.01 Date: 11/01/2019 | Result: 0.06 Date: 20/02/2019 | Result: N/D Date: 25/03/2019 | Result: 0.05 Date: 08/04/2019 | Result: <0.01 Date: 21/06/2019 |
| Total Nickel (Limit = 2 mg/l) |  | Result: 0.15 Date: 11/01/2019 | Result: 0.16 Date: 20/02/2019 | Result: 0.19 Date: 25/03/2019 | Result: 0.25 Date: 08/04/2019 | Result: 0.19 Date: 21/06/2019 |
| Total Zinc (Zn) (Limit = 2 mg/l) |  | Result: 0.14 Date: 11/01/2019 | Result: 0.18 Date: 20/02/2019 | Result: 0.07 Date: 25/03/2019 | Result: 0.18 Date: 08/04/2019 | Result: 0.08 Date: 21/06/2019 |
| Total Cyanide (CN) (Limit = 2 mg/l) |  | Result: N/D Date: 11/01/2019 | Result: N/D Date: 20/02/2019 | Result: N/D Date: 25/03/2019 | Result: N/D Date: 08/04/2019 | Result: N/D Date: 21/06/2019 |
| Total Sulphide (Limit = 3 mg/l) |  | Result: 1.75 Date: 11/01/2019 | Result: 0.53 Date: 20/02/2019 | Result: 0.55 Date: 25/03/2019 | Result: 0.49 Date: 08/04/2019 | Result: 0.48 Date: 21/06/2019 |
| Total Monohydric Phenols (Limit = 100 mg/l) |  | Result: 7.39 Date: 11/01/2019 | Result: 7.62 Date: 20/02/2019 | Result: 8.21 Date: 25/03/2019 | Result: 6.18 Date: 08/04/2019 | Result: 7.23 Date: 21/06/2019 |
| Total Monoxol OT (Limit = 30mg/l) |  | Result: N/D Date: 11/01/2019 | Result: N/D Date: 20/02/2019 | Result: N/D Date: 25/03/2019 | Result: N/D Date: 08/04/2019 | Result: N/D Date: 21/06/2019 |
| PH (Limit = 6.0 – 10.0) | Max Min | 8.4 6.86 |  |  |  |  |
| Temperature (Limit = 43 c) | Max | 28.9 |  |  |  |  |
|  | Min | 5.5 |  |  |  |  |
| Flow Rate l/s (Limit = 10l/s) | Max | 8 |  |  |  |  |
|  | Min | 0 |  |  |  |  |
| Daily Flow M3 (Limit = 1400 m3) | Max | 185 |  |  |  |  |
|  | Min | 0 |  |  |  |  |

| Week ending: | Parameters |  |  |
| --- | --- | --- | --- |
| 31/03/2019 | COD (limit 2500kg/day) | Total NH3-N (Limit 280kg/day) | Monohydric Phenols (Daily Increase, 7kg/day) |
| Monday | 411.74 | 241.16 | <1kg |
| Tuesday | 176.22 | 131.63 | <1kg |
| Wednesday | 234.30 | 168.30 | <1kg |
| Thursday | 371.70 | 170.81 | <1kg |
| Friday | 285.60 | 161.84 | <1kg |
| Saturday | 217.14 | 74.61 |  |
| Sunday |  |  |  |

Signed on behalf of the Operator:

Andrew Price

Dated:         30/07/2019