| Cardigan Tidal Flood Risk Management Scheme Environmental Statement Appendix 8.1 - Landscape and Visual Amenity Impact Assessment Methodology |  |  |
| --- | --- | --- |
| Client project no. CE0587 BUKL project no. 4021883 |  |  |
| Prepared for: Natural Resources Wales (NRW) October 2026 |  |  |
| Revision P04 |  |  |

Table of contents

APPENDIX 8.1: Landscape and Visual Amenity Impact Assessment Methodology	2

1.1	Introduction	2

1.2	Establishing the study area	2

1.3	Scoping	2

1.4	Establishing the baseline	3

1.5	Visual aids	4

1.6	Landscape effects	5

1.7	Visual effects	7

1.8	Magnitude of landscape and visual change	9

1.9	Significance of landscape and visual effects	10

1.10	Positive/negative	11

1.11	Cumulative effects	11

1.12	Glossary of terms	13

Details of document preparation and issue:

| Version | Prepared | Checked | Reviewed | Approved | Issue date | Status |
| --- | --- | --- | --- | --- | --- | --- |
| P01 | T. Dendy | H. Goodrick | M. Boothroyd | A. Humphreys | 18/12/2025 | D |
| P02 | H. Goodrick | H. Rowell | M. Boothroyd | A. Humphreys | 18/03/2026 | S3 |
| P03 | H. Goodrick | H. Rowell | M. Boothroyd | L. McCarthy | 27/03/2026 | S2 |
| P04 | H. Goodrick | M. Ellins | M. Boothroyd | A. Humphreys | 09/10/2026 | S5 |

| BUKL project no. 40218831883 Client’s reference no. CE0587 |  |  |  |
| --- | --- | --- | --- |
| File name: | 4021883-BUK-ZZ-00-RP-L-00002 |  |  |

Notice: This report was prepared by Binnies UK Limited (BUKL) solely for use by Natural Resources Wales (NRW). This report is not addressed to and may not be relied upon by any person or entity other than Natural Resources Wales (NRW) for any purpose without the prior written permission of BUKL. BUKL, its directors, employees and affiliated companies accept no responsibility or liability for reliance upon or use of this report (whether or not permitted) other than by Natural Resources Wales (NRW) for the purposes for which it was originally commissioned and prepared. In producing this report, BUKL has relied upon information provided by others. The completeness or accuracy of this information is not guaranteed by BUKL.

# APPENDIX 8.1: Landscape and Visual Amenity Impact Assessment Methodology

## Introduction

”Landscape and Visual Impact Assessment is a tool used to identify and assess the significance of and the effects of change resulting from development on both the landscape as an environmental resource in its own right and people’s views and visual amenity.” (Guidelines for Landscape and Visual Impact Assessment (Third Edition) (GLVIA3), paragraph 1.1). Wherever possible, identified effects are quantified, but the nature of landscape and visual assessment requires interpretation using professional judgement.  In order to provide a level of consistency to the assessment, the prediction of magnitude and assessment of significance of the residual landscape and visual effects have been based on pre-defined criteria.

GLVIA3 states that “professional judgement is a very important part of the LVIA” (paragraph 2.23) and that “in all cases there is a need for the judgements that are made to be reasonable and based on clear and transparent methods so that the reasoning applied at different stages can be traced and examined by others.” (paragraph 2.24).  It continues at paragraph 3.32 to state that “there are no hard and fast rules about what effects should be deemed ‘significant’ but LVIAs should always distinguish clearly between what are considered to be the significant and non-significant effects.”

Landscape and Visual Assessments are separate, though linked processes which GLVIA3 notes are “related but very different considerations”.  The assessment of the potential effect on the landscape is carried out as an effect on the environmental resource (i.e. the landscape).  Visual effects are assessed as an inter-related effect on people.

Landscape effects derive from changes in the physical landscape elements which may give rise to changes in its distinctive character and how this is experienced, including consideration of aesthetic and perceptual aspects.

Visual effects relate to changes that arise in the composition of available views as a result of changes to the landscape, to people’s responses to the changes and to the overall effects with respect to visual amenity.

## Establishing the study area

The study area has been established using guidance in GLVIA3 and NRW guidance note 46 (GN46): Using LANDMAP in Landscape and Visual Impact Assessments (NRW, updated March 2023). An initial search area was first set at 3km, as although the Scheme is of low vertical elevation, there is surrounding high ground from which the Scheme could be visible. A Zone of Theoretical Visibility (ZTV) was undertaken for this 3km search area, which shows where the Scheme would theoretically be visible from. The study area was then derived from the landscape character areas falling within (or partly within) the 3km search area, and which have intervisibility with the Scheme as shown on the ZTV (see Figure 8.2).

## Scoping

For Landscape Character, in the absence of a published landscape character assessment at a local level, the study area is divided into appropriately-scaled Landscape Character Areas (LCAs) by the Landscape Architect. Each LCA has its own separate defining characteristics which make up the fabric of the landscape and set them apart from one another. These LCAs were defined using desktop study including analysis of LANDMAP and other data sources, and verified on site by the Landscape Architect. They are shown on Figure 8.1. In line with GN46, as the development is of limited vertical elevation the search area was initially set at a 3km buffer from the proposed development. Character areas identified within the 3km buffer may extend beyond the buffer line, however no new character areas are identified which begin beyond the 3km buffer.

Of the LCAs identified, an initial scoping exercise has been undertaken to identify which of the LCAs may experience significant effects to their character as a result of the proposed development. This is because GLVIA3 states that, in line with a proportionate approach, LVIA should focus on receptors which may experience significant effects.

National Landscape Character Assessments and Marine Character Assessments published by NRW have been used to inform the baseline but are considered to cover too broad an area on which to assess the likely effects of the proposed development as receptors in their own right.

Potential receptors of visual amenity have been previously identified (see LVIA Scope: Scope for a landscape and visual impact assessment for Cardigan TFRMS, Binnies, 2024) and refined through consultation with the Local Planning Authority (email from Sarah Smyth, 14/09/23). The ZTV was used to identify the groups of people whose views may be affected by the proposed development.

## Establishing the baseline

The baseline for consideration of landscape and visual effects is evaluated through desk study (including LANDMAP analysis, landscape character assessments and ZTV analysis) and a site visit by the Landscape Architect and is the current situation at the time of the assessment, unless noted otherwise.  Operational developments and those under construction are considered as part of the baseline and included as part of the assessment of landscape and visual effects.

LANDMAP is a tool describing the Welsh landscape baseline, aimed at helping decision making and natural resource planning. LANDMAP is not a character assessment itself but is a collection of five aspect area datasets which describe the characteristics of the landscape at a local level, evaluate their importance and recommend locally appropriate management guidelines. The five aspects are: geological landscape; landscape habitats; visual and sensory; historic landscape; and cultural heritage.

The future baseline is considered to be changes to the landscape which are considered certain or likely to happen – including consented proposals which are not yet present in the landscape but are expected to be constructed.  These may or may not be included as part of the landscape and visual baseline depending on individual project circumstances and the approach and reasoning is set out within the assessment.

A key part of the visual amenity baseline is the establishment of the ZTV, which shows the areas from which the development is theoretically visible, and therefore helps identify the ‘visual receptors’, i.e. the people who may have views of the proposed development, as well as aiding viewpoint selection and indicating the development’s potential influence on the wider landscape.

The ZTV was mapped by application of Viewshed software on a 1m Digital Surface Model dataset, a Lidar dataset that includes locations and heights of buildings and vegetation as well as landform. The height of the development was set at 4.95m AOD and the observer height was set at 1.7m. On the ZTV figure 8.2 areas with theoretical visibility are shaded to indicate how much of the Scheme is visible from any given location. The presence of any colour means that part of the Scheme is visible; the darker the shade, the more of the Scheme is visible. To generate this shading, a series of points were placed along the top of the proposed flood defence wall in the Model, and the visibility shaded depending on the percentage of the points visible from each location.

It should be noted however that the ZTV shows only whether the proposed development will theoretically be visible; it does not show how easily noticeable or identifiable it is likely to be. In line with GLVIA, the ZTV is supplemented by a site visit by a Landscape Architect, in order to verify the ZTV and establish the extent of visibility.

The ZTV was used to identify the groups of people whose views may be affected by the proposed development, and to locate viewpoints to represent the visual amenity receptors. Photographs showing views from a number of representative viewpoints are used to aid the visual appraisal, and all photographic viewpoints are located within areas shown to have visibility in the ZTV. Viewpoint locations are selected following production of the ZTV and desktop study exercises and refined through the site visit.

Viewpoints frequented by members of the public, such as from public rights of way (PRoW) or settlements, as well as viewpoints from which the proposed development is likely to be prominent, are favoured as there are likely to represent a greater concentration of potentially affected users.

## Visual aids

A photographic record was captured by the Landscape Architect during the site visit to aid in the presentation of the LVIA. One visit was undertaken to collect photographs, during winter only.

The Landscape Institute published the Landscape Institute Technical Guidance Note (TGN) 06/19: Visual Representation of development proposals in September 2019 (under review from January 2024). TGN 06/19 has been used to inform this methodology, which provides guidance on appropriate photography and visualisation techniques for use within LVIA and LVA, and which itself accords with the principles of GLVIA. Both GLVIA and TGN 06/19 advocate a proportionate approach based on professional judgement.

Baseline photographs as well as Type 1 visualisations (annotated viewpoint photographs) are used to inform the appraisal process. TGN 06/19 describes that Type 1 visualisations are appropriate for use in LVIA to show the extent and position of the site and features. It is not considered that photomontage visualisations (either Type 3 or 4) or 3D wireline/model (Type 2) are proportionate. This is due to the limited nature and scale of the proposed works.

Viewpoints have been selected to show representative views experienced by identified receptors. Viewpoint locations were selected using the ZTV and desk-based study and are shown on Figure 8.3 LVIA Viewpoints. Locations were all publicly accessible and agreed in advance with the Local Planning Authority.  Locations were refined and finalised on site. Some locations were chosen as much to show where the site is not visible.

A Panasonic DMC-G5 Compact System Camera with a 25mm fixed lens was used to take the baseline photographs. The DMC-G5 has a sensor size of 17.3 x 13.0mm, giving a horizontal field of view of 38.2° when the 25mm fixed lens is used. This gives a similar horizontal field of view to a full frame sensor camera with a 50mm lens (FFS+50mm). In line with a proportionate approach, TGN 06/19 advises that a FFS+50mm camera and lens combination are not required when taking photographs to be used in Type 1 annotated viewpoint photographs. The camera was raised on a tripod set to take an image at 1.5m.

Whilst the camera height was set at 1.5m to be representative of most viewers, the viewer eye height used in the ZTV was set at 1.7m. 1.7m is the upper end of the range suggested in GLVIA3 and therefore represents the maximum extents of where the Scheme would be theoretically visible from.

As recommended by TGN 06/19, single frame annotated images are presented on A3 sheets (one image per A3 sheet), in the Photographic Report, Appendix 8.3. All images shown are full frame images; no images have been cropped. The viewpoint locations and directions of view are shown on Figure 8.3.

## Landscape effects

The starting point for any assessment is a desk-based assessment of published landscape studies, which may include landscape character assessments, sensitivity and capacity studies and/or landscape designation reviews. These documents are listed in the assessment references.

The landscape effects of the proposed development are considered against the key characteristics of the receiving landscape. The degree to which the proposed development changes the “distinct and recognisable pattern of elements, or characteristics, in the landscape that make one landscape different from another, rather than better or worse”’ (‘An Approach to Landscape Character Assessment’, Natural England, 2014), enables a judgement to be made as to the significance of the effect in landscape character terms.

Direct and indirect landscape effects are defined in GLVIA3.  Direct effects may be defined as resulting “directly from the development itself” (paragraph 3.22).  An indirect (or secondary) effect is one that results “from consequential change resulting from the development” (paragraph 3.22) and is often produced away from the site of the proposed development or as a result of a complex pathway or secondary association.  The direct or physical landscape effects of the proposed development would generally be limited to within the planning application boundary.  The indirect landscape effects are concerned with the visual effects and relate to effects associated with the introduction of the development seen in the context of the existing landscape and visual character of the view.

In order to reach an understanding of the effects of development upon the landscape resource it is necessary to consider different aspects of the landscape baseline including:

Landscape Fabric/Elements: The individual features of the landscape, such as hills, valleys, woods, hedges, tree cover, vegetation, buildings and roads for example which can usually be described and quantified.

Landscape key characteristics: The particularly notable elements or combinations of elements which make a particular contribution to defining or describing the character of an area, which may include experiential characteristics such as wildness and tranquility.

The sensitivity (high, medium, low) of the landscape to a particular development is considered on a case by case basis and considers the susceptibility of the landscape, which varies depending on the type of development proposed and the particular site location, and the landscape value (identified as national, regional, or community). As stated in GLVIA3, ‘LVIA sensitivity is similar to the concept of landscape sensitivity used in the wider arena of landscape planning, but is not the same’.

Landscape value: The importance attached to a landscape, often used as a basis for designation or recognition which expresses national or local authority consensus, because of its special qualities/attributes. The factors which are considered in landscape include aesthetic or perceptual aspects such as scenic beauty, tranquility or wildness or cultural associations as well as recreational/community value, conservation interests, landscape character and condition and representativeness/rarity.

Landscape susceptibility according to GLVIA3 means “the ability of the landscape to accommodate the proposed Development without undue consequences for maintenance of the baseline situation and/or the achievement of landscape planning policies and strategies”.  In LITGN-2024-01 published by the Landscape Institute in August 2024 (Notes and Clarifications on aspects of the 3rd Edition of Guidelines on Landscape and Visual Impact Assessment), the Landscape Institute clarifies that susceptibility means the ability of the landscape to accommodate the type or nature of the change and should not be confused with assessments of magnitude. Judgements on landscape susceptibility (high, medium, low) include references to both the physical and aesthetic characteristics of the type of proposed development, the scale of the change, and landscape features and characteristics that affect the landscape’s ability to accommodate the change.

Susceptibility of landscape character areas is influenced by their characteristics and are often considered (though often recorded as ‘sensitivity’ rather than susceptibility) within landscape character assessments and capacity studies.

Susceptibility of designated landscapes is influenced by the nature of the special qualities and purposes of designation and/or the valued elements, qualities or characteristics, indicating the degree to which these may be unduly affected by the development proposed.

The criteria and the detailed judgements regarding susceptibility and value of landscape receptors are identified within the sensitivity tables included within this assessment.

Sensitivity is judged taking into account the component judgments about the value and susceptibility of the receptor as illustrated by the table below.

|  |  | Susceptibility |  |  |
| --- | --- | --- | --- | --- |
|  |  | High | Medium | Low |
| Value | National | High | High-Medium | Medium |
|  | Regional | High-Medium | Medium | Medium-Low |
|  | Community | Medium | Medium-Low | Low |

The magnitude of landscape change arising from the proposed development at any particular location is assessed in terms of its size or scale, geographic extent of the area or receptor that is influenced and its duration and reversibility.

The scale of the change takes account of:

degree of loss or alteration to key landscape features/elements; characteristics; and for designated areas – special qualities and/or purposes of designation;

distance from the development;

landscape context to the development.

The approach to assessing effects on landscape character is to consider the key characteristics for the LCA within which the proposed development is located (host) and the adjacent LCAs (non-host) and identify which of these the proposed development would affect.  For the host LCAs, a large scale change in landscape character is likely to occur where key characteristics would be lost or substantially changed.  Where particular views are a key characteristic of a landscape area, large or medium scale landscape character effects may occur where the proposed development becomes a key feature of those views.

Where multiple landscape assessments of different scales cover the same geographical area, the effects are assessed on the assessment at the most appropriate scale.  In this case there is no published Landscape Character Assessment of a suitable scale.  Therefore, LCAs have been drawn up at a suitable scale within the study area for this assessment.  The resulting LCAs have then been scoped in or out of the LVIA as receptors to change, based on the likelihood of significant effects on the key characteristics.

Having established the size/scale of change (large, medium, small, negligible) to the landscape baseline, the geographic extent of the change is identified (wide, intermediate, localised or limited) and a judgement made as to the degree of change for each landscape receptor.

Duration and reversibility can be linked depending on the nature of the development. Reversibility is a judgement about the ability and practicality of the proposed development to be reversible (such as wind farms which are predominantly reversible), partially reversible to something similar (such as mineral extraction) or a permanent change in the landscape (such as housing). Duration reflects how long the change will last. The following definitions of duration have been applied in this assessment:

| - Duration | - Definition / Description |
| --- | --- |
| - Short term | - Effect continues during construction and up to 1 year following construction |
| - Medium Term | - Effect continues 1 to 5 years following construction |
| - Long term | - Effect continues between 5 and 10 years following construction |
| - Permanent | - Due to the subjectivity of human perception of timeframes, those effects that continue for greater than 10 years following construction can be defined as permanent. |

Magnitude is considered taking into account the three contributory factors as illustrated by the diagrams included below, see section 1.8.

## Visual effects

In order to identify the significance of a visual effect it is necessary to establish the relative sensitivity of the viewers (people) and the magnitude of the change they experience.  In this case sensitivity is a combination of both susceptibility of the viewer to the proposed change and the value of the views.

Those living within view of the proposed development are usually regarded as the highest susceptibility group as well as those engaged in outdoor pursuits for whom landscape experience is the primary objective.  The susceptibility of potential visual receptors will also vary depending on the activity of the receptor.  For visual receptors susceptibility and value are closely linked - the most valued views are also likely to be those where viewer’s expectations will be highest.

The value of public views, which is the focus of GLVIA3, is identified as national, regional or community and will vary depending on the nature, location and context of the view and the recognised importance of the view.  Considerations include cultural associations; designation or policy protection; views of or from landmarks; and/or the scenic quality of the view. The value attributed relates to the value of the view, e.g. a National Trail is nationally valued for access, but not always for the available views from every section.

Visual receptor susceptibility is defined as in accordance with the criteria below.

High - Local residents; users of outdoor recreation focussed on the appreciation of views including footpaths, beauty spots and picnic areas; people experiencing views to or from important features of physical, visual, cultural or historic interest.

Medium - Local road users and travellers on trains. People engaged in outdoor recreation with some appreciation of the landscape e.g. road cycling, nature conservation, golf and water based recreation.

Low - Users of facilities and commercial buildings (indoors) experiencing views from buildings. Workers both indoors and outside, as they are not engaged in an activity related to enjoyment of the view. Road and rail users on fast moving commuting or trunk routes.  Visual receptors where views are incidental to the activity and/or location.

Sensitivity is judged taking into account the component judgments about the value and susceptibility of the receptor as illustrated by the table below.

|  |  | Susceptibility |  |  |
| --- | --- | --- | --- | --- |
|  |  | High | Medium | Low |
| Value | National | High | High-Medium | Medium |
|  | Regional | High-Medium | High-Medium | Medium-Low |
|  | Community | High-Medium | Medium | Low |

The magnitude of visual change arising from the proposed development at any particular location is assessed in terms of its size or scale (large, medium, small, negligible), geographic extent of the area or receptor that is influenced (wide, intermediate, localised, limited) and its duration (short, medium, long, permanent).

Representative viewpoints are used as ‘samples’ on which to base judgements of the scale of effects on visual receptors. The wider extent of the effect and its duration are not captured in the viewpoint analysis (as a viewpoint cannot capture these factors for an entire route or area). As duration and extent are necessary considerations in determining magnitude of change; magnitude and significance judgements are provided for visual receptors and not for all representative viewpoints. The exceptions to this are specific viewpoints – where people visiting that location to look at the view are assessed as a visual receptor group.

With the exception of specific viewpoints, each route and receptor group will encompass a range of possible views, which might vary from no view of the development to very clear, close views. Therefore, effects are described in such a way as to identify where views towards the development are likely to arise and what the scale and duration and extent (wide, intermediate, localised, limited) of those views are likely to be. In some cases this is further informed by a nearby viewpoint and in others it will be informed with reference to ZTV studies, aerial photography and site visits. These individual effects are then considered together in order to reach a judgement of the effects on the visual receptors along that route, or in that place.

The scale of effect arising from the proposed development at any particular viewpoint reflects the degree to which the nature of the views from that location would be changed and is taking into account:

The distance of the viewpoint from the development;

the degree to which the development is visible or screened;

the angle of view in relation to main receptor activity or main focus of the view;

the horizontal and vertical field of view occupied by the development; and

the extent and nature of other built development visible.

The approach to assessing effects on views is to assume that the change would be seen in clear visibility and the assessment is carried out on that basis. Where there are operational (and consented) developments considered as part of the baseline, the visual effects consider the effects of adding the proposed development to that baseline.

Duration reflects how long the change will last and is rated in the same way as described above for landscape effects (see section 1.6).

For visual receptors moving through the landscape (e.g. road and rail users), the length of their journey during which they would see the development is reflected in the judgement of the geographic extent of effects.

Magnitude is considered taking into account the three contributory factors as illustrated by the diagrams included below in section 1.8.

## Magnitude of landscape and visual change

Scale of effect is the first factor in determining magnitude; which may be higher if the effect is particularly widespread and/or long lasting, or lower if it is constrained in geographic extent and/or timescale. The tables below illustrate how this judgement is considered as a two-step process. Firstly, scale and extent are combined to produce an initial ‘Stage 1’ magnitude judgement. Duration is then applied to determine the final magnitude of change.

## Significance of landscape and visual effects

The significance of any identified landscape or visual effect is assessed as major, moderate, minor or negligible. These categories are based on the consideration of sensitivity with the predicted magnitude of change.  The table below is not used as a prescriptive tool and illustrates the typical outcomes, allowing for the exercise of professional judgement. In some instances a particular parameter may be considered as having a determining effect on the analysis.

|  |  | Magnitude of Change |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Substantial | Moderate | Slight | Negligible |
| Receptor Sensitivity | High | Major | Major-Moderate | Moderate | Minor |
|  | Medium | Major-Moderate | Moderate | Moderate-Minor | Minor-Negligible |
|  | Low | Moderate | Moderate-Minor | Minor | Negligible |

Where the effect has been classified as Major or Major/Moderate this is considered to be equivalent to likely significant effects referred to in the EIA Regulations. Moderate effects could be significant; where ‘Moderate’ effects are predicted, professional judgement is applied to ensure that the potential for significant effects arising has been thoroughly considered. Where an effect falls at Moderate, the judgement on EIA significance will take account of the combination of receptor sensitivity and magnitude and the underlying considerations which informed those judgements; the scale and nature of change, geographic extent, duration/reversibility, and the importance or susceptibility of the receptor. Moderate-Minor effects are not considered to be significant.

## Positive/negative

Landscape and visual effects can be positive or negative and in some instances may be considered neutral. Neutral effects are those which overall are neither negative nor positive but incorporate a combination of both. To be judged neutral overall, the effect experienced must contain an equal balance of positive and negative impacts. If a receptor is judged to experience both positive and negative impacts which are not considered to be equally balanced, the overall effect will be positive or negative, whichever is greater.

The assessment of whether an effect is positive or negative is independent of magnitude of change and therefore significance of effect; the judgement of significance is made first before the judgement as to whether the change is negative or positive. As such, it is a possible occurrence that even though an effect may be judged to be significant in EIA terms, the overall outcome is judged to be neutral because positive and negative components of the change are considered to be balanced.

In line with LITGN-2024-01, whether an effect is positive, neutral or negative takes into account how the design responds to its environment and whether it is appropriate in the context of its surroundings, and is not based on the assessor’s judgement of ‘beauty’. The assessment includes justification as to why the effect is considered to be negative, positive or neutral. GLVIA3 indicates at paragraph 2.15 that this is a “particularly challenging” aspect of assessment, especially in the context of a changing landscape.

## Cumulative effects

In a broad generic sense, cumulative impacts “result from the incremental changes caused by other past, present or reasonably foreseeable actions together with the project”’  However, an assessment of cumulative effects should focus on whether there are any potential cumulative impacts which are reasonably foreseeable and which are likely to influence the decision making of the proposed development, rather than an assessment of every potential cumulative effect, which in practice means focussing on other nearby development proposals and the effects that might arise from the combined influence of those developments on landscape and visual receptors.

As recommended by the NatureScot cumulative guidance, this assessment focusses on the “additional cumulative change which would be brought about by the proposed development”.

As noted above, operational developments are included in the baseline, consented development which are expected to be constructed, form part of the future baseline and will be included as such. However, where there is some uncertainty regarding the future construction of consented developments, they may be considered as the first scenario of the cumulative assessment.

Proposals in planning are considered where there is good reason to assume that the timing of decisions may be similar and significant cumulative effects are likely. The assessment of effects is considered within the cumulative assessment.

The assessment is based on the same landscape and visual baseline and receptor groups as the main LVIA, and the methodology is also the same in terms of forming and expressing judgements.

Cumulative effects on landscape receptors arise from combined direct and/or indirect effects on the same receptor – such as two developments within the same character area; or one development within, and one visible from, a designated area.

Cumulative effects on visual receptors arise either from two (or more) developments both being visible from the same place; or from sequential views as people travel.

In order to simplify what may otherwise be a complex assessment, the following approaches are also used:

The cumulative assessment considers scenarios within which developments may be ‘grouped’ - for instance two nearby cumulative proposals may be considered in one scenario if it is considered that the cumulative effects arising if one or both are developed are likely to be similar.

Receptors judged to receive negligible or slight-negligible magnitude of change are not considered for cumulative effects on the basis that any significant effects arising would primarily be caused by the cumulative developments and would be unlikely to be contributed to by the proposed development.

Only those receptors judged likely to experience effects from the cumulative development(s) being considered within a given scenario are described within that scenario.

Qualitative assessment of design and aesthetic considerations arising as a result of cumulative development, and/or considerations set out within local guidance provided in relation to cumulative development, is also provided where relevant.

## Glossary of terms

| Term | Definition |
| --- | --- |
| CLVIA | Cumulative Landscape and Visual Impact Assessment. |
| Cumulative Effects | Cumulative effects are the additional effects arising from changes caused by a development in conjunction with other past, present or reasonably foreseeable actions. |
| Direct Effect | A direct (or primary) effect may be defined as an effect that is directly attributable to the development. |
| GLVIA3 | ‘Guidelines for Landscape and Visual Impact Assessment, Third Edition’, published jointly by the Landscape Institute and Institute of Environmental Management and Assessment 2013. |
| Indirect Effect | An indirect (or secondary) effect is an effect that results indirectly from the proposed project as a consequence of the direct effect, often occurring away from the site, or as a result of a sequence of interrelationships or a complex pathway. They may be separated by distance or in time from the source of the effects. |
| Key Characteristics | Those combinations of elements which are particularly important to the current character of the landscape and help to give an area its particularly distinctive sense of place. |
| LVIA | Landscape and Visual Impact Assessment. |
| Landscape Capacity | The amount of change which a particular landscape character type or area is able to accommodate without significant detrimental effects on its character. Capacity is likely to vary according to the type and nature of change proposed. |
| Landscape Character | The distinct and recognisable pattern of elements in the landscape that makes one landscape different from another, rather than better or worse. |
| Landscape Character Areas | These are single unique areas which are the discrete geographical areas of a particular landscape type. |
| Landscape Character Types | These are distinct types of landscape that are relatively homogeneous in character. They are generic in nature in that they may occur in different areas in different parts of the country, but wherever they occur, they share broadly similar combinations of geology, topography, drainage patterns, vegetation and historical land use and settlement pattern, and perceptual and aesthetic attributes. |
| Landscape Effects | Effects on the landscape as a resource in its own right. |
| Landscape Elements | Individual components which make up the landscape such as trees and hedges. |
| Landscape Features | Particularly prominent or eye-catching elements, like tree clumps, church towers or wooded skylines. |
| Landscape Quality or Condition | This is a measure of the physical state of the landscape. It may include the extent to which a typical character is represented in individual areas, the intactness of the landscape and the condition of individual elements. |
| Landscape Receptor | Defined aspects of the landscape resource that have the potential to be affected by a proposal. |
| Landscape Resource | The combination of elements that contribute to landscape context, character and value. |
| Landscape Value | The relative value or importance attached to different landscapes by society on account of their landscape qualities. |
| Level of Effect | Determined through the combination of sensitivity of the receptor and the proposed magnitude of change brought about by the development. |
| Magnitude (of effect) | A term that combines judgements about the size and scale of the effect, the extent of the area over which it occurs, whether it is reversible or irreversible and whether it is short or long term in duration. |
| Mitigation | Measures including any process, activity or design to avoid, reduce, remedy or compensate for negative environmental impact or effects of a development. |
| Photomontage | A visualisation which superimposes an image of a proposed development upon a photograph or series of photographs. |
| Residential Visual Amenity | A collective term describing the views and visual amenity from a residential property, relating to the type, nature, extent and quality of views that may be experienced from the property and its ‘domestic curtilage’ including gardens and access driveway. Residential Visual Amenity is only one component of the overall Residential Amenity, others being for example noise, shadow flicker and access amongst others. |
| Residual Effects | Potential environmental effects remaining after mitigation. |
| Sense of Place | The essential character and spirit of an area: genius loci literally means ‘spirit of the place’. |
| Sensitivity | A term applied to specific receptors, combining judgements of the susceptibility of the receptor to the specific type of change or development proposed and the value related to that receptor. |
| Significant Effects | It is a requirement of the EIA Regulations to determine the likely significant effects of development on the environment which should relate to the level of an effect and the type of effect. Where possible significant effects should be mitigated. The significance of an effect gives an indication as to the degree of importance (based on the magnitude of the effect and sensitivity of the receptor) that should be attached to the impact described. Whether an effect should be considered significant is not absolute and requires the application of professional judgement. |
| Type or Nature of Effect | Whether an effect is direct, indirect, temporary or permanent, positive (positive), neutral or negative (negative) or cumulative. |
| Visual amenity | Value of a particular place in terms of what is seen by visual receptors taking account of all available views and the total visual experience. |
| Visual Effect | Effects on specific views and on the general visual amenity experienced by people. |
| Visual Receptors | Individuals and/or defined groups of people who have the potential to be affected by a proposal. |
| Visualisation | Computer simulation, photomontage or other technique to illustrate the appearance of a development. |
| Wildness | A quality of appearing to be remote, inaccessible and rugged with little evidence of human influence. |
| Wireframe or Wireline | A computer generated line drawing of the DTM (Digital Terrain Model) and the proposed development from a known location. |
| Zone of Theoretical Visibility (ZTV) | Area within which a proposed development may have an influence or an effect on visual amenity. |

GLVIA3 page 91, paragraph 5.52

GLVIA3 page 120, paragraph 7.1 quoting Hyder, 1999 ‘ Guidelines for the assessment of indirect and cumulative impacts as well as impact interactions’

GLVIA3 page 121 paragraph 7.5.

Assessing the Cumulative Impact of Onshore Wind Energy Developments, NatureScot, 2021

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p155

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p156

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p156

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p157

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p157

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p157

The Landscape Institute; Technical Guidance Note 02/21 Assessing Landscape Value Outside National Designations

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p157

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p158

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p158

The Landscape Institute/Institute of Environmental Management and Assessment; Guidelines for Landscape and Visual Impact Assessment; Spon; 2013; p158