Natural Resources Wales

Chester Road

Buckley

Flintshire

CH7 3AJ

Attention of: Siân McGregor-Andrew – Regulatory Officer, Industry Regulation Team

28th October 2019

Dear Ms McGregor-Andrew

Re: Quarter 3 Monitoring Report for 2019 - Permit Number: EPR/BR9383ID/V010

Please find enclosed the completed emissions reporting forms for the period from 1st July 2019 to 30th September 2019.

I trust this information is satisfactory, however, please do not hesitate to contact me should you require any further information.

Yours sincerely

Graham Jones

Process and Energy Manager

Permit Number: 	EPR/BR9383ID/V010								Operator: 		Knauf Insulation Ltd

Facility:		Queensferry Mineral Fibre Works							Form Number: 	 Air1a

# Reporting of emissions to air for the quarter from 1st July to 30th September 2019 		Continuous Emissions Monitoring:

| Emission Point | Substance / Parameter | Emission Limit Value | Month 1 Average of daily average values | Month 1 Maximum Daily average value | Month 2 Average of daily average values | Month 2 Maximum Daily average value | Month 3 Average of daily average values | Month 3 Maximum Daily average value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Particulate Matter | 20 mg/m3 | 0.241 | 1.784 | 0.766 | 2.450 | 0.131 | 0.429 |
| A | Oxides of Nitrogen (expressed as NO2) | 300 mg/m3 | 152.18 | 206.15 | 130.57 | 144.31 | 145.38 | 168.68 |
| A | Oxides of Sulphur (expressed as SO2) | 1350 mg/m3 | 1221.26 | 1339.67 | 1225.76 | 1331.90 | 1207.43 | 1516.84 |
| A | Carbon Monoxide | 80 mg/m3 | 8.57 | 26.07 | 10.91 | 34.23 | 13.54 | 60.51 |

Month 1, July excludes data generated during CEMS instrument calibration on 16 July as advised in notification EP_EX_231.

Month 2, August includes a period where dust readings were low. This was corrected by adjustment and cleaning the Durag probe on 22 August as advised in notification EP_EX_235.

Month 3, September includes a breach of ELV for SO2 on 7th to 9th September. This was due to cross contamination of one of the batch components following a failure in the storage silos distribution system, as reported in notification EP_EX_236. The maximum daily average was during his period on 8/9/2019.

Signed …… 				Date……24/10/2018

(Authorised to sign as representative of Operator)

Permit Number: 	EPR/BR9383ID/V010			                                                	   Operator: 		Knauf Insulation Ltd

Facility:		Queensferry Mineral Fibre Works		                                                	   Form Number: 	Air1h

Reporting of emissions to air for the 3 month period for 1st July to 30th September 2019     Point A, Cupola Stack After Oxidizer

| Emission Point | Substance / Parameter | Emission Limit Value | Result 1 | Sample Date and Time | Result 2 | Sample Date and Time | Result 3 | Sample date and Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | ### Chlorides as HCl | 10 mg/m3 | 3.6 | 07/08/2019 13:18 to 14:18 | 10.4 | 07/08/2019 14:21 to 15:21 | 10.0 | 07/08/2019 15:24 to 16:24 |

The average result of 7.98 mg/m3 was within the ELV of 10 mg/m3 with a range of measurement uncertainty of +/- 0.59 mg/m3

Stability of the feed system has been further improved by reducing the flow rate of the carrier air from the lime powder blower.

Signed …… 				Date……24/10/2019

(Authorised to sign as representative of Operator)

Permit Number: 	EPR/BR9383ID/V010			                                                	   Operator: 		Knauf Insulation Ltd

Facility:		Queensferry Mineral Fibre Works		                                                	   Form Number: 	Air1c

Reporting of emissions to air for the 3 month period for 1st July to 30th September 2019     Point G, Cooling Zone Stack

| Emission Point | Substance / Parameter | Emission Limit Value | Result 1 | Sample Date and Time | Result 2 | Sample Date and Time | Result 3 | Sample date and Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G | Particulate Matter | 10 mg/m3 | 21.3 | 08/08/2019 10:37 to 11:39 | 8.7 | 08/08/2019 11:40 to 12:42 | 8.4 | 08/08/2019 12:45 to 13:17 |
| G | Ammonia | 10 mg/m3 | 0.4 | 08/08/2019 10:37 to 11:39 | 14.5 | 08/08/2019 11:40 to 12:42 | 23.5 | 08/08/2019 12:45 to 13:17 |

Note:  The monitoring results recorded concentrations of Particulate Matter & Ammonia in excess of the Emission Limit Values.  This was reported as an un-authorised release in notification EP_EX_239.

Signed …… 				Date……24/10/19

(Authorised to sign as representative of Operator)