PAN-007116 – Monte Carlo Modelling Report

Water Quality Modelling has been undertaken to measure the deterioration in Maenan Stream at point SH 78729 65746, this is the current permitted discharge point. Maenan Stream is not a designated WFD stretch. I have assumed ‘Good Class’ for Physiochemical parameters as per the established guidance (OI 50_12):

- Hydrology data:

  - QMean = 14515 m3/day

  - Q95 = 1468 m3/day

- Mean Quality and Standard Deviation of River Quality:

  - BOD Mean = 2.58	BOD Standard Deviation = 1.55 (these values are inferred from the ‘Mid Class Statistics’ data table)

  - Ammonia Mean = 0.26		Ammonia Standard Deviation = 0.15 (these values are inferred from the ‘Mid Class Statistics’ data table) – figures are multiplied by 1000 in the model to account for the fact the model can only complete to 2 decimal places

- Mean Flow and Standard Deviation of Flow were calculated using the maximum daily discharge volume of 17.5 m3/day:

  - Mean Flow = 8.47	Standard Deviation of Flow = 2.8

- Mean Quality and Standard Deviation of Quality were obtained from the ‘Mean and Standard Deviation Calculated from 95-Percentiles’ Table

Ammonia Modelling:

Conclusions:

Whilst a lot of assumptions are made in the data, this modelling exercise demonstrates that the currently permitted Maenan Stream is a more suitable discharge point than the proposed point. Modelling the impact of the discharge shows there will be a <2% deterioration in BOD and <1% deterioration in Ammonia in Maenan Stream.