- Introduction

  - Applicant Name: Associated British Ports

  - Marine Licence reference number: DML1946

  - Description of Operation: Port Talbot Harbour and Channel Maintenance Dredge

  - Dredge and disposal Site Location(s):

Port Talbot Harbour & Channel

Within the area bounded by joining the points listed below;

Harbour Approach Channel and Tidal Harbour

| 51 33.15N | 3 52.55W |
| --- | --- |
| 51 33.78N | 3 51.37W |
| 51 34.79N | 3 48.59W |
| 51 34.78N | 3 48.33W |
| 51 34.52N | 3 47.98W |
| 51 34.44N | 3 47.99W |
| 51 34.46N | 3 48.32W |
| 51 34.41N | 3 48.07W |
| 51 34.30N | 3 48.03W |
| 51 34.38N | 3 48.78W |
| 51 34.52N | 3 49.04W |
| 51 33.70N | 3 51.29W |
| 51 33.08N | 3 52.46W |

Dock Approach Channel

| 51 34.88N | 3 49.19W |
| --- | --- |
| 51 34.96N | 3 48.32W |
| 51 35.04N | 3 47.95W |
| 51 35.05N | 3 47.90W |
| 51 35.06N | 3 47.90W |
| 51 35 07N | 3 47.83W |
| 51 35.06N | 3 47.82W |
| 51 35.07N | 3 47.79W |
| 51 35.06N | 3 47.79W |
| 51 35.03N | 3 47.92W |
| 51 35.03N | 3 47.94W |
| 51 34.94N | 3 48.31W |
| 51 34.86N | 3 49.18W |

Deposit:

Swansea Bay (Outer), LU130 at:

Within a circle of 0.75 nautical miles radius centred on 51°31.30 N 03° 55.80 W

  - Activity period: On going maintenance dredging from 23rd December 2019  – 22nd December 2022

  - Biosecurity Manager/Officer: James Steer (on behalf of UK Dredging) & Martin Lightwood & Elton Prance (on behalf of ABP)

  - Approval Date: To be completed by NRW Marine Licensing Team

  - Document version number: 1.2

- Information related to the Environmental conditions affecting biosecurity

|  | Source environment/location | Receiving environment/location |
| --- | --- | --- |
| Marine Non-native species known to be present | Port Talbot Harbour and Channel Dredge Areas Known to be present at the dredge area. Crepidula Fornicata has been found within the harbour and surrounding area including Swansea Bay. Sargassum Muticum has been found within the harbour and to the west on Mumbles head. Styella clava has been found within the harbour, throughout much of Swansea bay and to the west at Mumbles head. Known to be present in near coastal locations: Eriocheir Sinesis has been found to the West of Port Talbot at Swansea. Spartina Townsendii has been found to the east of Port Talbot, at Kenfig nature reserve and to the west on the Neath river. Ficompoatus enigmaticus has been found to the West at Swansea. | Swansea Bay (outer) deposit LU130 As per Port Talbot Harbour and Channel Dredge Areas. Plus Spartina Townsendii has been found widely around the Mumbles also at Black Pill in Swansea Bay, on the Neath river near Briton Ferry, Ogmore-by-sea, Aberthaw and at various sites from Cardiff Bay to West of Newport. Additionally, Spartina Townsendii has been found extensively along the English coast from near Weston-Super-Mare past Clevedon around Avonmouth and reaching into the river Avon and both sides of the River Severn. Styella clava is known to be present in Swansea navigation channel also within the harbour, throughout much of Swansea bay to the west at Mumbles head, in the vicinity of Brynmill at Port Talbot and Cardiff Bay. Additionally, Styela Clava can be found at Watchet on the English coast. Ficompoatus enigmaticus has been found in and around Cardiff Bay and Penarth. Also found within Swansea Marina’s. Additionally, Ficompoatus enigmaticus has been found at Portishead and to the west of Avonmouth docks on the English coast. Crepidula Fornicata has been found at the Mumbles immediately outside the harbour and surrounding area including Swansea Bay also at various sites along the South Wales coast including Port Talbot, Porthcawl, St Donats, Lavernock and Cardiff. Aditionally, Crepidula Fornicata has been found at Woolacombe and Watchet on the English coast. Eriocheir Sinesis has been found in the vicinity of Brynmill to the West of Swansea Marina. Sargassum Muticum has been found along the south Mumbles, Swansea Marina and Port Talbot Harbour, to the east and west of Porthcawl, Penarth and at Cardiff Bay. Additionally, Sargassum Muticum has been found at various sites along the English coast including Woolacombe, Ilfracombe, Lynton and Watchet. Crassostrea gigas has been found to the west of Swansea at Oxwich Bay, at Swansea Marina, to the east and west of Porthcawl, near Porthkerry, Penarth and at Cardiff Bay. Aditionally, Crassostrea gigas has been found at Woolacombe and Watchet on the English coast and also at Flatholm. Diadumene Lineata has been found at Swansea marina, at Penarth marina and Cardiff Bay. Additionally, Diadumene Lineata has been found at Weston-Super-Mare on the English Coast. Additionally, Amphibalanus has been discovered at locations to the South East at areas including Weston-super-Mare, Sand Bay and Clevedon. |
| Above sourced from National Biodiversity Network dataset information. |  |  |

- Vessels & equipment involved in the operation (please list all vessels, barges and equipment to be used as part of the maintenance dredging operation – insert additional rows as necessary)

| Vessel name | Photo reference (Please insert images in the appendix) | Vessel type | Details, Risk factors & Justification e.g. Pathway, vessel speed, biofouling control, Inspection history, Internal treatment history etc.. | Risk: (High/Medium/Low) |
| --- | --- | --- | --- | --- |
| Insert vessel name here | See attached UK Dredging bio-security plan | e.g. Suction dredge | See IMO Biofouling Guidance | e.g. Medium |

Please see attached UK Dredging Bio-Security Plan for Vessels Names, Photo’s and Vessel Types.  A full bio-security risk assessment is also included detailing sources, pathways, mitigation/preventative measures and risk analysis; which is applicable to all vessels within the fleet.

  - Vessel & Disposal site Proposed activity log

In relation to maintenance dredging activities, marine invasive non-native species typically have three main vectors by which spread can potentially occur from one location to another. These are hull fouling, transfer in vessel ballast water and transfer within marine sediments. In order to better understand the potential risks associated to the current operation it is important to outline a brief history of vessel/equipment movements. This includes information such as vessel/equipment recent activity history (including any periods of inactivity) as well as information on ports visited and dredge/disposal site locations associated to the current licensable activity.

| Vessel name | Brief description of vessel activity over the last 12 months | Transit ports (list all ports to be visited on route to the current dredge activity if applicable) | Transit ports arrival and departures |  | Disposal site name & reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Arrival date | Departure date |  |
| Insert vessel name here | Insert name of all ports and dredge/disposal sites visited in the last 12 months | Port name | Date | Date | Disposal site name and reference no. |

Due to the operational nature of dredgers, transit ports are rarely visited.  Instead, dredgers carry out one dredge campaign (usually a few days in length) before heading to a different port to carry out another campaign.  Dredgers rarely stop for any length of time and when they do it is for maintenance reasons and within dock areas or dry-docks.  Due to the nature of dredging, work carried out by dredgers is often conducted in areas of low salinity.

- Activities which have a significant risk of introducing or spreading marine invasive non-native species (Please provide as much detail as possible of how each of the activities have the potential to increase the risk of introduction or onward spread of marine invasive non-native species – insert additional rows as necessary)

| Activity and brief description | Details of how this could potentially increase the risk of introducing and/or spreading MINNS | Mitigation and Biosecurity measures to be implemented |
| --- | --- | --- |
| Movement of vessels from one port to another | Risk of spreading INNS could be through - the transfer of ballast water/sediment between ports or between dredge area and deposit area. - bio-fouling of the ships hull, strum boxes and seawater intakes - the transfer of dredge arrisings between ports. | The prevention of introducing or spreading INNS through the transfer of ballast is governed by the Ballast Water Management Convention as measure are taken in accordance with each vessels approved Ballast Water Management Plan. In addition to the requirements of the BWMC, UK Dredging actively manages the risk by reducing the need to conduct ballasting operations to a minimum and where ballast tanks are rarely discharged, they are filled using fresh water from a treated shore source. To prevent bio-fouling of the ship, we use of certified anti-foul paint system, with maintenance and reapplication every 2 years (which is over and above Class requirement of 3 yearly). Copper anodes in sea-boxes or in sea-box flush systems prevents build-up of biofouling. Any biofouling removed from filters is disposed of or incinerated. In accordance with our marine licenses, we are prohibited from moving dredging wastes or arisings between ports and discharge everything prior to leaving a port or deposit area. We also ensure we clean the hopper, dredge equipment and plough prior to sailing from a port. We will photograph and report any fauna & flora foud within the dredge arisings. This will be identified, recorded and reported if INNS is discovered. |
| Dredging or the movement of dredge material between dredge area and deposit. | Increased risk if INNS has been found within dredge area and not deposit. | For this project, the deposit and dredge area are relatively close and so there is a high probability that they will contain similar species. However, any flora or fauna dredged will be photographed, identified, recorded and reported if INNS is discovered |

- Mitigation & Biosecurity Control Measures – Instructions for staff/contractors/site users (insert additional rows as necessary)

| Who | What | Where | When |
| --- | --- | --- | --- |
| Master / OOW | Record details of each dredge cycle. | Logbook | During normal operations. |
| Master | Wash down ship to remove dredge material | Vessel and dredge equipment | Prior to departure from port area. |
| Master / Mate | Refresh ballast water if it is anticipated that there will be a need to discharge ballast at the following port | Vessel | Between ports, as far from land and in as deep water as possible, |
| Master | Refresh clean hopper water carried as ballast. | Vessel hopper | On departure from port limits |
| Master | Pump out clean hopper water carried as ballast | Vessel hopper | Prior to arrival at destination port. |
| All | Report all flora and fauna dredged as part of operations | Vessel | At any time |
| UKD Bio-Security Manager | Identify reported flora and fauna. If INNS, record and report to port bio-security manager and as per bio-security plan | Management Office | As required |
| UKD Technical Manager | Ensure maintenance of anti-bio-foul paint systems, hull cleaning and inspection by class. | Dry dock | Every two years |

- Operational surveillance and reporting procedure

All ships crew instructed to report any flora and fauna found when dredging. INNS identification material has also been provided to the ships. Photographs will be supplied to the bio-security manager who will identify. If INNS discovered, this is recorded and reported to the port bio-security manager and to the GB NNS Secretariat.

- Contingency Plan (i.e. when changes to the plan necessary and have been identified - insert additional rows as necessary)

| Action | Responsibility | Location of Equipment |
| --- | --- | --- |
| The be completed if any contingency required |  |  |

- Location of biosecurity logbook

UKD Management Office

- Annual Review Date

22nd December 2020

Appendix

UK Dredging Biosecurity Plan

3 x Washdown Method Statements

Example Approved Ballast Water Management Plan

Statement of Compliance with the BWMC

The licence holder should update and submit a revised plan as a result of any amendments to the list of vessels and/or equipment to be utilised as part of current licence.