Irfon River Premier Leisure Park

Hydrology Data (at SN 96174 49490)

Watercourse Name – River Irfon, conf Cledan to conf R Irfon (GB109055037090)

Mean flow = 872640

95% exceedance flow = 48124.8

Discharge Data

Volume = 42.5m3/day

Mean = 21.18

Standard Deviation = 6.99

Waterbody Data

WFD/Mid Class Data (Type 1, High Class)

WFD Watercourse: GB109055037090

Ammonia: Mean = 0.072545	Standard Deviation = 0.190857

Midclass/High data used for BOD: Mean = 0.86		Standard Deviation = 0.52

Phosphate: Mean = 0.14	Standard Deviation = 0.36

Summary of Results

- To enable accurate modelling of Ammonia, WFD data and discharge quality data have been converted from mg/l to ug/l to account for the fact that MPER can only model to 2 decimal places.

- Modelling Ammonia @ 20mg/l showed there will be a 0% deterioration within the receiving watercourse

- Modelling BOD @ 20mg/l showed there will be a 0-1.1% deterioration within the receiving watercourse

- Modelling BOD @ 40mg/l showed there will be a 0-1.1% deterioration within the receiving watercourse

- Modelling Phosphate @ 8mg/l showed there will be a 0% deterioration within the receiving watercourse

- Due to the sensitivity of the receiving watercourse, I will put backstop emission limits on the permit (40mg/l BOD, 60mg/l SS and 20mg/l Ammonia), this will ensure 0% deterioration.

| MASS BALANCE CALCULATION: MONTE CARLO METHOD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version 2.5 |  |  |  |  |
| Calculations done on 04/04/2019 at 08.51 |  |  |  |  |
| Name of discharge | Premier Leisure Park - 41.85m3/day |  |  |  |
| Name of river | Irfon (Type 1) |  |  |  |
| Name of determinand | Ammonia @ 20mg/l High class |  |  |  |
| INPUT DATA |  |  | RESULTS |  |
| UPSTREAM RIVER DATA |  |  | RIVER DOWNSTREAM OF DISCHARGE |  |
| Mean flow | 872640.0 |  | Mean quality | 77.36 |
| 95% exceedence flow | 48124.8 |  | Standard deviation of quality | 229.10 |
| Mean quality | 72.50 |  | 90-percentile quality | 161.21 |
| Standard deviation of quality | 190.80 |  | 95-percentile quality | 271.98 |
| 90-percentile | 162.75 |  | 99-percentile quality | 728.73 |
| DISCHARGE DATA |  |  | DISCHARGE QUALITY |  |
| Mean flow | 20.86 |  | Mean quality | 13228.5 |
| Standard deviation of flow | 6.88 |  | Standard deviation of quality | 5258.1 |
| Mean quality | 13000.0 |  | 95-percentile quality | 23133.1 |
| Standard deviation of quality | 5300.0 |  | 99-percentile quality | 29551.6 |
| ... or 95-percentile | 22944.5 |  | 99.5-percentile quality | 31579.0 |
| -0.946236559 | % deterioration based off 90% |  |  |  |
| *6.703448276 | % deterioration based off Mean |  |  |  |

*The 6.7% deterioration based off Mean is because the standard deviation of the upstream river data is anomalously high. This shows there are some outliers in the WFD results. Due to the fact the deterioration is still <10% and the % deterioration based on the 90%ile is 0%, I can discount this result and proceed with a limit of 20mg/l for Ammonia.

| MASS BALANCE CALCULATION: MONTE CARLO METHOD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version 2.5 |  |  |  |  |
| Calculations done on 18/01/2019 at 09.20 |  |  |  |  |
| Name of discharge | Premium Leisure Park, 42.5 m3 |  |  |  |
| Name of river | Irfon (Type 1) |  |  |  |
| Name of determinand | BOD @ 20mg/l - High Class |  |  |  |
| INPUT DATA |  |  | RESULTS |  |
| UPSTREAM RIVER DATA |  |  | RIVER DOWNSTREAM OF DISCHARGE |  |
| Mean flow | 872640.0 |  | Mean quality | 0.87 |
| 95% exceedence flow | 48124.8 |  | Standard deviation of quality | 0.53 |
| Mean quality | 0.86 |  | 90-percentile quality | 1.50 |
| Standard deviation of quality | 0.52 |  | 95-percentile quality | 1.84 |
| 90-percentile | 1.50 |  | 99-percentile quality | 2.69 |
| DISCHARGE DATA |  |  | DISCHARGE QUALITY |  |
| Mean flow | 21.18 |  | Mean quality | 10.91 |
| Standard deviation of flow | 6.99 |  | Standard deviation of quality | 4.95 |
| Mean quality | 10.70 |  | 95-percentile quality | 20.32 |
| Standard deviation of quality | 5.00 |  | 99-percentile quality | 26.82 |
| ... or 95-percentile | 20.14 |  | 99.5-percentile quality | 28.92 |
| % Deterioration | 0 |  |  |  |
| % Deterioration based off mean | 1.162790698 |  |  |  |

| MASS BALANCE CALCULATION: MONTE CARLO METHOD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Version 2.5 |  |  |  |  |  |  |
| Calculations done on 23/01/2019 at 10.19 |  |  |  |  |  |  |
| Name of discharge | Premium Leisure Park, 42.5m3 |  |  |  |  |
| Name of river | Irfon (Type 1) |  |  |  |  |  |
| Name of determinand | BOD @ 40mg/l – High Class |  |  |  |  |
| INPUT DATA |  |  | RESULTS |  |  |  |
| UPSTREAM RIVER DATA |  |  | RIVER DOWNSTREAM OF DISCHARGE |  |  |  |
| Mean flow | 872640.0 |  | Mean quality |  | 0.87 |  |
| 95% exceedence flow | 48124.8 |  | Standard deviation of quality |  | 0.53 |  |
| Mean quality | 0.86 |  | 90-percentile quality |  | 1.50 |  |
| Standard deviation of quality | 0.52 |  | 95-percentile quality |  | 1.84 |  |
| 90-percentile | 1.50 |  | 99-percentile quality |  | 2.69 |  |
| DISCHARGE DATA |  |  | DISCHARGE QUALITY |  |  |  |
| Mean flow | 21.18 |  | Mean quality |  | 19.53 |  |
| Standard deviation of flow | 6.99 |  | Standard deviation of quality |  | 10.92 |  |
| Mean quality | 19.10 |  | 95-percentile quality |  | 40.56 |  |
| Standard deviation of quality | 11.10 |  | 99-percentile quality |  | 56.81 |  |
| ... or 95-percentile | 40.11 |  | 99.5-percentile quality |  | 62.23 |  |
| % Deterioration Based off 90%ile | 0 |  |  |  |  |  |
| % Deterioration Based off Mean | 1.16279 |  |  |  |  |  |

| MASS BALANCE CALCULATION: MONTE CARLO METHOD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version 2.5 |  |  |  |  |
| Calculations done on 23/01/2019 at 08.40 |  |  |  |  |
| Name of discharge | Premium Leisure Park |  |  |  |
| Name of river | Irfon |  |  |  |
| Name of determinand | Phosphate @ 8mg/l |  |  |  |
| INPUT DATA |  |  | RESULTS |  |
| UPSTREAM RIVER DATA |  |  | RIVER DOWNSTREAM OF DISCHARGE |  |
| Mean flow | 872640.0 |  | Mean quality | 0.14 |
| 95% exceedence flow | 48124.8 |  | Standard deviation of quality | 0.43 |
| Mean quality | 0.14 |  | 90-percentile quality | 0.30 |
| Standard deviation of quality | 0.36 |  | 95-percentile quality | 0.51 |
| 90-percentile | 0.30 |  | 99-percentile quality | 1.36 |
| DISCHARGE DATA |  |  | DISCHARGE QUALITY |  |
| Mean flow | 21.18 |  | Mean quality | 8.23 |
| Standard deviation of flow | 6.99 |  | Standard deviation of quality | 7.58 |
| Mean quality | 8.00 |  | 95-percentile quality | 22.64 |
| Standard deviation of quality | 8.00 |  | 99-percentile quality | 38.08 |
| ... or 95-percentile | 22.25 |  | 99.5-percentile quality | 43.84 |
| % Deterioration Based off 90%ile | 0.0 |  |  |  |
| % Deterioration based off Mean | 0 |  |  |  |