Permit Number: EPR/DP3735NP	                                          	Operator: 		Biogen Waen Limited

Facility Waen Anaerobic Digestion Site	                      	Form Number 		Air1 / 16/01/14

Reporting of emissions to air for the period from 01/01/2019 to 31/12/2019

| Emission Point | Substance/ Parameter | Emission Limit/Value | Reference Period | Result | Test Method | Sample Date and Times |  | Uncertainty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHP | Oxides of nitrogen (NO and NO2 expressed as NOx) | 500 mg/m3 | 1hour | 321 mg(N)m-3 | BS EN 14792 |  | 23/07/2019 10:12 -11:12 | ± 18 |
|  | Carbon monoxide | 1400 mg/m3 | 1hour | 738 mg(N)m-3 | BS EN 15058 |  | 23/07/2019 10:12 -11:12 | ±43 |
|  | Total Volatile Organic Compounds (VOCs) | 1000 mg/m3 | 1hour | 632 mg(N)m-3 | BS EN12619:1999 |  | 23/07/2019 10:12 -11:12 | ±30 |
|  | Sulphur dioxide | 350 mg/m3 | 1hour | 100.3 mg(N)m-3 | BS EN 14791 |  | 23/07/2019 10:12 -11:12 | ± 8.4 |

The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

Where an internationally recognised standard test method is used the reference, number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases, the principal technique is stated, for example gas chromatography.

For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.   The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Operator’s comment:

Signed …………………………………………. Date…16-01-2020……………………..

(Authorised to sign as representative of Operator)