| Permit Number: | PAN-005154 |
| --- | --- |
| Applicant name: | Mr John Harvey |
| Site Address: | Glanteifi, Cellan, Lampeter, SA48 8HU |
| Discharge Point NGR: | SN 59978 48762 |
| Respond by date: | 03/05/2019 (10 working days) |
| Permitting Officer: | Elliot Burge |

Brief Description of proposal:

The application is for a new discharge of secondary treated sewage effluent originating from a single domestic property and associated annex, the maximum daily discharge volume will be 2m3/day. The treatment system will be a package treatment plant that meets the British Standard BS 12566, capable of treating the effluent to 20:30:20 mg/l for BOD: Suspended Solids: Ammonia. The discharge will be made to an unnamed tributary of the Afon Teifi, the applicant has confirmed that the watercourse does not dry up at any point during the year. The tributary reaches the Teifi after approximately 60m.

The discharge replaces a septic tank discharge to surface water.

Habitats Screening Report – Designations or protected species/habitats:

The risk screening report can be found on the DMS here.

The Afon Teifi SAC and SSSI is within 500m of the proposed discharge point, a Form 1 HRA and Appendix 4 assessment will be completed and consulted on as appropriate.

MyMap also shows Eel in the main river approximately 60m from the proposed discharge point.

Risk Assessment Results (modelling or L1 Risk Assessment):

As the discharge is <5m3/day, there will be no water quality modelling for this application.

Miscellaneous:

There is a sewer line nearby to the property, however as it is a rising main it is not possible for the property to connect into the sewer. The nearest sewer connection point is >200m, therefore we will not ask the applicant to explore connection to the foul sewer.

NRM Responses:

Environment Management comments:

Officers name:
Date:

Biodiversity comments:

Officers name:
Date:

Fisheries comments:

Officers name:
Date:

Conservation/Ecology Comments:

Can you send the HRA Form 1 and SSSI Appendix 4 assessment to me please, as lead conservation officer for the Afon Teifi SSSI and SAC.

Officers name:  Jon Turner, SCO Ceredigion NRM Team
Date:  25/04/2019

Internal Consultation Responses from other NRW Teams:

Water Quality Planning:

N/A

Officers name:

Date:

Geoscience Response:

N/A

Officers name:
Date:

Flood Risk Management:

N/A

Officers name:
Date:

| External Consultations sent to: | Yes/No |
| --- | --- |
| National Parks | N |
| Water Company regarding Drinking Water Protected Zone / SPZ1 | N |
| Environmental Heath | N |

Any Permitting Responses:

Date: