Permit Number:	EPR/AB3697CN							Operator: 	Radnor Hills Mineral Water Company Ltd.

Facility:			Radnor Hills							Form Number:	Energy1

Reporting of Energy Usage for the year - 01/01/2019Yto 31/12/2019/01/2018Yto 31/12/201

| Energy Source | Energy Usage Quantity | Primary Energy (MWh) | Specific Usage (MWh/unit output) |  |
| --- | --- | --- | --- | --- |
| Electricity | 24566.6 kWh/day average | 8966.82 MWh | 0.623 MWh/m3 |  |
| Natural Gas | 3118 litres/day average | 1137993 Litres | 7.917 Litre/m3 |  |
| Fuel Oil | 4.06 litres/day average | 1482 Litres | 0.010 Litres/m3 |  |

Operator’s comments: Electricity: Supplied by Hudson Energy from Jan – Sep’19 whose energy is sourced from 100% renewable generation and accredited by Ofgem a REGOs certificate. From Oct’19 to present our electricity is being supplied by EON which is sourced from 100% renewable wind sources. The energy usage has an approximate daily figure taken from the annual totals. Significant difference from 2018 total energy of electricity and natural gas, this is due to the removal of manufacturing equipment during the year for the following areas: RV4 Hot fill PET Production Line, Bottle Blowing Production Line, RV2 PET Production Line. RV4 has been replaced by a canning manufacturing line known as C4 which due to the process requires less electricity than its predecessor. Electricity difference is 393.36MWh Natural Gas difference is 470013litres.

Signed …………………………………………………………….  		Date…11/02/2020   	(Authorised to sign as representative of Operator)

Permit Number: 	EPR/AB3697CN						Operator:	Radnor Hills Mineral Water Company Ltd.

Facility:			Radnor Hills						Form Number: 	Performance1

Reporting Performance Indicators for the period - 01/01/2019Yto 31/12/2019/

| Parameter |  | Units |  |
| --- | --- | --- | --- |
| Total Production Volume 143729.77 |  | m3 |  |
| Total Waste Volume 56835.7 |  | m3 |  |
| Total Materials Used 2,833,791.1 |  | kg |  |

Operator’s comments:

Signed …………………………………………………………….  		Date…11/02/2020……….   	(Authorised to sign as representative of Operator)

Permit Number: 	EPR/AB3697CN							Operator: 	Radnor Hills Mineral Water Company Ltd.

Facility:			Radnor Hills							Form Number:	Air1

Reporting of Emissions to Air for the period -        01/01/2019Yto 31/12/2019/MM/YYYY

| Emission Point | Source | Substance / Parameter | Emission Limit Value | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Boiler 1 – RV3 | Oxides of nitrogen (NO and NO2 expressed as NO2) | No Limit Set | - | 0ppm | MCERTS certified contractor and equipment, method to follow M1 | 30/05/19 | ±2% |
| A1 |  | Particulates |  |  | 58ppm |  |  | ±2% |
| A2 | Boiler 2 – Tetra | Oxides of nitrogen (NO and NO2 expressed as NO2) | No Limit Set | - | 0.9ppm |  | 30/05/19 | ±2% |
| A2 |  | Particulates |  |  | 79ppm |  |  | ±2% |
| A3 | Boiler 3 – Syrup CIP Set | Oxides of nitrogen (NO and NO2 expressed as NO2) | No Limit Set | - | 3.2ppm |  | 30/05/19 | ±2% |
| A3 |  | Particulates |  |  | 86.1 |  |  | ±2% |
| A4 | Boiler 4 – RV5 | Oxides of nitrogen (NO and NO2 expressed as NO2) | No Limit Set | - | 1.1ppm |  | 30/05/19 | ±2% |
| A4 |  | Particulates |  |  | 84ppm |  |  | ±2% |
| A4 | Boiler 5 – RV8 | Oxides of nitrogen (NO and NO2 expressed as NO2) | No Limit Set | - | 1.1ppm |  | 30/05/19 | ±2% |
| A4 |  | Particulates |  |  | 57ppm |  |  | ±2% |

Signed …………………………………………………………….  		Date   11/02/2020……   	(Authorised to sign as representative of Operator)

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference, number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases, the principal technique is stated, for example gas chromatography.

- For non-continuous measurements, the date and time of the sample that produced the result is given. For continuous measurements, the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Permit Number: 	EPR/AB3697CN							Operator: 	Radnor Hills Mineral Water Company Ltd.

Facility:			Radnor Hills							Form Number:	Water1

| Emission Point | Substance / Parameter | Emission Limit Value | Reference Period | Result [1] |  | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W1 | Biological Oxygen Demand | 5.00mg/l | Flow weighted sample or composite sample | Max | 11.30mg/l | SCA Blue Book 130 | 19/11/19 | 25% |
|  |  |  |  | Min | <1.00mg/l |  | 16/10/19 | 25% |
|  | Total Ammonia as N | 0.50mg/l | Flow weighted sample or composite sample | Max | 0.4mg/l | BS EN ISO 11732 | 05/01/19 | 5% |
|  |  |  |  | Min | 0.0mg/l |  | 25/01/19 |  |
|  | Soluble Reactive Phosphorus as PO4 | 0.20mg/l | Flow weighted sample or composite sample | Max | 4.10mg/l | BS EN ISO 15681-1 or BS EN ISO 15681-2 | 30/07/19 | 55% |
|  |  |  |  | Min | 0.00mg/l |  | 19/07/19 |  |
|  | Flow | 8.1m3hr | Integrated daily flow rate | Approx. Average/hr | 5.9m3/hr | MCERTS certified equipment, method as per M18 | AVERAGE | 55% |
|  | Flow | 194.4m3/day | Integrated daily flow rate | Approx. Average/day | 141.66m3/day | MCERTS certified equipment, method as per M18 | AVERAGE | 5% |
|  |  |  |  | Max | 193.4m3/day |  | 15/07/19 | 0% |
|  |  |  |  | Min | 0m3/day |  | 25/12/19 |  |
|  | Temperature | Max 25oC | Instantaneous | Max | 26.1oC | MCERTS certified contractor and equipment, method to follow M1 | 26/07/19 | 5% |
|  |  |  |  | Min | 11.5oC |  | 31/01/19 |  |
|  | pH | Between 6-9 | Instantaneous | Max | 8.17 | BS EN ISO 10523 | 20/07/19 | 5% |
|  |  |  |  | Min | 5.56 |  | 7/02/19 |  |

Reporting of Emissions to Water for the period - DD/M01/01/2019Yto 31/12/2019/2016

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference, number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases, the principal technique is stated, for example gas chromatography.

- For non-continuous measurements, the date and time of the sample that produced the result is given. For continuous measurements, the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date…11/02/2020   	(Authorised to sign as representative of Operator)

Permit Number: 	EPR/AB3697CN							Operator: 	Radnor Hills Mineral Water Company Ltd.

Facility:			Radnor Hills							Form Number:	WaterUsage1

Reporting of Emissions to Air for the period -        01/01/2019 to 31/12/2019

| Water Source | Usage (m3/year | Specific Usage (m3/unit output) |  |
| --- | --- | --- | --- |
| Mains water | 0m3 | 0m3 |  |
| Site borehole | 310584m3 | 2.16m3/m3 |  |
| River abstraction | 0m3 | 0m3 |  |
| TOTAL WATER USAGE | 310584m3 |  |  |

Operator’s comments:

Signed …………………………………………………………….  		Date……11/02/20…..

(authorised to sign as representative of Operator)