
[image: image1.png]Cyfoeth
Naturiol
Cymru

Natural
Resources
Wales




Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016
OHES Environmental Limited
Groundwater treatment remediation system serving
Certas Oil Depot
Pontwelly
Llandysul
SA44 4AJ
Permit number

EPR/BB3595ZK
Groundwater remediation treatment system serving Certas Oil Depot
Permit Number EPR/BB3595ZK
Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.  
The permit is for the discharge of 80m3/day of treated contaminated groundwater. The contaminated groundwater will be abstracted and subsequently remediated prior to being re-injected to ground via boreholes, the discharge will take place Monday-Friday. The treatment system comprises of an oil separator, suspended solids settlement and primary and secondary activated carbon filters. There are monitoring and reporting requirements within the permit.
The status log of the permit sets out the permitting history, including any changes to the permit reference number.
	Status log of the permit

	Description
	Date
	Comments

	Application PAN-006818
	Duly made 27/09/2019
	Application for discharge of remediated groundwater

	Additional information received
	08/11/2019
	Confirmation that ‘prevent’ requirements of the Environmental Permitting Regulations will be met

	Permit determined
	28/02/2020
	Permit issued to OHES Environmental Limited


End of introductory note
	Permit


The Environmental Permitting (England and Wales) Regulations 2016
Permit number 

EPR/BB3595ZK
The Natural Resources Body for Wales (“Natural Resources Wales”) hereby authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016 
OHES Environmental Limited (“the operator”),

whose registered office is

80 Station Parade
Harrogate

North Yorkshire

HG1 1HQ

company registration number 04320201
to operate a groundwater activity at
Certas Oil Depot

Pontwelly

Llandysul

SA44 4AJ

to the extent authorised by and subject to the conditions of this permit.
	Name
	Date

	Melissa Broad
	28/02/2020


Authorised on behalf of Natural Resources Wales
Conditions 

1 Management
1.1 General management

1.1.1 The operator shall manage and operate the activities:

(a) in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances and those drawn to the attention of the operator as a result of complaints; and

(b) using sufficient competent persons and resources.

1.1.2 Records demonstrating compliance with condition 1.1.1 shall be maintained. 

1.1.3 Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.
2
Operations
2.1
Permitted activities

2.1.1
The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”). 

2.2
The site 
2.2.1
The activities shall not extend beyond the site, being the land shown edged in green and the discharge shall be made at the points marked on the site plan at schedule 7 to this permit and as listed in table S3.2 (discharge points). 

2.3
Operating techniques

2.3.1
(a) 
The activities shall, subject to the conditions of this permit, be operated using the 



techniques and in the manner described in the documentation specified in 
schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources      Wales.

(b)
If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan, and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

2.3.2
The boreholes

(a)
shall not extend below the depth specified in table S1.1

(b)
shall comply with the minimum depth below ground level for un-perforated linings specified in table S1.1;

(c) shall not connect to any land drainage system;
(d) shall not be situated within 10 metres of any watercourse (including any ditch that runs dry for part of the year), or any other surface water;
(e) shall not be within 50 metres of a well or borehole used for water supply.

3
Emissions and monitoring
3.1
Emissions to water or land

3.1.1
There shall be no point source emissions to water, air or land except from the sources and emission points listed in schedule 3 tables S3.1, S3.2 and S3.3.

3.1.2
The limits given in schedule 3 shall not be exceeded.

3.2 Emissions of substances not controlled by emission limits

3.2.1
All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

3.2.2
For the activity referenced in schedule 1, table S1.1 the operator shall take appropriate measures as far as is reasonably practicable:

1 to prevent the input of hazardous substances to groundwater; and

1 where a non-hazardous pollutant is not controlled by an emission limit, to limit the input of such non-hazardous pollutants to groundwater so as to ensure that inputs do not cause pollution of groundwater. 
3.3
Monitoring 

3.3.1
The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

(a) point source emissions specified in tables S3.1 and S3.3;

(b) surface water or groundwater specified in table S3.4.

3.3.2
The operator shall maintain records of all monitoring required by this permit including records of the taking and analysis of samples, instrument measurements (periodic and continual), calibrations, examinations, tests and surveys and any assessment or evaluation made on the basis of such data.

3.3.3
Monitoring equipment, techniques, personnel and organisations employed for the emissions monitoring programme and the environmental or other monitoring specified in condition 3.3.1 shall have either MCERTS certification or MCERTS accreditation (as appropriate), where available, unless otherwise agreed in writing by Natural Resources Wales.

3.3.4
Permanent means of access shall be provided to enable sampling/monitoring to be carried out in relation to the emission points specified in schedule 3 tables S3.1, S3.2, S3.3 and S3.4 unless otherwise agreed in writing by Natural Resources Wales.
Installation of monitoring boreholes

3.3.5
A minimum of 3 monitoring boreholes shall be installed within 1 month of the date of issue of this permit at locations agreed in advance and in writing by Natural Resources Wales for the purpose of ‘requisite surveillance’ monitoring.

3.3.6
The monitoring borehole shall be installed to depths, by methods and according to a design agreed in advance and in writing by Natural Resources Wales.


3.3.7
The following details regarding the monitoring borehole shall be provided to Natural Resources Wales within 1 month of installation:

(a) casings/linings (length, diameter, material, type of grout or filter media and whether slotted or plain);

(b) depths and diameters of unlined sections;

(c) standing groundwater levels;

(d) details of strata encountered during drilling;

(e) reference levels in metres above ordnance datum;

(f) a location plan at a suitable scale showing the boreholes in relation to the point of discharge;

(g) national grid references of the borehole(s) in the form AB 12345 67890;

(h) any other information obtained from the borehole(s) relevant to the interpretation of water sample analysis.

4
Information

4.1
Records
4.1.1 All records required to be made by this permit shall:

(a) be legible;

(b) be made as soon as reasonably practicable;

(c) if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

(d) be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made.
4.1.2
The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.   

4.2
Reporting
4.2.1
The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

4.2.2
Within 28 days of the end of the reporting period the operator shall, unless otherwise agreed in writing by Natural Resources Wales, submit reports of the monitoring and assessment carried out in accordance with the conditions of this permit, as follows:

(a) in respect of the parameters and emission points specified in schedule 4 table S4.1;

(b) for the reporting periods specified in schedule 4 table S4.1 and using the forms specified in schedule 4 table S4.2; and

(c) giving the information from such results and assessments as may be required by the forms specified in those tables.

4.3
Notifications
4.3.1
Natural Resources Wales shall be notified without delay following the detection of:

(a) any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

(b) the breach of a limit specified in the permit; or
(c) any significant adverse environmental effects.  
(d)
any emergency discharge that has occurred.

4.3.2
Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.
4.3.3
Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules: 

Where the operator is a registered company:

(a) any change in the operator’s trading name, registered name or registered office address; and

(b)
any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

(a) any change in the operator’s name or address; and

(b) any steps taken with a view to the dissolution of the operator.

4.3.4
Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

(a) Natural Resources Wales shall be notified at least 14 days before making the change; and

(b) the notification shall contain a description of the proposed change in operation.

 4.4
Interpretation
4.4.1
In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

4.4.2
In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone. 
Schedule 1 – Operations

	Table S1.1 Activities

	Description of activity
	Limits of specified activity

	Groundwater activity: Reinjection to groundwater of water abstracted from boreholes subject to treatment for the purposes of ground remediation 
	Discharge to re-injection boreholes at the following grid references:

Discharge 1 - SN 41227 40075
Discharge 2 - SN 41227 40093
Discharge 3 - SN 41250 40099
At each outlet, boreholes shall not extend deeper than 6 metres below ground level.

Unperforated linings shall extend to the following minimum depths below ground level:

Discharge 1, 2 and 3 – 1 metre 

The discharge shall be made Monday-Friday only


	Table S1.2 Operating techniques

	Description of documentation
	Parts
	Date Received

	OHES Environmental – Environmental Permit Application
	Sections 3.4 and 3.5
	05/09/19

	Certas Oil Depot, Llandysul – Permit Application Queries
	All
	06/12/2019


Schedule 2 - Waste types, raw materials and fuels

Wastes are not accepted as part of the permitted activities and there are no restrictions on raw materials or fuels under this schedule.

 Schedule 3 – Emissions and monitoring

	Table S3.1 Point Source emissions to water (other than sewer) and land – emission limits and monitoring requirements

	Emission point ref. & location
	Parameter  
	Limit (incl. unit)
	Reference 

Period
	Monitoring frequency
	Compliance statistic

	Discharges 1/2/3 on site plan in schedule 7 – Discharge of treated groundwater via re-injection boreholes

	Maximum daily discharge volume
	80 m3/day
	Total daily volume
	Continuous
	Maximum

	
	Benzene
	1 µg/L
	Instantaneous (spot sample)
	Daily (Mon-Fri)
	Maximum

	
	Toluene
	4 µg/L
	Instantaneous (spot sample)
	Daily (Mon-Fri)
	Maximum

	
	Xylenes
	3 µg/L
	Instantaneous (spot sample)
	Daily (Mon-Fri)
	Maximum

	
	Visible oil or grease
	No significant trace present
	Instantaneous (spot sample)
	N/A
	No significant trace


	Table S3.2 Discharge points

	Effluent Name
	Discharge Points
	Discharge points NGR
	Receiving Environment



	Discharge of treated groundwater via re-injection boreholes
	Discharge 1
Discharge 2

Discharge 3 

	SN 41227 40075
SN 41227 40093
SN 41250 40099
	Groundwater via boreholes


	Table S3.3 Monitoring points

	Effluent and discharge points
	Monitoring type
	Monitoring point NGR
	Monitoring point reference

	Discharge of treated groundwater via re-injection boreholes
	Effluent sample point
	SN 41306 40110
	Secondary GAC vessel

	
	Flow Monitoring
	SN 41306 40112
	Flow


	Table S3.4 Surface water or groundwater monitoring requirements

	Location or description of monitoring points
	Parameters
	Monitoring frequency
	Monitoring standard or method



	Monitoring borehole MW1 at NGR SN 41200 40053

Monitoring borehole MW2 at NGR SN 41200 40085 

Monitoring borehole MW3 at NGR SN 41220 40101
Monitoring borehole MW4 at NGR SN 41233 40105

Monitoring borehole MW5 at 

NGR SN 41246 40110
	Benzene

Toluene

Xylenes
	Weekly 
	BS ISO 5667




Schedule 4 - Reporting

Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

	Table S4.1 Reporting of monitoring data

	Parameter
	Emission or monitoring point/reference
	Reporting period
	Period begins

	Benzene, Toluene, Xylenes
	Monitoring boreholes MW1, MW2, MW3, MW4, MW5
	Monthly
	Upon issue of permit

	Benzene, Toluene, Xylenes
	Secondary GAC vessel
	Monthly
	Upon issue of permit

	Total daily volume


	Flow
	Monthly
	Upon issue of permit


	Table S4.2 Reporting forms

	Media/parameter
	Reporting format

	Water and Land
	Form water 1 or other form as agreed in writing by Natural Resources Wales

	Total daily volume
	Photographic Record of Flow Meter Readings and corresponding dates as agreed in writing by Natural Resources Wales


Schedule 5 - Notification 

These pages outline the information that the operator must provide. 

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A 
	Permit Number
	EPR/BB3595ZK

	Name of operator
	

	Location of Facility
	Certas Oil Depot, Lewis Street, Pontwelly, Llandysul, SA44 4AJ

	Time and date of the detection 
	


	(a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution

	To be notified within 24 hours of detection

	Date and time of the event
	

	Reference or description of the location of the event 
	

	Description of where any release into the environment took place
	

	Substances(s) potentially released
	

	Best estimate of the quantity or rate of release of substances
	

	Measures taken, or intended to be taken, to stop any emission
	

	Description of the failure or accident.
	


	(b) Notification requirements for the breach of a limit

	To be notified within 24 hours of detection unless otherwise specified below

	Emission point reference/ source
	

	Parameter(s)
	

	Limit
	

	Measured value and uncertainty
	

	Date and time of monitoring
	

	Measures taken, or intended to be taken, to stop the emission
	


	Time periods for notification following detection of a breach of a limit

	Parameter
	Notification period

	
	

	
	

	
	


	(c) Notification requirements for the detection of any significant adverse environmental effect

	To be notified within 24 hours of detection

	Description of where the effect on the environment was detected
	

	Substances(s) detected
	

	Concentrations of substances detected
	

	Date of monitoring/sampling
	


Part B - to be submitted as soon as practicable
	Any more accurate information on the matters for notification under Part A.
	

	Measures taken, or intended to be taken, to prevent a recurrence of the incident
	

	Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission
	

	The dates of any unauthorised emissions from the facility in the preceding 24 months.
	


	Name*
	

	Post
	

	Signature
	

	Date
	


* authorised to sign on behalf of the operator
Schedule 6 - Interpretation 

“accident” means an accident that may result in pollution.

“annually” means once every year.
“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“ATU-BOD as O2” means the biochemical oxygen demand (measured after 5 days at 20o C with nitrification suppressed by the addition of allylthiourea).

“emissions to land” includes emissions to groundwater.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission or background concentration limit.
“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

 “groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“MCERTS” means Natural Resources Wales’s Monitoring Certification Scheme.

“Natural Resources Wales” means the Natural Resources Body for Wales established by article 3 of the Natural Resources Body for Wales (Establishment) Order 2012. The Natural Resources Body for Wales (Functions) Order 2013 transferred the relevant functions of the Countryside Council for Wales, and functions of the Environment Agency and the Forestry Commission in Wales to the Natural Resources Body for Wales.  
“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“year” means calendar year ending 31 December.

[image: image2.jpg]


Schedule 7 - Site plan
© Crown Copyright and database right 2020. Ordnance Survey licence number 100019741.
[image: image3.png]


END OF PERMIT

PAGE  
	Permit Number EPR/BB3595ZK
	  Page ii
-  -
- ii -
- ii -ii 
	



