| Permit Number: | PAN-005578 (South Site) + PAN-005579 (North Site) |
| --- | --- |
| Applicant name: | Mr Stephen Salter |
| Site Address: | Land adjacent to The Start (North and South Site), Hay on Wye, Hereford, HR3 5RS |
| Discharge Point NGR: | South Site: SO 22782 42654 North Site: SO 22936 43000 |
| Respond by date: | 08/07/2019 (10 working days) |
| Permitting Officer: | Elliot Burge |

Brief Description of proposal:

The proposal is to install two package treatment plants and infiltration systems both designed to the relevant British Standards. One PTP and infiltration system will serve the North of the campsite and one will serve the South. The systems will serve a small tent site, reception desk and 2 log cabins, all effluent will be domestic in nature as there are no dishwashers/washing machines on site.

The maximum daily discharge volume for each system will be 2m3/day and there will be a distance of 350+ meters between the two infiltration systems. The applicant has confirmed there will be a 10m+ buffer between the edge of the infiltration systems and the watercourse.

Habitats Screening Report – Designations or protected species/habitats:

The Risk Screening reports can be found in the relevant DMS folders:

PAN-005578 Screening

PAN-005579 Screening

Both screening reports have picked up the designated SAC and SSSI for the River Wye, the North Site has also picked up the River Wye (England) designation. There is also designated Ancient Semi Natural Woodland on the site of the proposal.

The relevant risk assessments will be completed and consulted on if necessary.

Risk Assessment Results (modelling or L1 Risk Assessment):

The scores for both L1 assessments is 32, this is due to their proximity to each other. Therefore, consultation with Geoscience is not required.

PAN-005578 Risk Assessment

PAN-005579 Risk Assessment

Miscellaneous:

There is a sewer catchment across the river, although the Highways Authority has denied the applicant consent to undertake engineering works on the bridge. The nearest connection point is also >100m away for both sites; therefore, it is not necessary for the applicant to demonstrate plausibility of connection to the foul sewer.

NRM Responses:

Environment Management comments:

Site visit carried out with colleagues, it appears that the proposal covers what was discussed onsite.  No further comments to add.

Thank you

Officers name: Julia Lloyd
Date: 26/06/19

Biodiversity comments:

Officers name:
Date:

Fisheries comments:

N/A

Officers name:
Date:

Conservation/Ecology Comments:

I assume that an HRA will be completed in due course, though I am assuming that no risks have been identified at development planning stage. The principle consideration would be Otter but again I assume this was considered at planning.

Advise that manage any non native species observed on site.

Ken Perry

26/6/19

Officers name:
Date:

Internal Consultation Responses from other NRW Teams:

Water Quality Planning:

N/A

Officers name:

Date:

Geoscience Response:

N/A

Officers name:
Date:

Flood Risk Management:

N/A

Officers name:
Date:

| External Consultations sent to: | Yes/No |
| --- | --- |
| National Parks |  |
| Water Company regarding Drinking Water Protected Zone / SPZ1 |  |
| Environmental Heath |  |

Any Permitting Responses:

Date: