| Permit Number: | PAN-009912 |
| --- | --- |
| Applicant name: | Dŵr Cymru Cyfyngedig |
| Site Address: | Hook low level inlet sewage pumping station, Hook, Pembrokeshire, SA62 4LU |
| Discharge Point NGR: | SM 98863 11054 |
| Respond by date: | 12/05/2020 (10 working days) |
| Permitting Officer: | Elliot Burge |

Brief Description of proposal:

This application is the final part of the scheme to expand Hook WwTW. This application is specifically for the inlet pumping station at Hook WwTW, there will be new discharges from a combined sewer overflow (CSO) and emergency overflow (EO) on the asset. The pumping station is designed to pass forward flows of 8l/s, flows in excess of this figure (during storm conditions) will discharge via the CSO, which utilises the same outfall as the final effluent discharge from the WwTW (SM 98863 11054). Event duration monitoring and reporting requirements will be on any permit, should one be issued for the CSO discharge. The EO will only operate during emergency conditions, defined as:

- electrical power failure

- mechanical breakdown of duty and standby pumps

- rising main failure

- blockage of downstream sewer

The EO will also utilise the same outfall if it discharges.

DCWW have submitted a ‘Pumping Station Key Protection Measures’ document which outlines the mitigating measures that will be in place to minimise the impact from the emergency discharge, that document can be found on the DMS here and will be in any permit if one is to be issued.

The scheme is outlined in detail within the Solutions Document on the DMS here.

Habitats Screening Report – Designations or protected species/habitats:

The risk screening report can be found on the DMS here.

The report has highlighted several designations.

As the site was assessed within the HRA for Hook WwTW (PAN-008185) and there is a net reduction in the discharge of storm sewage from the improvement scheme, no new HRA and Appendix 4 will be completed. The decision will be appropriately recorded within the Decision Document.

Risk Assessment Results (modelling or L1 Risk Assessment):

The proposed storm spills can be found on page 11 of the Solutions Document Linked in the first box.

To summarise:

- the asset is predicted to spill 5 times a year

- the total combined volume of these spills will be 22 cubic metres per year

The modelling outputs have been signed off and validated by our Water Quality Planning team.

Miscellaneous:

NRM Responses:

Environment Management comments:

Officers name:
Date:

Biodiversity comments:

Officers name:
Date:

Fisheries comments:

Officers name:
Date:

Conservation/Ecology Comments:

Officers name:
Date:

Internal Consultation Responses from other NRW Teams:

Water Quality Planning:

N/A

Officers name:

Date:

Geoscience Response:

N/A

Officers name:
Date:

Flood Risk Management:

N/A

Officers name:
Date:

| External Consultations sent to: | Yes/No |
| --- | --- |
| National Parks | N |
| Water Company regarding Drinking Water Protected Zone / SPZ1 | N |
| Environmental Heath | N |

Any Permitting Responses:

Date: