Permit Number: EPR/EP3230BW	Operator: EnviroWales Limited Facility:	Rassau Recycling Facility	     Form Number: Sewer1: 15/04/2014

Reporting of emissions to sewer for the period from 01/01/19 to 31/12/2019

| Emission Point | Substance / Parameter | Emission # Limit Value | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | Flow | 350m3/day | Daily average | 139.82 | Meter MCERTs |  |  |
| S1 | Suspended solids | 400 mg/l | Weekly average | 7.68 | BS EN 872 |  |  |
| S1 | Hydrocarbon oil | No visible staining | Daily samples |  | SCA blue book 77 |  |  |
| S1 | pH max | 11 | - | 9.2 | BS ISO 10523 |  |  |
| S1 | pH min | 6 |  | 7.2 |  |  |  |
| S1 | Sulphate | 2000 mg/l | Weekly average | 1009 | SCA blue book 136 |  |  |
| S1 | Antimony and its compounds (as Sb) | 2.5 mg/l | Weekly average | 0.03 | BS EN ISO 15586 |  |  |
| S1 | Arsenic and its compounds (as As) | 2.5 mg/l | Weekly average | 0.002 | BS EN ISO 11969 |  |  |
| S1 | Cadmium and its compounds (as Cd) | 0.25 mg/l | Weekly average | 0.01 | BS EN ISO 5961 |  |  |
| S1 | Copper and its compounds (as Cu) | 12.5 mg/l | Weekly average | 0.02 | BS 6068-2.29 |  |  |
| S1 | Lead and its compounds (as Pb) | 5 mg/l | Weekly average | 0.03 | BS EN ISO 15586 |  |  |
| S1 | Mercury and its compounds (as Hg) | 0.125 mg/l | Weekly average | 0.0022 | BS EN 1483 |  |  |
| S1 | Nickel and its compounds (as Ni) | 12.5 mg/l | Weekly average | 0.01 | BS 6068-2.29 |  |  |
| S1 | Zinc and its compounds (as Zn) | 12.5 mg/l | Weekly average | 0.03 | BS 6068-2.29 |  |  |

The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed        (Authorised to sign as representative of Operator)     Date:31/01/2020