| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-010729 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Ltd |
| Site Address: | Blaenycoed Farm, Ysbyty Ifan, Betws-y-coed, Gwynedd, LL24 0NY |
| Grid Reference: | Farm location: SH 81623 45994 Fields: 1 SH 82096 46327 2 SH 81870 46143 3 SH 82069 46550 4 SH 81853 46334 5 SH 81392 45886 6 SH 81467 46000 7 SH 81123 45900 8 SH 80865 45607 12 SH 80688 45483 20 SH 82392 46972 21 SH 82484 46849 22 SH 82659 47067 Storage: SH 81649 46016 SH 82016 46659 SH 81916 46235 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: TimAmgylcheddDwyforacMeirionnydd@cyfoethnaturiolcymru.gov.uk Environment team: TimAmgylcheddDwyforacMeirionnydd@cyfoethnaturiolcymru.gov.uk NLO: Ewan.Marshall-Harries@environment-agency.gov.uk Archaeological trust: jenny.emmett@heneb.co.uk |
| Date due: | 06/07/2020 |
| Are fields on Anglesey? If yes, then send application. | N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: £1018 Date cleared on MyNRW: 12/06/2020 Transaction number: 2078095 |
| Location map | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| Certificate of Agricultural Benefit | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form | Y N n/a | Comments |
| --- | --- | --- |
| 4a Multiple deployments Qu answered? | Y |  |
| 4b1 Has the NCP name and contact information been given? | Y |  |
| 4c Is the risk banding correct? | Y |  |
| 4d/4e Does the operator need to provide a risk assessment? | Y |  |
| 4f Has all of Table 2- waste types been completed? | Y |  |
| 4f Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| 4f Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Tools used: Google |
| 4g1 Does the farm address match the grid reference? | Y |  |
| 4g2 If it is agricultural land; has the holding number been given? | Y | 53/046/0016 Fields 20,21 and 22 - 53/046/0015 – not on application |
| 4h Has all of Table 3- parcels of land been completed? | Y |  |
| 4h Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 38 ha |
| 4g/4h Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| 4i Has consent been given to carry out the activity? | Y |  |
| 4k Has all of Table 4- previous land treatment been completed? | Y | PAN-005752 Doesn’t expire until 06/08/2020 |
| 4l Has all of Table 5- waste storage details been completed? | Y |  |
| 4l Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 6b/6c Have both Table 6 and Table 7 been completed? | Y |  |
| 8 Has confidentially been claimed? | N |  |
| 8 Has national security been claimed? | N |  |
| 9a If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| 9b Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| 9c Has a relevant person or approved person (Qu 9a) signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | N |  |
| any well, spring or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring, well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring, well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | SPA/SAC/SSSI Migneint-Arenig-Dduallt |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | 1 historic monument within farm buildings. This is a no spread zone. 7 ancient woodland 23 protected habitats mostly located outside of activity area. Lowland dry acid grassland and Lowland calcareous grassland, Purple moor grass and rush pastures, a Lowland dry acid grassland blanket bog intersect with fields 2, 7, 8 and 12. The grasslands are not sensitive to the activity. The other habitats fall within no spread zones. National Parks: site falls within Snowdonia 6 protected species: some of these falls within fields 20 and 21 however, these are aquatic species and therefore located in the adjacent watercourse. There are no spread zones in place around the watercourse to minimise the risk of any negative effects. 2 species of threatened Bryophyte flagged in screening report. Reported to potentially fall within fields 5, 6, 7, 13 and 14. The description states that the Bryophyte’s are found south of the river and in Afon Conwy gorge and therefore, not in the spreading area. All features are located outside of the proposed activity area unless specifically stated otherwise. |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Sludge from alum water treatment should only be applied to soils of pH 6 and above. Only fields which have soil pH 6 or above can receive alum-based sludge (DCWW Bala, DCWW Cilfor, DCWW Dolbenmaen, DCWW Eithin Fynydd, and DCWW Pen y Cefn). |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | N |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y | Some minor rounding errors that do not affect the final decision |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. The applicant has assessed potential risks with regards to having a detrimental effect on the SAC and SPA’s and has put in place measures to minimise these risks. These measures include: - Spreading will only take place subject to ground conditions and following the Code of Good Agricultural Practice (Defra, 2018), NVZ regulations and the permit holder’s Environmental Management System. - All waste will be stored, transported and spread in accordance with conditions set in SR2010No4 permit and CoGAP. - Waste streams have low potential to produce airborne dust and particulate matter - Apply according to PQA, RB209 and Soil Code - 20m non-spreading buffer zone applied to where designated area borders fields and to watercourses that run directly into protected site - Avoid sensitive spreading periods eg. Breeding season - Application during appropriate conditions and awareness of access issue - Spreading will not be carried out in any areas of a field that will be sub-soiled. - Machinery operations will take account of soil conditions, slopes etc. - Liquid spreading machinery will be turned off and lifted away from soil prior to turning at the end of each run. - Machinery will be checked daily when in use, regularly serviced and spreading equipment calibrated. - Umbilical hoses will be regularly checked for damage to prevent leaks. - Machinery turns will not be executed in the buffer strips. - Waste deliveries to field/stores will be supervised. - All spillages will be reported immediately to the NRW. - In prolonged adverse weather, waste will not be collected from the producer unless suitably permitted storage is available, or ground/weather conditions become favourable for land application. - In circumstances where the wastes cannot be stored or spread beyond normal capacities, wastes will be diverted to a local alternative deployment or DCWW sewage treatment works. |  |

Comments

Consultation responses: Archaeological officer has outlined 3 features to which the activity poses a direct threat. RFI sent to request a 10m buffer around these sites. 13/07/2020: Query sent to Archaeological officer regarding one of the features that was highlighted in their report. The Archaeological officer replied the same day and resolved the query.

MyMap and location assessment: No further comments

Agricultural benefit calculations: The Pen y Cefn waste stream sees a phosphate addition of 173.76 kg/ha to fields 3, 7, 8, 9, 11 and 12. This is justified by the fields phosphorus indices of 1. Some of the waste streams have a pH of below 5 which is slightly acidic, the receiving soil have pH ranging from 5.6 to 6.8 and will buffer the waste pH with no detrimental effect anticipated.

Request for information: 08/07/2020: RFI sent to query archaeological features and waterway buffer zones, storage, end date of previous deployment and the agricultural benefit statement. 10/07/2020: RFI response resolving all queries apart from one regarding Archaeological features. Email sent to Archaeological officer on 13/07/2020 to acquire further information. 15/07/2020: Further RFI sent about archaeological features. Applicant replied the same day and resolved the query.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 15/07/2020 |
| ## Effective date | 07/08/2020 |
| ## Permitting Officer Date | Sion Sleep 15/08/2020 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 15/07/2020 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 30 |
| Determination | 810 |
| Sign off | 10 |
| Issuing | 20 |
| Total | 870 |