# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Cerrigllwydion Hall operated by Oracle Environmental Experts Ltd.

The application number is PAN-009783.

The permit number is EPR/BB3792XU.

The permit is for the daily discharge of 20 m3/day of treated contaminated groundwater. The contaminated groundwater will be abstracted and subsequently remediated prior to being discharged directly into ground. The treatment system comprises an oil-water separator and 2 granulated carbon filters. There are monitoring and reporting requirements within the permit.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. |  |
| ### Responses to web publicising | No responses were received The decision was taken in accordance with our guidance. |  |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. |  |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. - The emission limits set within the permit ensure that there is no deterioration to groundwater in line with the Water Framework Directive. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. |  |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. |  |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. The below limits for Benzene, Toluene and Xylenes are set at the “Minimum Reporting Values” to ensure that there are no discernible concentrations of hazardous substances. The limit for naphthalene has been set at 1 µg/l based on what the system can treat to. A limit of Total Petroleum Hydrocarbons has also been included as a ‘capture all’ performance indicator, the limit has been set at 10μg/L based on what the treatment system is expected to achieve: - Benzene (1µg/L) - Toluene (4 µg/L) - Xylenes (3 µg/L) - Napathlene (1 µg/L) - TPH (10 µg/L) |  |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. These monitoring requirements have been imposed in order to ensure the “prevent” requirements of the Environmental Permitting Regulations are met. We made these decisions in accordance with the Environmental Permitting Regulations. |  |
| Reporting | We have specified reporting in the permit. We made these decisions in accordance with the “Prevent” requirements of EPR. Ongoing monitoring and reporting will ensure the operating techniques are valid in order to maintain levels of Benzene, Toluene and Xylenes under Minimum Reporting Values. It will also ensure that Naphthalene is under the EQS. |  |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. |  |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” |  |