Operating Techniques – Mold STW (CM0031001)

Standstill permit limit monitoring and reporting

Mold STW is subject to an AMP7 CHEM ND2 driver which specifies: Prevent further deterioration where there is risk to EQS. Monitoring alternative to standstill limits - Permit conditions for monitoring and reporting, from April 2020.

Dwr Cymru will be required to assess against a calculated Standstill permit limit, failure of which will be counted as a descriptive permit condition failure not a non-compliance. This assessment will take the form of effluent spot sampling together with upstream and downstream river sampling at current OSM frequency. Dwr Cymru will submit a summary report to the NRW on an annual basis and will commit to undertaking an investigation if the limits specified below are exceeded.

Dwr Cymru will look at trends in data to determine if stable or increasing or decreasing. Agreement to look at data and review if increased treatment is needed at AMP8 planning. If analysis of the sampling data indicates a significant threat to achieving the EQS downstream (using the 50/80 rule), appropriate numeric limits shall be derived and applied to the permit from AMP8.

Sampling Programme

| Effluent Discharge Point | Parameter | Limit (including unit) | Reference period | Monitoring Frequency | Compliance Statistic |
| --- | --- | --- | --- | --- | --- |
|  | Fluoranthene | 0.043 ug/l | Instantaneous (spot) | Monthly | 95%ile Compliance to be assessed as descriptive |
|  | Fluoranthene | 2.084 ug/l | Instantaneous (spot) | Monthly | Maximum Compliance to be assessed as Descriptive |

- Sampling will consist of 12 monthly spot samples (as per OSM requirement for non sanitary)

- Exceedance of the 95%ile limit to be subject to look up table

- Dwr Cymru to undertake investigation upon confirmation of the maximum number of samples being exceeded in the Look up table below or an exceedance of the maximum limit.

- Dwr Cymru to provide annual report to NRW 2 months after completion of the sampling programme.

- Details of any investigations to be included within the annual report.

- In addition to Final effluent sampling DCWW shall undertake 12 monthly samples of the watercourse upstream and downstream of the final effluent discharge point at a time to be agreed with the NRW

- Dwr Cymru to consider undertaking downstream river quality samples in yr5 to aid any modelling requirements.

Look Up Table

| Number of Samples taken in any period of 12 months | Maximum number of samples permitted to exceed limit for given parameter |
| --- | --- |
| 4-7 | 1 |
| 8-16 | 2 |
| 17-28 | 3 |