- Application type

New impoundment licence			☐

Removal of an existing impoundment			☐	Complete sections 2 and 4

To licence an existing impoundment				☐

Technical variation to an impoundment licence		☒

WA/055/0001/0007/V001

WA/055/0001/0007/V001

Give existing licence or pre- application reference number(s)

2.	Impoundment details 
All information should correspond with any maps and drawings submitted with this application

## 2.1 Provide details of all impoundment points. For structures spanning a watercourse, provide a National Grid Reference for each bank. If necessary, continue on a separate sheet and tick here to show that you have done this. ☐

| Impoundment location name / reference | National Grid Reference for each bank, looking downstream (12 digit) |  |
| --- | --- | --- |
|  | Left bank | Right bank |
| Intake | 288370 262767 | 288371 262781 |

2.2 Please provide a full description of the impoundment, outlining its purpose and how it will operate. If this information is detailed in a supporting document, provide the document title 
or reference in the space below.

If necessary, continue on a separate sheet and tick here to show that you have done this. ☐

## Hydro scheme intake comprising of intake and rocky ramps. ## Impoundment application is for a variation on a previous licence that was granted in February 2019. The change to the intake will be an increase the level of the intake crest by 0.5m meaning a taller weir and longer ponded reach. The reason is to enable the pipeline to be installed running downhill from the intake.

- Description of impoundment

Afon Clearwen

Afon Clearwen

3.1 Name of watercourse

## 3.2 Will your proposed impoundment result in a change to the wetted perimeter (downstream) or new submerged areas behind (upstream of) the impounding works?
No	☐	Yes	☒ 	If yes, ensure this is shown on any map or drawings submitted

## 3.3 Will the ponded area created by the impoundment be lined?	No	☒	Yes	☐
If yes, give details.

302.5 mOAD

3.4 Give the height of the impoundment structure, from the downstream toe to crest or top of spillway (in metres above Ordnance Datum). If the proposal involves an existing impoundment, state the change in height (in millimetres).

Weir Crest - 302.5 mOAD

Weir Crest - 302.5 mOAD

3.5 What is the overflow or crest level of the impoundment (in metres above Ordnance Datum)?

## 3.6 What is the proposed capacity of the impoundment when full to spillway level (in cubic metres)?

Minimal the bed level behind the weir will be only a small amount below the weir crest.

Minimal the bed level behind the weir will be only a small amount below the weir crest.

## 3.7 Will the proposal create a raised reservoir?				No	☒	Yes	☐

## 3.8 Does the proposal involve the controlled release of water to safeguard downstream flows? 	No		☐	Yes		☒	If yes, provide details

## The weir will only abstract the flow rate being released by Claerwen dam, so that the Arban flows are maintained in the river downstream of the impoundment at all times. This will involve water level sensors installed at the dam foot. The flows will be additionally safeguarded by two notches in the weir, the main notch in the main channel has a rocky ramp and the second notch maintains a flow through the other side of the island.

## 3.9 Is the impounded water to be used for a subsequent purpose? 	No	☐	Yes	☒

## If yes, provide details (for abstractions, state the daily and annual quantities in cubic metres).

## Water is abstracted, max flow rate is 3.7m3/s which means a max abstraction of 13320m3/h, 319680m3/day, and based on 365days – 116,683,200m3/year. ## The abstraction is limited to matching what is released by the Claerwen Dam therefore the abstracted flows will be less that the maximums stated.

## 3.10 How will the impounded area be filled initially, and subsequently refilled if applicable?

## The intake structures construction will by necessity include excavation of the structure footprint and this watercourse material will be both appropriate and sufficient to be used to backfill behind the concrete structure. This material will remain and no subsequent refilling will be needed, the watercourse itself via natural processes will retain a backfilled weir.

- Fish and eel passage
4.1 Confirm the fish species present at your site.

Brown Trout

Brown Trout

## 4.2 Complete the table below with full details of the measures you intend to take to safeguard these fish species.

|  | Intake | Outfall |
| --- | --- | --- |
| Type of fish screen | Coanda Screen | Vertical Bar Screen |
| Screen aperture size (mm) | 2mm | 10mm |
| Screen height and width (mm) | 1.2m x 14.1m | 3.2m x 11.7m |
| Type of upstream fish passage | Rocky Ramp |
| Proposed flow for fish pass (m/s) | 0.040 |
| Type of downstream fish passage | Down rocky ramp and over the top of screens is also navigable. |

- Construction, maintenance and operation

## 5.1 Provide details of maintenance or activities relating to the operation of the impoundment. If necessary, continue on a separate sheet and tick here to show that you have done this. ☐

## The coanda screens are self cleaning but some cleaning with a very long handled brush (eg brush on pole) is required on occasion, the screen will be checked weekly and if necessary, cleaned on this weekly check. In addition, the notches will also be checked and any debris cleared.

5.2 Provide details of diversion works or removal of existing works.

The small island at the intake provides great assistance for the diversion works. Water will be diverted onto one side of the island whilst the intake structure on the other site is constructed. Then water will then be diverted over the built side whilst the other side is constructed. One tonne sand bags lined with visqueen will be used to assist in diverting the water in this manner.

- Planning application

Have you sought advice on your planning application?

No	☐		Yes	☒

If yes, submit a copy of the Planning Authority’s response.

- Declaration

### Please see Guidance Note WRX for details of who can sign this section and note the information in that document relating to the Data Protection Act 1998.

### By signing below, you are declaring that as far as you know and believe the information given in this form, on any map and in any supporting or additional information, is true.

### Signed

Adam Cropper

### Print name

Agent

### Position

17.10.2019

### Date

Application Checklist

Please tick the following checklist items to indicate that you have included the required information. If any sections of the form are left blank and no supporting information submitted, where we have insufficient information to make a decision

on your application, we will return your application to you.

Essential:

Form WRA completed, if there have been any changes since pre-application an updated Form WRA is required										☒

Map with all impoundment points and new wetted perimeters/submerged areas 		☐

clearly marked

Drawings and Design Statement									☐

State number of continuation sheets (enter 0 if none included)

Where relevant:

Form WRD completed, if your proposal also requires an abstraction licence		☐

Stage 1 photo survey (contact us for more details on this requirement)			☐

Letter of authorisation from the applicant, allowing an agent to act as signatory		☐

Further information requested in our pre-application response letter to you			☐

Planning Authority response, where available							☐

Additional supporting information	- please list below: