Rest

Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Mr. Simon Stone

Crownhill Topsoil and Aggregates

Unit 1009

Ministry of Defence

Caerwent Training Area

Dinham Road

Caerwent

Caldicot

NP26 5XL

Permit number

EPR/BB3796HD

Crownhill Topsoil and Aggregates

Permit number EPR/BB3796HD

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.

This is a Tier 3 bespoke permit for a non-hazardous waste physical treatment facility.

Treatment operations are limited to manual and/or mechanical sorting, separation, screening, grading, blending, and bulking up of permitted waste for the purpose of recovery only.

Permitted waste types are predominantly non-hazardous inert wastes to produced recycled aggregates and soils.

The maximum quantity of waste accepted and processed on site shall not exceed 74,999 tonnes per year.

Drainage from the site includes:

- clean, rainfall dependant drainage from areas of the site not used in connection with the storage and/or treatment of waste, and

- integral discharge(s) from the permitted activity from external areas of the site used in connection with the storage and/or treatment of waste in accordance with the limits set out in Table S1.1 and Table S3.1.

There are to be no emissions to surface and/or groundwater from the buildings on site.

The monitoring and reporting for the point source emissions, surface water and ground water are required by Schedules 3 and 4 of this permit.

All emissions from the activity which are not controlled via actual emission limits, are controlled through the ‘emission of substances not controlled by emission limits condition’ and any all relevant process requirements set out in Tables S1.1 and S1.2.

The burning of any wastes, either in the open, inside buildings or in any form of incinerator is not permitted.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application EPR/BB3796HD/A001 | Duly made 04/10/19 | Application for inert waste transfer station with treatment. |
| Additional information received in response to Schedule 5 notice sent 06/12/19 | 17/01/20 | Consisting of: - Schedule 5 Notice Response - Updated Site Layout plan |
|  | 19/01/20 | Consisting of Updated Preliminary Surface and Groundwater Risk Assessment. |
|  | 20/01/20 | Consisting of: - Updated drainage strategy - appendices to the Surface and Groundwater Risk Assessment |
|  | 21/01/20 | Consisting of: - Outline building assessment - Updated Environmental Management System - Site drainage plan - Updated Site Layout plan |
| Additional information received in response to Schedule 5 notice sent 09/04/20 | 15/06/2020 | Consisting of response document (Schedule 5 Notice 09.04.20) and revised: - Document Reference List (CH001 Rev2) - Non-Technical Summary (CH002 Rev 2) - Environmental Management System (CH003 Rev2) - Drainage Strategy (CH005 Rev1) - Ecological Mitigation Plan (CH006 May 2020) - Great Crested Newt Survey (Rev 1 FINAL V2) - Biodiversity Information Search (Crownhill Topsoil report) - Preliminary Ground and Surface water Risk Assessment (CH007 Rev2) - Site Specific Risk Assessment (CH008 ISSUE 1b) - Dust Management Plan (CH009 Rev 1) - Crownhill Noise Calculation (Rev1) - Site Drainage (2020 EV-13-0901-103) - Site Layout (2020 Rev1 EV-13-0901-103) |
|  | 14/07/20 | Consisting of revised: - Environmental Management System (CH003 Rev3) - Noise and Vibration Management Plan (CH010 Rev1) |
|  | 10/08/20 | Consisting of: - Dust Management Plan(CH009 Rev 2) - Noise and Vibration Management Plan (CH010 Rev2) |
| Permit determined EPR/BB3796HD | 24/09/20 | Permit issued to Mr. Simon Stone |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/BB3796HD

The Natural Resources Body for Wales (“Natural Resources Wales”) authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016

Mr. Simon Stone (“the operator”),

of

Crownhill Topsoil and Aggregates

Unit 1009

Ministry of Defence

Caerwent Training Area

Dinham Road

Caerwent

Caldicot

NP26 5XL

to operate waste operations at

Crownhill Topsoil and Aggregates

Unit 1009

Ministry of Defence

Caerwent Training Area

Dinham Road

Caerwent

Caldicot

NP26 5XL

to the extent authorised by and subject to the conditions of this permit.

| Name | Date |
| --- | --- |
| Huw Davies | 24th September 2020 |

Authorised on behalf of Natural Resources Wales

Conditions

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

### The operator shall comply with the requirements of an approved competence scheme.

## Avoidance, recovery and disposal of wastes produced by the activities

### The operator shall take appropriate measures to ensure that:

### the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied to the generation of waste by the activities; and

### any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

### The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

## 2.2	The site

2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

## 2.3	Operating techniques

#### 2.3.1	(a)	The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan, and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

### 2.3.2	Waste shall only be accepted if:

### (a)	it is of a type and quantity listed in schedule 2 table(s) S2.1 [, S2.2 etc.]; and

### (b)	it conforms to the description in the documentation supplied by the producer and holder.

## 2.4	Pre-operational conditions

2.4.1	The activities shall not be brought into operation until the measures specified in schedule 1 table S1.3 have been completed.

# 3	Emissions and monitoring

## 3.1	Emissions to water, air or land

### 3.1.1	There shall be no point source emissions to water, air or land except from the sources and emission points listed in schedule 3 tables S3.1.

### 3.1.2	The limits given in schedule 3 shall not be exceeded.

## Emissions of substances not controlled by emission limits

### Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

## 3.3	Odour

#### 3.3.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

3.3.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to odour, submit to Natural Resources Wales for approval within the period specified, an odour management plan which identifies and minimises the risks of pollution from odour;

#### (b)	implement the approved odour management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.4	Noise and vibration

#### 3.4.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

## 3.5	Monitoring

### 3.5.1	The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

#### point source emissions specified in tables S3.1;

#### surface water or groundwater specified in table S3.2;

3.5.2	The operator shall maintain records of all monitoring required by this permit including records of the taking and analysis of samples, instrument measurements (periodic and continual), calibrations, examinations, tests and surveys and any assessment or evaluation made on the basis of such data.

3.5.3	Monitoring equipment, techniques, personnel and organisations employed for the emissions monitoring programme and the environmental or other monitoring specified in condition 3.3.1 shall have either MCERTS certification or MCERTS accreditation (as appropriate), where available, unless otherwise agreed in writing by Natural Resources Wales.

3.5.4	Permanent means of access shall be provided to enable sampling/monitoring to be carried out in relation to the emission points specified in schedule 3 tables S3.1 and S3.2 unless otherwise agreed in writing by Natural Resources Wales.

# 4	Information

## 4.1	Records

### All records required to be made by this permit shall:

### be legible;

### be made as soon as reasonably practicable;

### if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

### be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- off-site environmental effects; and

- matters which affect the condition of the land and groundwater.

4.1.2	The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

## 4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

### 4.2.2	Within one month of the end of each quarter, the operator shall submit to Natural Resources Wales using the form made available for the purpose, the information specified on the form relating to the site and the waste accepted and removed from it during the previous quarter.

### 4.2.3	Within 28 days of the end of the reporting period the operator shall, unless otherwise agreed in writing by Natural Resources Wales, submit reports of the monitoring and assessment carried out in accordance with the conditions of this permit, as follows:

- in respect of the parameters and emission points specified in schedule 4 table S4.1;

- for the reporting periods specified in schedule 4 table S4.1 and using the forms specified in schedule 4 table S4.2; and

- giving the information from such results and assessments as may be required by the forms specified in those tables.

## 4.3	Notifications

### 4.3.1	Natural Resources Wales shall be notified without delay following the detection of:

### any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

### the breach of a limit specified in the permit; or

### any significant adverse environmental effects.

### 4.3.2	Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

### 4.3.3	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.4	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

#### any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

#### any change in the operator’s name or address; and

#### any steps taken with a view to the dissolution of the operator.

#### In any other case:

#### the death of any of the named operators (where the operator consists of more than one named individual);

#### any change in the operator’s name(s) or address(es); and

#### any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.5	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

### Natural Resources Wales shall be notified at least 14 days before making the change; and

### the notification shall contain a description of the proposed change in operation.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone.

Schedule 1 - Operations

| ### Table S1.1 activities |  |  |
| --- | --- | --- |
| ### Activity reference | ### Description of activities for waste operations | ### Limits of activities |
| ### A1- Non-hazardous waste physical treatment facility | ### R13: Storage of waste pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced) ### R3: Recycling/reclamation of organic substances which are not used as solvents ### R5: Recycling/reclamation of other inorganic material | All waste must be stored and treated on an impermeable surface with sealed drainage. Any building used for the storage or treatment of waste must be enclosed on all sides and be maintained to prevent the ingress of rain or surface water. Treatment operations shall be limited to manual and/or mechanical: - sorting - separation - screening - crushing - grading - blending - bulking up of permitted waste for the purpose of recovery only. Storage of wastes listed in Table S2.1 shall not exceed 40,000 tonnes in total at any one time. ### Waste types as specified in Table S2.1. |
|  | ### Discharge of integral site drainage (process effluent) from the emission points indicated on the plan in Schedule 7. | Drainage consisting solely of: - clean, rainfall dependant drainage from areas of the site not used in connection with the storage and/or treatment of waste - run-off from the external areas of the site used to store and/or treat waste via a series of attenuation ponds and class 1 full retention hydrocarbon interceptor (point source emission as set out in Table S3.1). No visible sediments, oil, or grease shall be present in the discharge. There shall be no discharge from the buildings on site used to store and/or treat waste. |

| ### Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| ### Description | ### Parts | ### Date Received |
| ### Technical Guidance Document: ‘How to comply with your environmental permit’ | ### All relevant sections | ### N/A |
| ### Sector Guidance Note IPPC S5.06: Guidance for the Recovery and Disposal of Hazardous and Non-Hazardous Waste | ### All relevant sections | ### N/A |
| ### Response to Schedule 5 Notice dated 09/04/20 | ### CH002 - Non-Technical Summary Rev 2 ### Section: - 5.1 Process- paragraphs 1, 2 and 3 | ### 15/06/20 |
|  | ### CH005 - Drainage Strategy Rev1 ### Sections: - Attenuation Pond Inspection and Dredging |  |
|  | ### CH006 - Ecological Mitigation Plan - May 2020 ### Sections: - 6.1.1 Exclusion of Great Crested Newts - 6.1.6 Monitoring and Maintenance of Great Crested Newt Exclusion Fencing - 6.1.7 Contingencies in the event of a breach of the fence |  |
|  | ### CH003 - Environmental Management System Rev3 ### Sections: - 10.1.1 waste acceptance - 13.2.1 Attenuation Pond Inspection and Dredging | ### 14/07/20 |
|  | ### CH10- Noise and Vibration Control Plan – Rev 2 ### Sections: - all | ### 10/08/20 |
| ### Additional information | ### CH009 - Dust Management Plan- Rev 3 ### Sections: - all | ### In line with PO1 |

| ### Table S1.3 Pre-operational measures |  |
| --- | --- |
| ### Reference | ### Pre-operational measures |
| ### PO1 | ### Dust Management Plan - Complete dust baseline monitoring survey as set out in responses to Schedule 5 notice dated 09/04/20. - Update Dust Management Plan with details of training given for dust monitoring - Update Dust Management Plan including reference to the diary system for logging dust monitoring dates - Update Dust Management Plan with the results of the baseline monitoring survey. - Submit the updated Dust Management Plan to Natural Resources Wales for audit purposes before operations can begin. |

# Schedule 2 - Waste types, raw materials and fuels

| ### Table S2.1 Permitted waste types and quantities for Non-hazardous waste physical treatment facility |  |
| --- | --- |
| Maximum quantity | The maximum quantity for waste to be accepted on site shall not exceed 74,999 tonnes per year. |
| Exclusions | Notwithstanding the waste types set out in this table, wastes having any of the following characteristics shall not be accepted: - consisting solely or mainly of dusts, powders or loose fibres - sludges - odorous or odour producing - hazardous |
| ### Waste code | ### Description |
| ### 01 | ### WASTES RESULTING FROM EXPLORATION, MINING, QUARRYING, AND PHYSICAL AND CHEMICAL TREATMENT OF MINERALS |
| ### 01 04 | ### wastes from physical and chemical processing of non-metalliferous minerals |
| ### 01 04 08 | ### waste gravel and crushed rocks other than those mentioned in 01 04 07 |
| ### 01 04 09 | ### waste sand and clays |
| ### 10 | ### WASTES FROM THERMAL PROCESSES |
| ### 10 11 | ### wastes from manufacture of glass and glass products |
| ### 10 11 12 | ### waste glass other than those mentioned in 10 11 11 (consisting of clean glass only) |
| ### 10 12 | ### wastes from manufacture of ceramic goods, bricks, tiles and construction products |
| ### 10 12 08 | ### waste ceramics, bricks, tiles and construction products (after thermal processing) |
| ### 10 13 | ### wastes from manufacture of cement, lime and plaster and articles and products made from them |
| ### 10 13 14 | ### waste concrete and concrete sludge (consisting of waste concrete only) |
| ### 15 | ### WASTE PACKAGING, ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| ### 15 01 | ### packaging (including separately collected municipal packaging waste) |
| ### 15 01 07 | ### glass packaging (consisting of clean glass only) |
| ### 17 | ### CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| ### 17 01 | ### concrete, bricks, tiles and ceramics |
| ### 17 01 01 | ### concrete |
| ### 17 01 02 | ### bricks |
| ### 17 01 03 | ### tiles and ceramics |
| ### 17 01 07 | ### mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 06 |
| ### 17 02 | ### wood, glass and plastic |
| ### 17 02 02 | ### glass (consisting of clean glass only) |
| ### 17 03 | ### bituminous mixtures, coal tar and tarred products |
| ### 17 03 02 | ### bituminous mixtures other than those mentioned in 17 03 01 (consisting of road base and road planings (other than those containing coal tar) only) |
| ### 19 | ### WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND THE PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION AND WATER FOR INDUSTRIAL USE |

| ### 19 12 | ### wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| --- | --- |
| ### 19 12 05 | ### glass (consisting of clean glass only) |
| ### 19 12 09 | ### minerals (for example sand, stones) |
| ### 20 | ### MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| ### 20 01 | ### separately collected fractions (except 15 01) |
| ### 20 01 02 | ### glass (consisting of clean glass only) |
| ### 20 02 | ### garden and park wastes (including cemetery waste) |
| ### 20 02 02 | ### soil and stones |

# Schedule 3 – Emissions and monitoring

| ### Table S3.1 Point Source emissions to water (other than sewer) and land – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit (incl. unit) | ### Reference Period | ### Monitoring frequency | ### Monitoring standard or method |
| Discharge SW101 at NGR ST4634392074 | ### Total suspended solids | ### Outfall from interceptor | ### 60 mg/l | ### Instantaneous (spot sample) | ### Monthly ### During times of intense rainfall | ### As agreed in writing with Natural Resources Wales |
|  | ### Total petroleum hydrocarbons |  | ### 0.09 mg/l |  |  |  |

| ### Table S3.2 Surface water or groundwater monitoring requirements |  |  |  |
| --- | --- | --- | --- |
| ### Location or description of point of measurement | ### Parameter | ### Monitoring frequency | ### Monitoring standard or method |
| SW101 at NGR ST4634392074 SW103 at NGR ST4602290726 SW104 at NGR ST4605690679 | - Total suspended solids - Total dissolved solids - Hydrocarbons- EPH (C10-C40) - Hydrocarbons- PAH16 - Temperature (oC) - pH - Electrical Conductivity - Redox Potential - Biological oxygen demand - Ammoniacal-nitrogen - Chloride | Monthly | ### CH003 - Environmental Management System Rev2- methodology set out in section 10.1.6 Sampling Methodology |
|  | - Chemical oxygen demand - Ammonia - Arsenic - Boron - Cadmium - Chromium - Copper (dissolved and bioavailable) - Iron - Mercury - Manganese (dissolved and bioavailable) - Nickel (dissolved and bioavailable) - Lead (dissolved and bioavailable) - Zinc (dissolved and bioavailable) - TPH (speciated) | Twice yearly |  |
| - BH07 | - Total suspended solids - Total dissolved solids - Hydrocarbons- EPH (C10-C40) - Hydrocarbons- PAH16 - Temperature (oC) - pH - Electrical Conductivity (μS) - Redox Potential (mV) - Biological oxygen demand - Chemical oxygen demand - Choride - Ammonia - Ammoniacal Nitrogen - Arsenic - Boron - Cadmium - Chromium - Copper (dissolved and bioavailable) - Iron - Mercury - Manganese (dissolved and bioavailable) - Nickel (dissolved and bioavailable) - Lead (dissolved and bioavailable) - Zinc (dissolved and bioavailable - TPH (speciated) | Monthly | ### CH003 - Environmental Management System Rev2- methodology set out in section 10.1.6: - Groundwater on Site - Groundwater Off Site ### Environmental Quality Standards (EQS) ### Drinking Water Standards (DWS) |
| - BH101 - BH102 - BH104 - BH105 - BH106 - BH107 - BH130 |  | ### Twice yearly |  |

# Schedule 4 - Reporting

### Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

| ### Table S4.1 Reporting of monitoring data |  |  |  |
| --- | --- | --- | --- |
| ### Parameter | ### Emission or monitoring point/reference | ### Reporting period | ### Period begins |
| ### Emissions to water - Parameters as required by condition 3.5.1 | - SW101 at NGR ST4634392074 | ### Every 3 months for monthly testing ### Every 6 months for twice yearly testing | ### 01 January ### 01 April ### 01 July ### 01 October |
| ### Surface water monitoring - Parameters as required by condition 3.5.1 | - SW101 at NGR ST4634392074 - SW103 at NGR ST4602290726 - SW104 at NGR ST4605690679 |  |  |
| ### Groundwater monitoring - Parameters as required by condition 3.5.1 | - BH07 - BH101 - BH102 - BH104 - BH105 - BH106 - BH107 - BH130 |  |  |

| ### Table S4.2 Reporting forms |  |  |
| --- | --- | --- |
| ### Media/parameter | ### Reporting format | ### Date of form |
| ### Point source emissions | ### Form Water1 or other form as agreed in writing by Natural Resources Wales | ### 24/09/2020 |
| ### Surface water | ### Form Surfacewater1 or other form as agreed in writing by Natural Resources Wales |  |
| ### Groundwater | ### Form Groundwater1 or other form as agreed in writing by Natural Resources Wales |  |

# Schedule 5 - Notification

### These pages outline the information that the operator must provide.

### Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

## Part A

| ## Permit Number |  |
| --- | --- |
| ## Name of operator |  |
| ## Location of Facility |  |
| ## Time and date of the detection |  |

| ## (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Date and time of the event |  |
| ## Reference or description of the location of the event |  |
| ## Description of where any release into the environment took place |  |
| ## Substances(s) potentially released |  |
| ## Best estimate of the quantity or rate of release of substances |  |
| ## Measures taken, or intended to be taken, to stop any emission |  |
| ## Description of the failure or accident. |  |

| ## (b) Notification requirements for the breach of a limit |  |
| --- | --- |
| ## To be notified within 24 hours of detection unless otherwise specified below |  |
| ## Emission point reference/ source |  |
| ## Parameter(s) |  |
| ## Limit |  |
| ## Measured value and uncertainty |  |
| ## Date and time of monitoring |  |
| ## Measures taken, or intended to be taken, to stop the emission |  |

| ### Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| ### Parameter | ### Notification period |

| ## (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Description of where the effect on the environment was detected |  |
| ## Substances(s) detected |  |
| ## Concentrations of substances detected |  |
| ## Date of monitoring/sampling |  |

## Part B - to be submitted as soon as practicable

| ### Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| ### Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| ### Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| ### The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| ### Name* |  |
| --- | --- |
| ### Post |  |
| ### Signature |  |
| ### Date |  |

### * authorised to sign on behalf of the operator

Schedule 6 - Interpretation

“accident” means an accident that may result in pollution.

“Annex II” means Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

“building” means a construction that has the objective of providing sheltering cover and minimising emissions of noise, particulate matter, odour and litter.

“emissions to land” includes emissions to groundwater.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission or background concentration limit.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“groundwater protection zones 1 and 2” have the meaning given in the document titled Groundwater Protection: Policy and Practice" published by the Environment Agency in 2006.

“hazardous property” has the meaning in Annex lll of the Waste Framework Directive.

“hazardous substance” means a substance classified as hazardous as a consequence of fulfilling the criteria laid down in parts 2 to 5 of Annex I to Regulation (EC) No 1272/2008.

“hazardous waste” has the meaning given in the Hazardous Waste (Wales) Regulations 2005 (as amended).

“impermeable surface” means a surface or pavement constructed and maintained to a standard sufficient to prevent the transmission of liquids beyond the surface.

“MCERTS” means the Environment Agency’s Monitoring Certification Scheme.

“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“R” means a recovery operation provided for in Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“Waste code” means the six digit code referable to a type of waste in accordance with the list of wastes established by Commission Decision 2000/532/EC as amended from time to time (the ‘List of Wastes Decision’) and in relation to hazardous waste, includes the asterisk.

“Waste Framework Directive” or “WFD” means Waste Framework Directive 2008/98/EC of the European Parliament and of the Council on waste.

“year” means calendar year ending 31 December.

Schedule 7 - Site plan

END OF PERMIT

Permit Number: EPR/BB3796HD			Operator: Mr. Simon Stone

Facility: Crownhill Topsoil and Aggregates	Form Number: Water1

# Reporting of emissions to water (other than to sewer) and land for the period from DD/MM/YYYY to DD/MM/YYYY

| Emission Point | Substance / Parameter | Emission Limit Value | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)

Permit Number: EPR/BB3796HD			Operator: Mr. Simon Stone

Facility: Crownhill Topsoil and Aggregates	Form Number: Surfacewater1

# Reporting of groundwater monitoring for the period from DD/MM/YYYY to DD/MM/YYYY

| Monitoring Point | Substance / Parameter | # Trigger level | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)

Permit Number: EPR/BB3796HD			Operator: Mr. Simon Stone

Facility: Crownhill Topsoil and Aggregates	Form Number: Groundwater1

# Reporting of surface water monitoring for the period from DD/MM/YYYY to DD/MM/YYYY

| Monitoring Point | Substance / Parameter | # Trigger level | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)