[image: image1.jpg]° Cyfoeth
Naturiol
Cymru

Natural
Resources
Wales




OWNER/OCCUPIER – NOTICE OF INTENTION TO CARRY OUT OR PERMIT AN OPERATION LIKELY TO DAMAGE SPECIAL INTEREST ON A SITE OF SPECIAL SCIENTIFIC INTEREST (SSSI)

This form should be used by owners and occupiers of SSSI intending to carry out, or give permission for someone else to carry out, one or more activities listed on the list of Operations Likely to Damage the Special Interest (OLDSI list) for the site. On older sites this will be called the PDO list.
	Name of SSSI 
	Traeth Lafan SSSI (is located directly adjacent Llanfairfechan sea wall for the first 110m section of sea wall on the western side).


I/We hereby give notice under Section 28E (1) of the Wildlife and Countryside Act 1981 that I/we intend to carry out, cause or permit to be carried out the operation(s) specified below:-

	1.
	Details of proposed operation. Attach method statements, photographs or diagrams if these will help to explain your proposals. (Operations may be changes in management practices)

	The sea wall is of masonry construction and 1052m long, stones at the toe of the wall have dislodged following recent stormy weather and need replacing urgently before the wall is damaged further. There are twenty six small voids which needs repairing the exact details can be found in the Contractor method statement in the appendix. Maintaining the sea wall is paramount to keeping the North Wales Coastline Railway open. The works are expected to start as soon as all consents are to hand. Concrete for the works will be delivered in small ready mixed concrete wagon which will be banked onto the causeway. A small static concrete pump will be towed by pick up to the works location and all vehicles and pump equipment will remain on the concrete causeway path. 

The concrete causeway path will be cleaned down to remove algae/seaweed at start of each shift to ensure the path is safe to walk/drive on. This will be undertaken using a jet wash/bowser using fresh water, towed with pick up vehicle. Areas of work will be inspected by operatives and the voids will be prepared as required with the use of hand held breakers/sthil saws where loose material requires breaking/cutting out and removing. A small pump will be on hand should small pockets of water require pumping out of the repair area. Water will generally be pumped/discharged into a suitable drainage point on the causeway which won’t affect the area of repair. Concrete will be discharged into the voids via the concrete pump. Operatives will control the pump hose and monitor the discharge of concrete. The concrete will be trowel finished once void has been filled. Smaller voids will require use of a concrete mortar repair. This material will be mixed in buckets on the causeway and carried down in buckets to the repair location. Hand trowels will be used to apply the mortar repair to the damaged area. Plant/Equipment will generally consist of a small pickup type vehicle, water bowser/jet wash, Concrete mini mix wagon, static towable concrete pump, small hand held tools such as mechanical breakers, sthil saw, 2.6kva generator & small water pump.



	2.
	I/We intend to carry out this operation on the area of land detailed on the enclosed method statement. 

	
	The location(s) of the sea wall is SH68707 75684 to SH69650 76126 near Llanfairfechan, Conwy. 



	
	Please see Initial Environmental Appraisal, HRA, WFD and Method Statement Document Appendix for site photographs and method statement for whote details of the works.


* Tick as appropriate

You must supply enough information to allow us to locate the land where you wish to carry out the work. If your notice does not specify where the operation will take place we will be unable to accept it. An annotated map is most helpful. Please also include details of any relevant secondary works, such as access routes, material storage or disposal locations.
	3
	I/We propose to start the operation (or implement the changes) on/after
	As soon as all consents are received. 


	
	Work will continue until (if there is no end date strike out this section)
	For 3 weeks from start. 



	Note: Only owners and occupiers of the site can apply for consent. If you are not the owner or occupier, you will need to ask them for permission. The owner or occupier will then need to notify us and get consent before they give you permission.



	Name:
	Mr Andrew Mc Monagle

	Address :
	Alun Griffiths (Contractor) Ltd

Waterways House

Merthyr Road

Llanfoist

Abergavenny

Monmouthshire

NP7 9PE
Acting for: Network Rail

No. 1 Capital Quarter

Tyndall Street

Cardiff

CF10 4BZ

	Contact phone number:
	D +441873 857211

	Email:
	andrew.mcmonagle@alungriffiths.co.uk 

	Date:
	16/09/2020


Natural Resources Wales must process your application within 4 months of receipt of a duly made notice. We will usually be able to respond much more quickly than this if sufficient information is included.
Please return to SSSI.notices@cyfoethnaturiolcymru.gov.uk  or your local Natural Resources Wales office. 
