## Schedule 5 Notification

These pages outline the information that the operator must provide.

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A

| Permit Number | EPR/AB3092CV |
| --- | --- |
| Name of operator | Parc Adfer Operations Limited |
| Location of Facility | Parc Adfer Energy Recovery Facility Deeside |
| Time and date of the detection | 2nd October 2020 at 11:30 hrs to 11:59hrs |

| (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Date and time of the event | 2nd October 2020 at 11:30 hrs to 11:59hrs |
| Reference or description of the location of the event | At approximately 10:50 there was a sudden decline in the boiler steam flow and furnace temperature. Manual intervention of the combustion controller attempted to reverse this decline, but this was not successful. At 11:35 due to the continued instability in combustion conditions the combustion support burners were put into service. At 11:40 the poor combustion conditions continued and the resultant rise in CO resulted in an exceedance of the CO half-hour emission limit. During the investigation it was discovered that there was a blockage in the Ash Discharger. |
| Description of where any release into the environment took place | Plant emission point A1. |
| Substances(s) potentially released | Carbon Monoxide |
| Best estimate of the quantity or rate of release of substances | On the 2nd October 2020 the CEMS system showed a High Level of Carbon Monoxide emission for the period 11:30hrs to 12:00hrs approximately. |
| Measures taken, or intended to be taken, to stop any emission | With combustion conditions deteriorating the oil burners were fired at 11:35 and the decision to stop waste feeding and close the feed chute dampers was taken. The initial attempts to close the feed chute dampers failed due to waste in the feed chute. The feed chute dampers were finally closed at 12:28 and the boiler taken off-line. A safety inspection of the boiler was then carried out, which revealed a major blockage in the Ash Discharger and Discharger chute. |
| Description of the failure or accident. | A change in the quality of waste being fed to the system, as illustrated by the rapid decrease in the Net Calorific Value of the feedstock which resulted in an automatic increase in the feed rate due to the combustion on-time controller attempting to achieve the average IR temperature setpoint. This resulted in an over feeding of waste on to the combustion grate and the exacerbation of the poor combustion conditions. |

| (b) Notification requirements for the breach of a limit |  |
| --- | --- |
| To be notified within 24 hours of detection unless otherwise specified below |  |
| Emission point reference/ source | A1 stack - breach of ELV for CO for one HHA period. |
| Parameter(s) | Carbon Monoxide (CO) |
| Limit | 100 mg/Nm³ |
| Measured value and uncertainty | 371.24mg/NMm3 +- 10% |
| Date and time of monitoring | 02nd October 2020 at 11:30 hrs to 11:59hrs |
| Measures taken, or intended to be taken, to stop the emission | Initial use of oil burners followed by the cessation od waste feeding when the burners failed to adequately support the combustion process. Boiler was taken off-line for investigation. |

| Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| Parameter | Notification period |
| CO ELV 100 mg/m3 | 24 hours |

| (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Description of where the effect on the environment was detected | No effect on the environment detected |
| Substances(s) detected | None |
| Concentrations of substances detected | N/A |
| Date of monitoring/sampling | 02/10/20 |

Part B - to be submitted as soon as practicable

| Any more accurate information on the matters for notification under Part A. | A significant change in the moisture content of the waste combined with a concurrent blockage in one of the ash discharger units temporarily compromised the burn process. As soon as this was noticed the plant was brought off line. |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident | The moisture content was due to the intense downpours at the time. The ash discharger blockage was cleared and the plant kept off-line until the system was clear and normal operation could be safely reinitiated. |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission | N/A |
| The dates of any unauthorised emissions from the facility in the preceding 24 months. | 28th January 2020 8th February 2020 20th February 2020 23rd March 2020 11:30 to 11:59hrs 23rd March 2020 15:00 to 15:29hrs 7th April 21:00am to 21.29am 8th April 00:20am to 0.29am 8th April 10:00am to 10.29am 21st April 2020 at 15:00hrs to 15:29hrs 1st May 2020 at 07:49hrs 1st May 2020 at 14:09hrs 3rd May 2020 CO 15:00 to 15:29hrs140.92mg/Nm³. 30th May 2020 CO 15:00 to 15:29hrs 110.16mg/Nm³. 26th June 2020 CO 06:10 to 06:19hrs of 152.23mg/Nm³. 13th August 2020 CO 15:00 hrs to 15:29 hrs of 108.52 mg/Nm³. 20th August 2020 Lime spill of approximately 200kg during lime silo filling from a road tanker. |

| Name* | Jonathan Farebrother |
| --- | --- |
| Post | Plant Manager |
| Signature |  |
| Date | 16/10/2020 |

* authorised to sign on behalf of the operator