| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-011680 |
| Permit Reference Number: | EPR-KP3594VE |
| Permit Holder: | Trade Effluent Services Ltd |
| Site Address: | Brick Kiln Farm, Tremerichion, Denbighshire, CH7 5BL |
| Grid Reference: | Farm location: SJ 10667 75181 Fields: Brick Kiln 1 SJ 10797 75323 Brick Kiln 2 SJ 10704 75430 Brick Kiln 3 SJ 10492 75240 Brick Kiln 4 SJ 10380 75228 Brick Kiln 5 SJ 10440 75057 Brick Kiln 6 SJ 10186 74972 Brick Kiln 7 SJ 10189 75132 Rhualt 1 SJ 08806 74861 Rhualt 2 SJ 08498 74772 Rhualt 4 SJ 08475 74974 Storage: 1 SJ 17918 72521 2 SJ 08912 74947 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: DenbighshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Amy.Henderson@cyfoethnaturiolcymru.gov.uk Archaeological trust: abi.mccullough@cpat.org.uk |
| Date due: | 16/10/2020 |
| Are fields on Anglesey? If yes, then send application. | N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: £1018.00 Date cleared on MyNRW:02/10/20 Transaction number: 2081980 |
| Location map | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| Certificate of Agricultural Benefit | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form | Y N n/a | Comments |
| --- | --- | --- |
| 4a Multiple deployments Qu answered? | Y |  |
| 4b1 Has the NCP name and contact information been given? | Y |  |
| 4c Is the risk banding correct? | Y |  |
| 4d/4e Does the operator need to provide a risk assessment? | Y |  |
| 4f Has all of Table 2- waste types been completed? | Y |  |
| 4f Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| 4f Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Tools used: DMS |
| 4g1 Does the farm address match the grid reference? | Y |  |
| 4g2 If it is agricultural land; has the holding number been given? | Y | 56/202/0037 |
| 4h Has all of Table 3- parcels of land been completed? | Y |  |
| 4h Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 36.1 ha |
| 4g/4h Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| 4i Has consent been given to carry out the activity? | Y |  |
| 4k Has all of Table 4- previous land treatment been completed? | n/a |  |
| 4l Has all of Table 5- waste storage details been completed? | Y |  |
| 4l Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 6b/6c Have both Table 6 and Table 7 been completed? | Y |  |
| 8 Has confidentially been claimed? | N |  |
| 8 Has national security been claimed? | N |  |
| 9a If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| 9b Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| 9c Has a relevant person or approved person (Qu 9a) signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | Y | Brick keln fields and Rhualt 1 |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring(or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | SSSI Mwyn gloddfa |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | Brick Keln fields are in an SPZ1 and nitrogen vulnerable zone. 2 SINCs nearby no issue. |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Fats and oils, conductivity |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | Y |  |
| - Included the nutrient level in their environmental risk assessment | Y |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. |  |

Comments

Consultation responses: Comments made by waste reg officer observing SPZ1 covering most fields. Also mentioned the fields slope.

MyMap and location assessment: https://mapsinternal.cyfoethnaturiolcymru.gov.uk/Html5Viewer2101/index.html?viewer=MyMap_Permitting.MyMapPermittingHTML5&project=e98843d5837343eebb652b4bf8e3ab9a Majority of fields are in an SPZ1 zone and will have to be removed.

Agricultural benefit calculations: Closely match those of the applicant. Will deliver a maintenance amount of phosphate, nitrogen, potash and magnesium.

Request for information: Sent on the 15/10/20 Applicants application had several mistakes including tables from other apps, duplicate lab reports and no SNS data on field nitrogen. Waste stream- Croda widnes is being spread at 243t/ha but is giving, only 5 mg/kg of P, 9 mg/kg of K, sodium is above permissible level. fields in Brick Kiln and Rhault 1 are in a ground water SPZ1 and need to be removed. Applicant initially responded on 21/10/20- requesting to withdraw application, however assessment had been completed and applicant was not entitled to a refund. Learning this the applicant requested to continue processing. Covering officer Sion Sleep responded the same day. I responded to applicant on 27/10/20 confirming he could respond to RFI and I would assess response. Response on 28/10/20-With amended documents and answers to my main queries however the elimination of fields required me to reassess the fields. Following this I requested additional justification for spreading on slopes and buffering around a well that was 25ms from spreading area. Applicant response on 29/10/20 confirming my requests.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 03/11/20 |
| ## Effective date | 03/11/20 |
| ## Permitting Officer Date | Ethan Banfield 03/11/20 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 03/11/2020 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 30 |
| Determination | 750 |
| Sign off | 10 |
| Issuing |  |
| Total |  |