| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-012057 |
| Permit Reference Number: | EPR-KP3594VE |
| Permit Holder: | Trade Effluent Services Ltd |
| Site Address: | Brookhouse Farm Hanmer Whitchurch SY13 3EG |
| Grid Reference: | Farm location: SJ 4679339953 Fields: 34 SJ 45877 39282 35 SJ 45887 39440 37 SJ 46073 39385 38 SJ 46189 39507 36 SJ 45776 39493 31 SJ 46249 39200 30 SJ 46185 39075 29 SJ 45925 39067 18 SJ 45934 38779 25 SJ 45643 38479 16/17 SJ 46195 38708 26 SJ 45824 38386 39 SJ 46495 39354 32 SJ 46122 39207 Storage: SJ 4613639311 SJ 4613639311 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: North East WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: Wrexham & Flintshire WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Amy Henderson Amy.Henderson@cyfoethnaturiolcymru.gov.uk Archaeological trust: Clwyd- Powys abi.mccullough@cpat.org.uk |
| Date due: | 04/12/2020 |
| Are fields on Anglesey? If yes, then send application. | N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: £1018 Date cleared on MyNRW: 09/11/2020 Transaction number: 2084012 |
| Location map | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| Certificate of Agricultural Benefit | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form | Y N n/a | Comments |
| --- | --- | --- |
| 4a Multiple deployments Qu answered? | Y |  |
| 4b1 Has the NCP name and contact information been given? | Y |  |
| 4c Is the risk banding correct? | Y |  |
| 4d/4e Does the operator need to provide a risk assessment? | Y |  |
| 4f Has all of Table 2- waste types been completed? | Y |  |
| 4f Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| 4f Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Tools used: Google |
| 4g1 Does the farm address match the grid reference? | Y |  |
| 4g2 If it is agricultural land; has the holding number been given? | Y | 56/241/0025 |
| 4h Has all of Table 3- parcels of land been completed? | Y |  |
| 4h Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 50 ha |
| 4g/4h Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| 4i Has consent been given to carry out the activity? | Y |  |
| 4k Has all of Table 4- previous land treatment been completed? | Y | PAN-006671 |
| 4l Has all of Table 5- waste storage details been completed? | Y |  |
| 4l Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 6b/6c Have both Table 6 and Table 7 been completed? | Y |  |
| 8 Has confidentially been claimed? | N |  |
| 8 Has national security been claimed? | N |  |
| 9a If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y | The applicant hasn’t ticked 9a however they have given written permission to act on the relevant persons behalf. |
| 9b Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| 9c Has a relevant person or approved person (Qu 9a) signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring(or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | 1 Ramsar sites 2 SSSIs: Hanmer Mere and Llyn Bedydd |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | 19 listed buildings 17 historic monuments 2 scheduled ancient monuments 9 ancient woodland – 2 of which are adjacent to the proposed spreading area however, the applicant has put a 10m no spread zone in place, meaning there is no significant risk that the woodland will be detrimentally effected 4 local wildlife site (SINCs) – 1 of which is adjacent to the proposed spreading area however, the applicant has put a 10m no spread zone in place, meaning there is no significant risk that the site will be detrimentally affected The site is partially within a NVZ 4 protected habitats 1 protected species All features are listed outside of the proposed spreading area unless specifically stated otherwise |
| What is the slope of the fields? | There is a low risk of run-off because the proposed activity area is relatively flat with only a small amount of moderately sloping terrain (7-12 degrees) and steeply sloping terrain (>12 degrees). The applicant also plans to predominantly apply the waste subsoil injection which will decrease the likelihood of run-off further. |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Fats, oils and grease |
| - If yes; have they been tested for? | N | Although the applicant has not carried out the special testing requirements, they have justified this in their agricultural benefit statement and RFI. |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | Y |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | Y |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? | Y |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. The proposed permission is not likely to damage any of the flora, fauna or geological or physiological features which are of special interest because of the mitigation strategies set out below: - Hanmer Mere SSSI is 115m to the west of field 36 - Llyn Bedydd SSSI is located 40m to the east of field 39 - Waste spread in accordance with COGAP - Operations carried out using best available techniques - Spreading to be carried out during appropriate conditions - Regular checks of watercourses in the area surrounding the spreading operations - No travelling on a SSSI or protected site - No activities to occur inside the boundary without written permission - ESA clearly marked on deployment maps |  |

Comments

Consultation responses: No relevant comments

MyMap and location assessment: No further comments.

Agricultural benefit calculations: Although the Kelloggs and Rowan Foods waste streams have low pH (4.18 and 4.02 respectively) the soil analysis provided with this application show that the fields have sufficient buffering capacity.

Request for information: 07/12/20: RFI sent to query the spreading map, storage, agricultural benefit statement and Site-Specific Risk Assessment. 07/12/20: The applicant responded however, there were still some outstanding queries regarding the spreading map and agricultural benefit statement. 08/12/2020: Further RFI sent to clarify outstanding queries. 08/12/2020: The applicant responded clarifying all outstanding queries.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 10/12/2020 |
| ## Effective date | 10/12/2020 |
| ## Permitting Officer Date | Sion Sleep 10/12/2020 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 09/12/2020 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 45 |
| Determination | 750 |
| Sign off | 10 |
| Issuing | 15 |
| Total | 820 |