Water Framework Directive assessment: scoping template for activities in estuarine and coastal waters

Use this template to record the findings of the scoping stage of your Water Framework Directive (WFD) assessment for an activity in an estuary or coastal water.

If your activity will:

- take place in or affect more than one water body, complete a template for each water body

- include several different activities or stages as part of a larger project, complete a template for each activity as part of your overall WFD assessment

The WFD assessment guidance for estuarine and coastal waters will help you complete the table.

| Your activity | Description, notes or more information |
| --- | --- |
| Applicant name | Brian Briggs |
| Application reference number (where applicable) |  |
| Name of activity | Improvement of water control structures on Saline Lagoons at WWT Llanelli Wetland Centre |
| Brief description of activity | Works to the Saline Lagoons will repair the damaged banks and install a new water management system to improve water level control and reduce erosion pressures on the lagoon bunds by allowing water to move more quickly in and out of the lagoons during spring tides. The new water management system will replace the current system of twinwall pipes which is failing, and will consist of: - Two separate sluices, one in each of the two largest saline lagoons. Each of these sluices will consist of a 450mm diameter pipe installed within the bund and connecting two concrete headwalls measuring 2.5x2m on either side of the bund. These sluices will each have a footprint of around 10 square metres. To reduce erosion below the outfall spillway from each sluice, 10 tonnes of stone will also be installed, in addition to 4 gabion baskets. - An excavator will be used to repair banks, through minor re-profiling, around the saline lagoons where high tides and storms have caused damage or erosion. |
| Location of activity (central point XY coordinates or national grid reference) | Ordnance Survey National Grid Reference SS 52784 98280 |
| Footprint of activity (ha) | 0.002ha (20 square metres) |
| Timings of activity (including start and finish dates) | Two weeks’ work in total, undertaken between January and April 2021 |
| Extent of activity (for example size, scale frequency, expected volumes of output or discharge) | Each of the four headwalls will measure around 2.5m length x 2m width x 2m height, and the 450mm diameter twinwall connecting pipes will measure around 10m in length. 20 tonnes of stone will be installed to reduce erosion below the outfall slipways. |
| Use or release of chemicals (state which ones) | N/A |

| Water body1 | Description, notes or more information |
| --- | --- |
| WFD water body name | Burry Inlet Inner |
| Water body ID | GB531005913500 |
| River basin district name | Western Wales |
| Water body type (estuarine or coastal) | Estuarine |
| Water body total area (ha) | 11.17 km |
| Overall water body status (2015) | Poor |
| Ecological status | Poor |
| Chemical status | Good |
| Target water body status and deadline | Good Status by 2027 |
| Hydromorphology status of water body | Not Designated A/HMWB |
| Heavily modified water body and for what use | No |
| Higher sensitivity habitats present | Saltmarsh, intertidal substrate, seagrass beds, Nationally Important Intertidal Habitats (Wales) |
| Lower sensitivity habitats present | intertidal soft sediments like sand and mud |
| Phytoplankton status | Poor |
| History of harmful algae | Unknown |
| WFD protected areas within 2km | Carmarthen Bay and Estuaries SAC Burry Inlet SPA Burry Inlet Ramsar Site Burry Inlet and Loughor Estuary SSSI |

1 Water body information can be found in the Environment Agency’s catchment data explorer and the water body summary table. Magic maps provide additional information on habitats and protected areas. Links to these information sources can be found in the WFD assessment guidance for estuarine and coastal waters.

Specific risk information

Consider the potential risks of your activity to each of these receptors: hydromorphology, biology (habitats and fish), water quality and protected areas. Also consider invasive non-native species (INNS).

Section 1: Hydromorphology

Consider if hydromorphology is at risk from your activity.

Use the water body summary table to find out the hydromorphology status of the water body, if it is classed as heavily modified and for what use.

| Consider if your activity: | Yes | No | Hydromorphology risk issue(s) |
| --- | --- | --- | --- |
| Could impact on the hydromorphology (for example morphology or tidal patterns) of a water body at high status | Requires impact assessment | Impact assessment not required | N/A |
| Could significantly impact the hydromorphology of any water body | Requires impact assessment | Impact assessment not required | N/A |
| Is in a water body that is heavily modified for the same use as your activity | Requires impact assessment | Impact assessment not required | N/A |

Record the findings for hydromorphology and go to section 2: biology.

Section 2: Biology

Habitats

Consider if habitats are at risk from your activity.

Use the water body summary table and Magic maps, or other sources of information if available, to find the location and size of these habitats.

| Higher sensitivity habitats 2 | Lower sensitivity habitats 3 |
| --- | --- |
| chalk reef | cobbles, gravel and shingle |
| clam, cockle and oyster beds | intertidal soft sediments like sand and mud |
| intertidal seagrass | rocky shore |
| maerl | subtidal boulder fields |
| mussel beds, including blue and horse mussel | subtidal rocky reef |
| polychaete reef | subtidal soft sediments like sand and mud |
| Saltmarsh YES |  |
| subtidal kelp beds |  |
| subtidal seagrass |  |

2 Higher sensitivity habitats have a low resistance to, and recovery rate, from human pressures.

3 Lower sensitivity habitats have a medium to high resistance to, and recovery rate from, human pressures.

| Consider if the footprint4 of your activity is: | Yes | No | Biology habitats risk issue(s) |
| --- | --- | --- | --- |
| 0.5km2 or larger | Yes to one or more – requires impact assessment | No to all – impact assessment not required | Within 500m of saltmarsh habitat |
| 1% or more of the water body’s area |  |  |  |
| Within 500m of any higher sensitivity habitat |  |  |  |
| 1% or more of any lower sensitivity habitat |  |  |  |

4 Note that a footprint may also be a temperature or sediment plume. For dredging activity, a footprint is 1.5 times the dredge area.

Fish

Consider if fish are at risk from your activity, but only if your activity is in an estuary or could affect fish in or entering an estuary.

| Consider if your activity: | Yes | No | Biology fish risk issue(s) |
| --- | --- | --- | --- |
| Is in an estuary and could affect fish in the estuary, outside the estuary but could delay or prevent fish entering it or could affect fish migrating through the estuary | Continue with questions | Go to next section | N/A |
| Could impact on normal fish behaviour like movement, migration or spawning (for example creating a physical barrier, noise, chemical change or a change in depth or flow) | Requires impact assessment | Impact assessment not required | N/A |
| Could cause entrainment or impingement of fish | Requires impact assessment | Impact assessment not required | N/A |

Record the findings for biology habitats and fish and go to section 3: water quality.

Section 3: Water quality

Consider if water quality is at risk from your activity.

Use the water body summary table to find information on phytoplankton status and harmful algae.

| Consider if your activity: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| Could affect water clarity, temperature, salinity, oxygen levels, nutrients or microbial patterns continuously for longer than a spring neap tidal cycle (about 14 days) | Requires impact assessment | Impact assessment not required | N/A |
| Is in a water body with a phytoplankton status of moderate, poor or bad | Requires impact assessment | Impact assessment not required | N/A |
| Is in a water body with a history of harmful algae | Requires impact assessment | Impact assessment not required | N/A |

Consider if water quality is at risk from your activity through the use, release or disturbance of chemicals.

| If your activity uses or releases chemicals (for example through sediment disturbance or building works) consider if: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| The chemicals are on the Environmental Quality Standards Directive (EQSD) list | Requires impact assessment | Impact assessment not required | N/A |
| It disturbs sediment with contaminants above Cefas Action Level 1 | Requires impact assessment | Impact assessment not required | N/A |

| If your activity has a mixing zone (like a discharge pipeline or outfall) consider if: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| The chemicals released are on the Environmental Quality Standards Directive (EQSD) list | Requires impact assessment5 | Impact assessment not required | N/A |

5 Carry out your impact assessment using the Environment Agency’s surface water pollution risk assessment guidance, part of Environmental Permitting Regulations guidance.

Record the findings for water quality go on to section 4: WFD protected areas.

Section 4: WFD protected areas

Consider if WFD protected areas are at risk from your activity. These include:

| - special areas of conservation (SAC) | - bathing waters |
| --- | --- |
| - special protection areas (SPA) | - nutrient sensitive areas |
| - shellfish waters |  |

Use Magic maps to find information on the location of protected areas in your water body (and adjacent water bodies) within 2km of your activity.

| Consider if your activity is: | Yes | No | Protected areas risk issue(s) |
| --- | --- | --- | --- |

| Within 2km of any WFD protected area6 | Requires impact assessment | Impact assessment not required | Yes – it is within the: Carmarthen Bay and Estuaries SAC Burry Inlet SPA Burry Inlet Ramsar Site Burry Inlet and Loughor Estuary SSSI |
| --- | --- | --- | --- |

6 Note that a regulator can extend the 2km boundary if your activity has an especially high environmental risk.

Record the findings for WFD protected areas and go to section 5: invasive non-native species.

Section 5: Invasive non-native species (INNS)

Consider if there is a risk your activity could introduce or spread INNS.

Risks of introducing or spreading INNS include:

- materials or equipment that have come from, had use in or travelled through other water bodies

- activities that help spread existing INNS, either within the immediate water body or other water bodies

| Consider if your activity could: | Yes | No | INNS risk issue(s) |
| --- | --- | --- | --- |
| Introduce or spread INNS | Requires impact assessment | Impact assessment not required | No – vehicles and equipment will not have been used in other water bodies |

Record the findings for INNS and go to the summary section.

Summary

Summarise the results of scoping here.

| Receptor | Potential risk to receptor? | Note the risk issue(s) for impact assessment |
| --- | --- | --- |
| Hydromorphology | No |  |
| Biology: habitats | Yes | Saltmarsh habitat |
| Biology: fish | No |  |
| Water quality | No |  |
| Protected areas | Yes | Within an SPA/SAC/Ramsar/SSSI site |
| Invasive non-native species | No |  |

If you haven’t identified any receptors at risk during scoping, you don’t need to continue to the impact assessment stage and your WFD assessment is complete.

If you’ve identified one or more receptors at risk during scoping, you should continue to the impact assessment stage.

Include your scoping results in the WFD assessment document you send to your activity’s regulator as part of your application for permission to carry out the activity.