Part A

| Permit Number |  | PP31339GB |
| --- | --- | --- |
| Name of Operator |  | Enovert (North) Ltd |
| Location of Installation |  | Hafod Quarry, Bangor Road, Johnstown, Wrexham, LL14 6ET |
| Time and date of the detection |  | When data entered into Enovert database |
| (b) Notification requirements for the breach of a limit (within 24 hours unless otherwise agreed) |  |  |
| Emission point reference/ source | BH HA Perimeter Gas Monitoring Points Methane (%) CH4 >1% See Table 1 below. Uncertainty is +/- 0.5% |  |
| Parameter(s) |  |  |
| Limit |  |  |
| Measured value and uncertainty |  |  |
| Date and time of monitoring |  |  |
| Measures taken, or intended to be taken, to stop the emission | Perimeter Gas Monitoring will continue in accordance with Environmental Permit PP3139GB |  |

Part B (to be supplied as soon as practicable)

| Any more accurate information on the matters for notification under Part A. | A review of the historic Methane data for the boreholes that breached the compliance limit this month, reveal that these respective boreholes have consistently demonstrated elevated Methane concentrations throughout this time. |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident | Upon review and collation of the latest perimeter gas results, there were 11 breaches of the Methane compliance limit. Breaches where the methane to carbon dioxide ratio is much greater than or much less than 1.5 is very unlikely to be landfill gas. Much of the gas seen is believed to be mine gas. |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| The dates of any unauthorised emissions from the installation in the preceding 24 months. | Please see previous notifications. |
| Name* | Elizabeth Forrest |
| Post | Environmental Data Analyst |
| Date | 11.01.21 |

* authorised to sign on behalf Enovert (North) Ltd

| Sample Point | Date | CH4 (%) |
| --- | --- | --- |
| HAF GB04b | 15/12/2020 | 33.9 |
| HAF GB06a | 15/12/2020 | 40.2 |
| HAF GB07a | 15/12/2020 | 22 |
| HAF GB07b | 15/12/2020 | 3.8 |
| HAF GB08 | 15/12/2020 | 4.6 |
| HAF GB09 | 15/12/2020 | 22.3 |
| HAF GB11 | 15/12/2020 | 5.8 |
| HAF GB12 | 15/12/2020 | 19.8 |
| BH HA 10(B) | 15/12/2020 | 7.9 |
| BH HA 10(M) | 15/12/2020 | 16.6 |
| BH HA 10(T) | 15/12/2020 | 22.8 |

Table 1: Perimeter Gas Breaches (Methane) December 2020