Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Worldcare Recycling Limited

Worldcare Recycling

Plot 2

Tre Marl Industrial Estate

Conwy

LL31 9PN

Permit number

EPR/AB3998CJ

Worldcare Recycling

Permit number EPR/AB3998CJ

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.

This is a tier 2 bespoke permit based on standard rule set SR2010 No12 - Treatment of waste to produce soil, soil substitutes and aggregate.

This permit allows the operator to store waste at a specified location and treat it to produce soil, soil substitutes and aggregate.

Permitted wastes do not include hazardous wastes.

The total quantity of waste that can be stored and subsequently treated at the site under this permit shall be no more than 75,000 tonnes per year.

This permit does not permit the burning of any wastes, either in the open, inside buildings or in any form of incinerator.

If the site is located within a groundwater Source Protection Zone 1 or 2, then the wastes listed in Table S2.2 shall be stored and treated on an impermeable surface with a sealed drainage system. Wastes listed in Table S2.1 may be stored and treated on hard-standing or on an impermeable surface with sealed drainage system.

If the site is located outside a groundwater Source Protection Zone 1 or 2, then wastes listed in table S2.1 and S2.2 may be stored and treated on hard-standing or on an impermeable surface with sealed drainage system.

This permit does not allow any point source emission into surface waters or groundwater. However, under the emissions of substances not controlled by emission limits condition:

- Liquids may be discharged into a foul sewer subject to a consent issued by the local water company.

- Liquids may be taken off-site in a tanker for disposal or recovery.

- Clean surface water from roofs, or from areas of the site that are not being used in connection with storing and treating waste, may be discharged directly to surface waters, or to groundwater by seepage through the soil via a soakaway.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application EPR/AB3998CJ/A001 | Duly made 23/07/18 | Application for tier 2 bespoke permit based on SR2010 No12 - Treatment of waste to produce soil, soil substitutes and aggregate. |
| Additional information received in response to Schedule 5 notice | 21/09/18 | Confirmation of continuing completence |
| Permit determined | 02/10/18 | Permit issued to Worldcare Recycling Limited |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/AB3998CJ

The Natural Resources Body for Wales (“Natural Resources Wales”) authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016

Worldcare Recycling Limited (“the operator”),

whose registered office is

7 Maes Hyfryd

Glan Conwy

Colwyn Bay

Clwyd

Wales

LL28 5NF

company registration number 09628222

to operate waste operations at

Worldcare Recycling

Plot 2

Tre Marl Industrial Estate

Conwy

LL31 9PN

to the extent authorised by and subject to the conditions of this permit.

| Name | Date |
| --- | --- |
| Huw Davies | 02/10/2018 |

Authorised on behalf of Natural Resources Wales

Conditions

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

### The operator shall comply with the requirements of an approved competence scheme.

## Avoidance, recovery and disposal of wastes produced by the activities

### The operator shall take appropriate measures to ensure that:

### the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied to the generation of waste by the activities; and

### any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

### The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

## 2.2	The site

2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

## 2.3	Operating techniques

#### 2.3.1	(a)	The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan , and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

### 2.3.2	Waste shall only be accepted if:

### (a)	it is of a type and quantity listed in schedule 2 table(s) S2.1 [, S2.2 etc.];

### (b)	it conforms to the description in the documentation supplied by the producer and holder;

### (c) 	its chemical, physical and biological characteristics make it suitable for its intended treatment;

(d) 	any excavated soil from potentially contaminated sites has been shown by prior chemical analysis and assessment to be suitable for the intended use without significant risk of pollution; and

(e)	it is visually inspected on arrival and before it enters the treatment process to ensure that it complies with this permit.

2.3.3	 Any waste that does not comply with 2.3.2 shall be rejected and shall be:

(a)	removed from the site; or

(b)	moved to a designated quarantine area pending removal.

2.3.4 	Records demonstrating compliance with conditions 2.3.2 and 2.3.3 (including analysis and assessment of any excavated soil from potentially contaminated sites) shall be maintained.

# 3 	Emissions and monitoring

## 3.1	Emissions of substances not controlled by emission limits

### Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

3.1.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

#### (b)	implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3.1.3	All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

## 3.2	Odour

#### 3.2.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

3.2.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to odour, submit to Natural Resources Wales for approval within the period specified, an odour management plan which identifies and minimises the risks of pollution from odour;

#### (b)	implement the approved odour management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.3	Noise and vibration

#### 3.3.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

3.3.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to noise and vibration, submit to Natural Resources Wales for approval within the period specified, a noise and vibration management plan which identifies and minimises the risks of pollution from noise and vibration;

#### (b)	implement the approved noise and vibration management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

# 4	Information

## 4.1	Records

### All records required to be made by this permit shall:

### be legible;

### be made as soon as reasonably practicable;

### if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

### be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- off-site environmental effects; and

- matters which affect the condition of the land and groundwater.

4.1.2	The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

## 4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

4.2.2	Within one month of the end of each quarter, the operator shall submit to Natural Resources Wales using the form made available for the purpose, the information specified on the form relating to the site and the waste accepted and removed from it during the previous quarter.

## 4.3	Notifications

### 4.3.1	Natural Resources Wales shall be notified without delay following the detection of:

### any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

### the breach of a limit specified in the permit; or

### any significant adverse environmental effects.

### 4.3.2	Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

### 4.3.3	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.4	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

#### any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

#### any change in the operator’s name or address; and

#### any steps taken with a view to the dissolution of the operator.

#### In any other case:

#### the death of any of the named operators (where the operator consists of more than one named individual);

#### any change in the operator’s name(s) or address(es); and

#### any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.5	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

### Natural Resources Wales shall be notified at least 14 days before making the change; and

### the notification shall contain a description of the proposed change in operation.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone.

# Schedule 1 - Operations

| Table S1.1 activities |  |
| --- | --- |
| Description of activities for waste operations | Limits of activities |
| R13: Storage of waste pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where the waste is produced) R3: Recycling/reclamation of organic substances which are not used as solvents (including composting and other biological transformation processes) R5: Recycling/reclamation of other inorganic materials | Treatment of wastes listed in tables S2.1 and S2.2 consisting only of: - sorting; - separation; - screening; - crushing; and - blending of waste for recovery as a soil, soil substitute or aggregate. Secure storage of wastes listed in tables S2.1 and S2.2 pending treatment. Storage of wastes listed in table S2.1 shall not exceed 40,000 tonnes in total at any one time. Storage of wastes listed in table S2.2 shall not exceed 10,000 tonnes in total at any one time. If the site is located within a groundwater Source Protection Zones 1 or 2, then: - wastes listed in Table S2.2 shall be stored and treated on an impermeable surface with a sealed drainage system, and - wastes listed in Table S2.1 may be stored and treated on hard-standing or on an impermeable surface with sealed drainage system. If the site is located outside a groundwater Source Protection Zones 1 or 2, then wastes listed in tables S2.1 and S2.2 may be stored and treated on hard-standing or on an impermeable surface with sealed drainage system. Treatment of slags and ashes for recovery or a mix of recovery and disposal shall not exceed 75 tonnes per day. Waste types as specified in Table S2.1 and S2.2. |

| Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| Description | Parts | Date Received |
| Guidance Note EPR 1 How to comply with your environmental permit | All relevant sections | N/A |
| Sector Guidance Note IPPC S5.06: Guidance for the Recovery and Disposal of Hazardous and Non Hazardous Waste | All relevant sections | N/A |
| Application | Tier 2 Bespoke Locational Assessment, Version 1.0 | 09/07/18 |
| Application | Operating Techniques and Non Technical Summary, Ref: CEC/NT/WC/3, Version 2.0: Sections 2.1.3; 2.1.4; and 2.1.5 | 21/07/18 |

Schedule 2 - Waste types, raw materials and fuels

| Table S2.1 Permitted waste types and quantities |  |
| --- | --- |
| Maximum quantity | The maximum quantity for waste to be accepted on site shall not exceed 75,000 tonnes per year. |
| Exclusions | Wastes having any of the following characteristics shall not be accepted: - consisting solely or mainly of dusts, powders or loose fibres - hazardous wastes - wastes in liquid form |
| Waste code | Description |
| 01 | WASTES RESULTING FROM EXPLORATION, MINING, QUARRYING, AND PHYSICAL AND CHEMICAL TREATMENT OF MINERALS |
| 01 04 | wastes from physical and chemical processing of non-metalliferous minerals |
| 01 04 08 | waste gravel and crushed rocks other than those mentioned in 01 04 07 |
| 01 04 09 | waste sand and clays |
| 02 | WASTES FROM AGRICULTURE, HORTICULTURE, AQUACULTURE, FORESTRY, HUNTING AND FISHING, FOOD PREPARATION AND PROCESSING |
| 02 02 | wastes from the preparation and processing of meat, fish and other foods of animal origin |
| 02 02 02 | animal-tissue waste (consisting of shellfish shells from which the soft tissue or flesh has been removed only) |
| 10 | WASTES FROM THERMAL PROCESSES |
| 10 01 | wastes from power stations and other combustion plants (except 19) |
| 10 01 02 | coal fly ash (consisting of pulverised fuel ash only) |
| 10 11 | wastes from manufacture of glass and glass products |
| 10 11 12 | waste glass other than those mentioned in 10 11 11 (consisting of clean glass only) |
| 10 12 | wastes from manufacture of ceramic goods, bricks, tiles and construction products |
| 10 12 08 | waste ceramics, bricks, tiles and construction products (after thermal processing) |
| 10 13 | wastes from manufacture of cement, lime and plaster and articles and products made from them |
| 10 13 14 | waste concrete and concrete sludge (consisting of waste concrete only) |
| 15 | WASTE PACKAGING, ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | packaging (including separately collected municipal packaging waste) |
| 15 01 07 | glass packaging (consisting of clean glass only) |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 01 | concrete, bricks, tiles and ceramics |
| 17 01 01 | concrete |
| 17 01 02 | bricks |
| 17 01 03 | tiles and ceramics |
| 17 01 07 | mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 06 |
| 17 02 | wood, glass and plastic |
| 17 02 02 | glass (consisting of clean glass only) |
| 17 03 | bituminous mixtures, coal tar and tarred products |
| 17 03 02 | bituminous mixtures other than those mentioned in 17 03 01 (consisting of road base and road planings (other than those containing coal tar) only) |
| 17 05 | soil (including excavated soil from contaminated sites), stones and dredging spoil |
| 17 05 04 | soil and stones other than those mentioned in 17 05 03 |
| 17 05 08 | track ballast other than those mentioned in 17 05 07 |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND THE PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION AND WATER FOR INDUSTRIAL USE |
| 19 08 | wastes from waste water treatment plants not otherwise specified |
| 19 08 02 | waste from desanding (consisting of washed sewage grit free from sewage contamination only) |
| 19 08 99 | wastes not otherwise specified (consisting of stone filter media if free from sewage contamination only) |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 05 | glass (consisting of clean glass only) |
| 19 12 09 | minerals (for example sand, stones) |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | separately collected fractions (except 15 01) |
| 20 01 02 | glass (consisting of clean glass only) |
| 20 02 | garden and park wastes (including cemetery waste) |
| 20 02 02 | soil and stones |

| Table S2.2 Permitted waste types and quantities |  |
| --- | --- |
| Maximum quantity | The maximum quantity for waste to be accepted on site shall not exceed 75,000 tonnes per year. |
| Exclusions | Wastes having any of the following characteristics shall not be accepted: - consisting solely or mainly of dusts, powders or loose fibres - hazardous wastes - wastes in liquid form |
| Waste code | Description |
| 03 | WASTES FROM WOOD PROCESSING AND THE PRODUCTION OF PANELS AND FURNITURE, PULP, PAPER AND CARDBOARD |
| 03 01 | wastes from wood processing and the production of panels and furniture |
| 03 01 01 | waste bark and cork |
| 03 03 | wastes from pulp, paper and cardboard production and processing |
| 03 03 01 | waste bark and wood |
| 10 | WASTES FROM THERMAL PROCESSES |
| 10 01 | wastes from power stations and other combustion plants (except 19) |
| 10 01 01 | bottom ash, slag and boiler dust (excluding boiler dust mentioned in 10 01 04) (consisting of bottom ash and slag only) |
| 10 01 05 | calcium-based reaction wastes from flue-gas desulphurisation in solid form (consisting of gypsum only) |
| 10 01 07 | calcium-based reaction wastes from flue-gas desulphurisation in sludge form (consisting of gypsum only) |
| 10 01 15 | bottom ash, slag and boiler dust from co-incineration other than those mentioned in 10 01 14 (consisting of bottom ash and slag only from co-incineration only) |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 05 | soil (including excavated soil from contaminated sites), stones and dredging spoil |
| 17 05 06 | dredging spoil other than those mentioned in 17 05 05 |
| 17 08 | gypsum-based construction material |
| 17 08 02 | gypsum-based construction materials other than those mentioned in 17 08 01 (consisting of gypsum only) |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND THE PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION AND WATER FOR INDUSTRIAL USE |
| 19 05 | wastes from aerobic treatment of solid wastes |
| 19 05 03 | off-specification compost (consisting of compost from source segregated biodegradable waste only) |
| 19 09 | wastes from the preparation of water intended for human consumption or water for industrial use |
| 19 09 02 | sludges from water clarification |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 12 | other wastes (including mixtures of materials) from mechanical treatment of wastes other than those mentioned in 19 12 11 (consisting of treated bottom ash including IBA and slag other than that containing dangerous substances only) |
| 19 13 | wastes from soil and groundwater remediation |
| 19 13 02 | solid wastes from soil remediation other than those mentioned in 19 13 01 |
| 19 13 04 | sludges from soil remediation other than those mentioned in 19 13 03 |

Schedule 3 - Emissions and monitoring

There are no emission limits or associated monitoring requirements.

Schedule 4 - Reporting

There is no reporting under this schedule.

# Schedule 5 - Notification

These pages outline the information that the operator must provide.

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A

| Permit Number |  |
| --- | --- |
| Name of operator |  |
| Location of Facility |  |
| Time and date of the detection |  |

| (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Date and time of the event |  |
| Reference or description of the location of the event |  |
| Description of where any release into the environment took place |  |
| Substances(s) potentially released |  |
| Best estimate of the quantity or rate of release of substances |  |
| Measures taken, or intended to be taken, to stop any emission |  |
| Description of the failure or accident. |  |

| (b) Notification requirements for the breach of a limit |  |
| --- | --- |
| To be notified within 24 hours of detection unless otherwise specified below |  |
| Emission point reference/ source |  |
| Parameter(s) |  |
| Limit |  |
| Measured value and uncertainty |  |
| Date and time of monitoring |  |
| Measures taken, or intended to be taken, to stop the emission |  |

| Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| Parameter | Notification period |

| (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Description of where the effect on the environment was detected |  |
| Substances(s) detected |  |
| Concentrations of substances detected |  |
| Date of monitoring/sampling |  |

Part B - to be submitted as soon as practicable

| Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| Name* |  |
| --- | --- |
| Post |  |
| Signature |  |
| Date |  |

* authorised to sign on behalf of the operator

Schedule 6 - Interpretation

“accident” means an accident that may result in pollution.

“Annex II” means Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission or background concentration limit.

“emissions to land” includes emissions to groundwater.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater protection zones 1 and 2” have the meaning given in the document titled "Groundwater Protection: Policy and Practice" published by the Environment Agency in 2006.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“hardstanding” is a compacted solid surface capable of withstanding the operation and the loading / unloading of wastes.

“hazardous property” has the meaning in Annex lll of the Waste Framework Directive

“hazardous substance” means a substance classified as hazardous as a consequence of fulfilling the criteria laid down in parts 2 to 5 of Annex I to Regulation (EC) No 1272/2008

“hazardous waste” has the meaning given in the Hazardous Waste (Wales) Regulations 2005 (as amended)

“IBA” means incinerator bottom ash derived from the incineration or pyrolosis of waste.

“impermeable surface” means a surface or pavement constructed and maintained to a standard sufficient to prevent the transmission of liquids beyond the pavement surface, and should be read in conjunction with the term “sealed drainage system” (below).

“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“R” means a recovery operation provided for in Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“sealed drainage system” in relation to an impermeable surface, means a drainage system with impermeable components which does not leak and which will ensure that:

- no liquid will run off the surface otherwise than via the system;

- except where they may lawfully be discharged to foul sewer, all liquids entering the system are collected in a sealed sump.

“secure storage” means storage where waste cannot escape and members of the public do not have access to it.

“waste code” means the six digit code referable to a type of waste in accordance with the list of wastes established by Commission Decision 2000/532/EC as amended from time to time (the ‘List of Wastes Decision’) and in relation to hazardous waste, includes the asterisk.

“Waste Framework Directive” or “WFD” means Waste Framework Directive 2008/98/EC of the European Parliament and of the Council on waste.

“year” means calendar year ending 31 December.

Schedule 7 - Site plan

END OF PERMIT