# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Garwnant operated by Forest Holidays .

The application number is PAN-011902.

The permit number is EPR/BB3790CL

The permit has been varied to add hot tub effluent through the same treatment system as sewage effluent increasing the total volume discharged to 58.5 m3/day. This is an increase of 8.5m3.

The discharge point will move to a point immediately downstream of the Llwyn-on dam, approximately 3km downstream of the current discharge point: SO 00462 14015. Therefore, the discharge point is below the DCWW abstraction point and not within the drinking water protected area as it is now.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

During the determination of this permit variation, the UK left the EU.  However, reference is made in this document to various EU directives, which as implemented in UK domestic legislation are still applicable.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation, responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. |  |
| ### Responses to consultation | The consultation responses (Annex 2) were taken into account in the decision. No responses were received from web publicising. The decision was taken in accordance with our guidance. |  |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. |  |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. |  |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the CRoW Act. | The application is within the relevant distance criteria of the protected species: White-clawed Freshwater Crayfish. We have not formally consulted on the application. The decision was taken in accordance with our guidance. |  |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site by virtue of the location of the project. There are no Natura 2000 or RAMSAR sites located within a 10km screening distance which are hydrologically connected to the discharge. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment required additional National Resources Wales assessment. In addition to the water quality modelling carried out as part of the operator’s H1 Risk Assessment, Natural Resources Wales carried out its own water quality modelling. This was carried out in order to assess the impact of the discharge on the receiving watercourse and calculate the required limits for Ammonia, BOD, Phosphorus and Bromide. Our modelling concluded that the limits proposed by the applicant were acceptable. Please see “PAN-011902 WFD Assessment.” The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment or similar methodology supplied by the operator and reviewed by ourselves, all emissions may be categorised as environmentally insignificant. |  |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. |  |
| The permit conditions |  |  |  |
| Incorporating the application | We have specified that the applicant must operate the permit in accordance with descriptions in the application, including all additional information received as part of the determination process. These are specified in the Operating Techniques table in the permit. |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - ATU-Bod as O2 – 10 mg/l as a Maximum. - Suspended solids (measured after drying at 105oC) – 30 mg/l as Maximum - Ammoniacal nitrogen (expressed as N) – 9 mg/l as a Maximum - Bromide set a maximum of 1 mg/l The above limits have been set to protect the receiving Taf Fawr water course in line with our WFD assessment. These limits correspond with the limits within the applicant’s H1 Assessment. |  |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. These monitoring requirements have been imposed in order to comply with TGN 7.01 Section 3 Requirements. The operator will be required to carry out flow monitoring in accordance with these requirements. We made these decisions in accordance with the Environmental Permitting Regulations 2016 and TGN EPR 7.01. “How to comply with your permit. Based on the information in the application we are satisfied that the operator’s techniques, personnel and equipment have either MCERTS certification or MCERTS accreditation as appropriate. |  |
| Reporting | We have specified reporting in the permit. Reporting frequencies are in line with requirements set out in EPR 7.01. We made these decisions in accordance with the Environmental Permitting Regulations 2016 and TGN 7.01. |  |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. |  |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” |  |

# Annex 2: Consultation responses

Summary of responses to consultation, and the way in which we have taken these into account in the determination process.

Response received from

Brecon Beacons National Park Authority

Brief summary of issues raised

Response received on 15/02/2021 stating that they have no comments to make in regard to the permit application on the basis that the details shown are as those approved by the Local Planning Authority.

Summary of actions taken or show how this has been covered

If the foul discharge details change from what was approved by the Local Planning Authority the applicant will need to ensure that they have the appropriate permissions in place prior to discharge.