| Permit Number: | PAN-013043 |
| --- | --- |
| Applicant name: | Formaction Ltd |
| Site Address: | Units 14 to 18, Hepworth Business Park, Coedcae Lane, Pontyclun, CF72 9FQ |
| Discharge Point NGR: | ST 02731 82413 |
| Respond by date: | 25/03/2021 (10 working days) |
| Permitting Officer: | Tara Al-Far |

Brief Description of proposal:

The application is for a new water discharge activity permit regulating the discharge of treated sewage effluent from 5 industrial factory/office units at Hepworth Business Park. The units contain a total of 49 toilet facilities between them and it has been stated that there is no canteen on the site.

The sewage effluent is to be treated in a Klargester Biodisk (Model BE) package treatment plant that meets British Standards. A maximum daily volume of 5m3 treated effluent will then be discharged into an unnamed tributary of Nant Melyn.

Pre-application advice has been given regarding connection to sewer through which it has been determined that the closest connection point is some 750 m away from the site boundary.

Habitats Screening Report – Designations or protected species/habitats:

A risk screening report is available to view on the DMS here.

No designated sites were identified within the screening criteria.

Risk Assessment Results (modelling or L1 Risk Assessment):

The discharge is under 5m3/day therefore no river quality modelling has been conducted.

Miscellaneous:

The receiving watercourse is categorised as high flood risk.

Environment Team Responses:

Environment Officer comments:

Do have concerns that proposed discharge will be going into a small brook which when I checked 2 weeks ago had relatively small flow in it, which discharges into the Nant Melyn and then to the Ely, which is a failing watercourse under WFD.  Also there is no mention as to where their surface water drainage will go, and whether any interceptors are to be installed to ensure no fuel or oil ends up in the watercourse.  I commented on this during the planning consultation, but it does not appear the applicant has updated their plans.  Can you ask them to confirm the surface water will not be going to the treatment plant?

Officers name: Natalie Wrangham
Date: 18/03/2021

Biodiversity comments:

N/A

Officers name:
Date:

Fisheries comments:

N/A

Officers name:
Date:

Conservation/Ecology Comments:

N/A

Officers name:
Date:

Internal Consultation Responses from other NRW Teams:

Water Quality Planning:

N/A

Officers name:

Date:

Geoscience Response:

N/A

Officers name:
Date:

Flood Risk Management:

N/A

Officers name:
Date:

| External Consultations sent to: | Yes/No |
| --- | --- |
| National Parks | N |
| Water Company regarding Drinking Water Protected Zone / SPZ1 | N |
| Environmental Heath | N |

Any Permitting Responses:

Date: