Mr G & H Adams EPR/EP3232AD Coomb Farm Poultry Unit

Broiler Production IC1 & IC2

| Relevant BAT | Compliant | Technique employed |
| --- | --- | --- |
| BAT 1 | Yes | EMS in place and regularly reviewed |
| BAT 2 | Yes | 2a) N/A. 2b) Training in place. 2c) Emergency plan in place and regularly reviewed. 2d) Routine maintenance and inspections carried out and logged. 2e) Sealed carcass storage. |
| BAT 3 | Yes | Nutritional strategy employed, protein levels reduced during growing phase |
| BAT 4 | Yes | Nutritional strategy employed, phosphorus levels reduced during growing phase |
| BAT 5 | Yes | 5a) Water consumption recorded. 5b) Leaks repaired immediately. 5c) High pressure cleaning used. 5d) Nipple drinkers with spill cups fitted. 5e) Drinker heights and pressures checked daily. 5f) not applicable |
| BAT 6 | Yes | 6a) Yard areas kept clean 6b) Water usage minimised 6c) Clean and dirty water segregated |
| BAT 7 | Yes | 7a) Dirty water drains to sealed underground tanks. 7b) N/A. 7c) Waste- water land-spread by vacuum tanker. |
| BAT 8 | Yes | 8a) High efficiency heating/cooling and ventilation systems. 8b) Computer controlled heating/cooling and ventilation systems. 8c) Walls and ceilings insulated. 8d) Low energy lighting used. |
| BAT 9 | Yes | Noise management plan in place and reviewed regularly. |
| BAT 10 | Yes | Operational techniques given in noise management plan. |
| BAT 11 | Yes | 11 a 1) Use of wood shavings for bedding. 11a 2) Fresh litter applied by hand. 11a 4) Use of pelleted feed. |
| BAT 12 | Yes | Odour management plan in place and reviewed regularly. |
| BAT 13 | Yes | Odour reduction techniques given in odour management plan. |
| BAT 14 | Yes | Litter stored in “A” shaped heaps reducing surface area. |
| BAT 15 | Yes | Litter stored in “A” shaped temporary field heaps away from surface or underground water courses. |
| BAT 19 | N/A |  |
| BAT 23 | Yes | Ammonia emission reduction by estimation |
| BAT 24 | Yes | 24 b) Estimation of total nitrogen and phosphorus content by manure analysis. |
| BAT 25 | Yes | Ammonia levels estimated by using the standard emission factor for broilers of 0.034kg NH3/bird place. |
| BAT 26 | N/A |  |
| BAT 27 | Yes | Estimated and reported annually using standard emission factor 0.1 |
| BAT 29 | Yes | Water consumption and electric consumption metered and recorded. Fuel usage recorded by invoices. Flock records detailing placement numbers, losses and numbers exported. Feed consumption records kept by flock. Tonnages, dates and destination of litter used/exported recorded. |
| BAT 32 | Yes | 32a) Forced ventilation system with non leaking drinkers. BAT-AEL 0.01-0.08 kg NH3/animal place/year |