# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Bwl MX Motocross Track operated by Mynachdy Limited.

The application number is PAN-012300

The permit number is EPR/CB3095CP

The discharge is for an existing discharge of rainfall related runoff via site drainage at Bwl MX Motocross Track. The site has a main track, a secondary small junior track, paddocks, a car park, and changing/welfare facilities. The site is a total of 5.9ha with approximately 4ha consisting of positively drained non-vegetated soil.

The discharge is treated in a system comprising of silt fencing, settlement ponds, spreader bars, reed beds and straw bales and is eventually released into an unnamed tributary of the Y Ffrwd. The activity was previously permitted by EPR-WB3390HY that was lapsed due to the liquidation of the previous company and failure to transfer the permit correctly.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

During the determination of this permit variation, the UK left the EU.  However, reference is made in this document to various EU directives, which as implemented in UK domestic legislation are still applicable

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation and web publicising responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Receipt of submission |  |  |  |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. |  |
| ### Responses to consultation and web publicising | The web publicising and consultation responses (Annex 2) were taken into account in the decision. The decision was taken in accordance with our guidance. |  |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. |  |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. |  |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the CRoW Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . Appendix 4 is not required because there is no conceivable impact pathway to any SSSI. Formal consultation has been carried out with Rhondda Cynon Taf Council Ecologist The consultation responses (Annex 2) were taken into account in the permitting decision. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. |  |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. |  |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. An emission limit of 100 mg/l for Suspended Solids has been included in the permit. This was advised by WQ policy and WQ planning consultees and agreed with the relevant Fisheries Officer. |  |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. |  |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” |  |

# Annex 2: Consultation and web publicising responses

Response received from

Dŵr Cymru Welsh Water

Brief summary of issues raised

The application falls outside of a Drinking Water Catchment and therefore we have no further comment.

Summary of actions taken or show how this has been covered

N/A

Response received from

RCT Council Ecologist

Brief summary of issues raised

If NRW is satisfied that the treatment facility is sufficiently robust and controllable to ensure water quality of the receiving watercourse is not compromised that would be enough to be confident that the nature conservation status of the downstream SINC will also be protected.

Summary of actions taken or show how this has been covered

N/A