Ms Lara Cubley

Natural Resources Wales (NRW)

Chester Road

Buckley

CH7 3AG

BY EMAIL

Dear Lara,

Sundorne Products Limited – Bryn Posteg LANDFILL SITE

1st Quarterly Monitoring Report 2021

Please find enclosed a summary of the monitoring data for leachate, groundwater, surface water and landfill gas collected at Bryn Posteg Landfill Site for the 1st Quarter of 2021, review period January to March 2021. This letter summarises the main findings with reference to the compliance limits set in the Site Permit (reference BU7766IC).

LEACHATE

Leachate levels

In accordance with condition 3.7.1 and Table S3.1, leachate levels at the 10 leachate monitoring points across the site are to be reported on a quarterly basis. Leachate level monitoring data for this quarter are provided in Appendix A. A review of the data indicates that LCP1, LCP2-A, LCP7, LCP8, RMPL9B-A RMPL9D-A all marginally exceeded the 1 metre compliance level during the period. However, these were all less than 10cm and are within the error measurement for the dip tape. For Health and Safety reasons, access to LCP8 was prevented during January and February.

Leachate quality

Leachate quality monitoring data is provided in Appendix A in accordance with Table S3.9.

Treated leachate quality

In accordance with Table S3.4, treated leachate quality is provided in Appendix A.

SURFACE WATER

In accordance with condition 3.7.1 and Table S3.3, surface water emissions data is presented in Appendix B. There have been six failures during the quarter. These occurred in March, in P1 with elevated ammonia and suspended solids measurements of 0.8 and 1272 mg/l against a compliance level of 0.25mg/l and 50mg/l respectively. P2 also had failures in February and March with elevated ammonia results of 0.4 and 0.8 mg/l against a compliance limit of 0.25mg/l and suspended solids results of 1226 and 52mg/l against a compliance limit of 50 mg/l.

GROUNDWATER

Groundwater level

Groundwater levels are recorded on a weekly basis in accordance with Table S3.10 and are provided in Appendix C.

Groundwater quality sampling

Groundwater quality sampling is undertaken on a monthly basis in accordance with Table Table S3.5 and S3.10 of the Permit. The groundwater quality monitoring data has been assessed against the compliance limits set in the Permit and are provided in Appendix C. There was one exceedance recorded during the review period January to March 2021, which occurred on the 15th March 2020 in W1, with elevated chloride levels of 612 mg/l, respectively. W1 has a history of high chloride levels which have been correlated with winter salt spreading on the adjacent highway.

LANDFILL GAS

External Monitoring Boreholes

In accordance with Table S3.6, the perimeter boreholes around the site are monitored for gas emissions on a monthly basis. The result of this monitoring is provided in Appendix D.

In-Waste Gas Monitoring

In accordance with Table S3.8, the monthly landfill gas collection system data is provided in Appendix D. A full audit on the gas extraction system has been undertaken in March 2021. This report is currently being finalised and will be sent over to you as soon as it is available.

Summary data from the inlet into the gas compound is also provided in Appendix D. As previously explained, this summary is not an accurate representation of the data as it is providing erroneous readings during power loss of reset cycles.

DUST

Appendix E provides the dust analysis results for this quarter. There were no breaches on compliance.

Meteorological Data

Appendix F provides the meteorological data for the past quarter. This includes wind speed, direct, temperature, and rainfall readings taken from the onsite weather station at Bryn Posteg.

I trust that the above meets your requirements; please do not hesitate to contact me if you have any queries.

Yours sincerely,

Deborah Hall BSc MSc MRSB

Environment, Health and Safety Compliance Manager

Potters Waste Management

Appendix A

Leachate Levels

| Well no | Date | Depth to base | Depth to liquid | Leachate Head |
| --- | --- | --- | --- | --- |
| LCP1 | 04/01/2021 | 7.2 | 6.14 | 1.06 |
| LCP1 | 04/02/2021 | 7.2 | 6.18 | 1.02 |
| LCP1 | 03/03/2021 | 7.2 | 6.21 | 0.99 |
| LCP2-A | 04/01/2021 | 31.9 | 31.02 | 0.88 |
| LCP2-A | 04/02/2021 | 31.9 | 30.95 | 0.95 |
| LCP2-A | 03/03/2021 | 31.9 | 30.89 | 1.01 |
| LCP3-A | 04/01/2021 | 34.8 | 33.94 | 0.86 |
| LCP3-A | 04/02/2021 | 34.8 | 33.98 | 0.82 |
| LCP3-A | 03/03/2021 | 34.8 | 33.85 | 0.95 |
| LCP6-A | 04/01/2021 | 24.97 | 24.01 | 0.96 |
| LCP6-A | 04/02/2021 | 24.97 | 24.05 | 0.92 |
| LCP6-A | 03/03/2021 | 24.97 | 24 | 0.97 |
| LCP7 | 04/01/2021 | 16.5 | 15.64 | 0.86 |
| LCP7 | 04/02/2021 | 16.5 | 15.44 | 1.06 |
| LCP7 | 03/03/2021 | 16.5 | 15.52 | 0.98 |
| LCP8 | 04/01/2021 | 13.02 | No access | - |
| LCP8 | 04/02/2021 | 13.02 | No access | - |
| LCP8 | 03/03/2021 | 13.02 | 12.01 | 1.01 |
| RMPL9A -A | 04/01/2021 | 26.38 | 25.39 | 0.99 |
| RMPL9A -A | 04/02/2021 | 26.38 | 25.43 | 0.95 |
| RMPL9A -A | 03/03/2021 | 26.38 | 25.71 | 0.67 |
| RMPL9B-A | 04/01/2021 | 31.8 | 30.75 | 1.05 |
| RMPL9B-A | 04/02/2021 | 31.8 | 30.89 | 0.91 |
| RMPL9B-A | 03/03/2021 | 31.8 | 30.84 | 0.96 |
| RMPL9C-A | 04/01/2021 | 24.5 | 23.61 | 0.89 |
| RMPL9C-A | 04/02/2021 | 24.5 | 23.54 | 0.96 |
| RMPL9C-A | 03/03/2021 | 24.5 | 23.62 | 0.88 |
| RMPL9D-A | 04/01/2021 | 24.1 | 23.08 | 1.02 |
| RMPL9D-A | 04/02/2021 | 24.1 | 23.00 | 1.10 |
| RMPL9D-A | 03/03/2021 | 24.1 | 23.05 | 1.05 |

Leachate Quality

| 07/01/2021 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample point | units | LCP1 | LCP2A | LCP3A | LCP6A | LCP7 | LCP8 | RMPL9A-A | RMPL9B-A | RMPL9C-A | RMPL9D-A |
| pH |  | 6.1 | 7.9 | Dry | 7.5 | Dry | No Access | 8.2 | 8 | Dry | 8.1 |

| 15/02/2021 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample point | units | LCP1 | LCP2A | LCP3A | LCP6A | LCP7 | LCP8 | RMPL9A-A | RMPL9B-A | RMPL9C-A | RMPL9D-A |
| pH |  | Dry | 8.2 | Dry | Dry | 7.8 | No access | Dry | Dry | Dry | Dry |

| 15/03/2021 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample point | units | LCP1 | LCP2A | LCP3A | LCP6A | LCP7 | LCP8 | RMPL9A-A | RMPL9B-A | RMPL9C-A | RMPL9D-A |
| Ammoniacal Nitrogen (as N) | mg/l | 54 | Dry | 2201 | 564 | 547 | 899 | 324 | 2107 | 1092 | 746 |
| Chloride | mg/l | 176 | Dry | 8528 | 1121 | 884 | 1710 | 459 | 3179 | 1557 | 768 |
| pH |  | 7.6 | Dry | 8.1 | 8.2 | 8.2 | 8.3 | 8.6 | 8.2 | 8.2 | 8.1 |

Treated Leachate Quality

| Sample point | units | Treated Leachate |  |  |
| --- | --- | --- | --- | --- |
| Date |  | 07/01/2021 | 15/02/2021 | 15/03/2021 |
| pH |  | 6.9 | 7.1 | 8.2 |
| Sulphate (as SO4) | mg/l | 104 | 86 | 73 |
| Sulphide | mg/l | <0.01 | <1.00 | <0.25 |
| Ammoniacal Nitrogen (as N) | mg/l | 91.2 | 18.9 | 144.2 |
| COD | mg/l | 210 | 318 | 516 |
| Suspended Solids | mg/l | 20 | 390 | 58 |
| Alkalinity- Carbonate as CaCO3 | mg/l | 273.8 | 65 | - |
| Dissolved Methane | mg/l | 0.51 | <0.050 | <0.050 |
| Oil and Grease |  | ND | ND | ND |

Appendix B

Surface Water Quality

| 07/01/2021 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sample Point | Ammoniacal Nitrogen | Suspended Solids | BOD | pH |
| unit | mg/l | mg/l | mg/l |  |
| P1 | 0.2 | 12 | 5 | 6.4 |
| P2 | 0.2 | 8 | 5.4 | 6.6 |
| 15/02/2021 |  |  |  |  |
| Sample Point | Ammoniacal Nitrogen | Suspended Solids | BOD | pH |
| unit | mg/l | mg/l | mg/l |  |
| P1 | <0.1 | 16 | <2 | 7.6 |
| P2 | 0.4 | 1226 | 9 | 8 |
| 15/03/2021 |  |  |  |  |
| Sample Point | Ammoniacal Nitrogen | Suspended Solids | BOD | pH |
| unit | mg/l | mg/l | mg/l |  |
| P1 | 0.8 | 1272 | <2 | 7.3 |
| P2 | 0.8 | 52 | <2 | 7.5 |

Appendix C

Groundwater Levels

| Borehole | G1/W1 |  | W2 |  | G13/W3 |  | G16/ W4 |  | G18/W5 |  | W6 |  | W7 |  | W8 |  | G32/W9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dip to water | Level | Dip to water | Level | Dip to water | Level | Dip to water | Level | Dip to water | Level | Dip to water | Level | Dip to water | Level | Dip to water | Level | Dip to water | Level |
| 06/01/2021 | 10.09 | 309.91 | 2.4 | 330.5 | 19.57 | 303.06 | 14.46 | 304.34 | 3.96 | 312.94 | 5.37 | 310.13 | 17.02 | 300.93 | >31.9 |  | 9.09 | 320.31 |
| 11/02/2021 | 10.09 | 390.91 | frozen |  | 19.94 | 302.69 | 15.05 | 303.75 | 4.06 | 312.84 | 5.53 | 309.97 | 17.30 | 300.65 | >31.9 |  | 9.05 | 320.35 |
| 11/03/2021 | 9.21 | 310.79 | 2.62 | 330.28 | 19.99 | 302.64 | 15.98 | 302.82 | 3.99 | 312.91 | 5.56 | 309.94 | 18.42 | 299.53 | >31.9 |  | 9.37 | 320.03 |

Groundwater Quality – Monthly samples taken in accordance with Table S3.10

| 07/01/2021 |  |  |  |  | 15/02/2021 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample point | Sulphate (as SO4) | Electrical Conductivity | pH |  | Sample point | Sulphate (as SO4) | Electrical Conductivity | pH |
| unit | mg/l | µS/cm |  |  | unit | mg/l | µS/cm |  |
| GW01 | 10 | 1078 | 6 |  | GW01 | 17 | 2140 | 6.7 |
| GW02 | <4 | 418 | 7 |  | GW02 |  |  |  |
| GW03 | 29 | 200 | 6.2 |  | GW03 | 30 | 173 | 6.7 |
| GW04 | 41 | 306 | 6.4 |  | GW04 | 38 | 310 | 7.2 |
| GW05 | 37 | 225 | 6 |  | GW05 | 36 | 200 | 6.8 |
| GW06 | 6 | 159 | 5.9 |  | GW06 | 6 | 130 | 7 |
| GW07 | - | - | - |  | GW07 | 16 | 350 | 7.8 |
| GW08 | 26 | 280 | 6.1 |  | GW08 | 27 | 336 | 8.2 |
| GW09 | 25 | 202 | 6 |  | GW09 | 31 | 183 | 7.9 |
| GW10 | 58 | 318 | 5.9 |  | GW10 | 50 | 295 | 7.4 |
| GWD | 6 | 168 | 6.2 |  | GWD | 7 | 141 | 8.9 |

Groundwater Quality – Quarterly Sample taken 15 March 2020 in accordance with Table S3.5 and S3.10.

| Monitoring Point | Column1 | W01 | W02 | W03 | W04 | W05 | W06 | W07 | W08 | W09 | W10 | GWD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arsenic | µg/l | 2.9 | 0.2 | 1.7 | 2.2 | <0.2 | 1.5 | 1.9 | 1 | 0.9 | 1.6 | 0.8 |
| Cadmium | µg/l | 0.12 | <0.04 | <0.04 | <0.04 | 0.3 | <0.04 | <0.04 | <0.04 | 0.19 | 0.23 | 0.07 |
| Chromium | µg/l | <0.04 | <0.04 | 0.05 | <0.04 | 0.29 | <0.04 | 0.13 | <0.04 | <0.04 | <0.04 | <0.04 |
| Lead | µg/l | <0.01 | <0.01 | 0.07 | 0.24 | 0.29 | <0.01 | 0.6 | <0.01 | <0.01 | <0.01 | 0.8 |
| Mercury | µg/l | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 |
| Selenium | µg/l | <0.5 | <0.5 | <0.5 | <0.5 | <0.5 | <0.5 | <0.5 | <0.5 | <0.5 | <0.5 | <0.5 |
| Copper | µg/l | 6.94 | 1.33 | 6.25 | 3.38 | 12.93 | 3.43 | 1.73 | 1.23 | 5.82 | 5.73 | 8.13 |
| Nickel | µg/l | 2.4 | 0.8 | 0.9 | 3.8 | 8.7 | 0.6 | 0.7 | 0.3 | 2.1 | 13.4 | 1.3 |
| Zinc | µg/l | 11.5 | 3.6 | 4.3 | 6.2 | 44.3 | 12.4 | 38 | 2 | 118.2 | 51.2 | 15.3 |
| Iron | µg/l | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 |
| Manganese | µg/l | 950 | 17 | 41 | 2223 | 2082 | 8 | 1218 | 759 | 91 | 7381 | 30 |
| Silver | µg/l | <2 | <2 | <2 | <2 | <2 | <2 | <2 | <2 | <2 | <2 | <2 |
| Calcium | mg/l | 30 | 9 | 9 | 29 | 13 | 9 | 44 | 38 | 7 | 23 | 6 |
| Magnesium | mg/l | 6 | 0 | 2 | 7 | 3 | 1 | 7 | 10 | 2 | 8 | 1 |
| Sodium | mg/l | 370 | 33 | 6 | 11 | 9 | 7 | 18 | 11 | 13 | 8 | 5 |
| Potassium | mg/l | 2 | 3 | 2 | 2 | 3 | 1 | 3 | 1 | 1 | 1 | 1 |
| Phenol | µg/l | <0.50 | <0.50 | <0.50 | <0.50 | <0.50 | <0.50 | <0.50 | <0.50 | <0.50 | <0.50 | <2.00 |
| Total Cyanide | mg/l | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 |
| Sulphate as SO4 | mg/l | 20 | <4 | 17 | 38 | 33 | 7 | 26 | 27 | 31 | 52 | 8 |
| Ammoniacal Nitrogen as N | mg/l | 0.6 | 0.6 | 0.7 | 1.7 | 0.5 | 1 | 0.5 | 0.5 | 0.8 | 1.2 | 0.5 |
| Chloride | mg/l | 612.1 | 32.9 | 10.9 | 13 | 10.6 | 12 | 14.2 | 16.1 | 18.9 | 11.8 | 11.7 |
| Dissolved Oxygen | mg/l | 8.3 | 6.7 | 8.8 | 8.1 | 9.2 | 9.3 | 8.9 | 8.4 | 9.5 | 1.4 | 8.9 |
| Alkalinity - Carbonate as CaCO3 | mg/l | 25 | 49.1 | 14.7 | 78 | 25.1 | 30.8 | 125.9 | 104.2 | 3.8 | 61.9 | 11.4 |
| Alkalinity - bicarbonate | mg/l | 30.5 | 59.9 | 17.9 | 95.2 | 30.6 | 37.6 | 153.6 | 127.1 | 4.6 | 75.5 | 13.9 |
| Nitrate | mg/l | 0.3 | 0.6 | 4.8 | 1 | 3.7 | 1 | 0.6 | 0.2 | 1.7 | 0.9 | 3.5 |
| EC | µS/cm | 1810 | 255 | 152 | 320 | 211 | 146 | 396 | 354 | 207 | 307 | 124 |
| Mecoprop | µg/l | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 |
| 2,4-D | µg/l | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 | <0.02 |
| pH |  | 7.2 | 7.7 | 7.2 | 7.8 | 7.3 | 7.3 | 8.1 | 8 | 6.4 | 7.7 | 6.6 |

| Monitoring Point |  | W01 | W02 | W03 | W04 | W05 | W06 | W07 | W08 | W09 | W10 | GWD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Naphthalene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Acenaphthylene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Acenaphthene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Fluorene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Phenanthrene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Anthracene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Fluoranthene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Pyrene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Benzo (a) anthracene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Chrysene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Benzo (b) fluoranthene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Benzo (k) fluoranthene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Benzo (a) pyrene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| pyrene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Dibenzo (ah) anthrancene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Benzo (ghi) perylene | ng/l | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <10 | <40 |
| Polynuclear Aromatic Hydrocarbons (total) | ng/l | <160 | <160 | <160 | <160 | <160 | <160 | <160 | <160 | <160 | <160 | <640 |

| Monitoring Point |  | W01 | W02 | W03 | W04 | W05 | W06 | W07 | W08 | W09 | W10 | GWD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Benzene | µg/l | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 |
| Ethylbenzene | µg/l | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 |
| m & p - Xylene | µg/l | <2 | <2 | <2 | <2 | <2 | <2 | <2 | <2 | <2 | <2 | <2 |
| o - Xylene | µg/l | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 |
| Toulene | µg/l | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 |
| MTBE | µg/l | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 | <1 |

Appendix D

Perimeter Gas Monitoring

| ID | DATE | CH4 | CO2 | O2 | BALANCE | CH4 %LEL | PEAKCH4 | BARO | REL.PRESSURE | CO | H2S | CH4/CO2 | Res Nitrogen | Flow l/h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BPW00901 | 06/01/2021 00:00 | 1 | 0.2 | 19.1 | 79.7 | 20 | 1 | 986 | -0.72 | 0 | 0 | 5.00 |  | 0 |
| BPW00902 | 06/01/2021 00:00 | 0 | 4.4 | 18.8 | 76.8 | 0 | 0 | 986 | -0.83 | 0 | 0 | 0.00 |  | 0.1 |
| BPW00903 | 06/01/2021 00:00 | 0 | 0.5 | 21.8 | 77.7 | 0 | 0 | 986 | -0.88 | 0 | 0 | 0.00 |  | 0.1 |
| BPW00907 | 06/01/2021 00:00 | 0 | 0.2 | 22.3 | 77.5 | 0 | 0 | 986 | -0.98 | 0 | 0 | 0.00 |  | 0.1 |
| BPW00908 | 06/01/2021 00:00 | 0 | 0.6 | 18 | 81.4 | 0 | 0 | 986 | -0.95 | 0 | 0 | 0.00 |  | -1.2 |
| BPW00909 | 06/01/2021 00:00 | 0 | 1.8 | 22.1 | 76.1 | 0 | 0 | 986 | -18.07 | 0 | 0 | 0.00 |  | 0 |
| BPW00910 | 06/01/2021 00:00 | 0 | 1.8 | 19.1 | 79.1 | 0 | 0 | 986 | -1.38 | 0 | 0 | 0.00 |  | 0.1 |
| BPW00911 | 06/01/2021 00:00 | 0 | 0.7 | 22.2 | 77.1 | 0 | 0 | 986 | -17.88 | 0 | 0 | 0.00 |  | -3.3 |
| BPW00912 | 06/01/2021 00:00 | 71.5 | 6.5 | 1 | 21 | >>>> | 71.5 | 986 | -1.08 | 0 | 0 | 11.00 |  | 0.5 |
| BPW00913 | 06/01/2021 00:00 | 0.1 | 0.1 | 22.5 | 77.3 | 2 | 0.1 | 986 | -1.17 | 0 | 0 | 1.00 |  | 0.1 |
| BPW00914 | 06/01/2021 00:00 | 0.1 | 3.5 | 18.7 | 77.7 | 2 | 0.1 | 986 | -1.41 | 0 | 0 | 0.03 |  | 0.1 |
| BPW00915 | 06/01/2021 00:00 | 0 | 1.9 | 20 | 78.1 | 0 | 0 | 986 | -19.28 | 0 | 0 | 0.00 |  | 0.1 |
| BPW00916 | 06/01/2021 00:00 | 0 | 0.1 | 22.5 | 77.4 | 0 | 0 | 986 | -1.21 | 0 | 0 | 0.00 |  | 0 |
| BPW00917 | 06/01/2021 00:00 | 0 | 0.2 | 22.5 | 77.3 | 0 | 0 | 986 | -1.12 | 0 | 0 | 0.00 |  | 0 |
| BPW00918 | 06/01/2021 00:00 | 0 | 1.4 | 21.8 | 76.8 | 0 | 0 | 986 | -17.5 | 0 | 0 | 0.00 |  | 0.1 |
| BPW00919 | 06/01/2021 00:00 | 59.1 | 9.9 | 5.5 | 25.5 | 1182 | 59.1 | 986 | -1.16 | 0 | 0 | 5.97 |  | 0.1 |
| BPW00920 | 06/01/2021 00:00 | 28.2 | 5.5 | 0.4 | 65.9 | 564 | 28.2 | 986 | -22.5 | 0 | 46 | 5.13 |  | -1.4 |
| BPW00938 | 06/01/2021 00:00 | 0.3 | 4.1 | 16.3 | 79.3 | 6 | 0.3 | 986 | -1.05 | 0 | 0 | 0.07 |  | 0 |
| BPW00921 | 06/01/2021 00:00 | 30.8 | 10.2 | 0.3 | 58.7 | 616 | 30.8 | 986 | -0.93 | 0 | 0 | 3.02 |  | 0 |
| BPW00922 | 06/01/2021 00:00 | 65.2 | 18.6 | 0.2 | 16 | 1304 | 65.2 | 986 | -0.79 | 0 | 0 | 3.51 |  | 0 |
| BPW00923 | 06/01/2021 00:00 | 24.3 | 6.8 | 0.1 | 68.8 | 486 | 24.3 | 986 | -52.36 | 0 | 10 | 3.57 |  | 0 |
| BPW00924 | 06/01/2021 00:00 | 0.8 | 1.7 | 19.5 | 78 | 16 | 0.8 | 986 | -9.72 | 0 | 0 | 0.47 |  | -2.3 |
| BPW00925 | 06/01/2021 00:00 | 18.1 | 10.7 | 0.9 | 70.3 | 362 | 18.1 | 986 | -27.88 | 0 | 0 | 1.69 |  | 0.1 |
| BPW00926 | 06/01/2021 00:00 | 0.1 | 4.6 | 17.6 | 77.7 | 2 | 0.1 | 986 | -1.12 | 0 | 0 | 0.02 |  | 0.1 |
| BPW00927 | 06/01/2021 00:00 | 0 | 0.4 | 21.4 | 78.2 | 0 | 0 | 983 | -0.88 | 0 | 0 | 0.00 |  | 0.1 |
| BPW00929 | 06/01/2021 00:00 | 13.8 | 2.2 | 3.5 | 80.5 | 276 | 13.8 | 983 | -48.17 | 0 | 0 | 6.27 |  | 0.1 |
| BPW00930 | 06/01/2021 00:00 | 0.1 | 0.2 | 22.1 | 77.6 | 2 | 0.1 | 983 | -1.33 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00931 | 06/01/2021 00:00 | 0.1 | 0.7 | 21.8 | 77.4 | 2 | 0.1 | 983 | -1.1 | 0 | 0 | 0.14 |  | 0.1 |
| BPW00932 | 06/01/2021 00:00 | 0.2 | 0.9 | 21.9 | 77 | 4 | 0.2 | 983 | -1.19 | 0 | 0 | 0.22 |  | 0.1 |
| BPW00939 | 06/01/2021 00:00 | 0.1 | 1.5 | 21.4 | 77 | 2 | 0.1 | 983 | -1.1 | 0 | 0 | 0.07 |  | 0.1 |
| BPW00940 | 06/01/2021 00:00 | 0.1 | 1.9 | 18.8 | 79.2 | 2 | 0.1 | 983 | -1.31 | 0 | 0 | 0.05 |  | 0.1 |
| BPW00941 | 06/01/2021 00:00 | 0.1 | 0.6 | 22 | 77.3 | 2 | 0.1 | 983 | -1.14 | 0 | 0 | 0.17 |  | 0.1 |
| BPW00942 | 06/01/2021 00:00 | 0.1 | 0.4 | 22.4 | 77.1 | 2 | 0.1 | 983 | -1.16 | 0 | 0 | 0.25 |  | 0.1 |
| BPW00936 | 06/01/2021 00:00 | 0.6 | 0.9 | 20.9 | 77.6 | 12 | 0.6 | 983 | -1.31 | 0 | 0 | 0.67 |  | 0.1 |
| BPW00937 | 06/01/2021 00:00 | 0.1 | 0.2 | 22.4 | 77.3 | 2 | 0.1 | 983 | -1.24 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00901 | 11/02/2021 00:00 | 1 | 0.2 | 21.6 | 77.2 | 20 | 1 | 987 | -1.03 | 0 | 0 | 5.00 |  | 0 |
| BPW00902 | 11/02/2021 00:00 | 0.1 | 2.2 | 21.1 | 76.6 | 2 | 0.1 | 987 | -1.16 | 0 | 0 | 0.05 |  | 0.1 |
| BPW00903 | 11/02/2021 00:00 | 0.1 | 0.6 | 21.5 | 77.8 | 2 | 0.1 | 987 | -2.33 | 0 | 0 | 0.17 |  | 0.1 |
| BPW00907 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.4 | 77.3 | 2 | 0.1 | 987 | -1.26 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00908 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.3 | 77.4 | 2 | 0.1 | 987 | -1.35 | 0 | 0 | 0.50 |  | 0 |
| BPW00909 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.3 | 77.4 | 2 | 0.1 | 987 | -1.26 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00910 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.4 | 77.3 | 2 | 0.1 | 987 | -1.28 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00911 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.5 | 77.2 | 2 | 0.1 | 987 | -1.29 | 0 | 0 | 0.50 |  | 0 |
| BPW00912 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.6 | 77.1 | >>>> | 0.1 | 987 | -1.29 | 0 | 0 | 0.50 |  | 0 |
| BPW00913 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.7 | 77 | 2 | 0.1 | 987 | -1.33 | 0 | 0 | 0.50 |  | 0 |
| BPW00914 | 11/02/2021 00:00 | 0.1 | 1.9 | 20.8 | 77.2 | 2 | 0.1 | 987 | -1.38 | 0 | 0 | 0.05 |  | 0.1 |
| BPW00915 | 11/02/2021 00:00 | 0.1 | 1.8 | 20.6 | 77.5 | 2 | 0.1 | 987 | -1.34 | 0 | 0 | 0.06 |  | 0.1 |
| BPW00916 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.5 | 77.2 | 2 | 0.1 | 987 | -1.28 | 0 | 0 | 0.50 |  | 0 |
| BPW00917 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.5 | 77.2 | 2 | 0.1 | 987 | -1.41 | 0 | 0 | 0.50 |  | 0 |
| BPW00918 | 11/02/2021 00:00 | 0.1 | 1.2 | 22 | 76.7 | 2 | 0.1 | 987 | -1.26 | 0 | 0 | 0.08 |  | 0 |
| BPW00919 | 11/02/2021 00:00 | 0.1 | 0.4 | 22.3 | 77.2 | 2 | 0.1 | 987 | -1.33 | 0 | 0 | 0.25 |  | 0 |
| BPW00938 | 11/02/2021 00:00 | 2.1 | 1.3 | 20.8 | 75.8 | 42 | 2.1 | 987 | -1.33 | 0 | 0 | 1.62 |  | 0 |
| BPW00921 | 11/02/2021 00:00 | 10.3 | 2.8 | 18.9 | 68 | 206 | 10.3 | 987 | -1.34 | 0 | 0 | 3.68 |  | 0 |
| BPW00924 | 11/02/2021 00:00 | 0.1 | 2.8 | 18.2 | 78.9 | 2 | 0.1 | 987 | -1.36 | 0 | 0 | 0.04 |  | 0 |
| BPW00926 | 11/02/2021 00:00 | 0.1 | 2.7 | 19.2 | 78 | 2 | 0.1 | 987 | -1.45 | 0 | 0 | 0.04 |  | 0 |
| BPW00929 | 11/02/2021 00:00 | 20.2 | 5.2 | 0.8 | 73.8 | 404 | 20.2 | 986 | -93.42 | 8 | 102 | 3.88 |  | 0.1 |
| BPW00930 | 11/02/2021 00:00 | 0.1 | 0.2 | 20.9 | 78.8 | 2 | 0.1 | 986 | -2.59 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00931 | 11/02/2021 00:00 | 0.1 | 0.6 | 22.2 | 77.1 | 2 | 0.1 | 986 | -1.52 | 0 | 0 | 0.17 |  | 0.1 |
| BPW00932 | 11/02/2021 00:00 | 0.1 | 1.2 | 22.3 | 76.4 | 2 | 0.1 | 986 | -1.33 | 0 | 0 | 0.08 |  | 0.1 |
| BPW00939 | 11/02/2021 00:00 | 0.1 | 0.3 | 22.3 | 77.3 | 2 | 0.1 | 986 | -1.38 | 0 | 0 | 0.33 |  | 0.1 |
| BPW00940 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.5 | 77.2 | 2 | 0.1 | 986 | -1.43 | 0 | 0 | 0.50 |  | 0 |
| BPW00941 | 11/02/2021 00:00 | 0.1 | 0.5 | 22.2 | 77.2 | 2 | 0.1 | 986 | -1.43 | 0 | 0 | 0.20 |  | 0.1 |
| BPW00942 | 11/02/2021 00:00 | 0.1 | 0.2 | 22.3 | 77.4 | 2 | 0.1 | 986 | -1.43 | 0 | 0 | 0.50 |  | 0 |
| BPW00936 | 11/02/2021 00:00 | 2.2 | 1.1 | 20.9 | 75.8 | 44 | 2.2 | 986 | -1.41 | 0 | 0 | 2.00 |  | 0 |
| BPW00937 | 11/02/2021 00:00 | 0.1 | 1.2 | 21.1 | 77.6 | 2 | 0.1 | 986 | -1.55 | 0 | 0 | 0.08 |  | 0.1 |
| BPW00901 | 11/03/2021 00:00 | 1 | 0.1 | 21.7 | 77.2 | 20 | 1 | 957 | -0.4 | 0 | 0 | 10.00 |  | 0 |
| BPW00902 | 11/03/2021 00:00 | 0.1 | 0.6 | 22 | 77.3 | 2 | 0.1 | 957 | -0.57 | 0 | 0 | 0.17 |  | 0 |
| BPW00903 | 11/03/2021 00:00 | 0.1 | 2.7 | 11.9 | 85.3 | 2 | 0.1 | 957 | -0.71 | 0 | 0 | 0.04 |  | 0.1 |
| BPW00907 | 11/03/2021 00:00 | 0.1 | 2.1 | 20.4 | 77.4 | 2 | 0.1 | 957 | -0.55 | 0 | 0 | 0.05 |  | 0.1 |
| BPW00908 | 11/03/2021 00:00 | 0.1 | 0.2 | 21.8 | 77.9 | 2 | 0.1 | 957 | -0.6 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00909 | 11/03/2021 00:00 | 0.1 | 0.6 | 21.8 | 77.5 | 2 | 0.1 | 957 | -0.53 | 0 | 0 | 0.17 |  | 0.1 |
| BPW00910 | 11/03/2021 00:00 | 0.1 | 2.1 | 18.1 | 79.7 | 2 | 0.1 | 957 | -14.14 | 0 | 0 | 0.05 |  | 0.1 |
| BPW00911 | 11/03/2021 00:00 | 0.1 | 0.4 | 21.7 | 77.8 | 2 | 0.1 | 957 | -0.52 | 0 | 0 | 0.25 |  | 0.1 |
| BPW00912 | 11/03/2021 00:00 | 66.7 | 5.2 | 2.3 | 957 | >>>> | 66.7 | 957 | -5.22 | 0 | 0 | 12.83 |  | 0.1 |
| BPW00913 | 11/03/2021 00:00 | 0.1 | 0.1 | 22 | 77.8 | 2 | 0.1 | 957 | -0.55 | 0 | 0 | 1.00 |  | 0.1 |
| BPW00914 | 11/03/2021 00:00 | 1.1 | 0.2 | 21.9 | 76.8 | 22 | 1.1 | 957 | -27.6 | 0 | 0 | 5.50 |  | -0.9 |
| BPW00915 | 11/03/2021 00:00 | 0.1 | 2.3 | 19.5 | 78.1 | 2 | 0.1 | 957 | -0.59 | 0 | 0 | 0.04 |  | 0.1 |
| BPW00916 | 11/03/2021 00:00 | 0.1 | 0.1 | 22.1 | 77.7 | 2 | 0.1 | 957 | -0.4 | 0 | 0 | 1.00 |  | 0.1 |
| BPW00917 | 11/03/2021 00:00 | 0.1 | 0.1 | 22.2 | 77.6 | 2 | 0.1 | 957 | -0.6 | 0 | 0 | 1.00 |  | 0.1 |
| BPW00918 | 11/03/2021 00:00 | 0.1 | 2 | 20.9 | 77 | 2 | 0.1 | 957 | -24.58 | 0 | 0 | 0.05 |  | 0.1 |
| BPW00919 | 11/03/2021 00:00 | 0.1 | 0.2 | 22.4 | 77.3 | 2 | 0.1 | 957 | -0.33 | 0 | 0 | 0.50 |  | 0 |
| BPW00920 | 11/03/2021 00:00 | 22.6 | 4.7 | 0.3 | 72.4 | 452 | 22.6 | 957 | -0.59 | 0 | 20 | 4.81 |  | 0.7 |
| BPW00938 | 11/03/2021 00:00 | 66.5 | 13.3 | 1.2 | 19 | 1330 | 66.5 | 958 | -0.67 | 0 | 2 | 5.00 |  | 0.1 |
| BPW00921 | 11/03/2021 00:00 | 1.3 | 2.6 | 16.8 | 79.3 | 26 | 1.3 | 958 | -0.65 | 0 | 0 | 0.50 |  | 0 |
| BPW00922 | 11/03/2021 00:00 | 45.9 | 17.7 | 0.4 | 36 | 918 | 45.9 | 958 | -0.63 | 0 | 0 | 2.59 |  | 0 |
| BPW00923 | 11/03/2021 00:00 | 10.3 | 6.2 | 1.8 | 81.7 | 206 | 10.3 | 958 | -109.2 | 0 | 0 | 1.66 |  | 0 |
| BPW00924 | 11/03/2021 00:00 | 0.1 | 1.8 | 20.2 | 77.9 | 2 | 0.1 | 958 | -0.57 | 0 | 0 | 0.06 |  | 0.1 |
| BPW00925 | 11/03/2021 00:00 | 12.2 | 8 | 1.5 | 78.3 | 244 | 12.2 | 958 | -1.44 | 0 | 0 | 1.53 |  | 0.1 |
| BPW00926 | 11/03/2021 00:00 | 0.1 | 2.7 | 15.8 | 81.4 | 2 | 0.1 | 958 | -0.67 | 0 | 0 | 0.04 |  | 0.1 |
| BPW00927 | 11/03/2021 00:00 | 0 | 0.4 | 22.1 | 77.5 | 0 | 0 | 983 | -0.66 | 0 | 0 | 0.00 |  | 0.1 |
| BPW00929 | 11/03/2021 00:00 | 29.8 | 5.4 | 6.5 | 58.3 | 596 | 29.8 | 958 | -0.62 | 20 | 163 | 5.52 |  | 0.1 |
| BPW00930 | 11/03/2021 00:00 | 0.7 | 0.6 | 20.6 | 78.1 | 14 | 0.7 | 958 | -0.43 | 0 | 0 | 1.17 |  | 0.1 |
| BPW00931 | 11/03/2021 00:00 | 0.1 | 1.9 | 18.2 | 79.8 | 2 | 0.1 | 958 | -0.41 | 0 | 0 | 0.05 |  | 0.1 |
| BPW00932 | 11/03/2021 00:00 | 0.1 | 0.2 | 21.8 | 77.9 | 2 | 0.1 | 958 | -0.53 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00933 | 11/03/2021 00:00 | 0.1 | 2.5 | 16.1 | 81.3 | 2 | 0.1 | 959 | -0.64 | 0 | 0 | 0.04 |  | 0.1 |
| BPW00939 | 11/03/2021 00:00 | 0.1 | 0.2 | 22.5 | 77.2 | 2 | 0.1 | 958 | -0.78 | 0 | 0 | 0.50 |  | 0.1 |
| BPW00940 | 11/03/2021 00:00 | 14.8 | 10.1 | 0.4 | 74.7 | 296 | 14.8 | 958 | -0.55 | 0 | 0 | 1.47 |  | 0.1 |
| BPW00941 | 11/03/2021 00:00 | 0.3 | 1.5 | 16.2 | 82 | 6 | 0.3 | 958 | -0.53 | 0 | 0 | 0.20 |  | 0.1 |
| BPW00935 | 11/03/2021 00:00 | FLOODED |  |  |  |  |  |  |  |  |  |  |  |  |
| BPW00942 | 11/03/2021 00:00 | 0.1 | 0.1 | 21.7 | 78.1 | 2 | 0.1 | 958 | -0.48 | 0 | 0 | 1.00 |  | 0.1 |
| BPW00936 | 11/03/2021 00:00 | 8.9 | 2.5 | 17.1 | 71.5 | 178 | 8.9 | 958 | -0.55 | 0 | 0 | 3.56 |  | 0.1 |
| BPW00937 | 11/03/2021 00:00 | 0.2 | 1.5 | 20.8 | 77.5 | 4 | 0.2 | 958 | -0.5 | 0 | 0 | 0.13 |  | 0 |

Gas Collection System Data

| ID | DATE | CH4 | CO2 | O2 | BALANCE | CH4 %LEL | PEAKCH4 | BARO | REL.PRESSURE | CO | H2S | CH4/CO2 | Res Nitrogen |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | % | % | % | % | % | % | mb | mb | ppm | ppm | % | % |
| BPIW0203 | 17/12/2020 16:04 | 0 | 0.1 | 20.8 | 79.1 | 0 |  | 974 | 0.38 | 0 | 0 | 0 | 0.48 |
| BPIW0203 | 23/12/2020 11:47 | 0 | 0.1 | 21.3 | 78.6 | 0 |  | 965 | 0.24 | 2 | 0 | 0 | 0 |
| -------- | 09/01/2021 11:20 | 56.1 | 37.8 | 4.2 | 1.9 | >>>> |  | 988 | -5.75 | 8 | 0 | 1.48 | 0 |
| -------- | 21/01/2021 15:30 | 41.4 | 34.2 | 0.9 | 23.5 | >>>> | 41.5 | 946 | -103.14 | 5 | 5 | 1.21 | 20.1 |
| BPSP0001 | 22/01/2021 09:40 | 42.8 | 32 | 2.1 | 23.1 | >>>> | 42.8 | 955 | -127.25 | 5 | 26 | 1.34 | 15.16 |
| BPSP0002 | 22/01/2021 14:11 | 46.2 | 33.5 | 2 | 18.3 | >>>> | 46.2 | 954 | -101.03 | 4 | 53 | 1.38 | 10.74 |
| BPSP0003 | 22/01/2021 14:13 | 52.2 | 32.2 | 0.3 | 15.3 | >>>> | 52.2 | 954 | -100.3 | 2 | 7 | 1.62 | 14.17 |
| BPSP0004 | 22/01/2021 14:14 | 40.7 | 32.8 | 1.8 | 24.7 | >>>> | 41.1 | 954 | -99.04 | 8 | 57 | 1.24 | 17.9 |
| BPSP0005 | 22/01/2021 14:16 | 37.2 | 25.9 | 0.5 | 36.4 | >>>> | 37.4 | 954 | -99.61 | 2 | 3 | 1.44 | 34.51 |
| BPIW0204 | 22/01/2021 14:19 | 3.4 | 1.4 | 20.6 | 74.6 | 68 | 3.3 | 954 | -32.38 | 0 | 0 | 2.43 | 0 |
| BPIW0206 | 22/01/2021 14:22 | 53.2 | 29.1 | 0.7 | 17 | >>>> | 53.2 | 954 | -61.86 | 2 | 0 | 1.83 | 14.35 |
| BPIW0211 | 22/01/2021 14:24 | 23.6 | 19.3 | 8.1 | 49 | >>>> | 24.3 | 954 | -72.05 | 1 | 0 | 1.22 | 18.38 |
| BPIW0212 | 22/01/2021 14:29 | 50.9 | 31 | 2.3 | 15.8 | >>>> | 51 | 954 | -60.87 | 4 | 5 | 1.64 | 7.11 |
| BPIW0213 | 22/01/2021 14:32 | 11.9 | 6.3 | 18.1 | 63.7 | >>>> | 11.9 | 955 | -30.37 | 2 | 0 | 1.89 | 0 |
| BPIW0214 | 22/01/2021 14:34 | 50.8 | 24.6 | 6.3 | 18.3 | >>>> | 49.5 | 955 | 0.41 | 11 | 11 | 2.07 | 0 |
| BPIW0024 | 22/01/2021 14:38 | 49.3 | 30.9 | 0.7 | 19.1 | >>>> | 49.3 | 955 | -98.77 | 2 | 3 | 1.6 | 16.45 |
| BPIW0024 | 22/01/2021 14:38 | 49.3 | 30.9 | 1.2 | 18.6 | >>>> |  | 955 | 0.37 | 3 | 2 | 1.6 | 14.06 |
| BPIW0099 | 22/01/2021 14:41 | 56.8 | 38.8 | 0.1 | 4.3 | >>>> | 56.8 | 954 | -95.4 | 14 | 5 | 1.46 | 3.92 |
| BPIW0055 | 22/01/2021 14:43 | 51.4 | 35.6 | 0.4 | 12.6 | >>>> | 51.4 | 954 | -99.62 | 1 | 11 | 1.44 | 11.09 |
| BPIW0062 | 22/01/2021 14:50 | 64.7 | 38.9 | 0.1 | 0 | >>>> | 64.7 | 954 | -97.33 | 2 | 33 | 1.66 | 0 |
| BPIW0025 | 22/01/2021 14:54 | 63 | 38.8 | 0.1 | 0 | >>>> | 63.1 | 954 | -91.75 | 6 | 36 | 1.62 | 0 |
| BPIW0030 | 22/01/2021 14:56 | 57.2 | 36.2 | 1.2 | 5.4 | >>>> | 57.1 | 954 | -91.39 | 13 | 1 | 1.58 | 0.86 |
| BPIW0054 | 22/01/2021 14:58 | 44.3 | 35.1 | 0.2 | 20.4 | >>>> | 44.3 | 954 | -52.51 | 4 | 16 | 1.26 | 19.64 |
| BPIW0032 | 22/01/2021 15:01 | 38.6 | 28.6 | 6 | 26.8 | >>>> | 38.6 | 953 | -13.38 | 3 | 0 | 1.35 | 4.12 |
| BPIW0063 | 22/01/2021 15:04 | 59.3 | 38.5 | 0.3 | 1.9 | >>>> | 59.2 | 953 | -77.41 | 3 | 24 | 1.54 | 0.77 |
| BPIW063A | 22/01/2021 15:06 | 43.6 | 29.7 | 5.9 | 20.8 | >>>> | 44.1 | 954 | -54.86 | 3 | 2 | 1.47 | 0 |
| BPIW0087 | 22/01/2021 15:08 | 65 | 38.7 | 0.1 | 0 | >>>> | 65 | 954 | -98.1 | 23 | 12 | 1.68 | 0 |
| BPIW0083 | 22/01/2021 15:16 | 29 | 30.5 | 0.1 | 40.4 | >>>> | 29.1 | 954 | -53.96 | 2 | 2 | 0.95 | 40.02 |
| BPIW0323 | 22/01/2021 15:19 | 64.2 | 37.2 | 0.5 | 0 | >>>> | 64.2 | 954 | -23.47 | 4 | 41 | 1.73 | 0 |
| BPIW0088 | 22/01/2021 15:21 | 42 | 32.4 | 0.7 | 24.9 | >>>> | 42 | 954 | -95.92 | 4 | 0 | 1.3 | 22.25 |
| BPIW0082 | 22/01/2021 15:29 | 61.2 | 42.1 | 0.1 | 0 | >>>> | 61.6 | 953 | -14.26 | 9 | 94 | 1.45 | 0 |
| BPIW0035 | 22/01/2021 15:31 | 62.3 | 41.7 | 0.1 | 0 | >>>> | 62.3 | 953 | -2.6 | 10 | 140 | 1.49 | 0 |
| BPIW0035 | 22/01/2021 15:33 | 62.4 | 41.7 | 0.1 | 0 | >>>> | 62.4 | 953 | 0.45 | 9 | 49 | 1.5 | 0 |
| BPIW0090 | 22/01/2021 15:37 | 47.8 | 33.9 | 4.3 | 14 | >>>> | 47.8 | 953 | -57.68 | 8 | 62 | 1.41 | 0 |
| BPIW0093 | 22/01/2021 15:40 | 64.1 | 39.8 | 0.1 | 0 | >>>> | 64.1 | 954 | -89.74 | 12 | 50 | 1.61 | 0 |
| BPIW0094 | 22/01/2021 15:44 | 60.9 | 40.2 | 0.6 | 0 | >>>> | 60.9 | 954 | -89.56 | 11 | 76 | 1.51 | 0 |
| BPIW0095 | 22/01/2021 15:46 | 62.7 | 41.6 | 0.1 | 0 | >>>> | 62.7 | 954 | -85.25 | 16 | 396 | 1.51 | 0 |
| BPIW0300 | 23/01/2021 10:49 | 63.3 | 39.1 | 1.3 | 0 | >>>> | 63.5 | 958 | -64.45 | 14 | 112 | 1.62 | 0 |
| BPIW0301 | 23/01/2021 10:55 | 60.3 | 43.8 | 0.1 | 0 | >>>> | 60.2 | 958 | -1.89 | 18 | 313 | 1.38 | 0 |
| BPIW0302 | 23/01/2021 10:56 | 58.8 | 44.4 | 0.1 | 0 | >>>> | 58.7 | 957 | -65.95 | 15 | 143 | 1.32 | 0 |
| BPIW0304 | 23/01/2021 10:59 | 58.2 | 42.2 | 0.9 | 0 | >>>> | 58.3 | 958 | -64.23 | 24 | 239 | 1.38 | 0 |
| BPIW0305 | 23/01/2021 11:01 | 22.1 | 21.9 | 2.7 | 53.3 | >>>> | 22.2 | 957 | -21.48 | 8 | 2 | 1.01 | 43.09 |
| BPIW0306 | 23/01/2021 11:03 | 66.5 | 36.5 | 0.2 | 0 | >>>> | 66.6 | 957 | -41.79 | 7 | 67 | 1.82 | 0 |
| BPIW0307 | 23/01/2021 11:05 | 60.2 | 43.5 | 0.2 | 0 | >>>> | 60.1 | 957 | -60.67 | 26 | 243 | 1.38 | 0 |
| BPIW0308 | 23/01/2021 11:10 | 26.3 | 25.8 | 3 | 44.9 | >>>> | 26.4 | 957 | -6.48 | 16 | 1 | 1.02 | 33.56 |
| BPIW0309 | 23/01/2021 11:12 | 0.3 | 6.6 | 18.4 | 74.7 | 6 | 0.3 | 957 | -60.59 | 6 | 0 | 0.05 | 5.15 |
| BPIW0310 | 23/01/2021 11:14 | 57.7 | 38.3 | 1.5 | 2.5 | >>>> | 57.7 | 958 | -50.46 | 6 | 16 | 1.51 | 0 |
| BPIW0311 | 23/01/2021 11:18 | 0 | 0.1 | 22.4 | 77.5 | 0 | 23 | 958 | -45.35 | 2 | 0 | 0 | 0 |
| BPIW0312 | 23/01/2021 11:23 | 0.1 | 0.1 | 22.3 | 77.5 | 2 | 6 | 958 | -39.84 | 2 | 0 | 1 | 0 |
| BPIW0313 | 23/01/2021 11:25 | 54.8 | 39.7 | 1.4 | 4.1 | >>>> | 55.4 | 958 | -35.17 | 38 | 242 | 1.38 | 0 |
| BPIW0314 | 23/01/2021 11:28 | 39.4 | 26.4 | 6.7 | 27.5 | >>>> | 47 | 958 | -72.52 | 9 | 93 | 1.49 | 2.17 |
| BPIW0315 | 23/01/2021 11:30 | 37.6 | 29 | 2.6 | 30.8 | >>>> | 37.9 | 958 | -56.28 | 9 | 1 | 1.3 | 20.97 |
| BPIW0316 | 23/01/2021 11:32 | 57.6 | 39.2 | 1.1 | 2.1 | >>>> | 57.6 | 958 | -78.26 | 9 | 4 | 1.47 | 0 |
| BPIW0317 | 23/01/2021 11:35 | 9.3 | 7.3 | 18.5 | 64.9 | >>>> | 9.3 | 958 | -86.47 | 5 | 1 | 1.27 | 0 |
| BPIW0318 | 23/01/2021 11:40 | 61.2 | 42.1 | 0.1 | 0 | >>>> | 61.2 | 957 | -15.95 | 10 | 189 | 1.45 | 0 |
| BPIW0319 | 23/01/2021 11:46 | 60.9 | 34.9 | 0.7 | 3.5 | >>>> | 60.9 | 958 | -80.21 | 7 | 267 | 1.74 | 0.85 |
| BPIW0320 | 23/01/2021 11:49 | 66 | 37.8 | 0.1 | 0 | >>>> | 66 | 958 | -48.95 | 8 | 54 | 1.75 | 0 |
| BPIW0321 | 23/01/2021 11:51 | 33.9 | 21.9 | 8.4 | 35.8 | >>>> | 33.9 | 958 | -80.83 | 6 | 41 | 1.55 | 4.05 |
| BPIW0322 | 23/01/2021 11:53 | 70.6 | 33.1 | 0.1 | 0 | >>>> | 70.6 | 958 | -81.28 | 7 | 1 | 2.13 | 0 |
| BPIW0325 | 23/01/2021 12:01 | 60.8 | 42.6 | 0.1 | 0 | >>>> | 60.8 | 958 | -37.46 | 15 | 72 | 1.43 | 0 |
| BPIW0326 | 23/01/2021 12:04 | 61.9 | 39.8 | 0.1 | 0 | >>>> | 62 | 958 | -24.2 | 11 | 42 | 1.56 | 0 |
| BPIW0327 | 23/01/2021 13:41 | 60.8 | 42.3 | 0.1 | 0 | >>>> | 60.7 | 956 | -0.75 | 7 | 133 | 1.44 | 0 |
| BPIW0033 | 23/01/2021 13:43 | 54.6 | 37.5 | 2.4 | 5.5 | >>>> | 54.6 | 957 | 0.93 | 4 | 0 | 1.46 | 0 |
| BPIW059A | 23/01/2021 13:47 | 59.4 | 39.5 | 0.1 | 1 | >>>> | 59.4 | 957 | -19.48 | 7 | 53 | 1.5 | 0.62 |
| BPIW056A | 23/01/2021 13:50 | 54.5 | 38.2 | 0.1 | 7.2 | >>>> | 54.5 | 957 | -47.65 | 5 | 45 | 1.43 | 6.82 |
| BPIW0007 | 23/01/2021 13:52 | 56.7 | 40 | 1 | 2.3 | >>>> | 56.8 | 957 | -4.05 | 7 | 10 | 1.42 | 0 |
| BPIW0068 | 23/01/2021 14:00 | 60.9 | 42.4 | 0.1 | 0 | >>>> | 60.8 | 956 | -1.81 | 7 | 128 | 1.44 | 0 |
| BPIW0084 | 23/01/2021 14:03 | 46.7 | 36.5 | 0.3 | 16.5 | >>>> | 46.7 | 956 | -1.36 | 5 | 27 | 1.28 | 15.37 |
| BPIW0085 | 23/01/2021 14:05 | 57.4 | 40.7 | 0.1 | 1.8 | >>>> | 57.4 | 957 | -3.84 | 9 | 73 | 1.41 | 1.42 |
| BPIW0067 | 23/01/2021 14:08 | 28.5 | 29.9 | 0.8 | 40.8 | >>>> | 29.8 | 956 | -2.21 | 7 | 3 | 0.95 | 37.78 |
| BPIW0066 | 23/01/2021 14:11 | 15.2 | 23.2 | 2 | 59.6 | >>>> | 15.3 | 956 | -10.17 | 7 | 1 | 0.66 | 52.04 |
| BPIW0069 | 23/01/2021 14:13 | 43.6 | 36.6 | 0.5 | 19.3 | >>>> | 43.6 | 956 | -20.6 | 8 | 14 | 1.19 | 17.41 |
| BPIW0069 | 23/01/2021 14:13 | 43.5 | 36.5 | 0.5 | 19.5 | >>>> |  | 956 | -20.97 | 8 | 14 | 1.19 | 17.61 |
| BPIW0086 | 23/01/2021 14:19 | 49.7 | 34.7 | 0.1 | 15.5 | >>>> | 49.7 | 956 | -30.05 | 5 | 25 | 1.43 | 15.12 |
| BPIW0019 | 23/01/2021 14:21 | 43.9 | 31.1 | 1.3 | 23.7 | >>>> | 44 | 956 | -13.94 | 5 | 16 | 1.41 | 18.79 |
| BPIW0052 | 23/01/2021 14:24 | 29.5 | 21.9 | 9.5 | 39.1 | >>>> | 29.5 | 956 | -63.8 | 4 | 0 | 1.35 | 3.19 |
| BPIW0021 | 23/01/2021 14:26 | 66.4 | 38.3 | 0.1 | 0 | >>>> | 66.4 | 956 | -102.71 | 7 | 63 | 1.73 | 0 |
| BPIW0039 | 23/01/2021 14:29 | 52 | 35.6 | 0.1 | 12.3 | >>>> | 52.1 | 957 | -77.56 | 11 | 9 | 1.46 | 11.92 |
| BPIW0037 | 23/01/2021 14:32 | 68.3 | 36.3 | 0.1 | 0 | >>>> | 68.3 | 957 | 1.01 | 6 | 41 | 1.88 | 0 |
| BPIW037A | 23/01/2021 14:35 | 68 | 37 | 0.1 | 0 | >>>> | 68 | 957 | 1.14 | 5 | 68 | 1.84 | 0 |
| BPIW053A | 23/01/2021 14:37 | 24.2 | 26.4 | 0.6 | 48.8 | >>>> | 24.3 | 957 | -73.41 | 6 | 0 | 0.92 | 46.53 |
| BPIW0038 | 23/01/2021 14:49 | 66.9 | 37.5 | 0.1 | 0 | >>>> | 66.9 | 957 | -59.03 | 4 | 60 | 1.78 | 0 |
| BPIW0022 | 23/01/2021 14:53 | 65.1 | 39.1 | 0.1 | 0 | >>>> | 65.1 | 867 | -100.33 | 5 | 49 | 1.66 | 0 |
| BPIW040A | 23/01/2021 14:58 | 40.6 | 32.3 | 0.2 | 26.9 | >>>> | 40.6 | 957 | -18.03 | 9 | 45 | 1.26 | 26.14 |
| BPIW041B | 23/01/2021 15:07 | 40.1 | 26.8 | 2.8 | 30.3 | >>>> | 40.7 | 960 | -100.39 | 4 | 28 | 1.5 | 19.72 |
| BPIW013A | 23/01/2021 15:12 | 6.2 | 3.6 | 20.8 | 69.4 | >>>> | 6 | 958 | -101.24 | 1 | 0 | 1.72 | 0 |
| BPIW0044 | 23/01/2021 15:17 | 67 | 37.1 | 0.2 | 0 | >>>> | 67.1 | 958 | -10.47 | 3 | 10 | 1.81 | 0 |
| BPIW020A | 23/01/2021 15:20 | 60 | 40.6 | 0.1 | 0 | >>>> | 60 | 957 | -91.23 | 6 | 39 | 1.48 | 0 |
| BPIW0045 | 23/01/2021 15:28 | 62.7 | 41.8 | 0 | 0 | >>>> | 62.5 | 957 | -4.95 | 5 | 74 | 1.5 | 0 |
| BPIW045A | 23/01/2021 15:33 | 59.2 | 43.1 | 0.3 | 0 | >>>> | 59.2 | 957 | 0.54 | 5 | 0 | 1.37 | 0 |
| BPIW046A | 23/01/2021 15:35 | 0 | 0.1 | 21.8 | 78.1 | 0 | 0.1 | 958 | -14.18 | 2 | 0 | 0 | 0 |
| BPIW0011 | 23/01/2021 15:37 | 42.4 | 35.9 | 0.3 | 21.4 | >>>> | 42.4 | 958 | -97.19 | 11 | 28 | 1.18 | 20.27 |
| BPIW011A | 23/01/2021 15:39 | 38.1 | 32.9 | 0.3 | 28.7 | >>>> | 38 | 958 | -59.06 | 7 | 1 | 1.16 | 27.57 |
| BPIW0017 | 24/01/2021 10:42 | 57 | 38.4 | 1.1 | 3.5 | >>>> | 56.6 | 956 | -36.27 | 7 | 4 | 1.48 | 0 |
| BPIW0016 | 24/01/2021 10:45 | 55.7 | 36.7 | 0.2 | 7.4 | >>>> | 55.7 | 956 | -67.73 | 3 | 2 | 1.52 | 6.64 |
| BPIW0018 | 24/01/2021 10:47 | 13.1 | 21.7 | 0.1 | 65.1 | >>>> | 13.8 | 956 | -86.43 | 10 | 0 | 0.6 | 64.72 |
| BPIW0014 | 24/01/2021 11:27 | 46.3 | 32 | 5.6 | 16.1 | >>>> | 44 | 956 | 0.49 | 3 | 19 | 1.45 | 0 |
| BPIW0106 | 24/01/2021 11:30 | 41.4 | 33.9 | 0.4 | 24.3 | >>>> | 41.4 | 955 | -33.23 | 3 | 0 | 1.22 | 22.79 |
| BPIW0047 | 24/01/2021 11:32 | 47.8 | 36.4 | 0.3 | 15.5 | >>>> | 47.8 | 955 | -33.13 | 4 | 1 | 1.31 | 14.37 |
| BPIW0049 | 24/01/2021 11:35 | 15.9 | 10.1 | 12.8 | 61.2 | >>>> | 15.9 | 955 | -12.81 | 0 | 0 | 1.57 | 12.82 |
| BPIW0048 | 24/01/2021 11:37 | 58.4 | 39.2 | 1.4 | 1 | >>>> | 58.1 | 955 | -12.6 | 3 | 19 | 1.49 | 0 |
| BPIW0043 | 24/01/2021 11:40 | 17.7 | 23.2 | 0.5 | 58.6 | >>>> | 18.1 | 955 | 0.07 | 7 | 0 | 0.76 | 56.71 |
| BPIW0209 | 24/01/2021 11:43 | 18.7 | 9.3 | 15.2 | 56.8 | >>>> | 22.1 | 955 | -25.5 | 0 | 0 | 2.01 | 0 |
| BPIW0208 | 24/01/2021 11:46 | 31.7 | 28.6 | 0.7 | 39 | >>>> | 31.6 | 955 | -128.2 | 1 | 0 | 1.11 | 36.35 |
| BPIW208A | 24/01/2021 11:47 | 0 | 0.1 | 22.3 | 77.6 | 0 | 0 | 956 | -12.15 | 0 | 0 | 0 | 0 |
| BPIW0216 | 24/01/2021 11:50 | 23.8 | 30 | 0.6 | 45.6 | >>>> | 23.9 | 956 | -94.45 | 1 | 0 | 0.79 | 43.33 |
| BPIW0207 | 24/01/2021 11:53 | 0.6 | 0.9 | 21.9 | 76.6 | 12 | 0.6 | 956 | 0.5 | 0 | 0 | 0.67 | 0 |
| BPIW0215 | 24/01/2021 11:58 | 0 | 0.1 | 22.4 | 77.5 | 0 | 0 | 956 | -30.41 | 0 | 0 | 0 | 0 |
| BPIW0202 | 24/01/2021 12:00 | 27.1 | 16.8 | 8.6 | 47.5 | >>>> | 26.8 | 956 | -77.23 | 0 | 0 | 1.61 | 14.99 |
| BPIW0203 | 24/01/2021 12:03 | 30.4 | 24.7 | 0.2 | 44.7 | >>>> | 30.4 | 956 | -55.21 | 0 | 1 | 1.23 | 43.94 |
| BPSP0001 | 24/02/2021 09:15 | 57.6 | 35.6 | 1.3 | 5.5 | >>>> | 57.6 | 978 | -50.45 | 13 | 41 | 1.62 | 0.59 |
| BPSP0002 | 24/02/2021 09:17 | 58.5 | 35.5 | 1.3 | 4.7 | >>>> | 58.5 | 977 | -41.23 | 13 | 50 | 1.65 | 0 |
| BPSP0003 | 24/02/2021 09:19 | 57.1 | 36.8 | 1.7 | 4.4 | >>>> | 57.1 | 977 | -37 | 14 | 67 | 1.55 | 0 |
| BPSP0004 | 24/02/2021 09:21 | 57.4 | 37 | 1.8 | 3.8 | >>>> | 57.4 | 977 | -35.28 | 14 | 79 | 1.55 | 0 |
| BPSP0004 | 24/02/2021 09:21 | 57.5 | 37 | 1.8 | 3.7 | >>>> |  | 977 | -35.12 | 14 | 79 | 1.55 | 0 |
| BPSP0005 | 24/02/2021 09:23 | 55.6 | 26.4 | 0.1 | 17.9 | >>>> | 55.6 | 977 | -33.42 | 5 | 5 | 2.11 | 17.52 |
| BPSP0009 | 24/02/2021 09:26 | 67.3 | 13.3 | 5.3 | 14.1 | >>>> | 66.4 | 977 | -5.17 | 12 | 2 | 5.06 | 0 |
| BPIW0206 | 24/02/2021 09:29 | 69.1 | 30.6 | 0.1 | 0.2 | >>>> | 69.1 | 977 | -14.3 | 6 | 7 | 2.26 | 0 |
| BPIW0211 | 24/02/2021 09:32 | 75.4 | 32.1 | 0.1 | 0 | >>>> | 75.4 | 977 | -10.86 | 10 | 4 | 2.35 | 0 |
| BPIW0212 | 24/02/2021 09:35 | 74.2 | 34.2 | 0.1 | 0 | >>>> | 74.3 | 977 | -22.56 | 7 | 11 | 2.17 | 0 |
| BPIW0213 | 24/02/2021 09:38 | 75.5 | 33.4 | 0.1 | 0 | >>>> | 75.5 | 978 | -13.59 | 5 | 9 | 2.26 | 0 |
| BPIW0214 | 24/02/2021 09:40 | 68.8 | 31.5 | 2 | 0 | >>>> | 67.8 | 978 | 0.27 | 6 | 10 | 2.18 | 0 |
| BPIW0024 | 24/02/2021 09:43 | 62.7 | 36.8 | 0.7 | 0 | >>>> | 62.6 | 979 | -25.43 | 5 | 0 | 1.7 | 0 |
| BPIW0099 | 24/02/2021 09:46 | 69.9 | 40 | 0.2 | >>>> | >>>> | 69.9 | 977 | -24.88 | 20 | 8 | 1.75 | 0 |
| BPIW0055 | 24/02/2021 09:48 | 71.9 | 38.3 | 0.1 | >>>> | >>>> | 71.8 | 977 | -24.08 | 4 | 10 | 1.88 | 0 |
| BPIW0062 | 24/02/2021 11:03 | 71.7 | 38.9 | 0.1 | >>>> | >>>> | 71.7 | 978 | -22.58 | 4 | 37 | 1.84 | 0 |
| BPIW0025 | 24/02/2021 11:06 | 71.2 | 39.4 | 0.1 | >>>> | >>>> | 71.1 | 978 | -20.06 | 7 | 48 | 1.81 | 0 |
| BPIW0030 | 24/02/2021 11:08 | 68.7 | 37.8 | 0.6 | 0 | >>>> | 68.7 | 977 | -19.86 | 22 | 3 | 1.82 | 0 |
| BPIW0054 | 24/02/2021 11:09 | 67.5 | 38.2 | 0.8 | 0 | >>>> | 67.5 | 977 | -7.58 | 7 | 1 | 1.77 | 0 |
| BPIW0032 | 24/02/2021 11:14 | 54.2 | 35.7 | 3.4 | 6.7 | >>>> | 53.7 | 976 | 0.96 | 5 | 0 | 1.52 | 0 |
| BPIW0063 | 24/02/2021 11:16 | 69.9 | 40.9 | 0.1 | >>>> | >>>> | 69.9 | 976 | -20.24 | 7 | 27 | 1.71 | 0 |
| BPIW063A | 24/02/2021 11:39 | 69.3 | 41.3 | 0.2 | >>>> | >>>> | 69.2 | 977 | -4.61 | 7 | 6 | 1.68 | 0 |
| BPIW0087 | 24/02/2021 11:42 | 70.1 | 41 | 0 | >>>> | >>>> | 70.1 | 976 | -23 | 37 | 6 | 1.71 | 0 |
| BPIW0083 | 24/02/2021 11:44 | 65.1 | 38.7 | 0 | 0 | >>>> | 65 | 976 | -13.91 | 6 | 3 | 1.68 | 0 |
| BPIW0323 | 24/02/2021 11:46 | 69.4 | 38.4 | 0.9 | 0 | >>>> | 69.7 | 977 | -10.94 | 7 | 64 | 1.81 | 0 |
| BPIW0088 | 24/02/2021 11:49 | 67.4 | 39.1 | 0 | 0 | >>>> | 67.4 | 977 | -23.57 | 8 | 6 | 1.72 | 0 |
| BPIW0082 | 24/02/2021 11:53 | 69.4 | 42.7 | 0.1 | >>>> | >>>> | 69.4 | 977 | 0.43 | 13 | 123 | 1.63 | 0 |
| BPIW0035 | 24/02/2021 11:55 | 69.7 | 42.3 | 0.1 | >>>> | >>>> | 69.7 | 977 | -9.3 | 18 | 99 | 1.65 | 0 |
| BPIW0090 | 24/02/2021 11:58 | 69.1 | 42.7 | 0.1 | >>>> | >>>> | 69.1 | 977 | -10.62 | 13 | 74 | 1.62 | 0 |
| BPIW0093 | 24/02/2021 12:01 | 67.5 | 40.1 | 0.9 | 0 | >>>> | 67.4 | 978 | 0.35 | 17 | 11 | 1.68 | 0 |
| BPIW0094 | 24/02/2021 12:04 | 64.2 | 41.2 | 4.1 | 0 | >>>> | 64.6 | 978 | -18.73 | 10 | 77 | 1.56 | 0 |
| BPIW0095 | 24/02/2021 12:08 | 69.7 | 42.3 | 0 | >>>> | >>>> | 69.7 | 977 | -21.84 | 23 | 335 | 1.65 | 0 |
| BPIW0300 | 24/02/2021 12:37 | 66.9 | 40.3 | 0.6 | 0 | >>>> | 82 | 978 | -16.65 | 25 | 0 | 1.66 | 0 |
| BPIW0301 | 24/02/2021 12:44 | 4.9 | 5.3 | 20.9 | 68.9 | 98 | 7.3 | 978 | -17.94 | 6 | 0 | 0.92 | 0 |
| BPIW0303 | 24/02/2021 12:47 | 62.2 | 43.7 | 0.7 | 0 | >>>> | 75.5 | 977 | -20.42 | 16 | 0 | 1.42 | 0 |
| BPIW0304 | 24/02/2021 12:49 | 62.8 | 42.1 | 0.8 | 0 | >>>> | 70.4 | 977 | -19.78 | 19 | 0 | 1.49 | 0 |
| BPIW0305 | 24/02/2021 12:52 | 69 | 38.1 | 0.5 | 0 | >>>> | 74.5 | 978 | -20.89 | 7 | 0 | 1.81 | 0 |
| BPIW0306 | 24/02/2021 14:02 | 69 | 36.5 | 0.9 | 0 | >>>> | 78.5 | 977 | -21.72 | 8 | 0 | 1.89 | 0 |
| BPIW0307 | 24/02/2021 14:04 | 62.4 | 43.8 | 0.6 | 0 | >>>> | 71 | 977 | -16.61 | 29 | 0 | 1.42 | 0 |
| BPIW0308 | 24/02/2021 14:06 | 63.8 | 44.6 | 0.3 | 0 | >>>> | 71.3 | 976 | -8.23 | 23 | 0 | 1.43 | 0 |
| BPIW0309 | 24/02/2021 14:08 | 4.8 | 6.6 | 20.3 | 68.3 | 96 | 5.2 | 977 | -21.2 | 7 | 0 | 0.73 | 0 |
| BPIW0310 | 24/02/2021 14:12 | 66.5 | 41 | 0.4 | 0 | >>>> | 77.4 | 976 | -4.18 | 6 | 0 | 1.62 | 0 |
| BPIW0311 | 24/02/2021 14:14 | 64.6 | 40.3 | 0.9 | 0 | >>>> | 71.6 | 976 | -23.01 | 7 | 0 | 1.6 | 0 |
| BPIW0312 | 24/02/2021 14:16 | 68.6 | 27 | 2.7 | 1.7 | >>>> | 72.6 | 977 | -32.87 | 3 | 0 | 2.54 | 0 |
| BPIW0313 | 24/02/2021 14:22 | 64.5 | 43.4 | 0.4 | 0 | >>>> | 73 | 976 | 0.3 | 25 | 0 | 1.49 | 0 |
| BPIW0314 | 24/02/2021 14:24 | 59.8 | 36.1 | 2.7 | 1.4 | >>>> | 66.1 | 977 | -29.98 | 8 | 0 | 1.66 | 0 |
| BPIW0315 | 24/02/2021 14:26 | 64.1 | 39.6 | 1.1 | 0 | >>>> | 70.9 | 976 | -26.13 | 8 | 0 | 1.62 | 0 |
| BPIW0316 | 24/02/2021 14:28 | 62.1 | 39.1 | 1.7 | 0 | >>>> | 69.1 | 977 | -47.28 | 8 | 0 | 1.59 | 0 |
| BPIW0317 | 24/02/2021 14:30 | 10.1 | 8.6 | 19.2 | 62.1 | >>>> | 10 | 976 | -42.59 | 2 | 0 | 1.17 | 0 |
| BPIW0318 | 24/02/2021 14:32 | 62.2 | 37.7 | 1.9 | 0 | >>>> | 71.7 | 976 | -42.4 | 7 | 0 | 1.65 | 0 |
| BPIW0319 | 24/02/2021 14:34 | 52.9 | 26.4 | 6 | 14.7 | >>>> | 54 | 977 | -75.65 | 4 | 0 | 2 | 0 |
| BPIW0320 | 24/02/2021 14:41 | 69.3 | 41.5 | 0 | >>>> | >>>> | 69.1 | 977 | -84.81 | 7 | 66 | 1.67 | 0 |
| BPIW0321 | 24/02/2021 14:43 | 48.2 | 26.6 | 6.7 | 18.5 | >>>> | 48 | 976 | -89.03 | 3 | 32 | 1.81 | 0 |
| BPIW0322 | 24/02/2021 14:45 | 64.5 | 34.9 | 1.3 | 0 | >>>> | 64.8 | 976 | -89.6 | 4 | 1 | 1.85 | 0 |
| BPIW0325 | 24/02/2021 15:28 | 68.6 | 43.6 | 0 | >>>> | >>>> | 68.6 | 977 | 0.34 | 16 | 60 | 1.57 | 0 |
| BPIW0326 | 24/02/2021 15:33 | 70.3 | 40.9 | 0 | >>>> | >>>> | 70.1 | 976 | 0.22 | 8 | 48 | 1.72 | 0 |
| BPIW0327 | 24/02/2021 15:37 | 65.9 | 45.6 | 0 | >>>> | >>>> | 65.9 | 976 | 1.04 | 14 | 131 | 1.45 | 0 |
| BPIW0327 | 24/02/2021 15:38 | 66.1 | 45.6 | 0 | >>>> | >>>> | 66.1 | 975 | -18.63 | 14 | 130 | 1.45 | 0 |
| BPIW0033 | 24/02/2021 15:44 | 53.3 | 36.2 | 3 | 7.5 | >>>> | 53.2 | 975 | 0.24 | 7 | 1 | 1.47 | 0 |
| BPIW059A | 24/02/2021 15:46 | 69.1 | 42.2 | 0 | >>>> | >>>> | 69.1 | 975 | -9.77 | 12 | 52 | 1.64 | 0 |
| BPIW056A | 24/02/2021 15:49 | 66 | 40.6 | 0 | 0 | >>>> | 66 | 975 | -44.19 | 10 | 43 | 1.63 | 0 |
| BPIW0007 | 24/02/2021 15:50 | 67.3 | 43.9 | 0 | >>>> | >>>> | 67.2 | 975 | -4.16 | 13 | 30 | 1.53 | 0 |
| BPIW0068 | 25/02/2021 08:32 | 64.4 | 43.7 | 0.1 | 0 | >>>> | 64.4 | 984 | -1.44 | 13 | 147 | 1.47 | 0 |
| BPIW0084 | 25/02/2021 08:35 | 64.3 | 43.2 | 0.1 | 0 | >>>> | 64.3 | 985 | -0.96 | 10 | 47 | 1.49 | 0 |
| BPIW0085 | 25/02/2021 08:37 | 56.6 | 42 | 0 | 1.4 | >>>> | 56.5 | 985 | -4.37 | 13 | 77 | 1.35 | 1.4 |
| BPIW0067 | 25/02/2021 08:39 | 3.3 | 2.6 | 17.9 | 76.2 | 66 |  | 985 | 0.3 | 6 | 14 | 1.27 | 8.54 |
| BPIW0067 | 25/02/2021 08:41 | 42.9 | 37 | 1.5 | 18.6 | >>>> | 42.9 | 985 | -0.87 | 12 | 3 | 1.16 | 12.93 |
| BPIW0066 | 25/02/2021 08:43 | 67.9 | 42.5 | 0 | >>>> | >>>> | 67.8 | 984 | -1.36 | 9 | 374 | 1.6 | 0 |
| BPIW0069 | 25/02/2021 08:46 | 59.5 | 41.8 | 0.3 | 0 | >>>> | 59.5 | 985 | -10.57 | 13 | 15 | 1.42 | 0 |
| BPIW0086 | 25/02/2021 08:50 | 5.7 | 4.4 | 20.9 | 69 | >>>> | 5.7 | 984 | -0.68 | 7 | 0 | 1.3 | 0 |
| BPIW0019 | 25/02/2021 08:53 | 49.9 | 32.4 | 1.1 | 16.6 | >>>> | 49.9 | 984 | -54.24 | 7 | 10 | 1.54 | 12.44 |
| BPIW0052 | 25/02/2021 08:56 | 66.6 | 40.4 | 0.7 | 0 | >>>> | 66.7 | 984 | -11.16 | 8 | 7 | 1.65 | 0 |
| BPIW0021 | 25/02/2021 08:58 | 70.8 | 39.6 | 0.1 | >>>> | >>>> | 70.7 | 985 | -83.79 | 11 | 53 | 1.79 | 0 |
| BPIW0039 | 25/02/2021 09:00 | 52.3 | 35.5 | 0.1 | 12.1 | >>>> | 52.3 | 985 | -67.94 | 14 | 9 | 1.47 | 11.72 |
| BPIW0037 | 25/02/2021 09:03 | 65.7 | 36.7 | 1.5 | 0 | >>>> | 65.5 | 985 | -72.93 | 8 | 36 | 1.79 | 0 |
| BPIW037A | 25/02/2021 09:06 | 71.8 | 38.7 | 0 | >>>> | >>>> | 71.8 | 985 | -19.38 | 9 | 65 | 1.86 | 0 |
| BPIW053A | 25/02/2021 09:08 | 19.8 | 13 | 15.9 | 51.3 | >>>> | 19.9 | 985 | -6.38 | 7 | 0 | 1.52 | 0 |
| BPIW0038 | 25/02/2021 11:46 | 60.2 | 36.9 | 0.1 | 2.8 | >>>> | 60.2 | 987 | -72.23 | 9 | 38 | 1.63 | 2.42 |
| BPIW0022 | 25/02/2021 11:49 | 70.2 | 38.9 | 0 | 0 | >>>> | 70.2 | 987 | -86.19 | 9 | 46 | 1.8 | 0 |
| BPIW042B | 25/02/2021 11:51 | 27.7 | 16.9 | 11.8 | 43.6 | >>>> | 27.7 | 987 | -8.03 | 64 | 0 | 1.64 | 0 |
| BPIW041B | 25/02/2021 12:00 | 54.3 | 28.7 | 1.1 | 15.9 | >>>> | 54.3 | 987 | -79.38 | 7 | 15 | 1.89 | 11.74 |
| BPIW013A | 25/02/2021 12:02 | 71.5 | 35 | 0 | 0 | >>>> | 71.5 | 987 | -79.92 | 8 | 13 | 2.04 | 0 |
| BPIW0044 | 25/02/2021 12:05 | 68.1 | 35.9 | 0.5 | 0 | >>>> | 68.1 | 988 | -58.3 | 6 | 3 | 1.9 | 0 |
| BPIW020A | 25/02/2021 12:07 | 66.3 | 41.3 | 0.4 | 0 | >>>> | 66.2 | 987 | -78.79 | 9 | 2 | 1.61 | 0 |
| BPIW0045 | 25/02/2021 12:11 | 68.1 | 42.8 | 0.1 | >>>> | >>>> | 68 | 986 | -10.63 | 8 | 73 | 1.59 | 0 |
| BPIW045A | 25/02/2021 12:20 | 66.3 | 44.1 | 0.1 | >>>> | >>>> | 66.3 | 986 | 0.32 | 8 | 0 | 1.5 | 0 |
| BPIW046A | 25/02/2021 12:23 | 66.1 | 44.8 | 0.1 | >>>> | >>>> | 66 | 987 | -19.18 | 24 | 33 | 1.48 | 0 |
| BPIW0011 | 25/02/2021 12:26 | 63 | 42.5 | 0 | 0 | >>>> | 63 | 987 | -87.18 | 17 | 40 | 1.48 | 0 |
| BPIW011A | 25/02/2021 12:28 | 59.2 | 39.8 | 0.8 | 0.2 | >>>> | 59.9 | 987 | -66.84 | 10 | 7 | 1.49 | 0 |
| BPIW0017 | 25/02/2021 12:31 | 63.8 | 38.5 | 1.6 | 0 | >>>> | 63.7 | 987 | -84.33 | 12 | 6 | 1.66 | 0 |
| BPIW0016 | 25/02/2021 12:33 | 68.3 | 38 | 0.3 | 0 | >>>> | 68.3 | 988 | -84.67 | 8 | 4 | 1.8 | 0 |
| BPIW0018 | 25/02/2021 12:35 | 65.1 | 29.9 | 0 | 5 | >>>> | 65.1 | 989 | -40.33 | 9 | 5 | 2.18 | 5 |
| BPIW0104 | 25/02/2021 12:37 | 71.5 | 37.4 | 0 | 0 | >>>> | 71.4 | 988 | -87.64 | 6 | 17 | 1.91 | 0 |
| BPIW0014 | 25/02/2021 12:43 | 65.2 | 39.1 | 1.3 | 0 | >>>> | 66.5 | 988 | -58.86 | 10 | 34 | 1.67 | 0 |
| BPIW0106 | 25/02/2021 12:46 | 67.6 | 38.2 | 0 | 0 | >>>> | 67.6 | 987 | -79.81 | 8 | 9 | 1.77 | 0 |
| BPIW0047 | 25/02/2021 12:48 | 69.6 | 38.6 | 0 | 0 | >>>> | 69.6 | 988 | -80.55 | 8 | 4 | 1.8 | 0 |
| BPIW0049 | 25/02/2021 13:53 | 19.3 | 6 | 18.1 | 56.6 | >>>> | 18.7 | 988 | 0.31 | 5 | 0 | 3.22 | 0 |
| BPIW0048 | 25/02/2021 13:55 | 67.8 | 41.1 | 0.1 | 0 | >>>> | 67.8 | 988 | -35.81 | 7 | 0 | 1.65 | 0 |
| BPIW0043 | 25/02/2021 13:58 | 69.1 | 39.9 | 0.1 | 0 | >>>> | 69 | 988 | -12.14 | 12 | 5 | 1.73 | 0 |
| BPIW0209 | 25/02/2021 14:00 | 85.8 | 23 | 0.1 | 0 | >>>> | 86.6 | 988 | -4.61 | 9 | 8 | 3.73 | 0 |
| BPIW0208 | 25/02/2021 14:02 | 66.1 | 32.3 | 0.1 | 1.5 | >>>> | 66.2 | 988 | -80.15 | 10 | 8 | 2.05 | 1.12 |
| BPIW208A | 25/02/2021 14:05 | 4.4 | 2.3 | 22 | 71.3 | 88 | 4.5 | 989 | -44.01 | 2 | 0 | 1.91 | 0 |
| BPIW0216 | 25/02/2021 14:07 | 74.8 | 33.6 | 0.1 | 0 | >>>> | 74.8 | 989 | -49.5 | 15 | 3 | 2.23 | 0 |
| BPIW0207 | 25/02/2021 14:09 | 24.6 | 12.9 | 16.3 | 46.2 | >>>> | 23.1 | 988 | -0.02 | 6 | 1 | 1.91 | 0 |
| BPIW0215 | 25/02/2021 14:11 | 69.3 | 30.9 | 0.1 | 0 | >>>> | 69.3 | 988 | -49.82 | 10 | 26 | 2.24 | 0 |
| BPIW0202 | 25/02/2021 14:13 | 72.6 | 22.6 | 0 | 4.8 | >>>> | 72.7 | 988 | -53.94 | 7 | 10 | 3.21 | 4.8 |
| BPIW0203 | 25/02/2021 14:15 | 56 | 25.6 | 0 | 18.4 | >>>> | 56 | 988 | -64.19 | 5 | 2 | 2.19 | 18.4 |
| -------- | 23/03/2021 15:58 | 0.3 | 0.1 | 21 | 78.6 | 6 |  | 982 | 0.02 | 56 | 0 | 3 | 0 |
| BPSP0001 | 24/03/2021 09:24 | 48.5 | 33.2 | 1.6 | 16.7 | >>>> | 48.5 | 981 | -79.94 | 9 | 23 | 1.46 | 10.65 |
| BPSP0002 | 24/03/2021 10:57 | 47.6 | 33.7 | 1.5 | 17.2 | >>>> | 47.5 | 981 | -81.48 | 8 | 36 | 1.41 | 11.53 |
| BPSP0003 | 24/03/2021 10:59 | 68 | 33.2 | 0.6 | 0 | >>>> | 68.1 | 982 | -82.08 | 5 | 2 | 2.05 | 0 |
| BPSP0004 | 24/03/2021 11:01 | 51.1 | 35.5 | 1.3 | 12.1 | >>>> | 51 | 982 | -80.96 | 9 | 41 | 1.44 | 7.19 |
| BPSP0005 | 24/03/2021 11:03 | 47.2 | 27.9 | 0.2 | 24.7 | >>>> | 47.2 | 981 | -80.96 | 4 | 2 | 1.69 | 23.94 |
| BPIW0204 | 24/03/2021 11:05 | 76.7 | 16.4 | 2.6 | 4.3 | >>>> | 75.8 | 981 | -1.22 | 4 | 0 | 4.68 | 0 |
| BPIW0206 | 24/03/2021 11:08 | 47.1 | 28.3 | 0.8 | 23.8 | >>>> | 47.1 | 981 | -64.47 | 4 | 0 | 1.66 | 20.78 |
| BPIW0211 | 24/03/2021 11:11 | 48.1 | 24.7 | 5.1 | 22.1 | >>>> | 48.1 | 981 | -33.61 | 3 | 0 | 1.95 | 2.82 |
| BPIW0212 | 24/03/2021 11:15 | 53.4 | 32.4 | 0.1 | 14.1 | >>>> | 53.4 | 981 | -68.68 | 7 | 3 | 1.65 | 13.72 |
| BPIW0213 | 24/03/2021 11:18 | 14.2 | 6.5 | 18.5 | 60.8 | >>>> | 14.3 | 982 | -27.26 | 2 | 0 | 2.18 | 0 |
| BPIW0214 | 24/03/2021 11:20 | 59.3 | 27.5 | 4.4 | 8.8 | >>>> | 59 | 982 | -59.16 | 5 | 7 | 2.16 | 0 |
| BPIW0024 | 24/03/2021 11:23 | 68.9 | 33.5 | 0.9 | 0 | >>>> | 68.9 | 982 | -82.26 | 6 | 2 | 2.06 | 0 |
| BPIW0099 | 24/03/2021 11:26 | 67.2 | 39.1 | 0.1 | 0 | >>>> | 67.2 | 981 | -77.84 | 23 | 2 | 1.72 | 0 |
| BPIW0055 | 24/03/2021 11:28 | 60 | 36.1 | 0.6 | 3.3 | >>>> | 60 | 981 | -82.34 | 4 | 3 | 1.66 | 1.03 |
| BPIW0055 | 24/03/2021 11:36 | 45.4 | 27.3 | 7.3 | 20 | >>>> | 59.3 | 981 | -79.78 | 3 | 0 | 1.66 | 0 |
| BPIW0025 | 24/03/2021 11:39 | 70.5 | 39.7 | 0.1 | >>>> | >>>> | 70.5 | 981 | -75.11 | 8 | 27 | 1.78 | 0 |
| BPIW0030 | 24/03/2021 11:41 | 70.1 | 39.4 | 0.1 | 0 | >>>> | 70 | 981 | -73.98 | 20 | 75 | 1.78 | 0 |
| BPIW0054 | 24/03/2021 11:43 | 51.5 | 35.5 | 0.2 | 12.8 | >>>> | 51.4 | 980 | -29.96 | 7 | 1 | 1.45 | 12.04 |
| BPIW0032 | 24/03/2021 11:49 | 39.8 | 28 | 7.1 | 25.1 | >>>> | 39.8 | 980 | -10.21 | 5 | 0 | 1.42 | 0 |
| BPIW0063 | 24/03/2021 11:53 | 62.1 | 38.9 | 1 | 0 | >>>> | 62.1 | 979 | -75.6 | 6 | 1 | 1.6 | 0 |
| BPIW063A | 24/03/2021 11:55 | 19.7 | 11.9 | 16.8 | 51.6 | >>>> | 19.8 | 981 | -18.26 | 3 | 0 | 1.66 | 0 |
| BPIW0087 | 24/03/2021 11:57 | 70.3 | 41.5 | 0.1 | >>>> | >>>> | 70.3 | 980 | -81.27 | 29 | 2 | 1.69 | 0 |
| BPIW0083 | 24/03/2021 12:02 | 34.1 | 31.8 | 0.1 | 34 | >>>> | 34.1 | 980 | -43.2 | 4 | 2 | 1.07 | 33.62 |
| BPIW0323 | 24/03/2021 12:04 | 52.8 | 32.5 | 4.7 | 10 | >>>> | 59.2 | 980 | -53.61 | 5 | 46 | 1.62 | 0 |
| BPIW0088 | 24/03/2021 12:07 | 54 | 35.5 | 1.1 | 9.4 | >>>> | 54 | 980 | -83.77 | 6 | 1 | 1.52 | 5.24 |
| BPIW0090 | 24/03/2021 12:39 | 55.5 | 36.7 | 3.5 | 4.3 | >>>> | 55.4 | 980 | -46.22 | 10 | 93 | 1.51 | 0 |
| BPIW0035 | 24/03/2021 12:43 | 69.6 | 42.4 | 0 | >>>> | >>>> | 69.5 | 980 | -16.47 | 16 | 108 | 1.64 | 0 |
| BPIW0082 | 24/03/2021 12:46 | 66.6 | 42 | 0.7 | 0 | >>>> | 66.6 | 980 | -9.33 | 19 | 102 | 1.59 | 0 |
| -------- | 24/03/2021 14:27 | 67.9 | 42.1 | 0.8 | >>>> | >>>> | 67.9 | 980 | -69.2 | 15 | 47 | 1.61 | 0 |
| -------- | 24/03/2021 14:30 | 68.2 | 43.6 | 0 | >>>> | >>>> | 68.3 | 980 | -46.57 | 15 | 1 | 1.56 | 0 |
| -------- | 24/03/2021 14:36 | 7.7 | 4.5 | 18 | 69.8 | >>>> |  | 979 | 0.64 | 7 | 0 | 1.71 | 1.76 |
| BPIW0300 | 24/03/2021 15:06 | 68 | 44 | 0.1 | >>>> | >>>> | 68 | 981 | -66.33 | 29 | 104 | 1.55 | 0 |
| BPIW0301 | 24/03/2021 15:09 | 65.2 | 45.7 | 0 | >>>> | >>>> | 65.4 | 981 | -50.43 | 21 | 70 | 1.43 | 0 |
| BPIW0303 | 24/03/2021 15:12 | 32.3 | 25.9 | 10.1 | 31.7 | >>>> | 38.3 | 980 | -64.85 | 11 | 5 | 1.25 | 0 |
| BPIW0304 | 24/03/2021 15:14 | 65.2 | 44.9 | 0.7 | >>>> | >>>> | 65.2 | 980 | -64.76 | 23 | 188 | 1.45 | 0 |
| BPIW0305 | 24/03/2021 15:16 | 26.1 | 26.3 | 3.2 | 44.4 | >>>> | 26.1 | 980 | -35.2 | 10 | 1 | 0.99 | 32.3 |
| BPIW0306 | 24/03/2021 15:19 | 30.5 | 20.9 | 11.9 | 36.7 | >>>> | 30.5 | 980 | -69.1 | 11 | 16 | 1.46 | 0 |
| BPIW0307 | 24/03/2021 15:22 | 65.2 | 46.5 | 0 | >>>> | >>>> | 65.1 | 980 | -63.38 | 32 | 201 | 1.4 | 0 |
| BPIW0308 | 24/03/2021 15:24 | 56.3 | 39.5 | 1.8 | 2.4 | >>>> | 56.3 | 979 | -3.12 | 20 | 108 | 1.43 | 0 |
| BPIW0309 | 24/03/2021 15:26 | 4.1 | 2.9 | 21.6 | 71.4 | 82 | 4.1 | 980 | -68.86 | 6 | 1 | 1.41 | 0 |
| BPIW0310 | 24/03/2021 15:29 | 44.7 | 32.4 | 6.5 | 16.4 | >>>> | 44.7 | 980 | -69.5 | 20 | 1 | 1.38 | 0 |
| BPIW0311 | 24/03/2021 15:32 | 52.4 | 36.3 | 4.1 | 7.2 | >>>> | 52.4 | 980 | -46.61 | 11 | 10 | 1.44 | 0 |
| BPIW0312 | 24/03/2021 15:34 | 66.2 | 44.3 | 0 | >>>> | >>>> | 66.2 | 980 | -5.05 | 16 | 123 | 1.49 | 0 |
| BPIW0313 | 24/03/2021 15:37 | 66.6 | 45.4 | 0 | >>>> | >>>> | 66.6 | 980 | -2.03 | 35 | 238 | 1.47 | 0 |
| BPIW0314 | 24/03/2021 15:39 | 65.6 | 39.7 | 1.2 | 0 | >>>> | 65.9 | 980 | -63.62 | 17 | 92 | 1.65 | 0 |
| BPIW0315 | 24/03/2021 15:41 | 56.3 | 40.9 | 0.2 | 2.6 | >>>> | 56.2 | 980 | -42.51 | 12 | 23 | 1.38 | 1.84 |
| BPIW0316 | 24/03/2021 15:44 | 66.9 | 43.5 | 0.2 | >>>> | >>>> | 66.9 | 979 | -67.28 | 15 | 12 | 1.54 | 0 |
| BPIW0317 | 24/03/2021 15:48 | 57.9 | 39 | 4.2 | 0 | >>>> | 58.3 | 980 | -67.18 | 13 | 21 | 1.48 | 0 |
| BPIW0318 | 24/03/2021 15:51 | 45.8 | 37.7 | 1.6 | 14.9 | >>>> | 46 | 980 | -54.39 | 15 | 107 | 1.21 | 8.85 |
| BPIW0319 | 24/03/2021 15:54 | 67.7 | 38.7 | 0.4 | 0 | >>>> | 67.6 | 980 | -33.44 | 12 | 184 | 1.75 | 0 |
| BPIW0095 | 24/03/2021 15:57 | 70.3 | 43.4 | 0 | >>>> | >>>> | 70.2 | 980 | -67.47 | 25 | 213 | 1.62 | 0 |
| BPIW0320 | 24/03/2021 16:00 | 70.8 | 42.1 | 0 | >>>> | >>>> | 70.8 | 980 | -67.83 | 14 | 32 | 1.68 | 0 |
| BPIW0321 | 24/03/2021 16:06 | 71.2 | 35.8 | 0.9 | 0 | >>>> | 73.8 | 980 | -67.04 | 10 | 1 | 1.99 | 0 |
| BPIW0322 | 24/03/2021 16:08 | 69.6 | 41.1 | 0.3 | >>>> | >>>> | 70.1 | 980 | -66.62 | 11 | 2 | 1.69 | 0 |
| BPIW0325 | 24/03/2021 16:15 | 66.1 | 43.8 | 0.4 | >>>> | >>>> | 66.2 | 980 | -66.96 | 18 | 28 | 1.51 | 0 |
| BPIW0326 | 24/03/2021 16:17 | 64.3 | 42.5 | 0.2 | 0 | >>>> | 64.2 | 980 | -60.54 | 13 | 19 | 1.51 | 0 |
| BPIW0327 | 24/03/2021 16:20 | 55.9 | 41.8 | 0 | 2.3 | >>>> | 55.9 | 979 | -5.16 | 15 | 45 | 1.34 | 2.3 |
| BPIW0033 | 25/03/2021 09:06 | 57.1 | 37.6 | 2.4 | 2.9 | >>>> | 57.1 | 978 | 0.43 | 8 | 0 | 1.52 | 0 |
| BPIW059A | 25/03/2021 09:08 | 57.4 | 39.6 | 0 | 3 | >>>> | 57.4 | 977 | -20.43 | 10 | 40 | 1.45 | 3 |
| BPIW056A | 25/03/2021 09:10 | 42.6 | 35.3 | 0.1 | 22 | >>>> | 42.6 | 977 | -66.72 | 8 | 17 | 1.21 | 21.62 |
| BPIW0007 | 25/03/2021 09:12 | 40.7 | 30.7 | 6.1 | 22.5 | >>>> | 40.7 | 977 | -9.55 | 9 | 0 | 1.33 | 0 |
| BPIW0068 | 25/03/2021 09:15 | 43.2 | 37.7 | 0.7 | 18.4 | >>>> | 43.2 | 977 | -9.24 | 11 | 4 | 1.15 | 15.75 |
| BPIW0084 | 25/03/2021 09:18 | 31.3 | 32 | 0 | 36.7 | >>>> | 31.3 | 977 | -4.87 | 7 | 6 | 0.98 | 36.7 |
| BPIW0085 | 25/03/2021 09:20 | 32.2 | 31.5 | 3.3 | 33 | >>>> | 32.2 | 977 | -12.7 | 12 | 0 | 1.02 | 20.53 |
| BPIW0067 | 25/03/2021 09:22 | 15.8 | 22.2 | 5.7 | 56.3 | >>>> | 15.8 | 977 | -2.04 | 12 | 0 | 0.71 | 34.75 |
| BPIW0066 | 25/03/2021 09:25 | 45.6 | 39.8 | 0 | 14.6 | >>>> | 45.7 | 976 | -1.74 | 9 | 117 | 1.15 | 14.6 |
| BPIW0069 | 25/03/2021 09:27 | 49.6 | 39.6 | 0.3 | 10.5 | >>>> | 49.6 | 977 | -11.57 | 11 | 11 | 1.25 | 9.37 |
| BPIW0086 | 25/03/2021 09:29 | 6.5 | 5.2 | 20.1 | 68.2 | >>>> | 6.5 | 976 | 0.49 | 7 | 0 | 1.25 | 0 |
| BPIW0019 | 25/03/2021 09:31 | 36.2 | 28.2 | 2.4 | 33.2 | >>>> | 36.2 | 977 | -39.83 | 5 | 1 | 1.28 | 24.13 |
| BPIW0052 | 25/03/2021 09:33 | 28.8 | 26.6 | 4.8 | 39.8 | >>>> | 28.8 | 976 | -14.73 | 7 | 0 | 1.08 | 21.66 |
| BPIW0021 | 25/03/2021 09:36 | 70.9 | 40.2 | 0 | >>>> | >>>> | 70.8 | 976 | -86.42 | 10 | 52 | 1.76 | 0 |
| BPIW0039 | 25/03/2021 09:38 | 57.1 | 36.5 | 0.1 | 6.3 | >>>> | 57.1 | 977 | -71.44 | 14 | 8 | 1.56 | 5.92 |
| BPIW0039 | 25/03/2021 09:39 | 57 | 36.4 | 1 | 5.6 | >>>> |  | 977 | 0.37 | 10 | 6 | 1.57 | 1.82 |
| BPIW0037 | 25/03/2021 09:41 | 3.2 | 2.4 | 21.9 | 72.5 | 64 | 3.2 | 976 | -40.14 | 3 | 0 | 1.33 | 0 |
| BPIW037A | 25/03/2021 09:47 | 61 | 36.8 | 0.2 | 2 | >>>> | 60.8 | 976 | -76.84 | 8 | 19 | 1.66 | 1.24 |
| BPIW053A | 25/03/2021 09:50 | 9.7 | 6.3 | 18.7 | 65.3 | >>>> | 9.7 | 977 | -6.95 | 13 | 0 | 1.54 | 0 |
| BPIW0038 | 25/03/2021 11:21 | 47.2 | 34.8 | 0.1 | 17.9 | >>>> | 47.2 | 976 | -74.21 | 7 | 27 | 1.36 | 17.52 |
| BPIW0022 | 25/03/2021 11:23 | 69.4 | 38.7 | 0.1 | 0 | >>>> | 69.4 | 977 | -88.09 | 9 | 64 | 1.79 | 0 |
| BPIW040A | 25/03/2021 11:25 | 62.3 | 36.4 | 0.1 | 1.2 | >>>> | 62.2 | 976 | -23.59 | 14 | 34 | 1.71 | 0.82 |
| BPIW042B | 25/03/2021 11:27 | 65.5 | 39 | 0.1 | 0 | >>>> | 65.5 | 977 | -27.52 | 314 | 10 | 1.68 | 0 |
| BPIW0012 | 25/03/2021 11:29 | 66.5 | 42.6 | 0 | 0 | >>>> | 66.5 | 976 | -31.9 | 22 | 0 | 1.56 | 0 |
| BPIW041B | 25/03/2021 11:31 | 68 | 31.5 | 1.1 | 0 | >>>> | 67.8 | 976 | -60.54 | 10 | 17 | 2.16 | 0 |
| BPIW013A | 25/03/2021 11:33 | 69.6 | 34.7 | 0.3 | 0 | >>>> | 69.6 | 977 | -87.23 | 8 | 4 | 2.01 | 0 |
| BPIW0044 | 25/03/2021 11:36 | 49.4 | 31.1 | 4.1 | 15.4 | >>>> | 49.4 | 976 | -52.32 | 7 | 2 | 1.59 | 0 |
| BPIW020A | 25/03/2021 11:38 | 66 | 41.6 | 0.4 | 0 | >>>> | 66 | 976 | -58.98 | 10 | 2 | 1.59 | 0 |
| BPIW0045 | 25/03/2021 11:40 | 67.4 | 43.8 | 0.1 | >>>> | >>>> | 67.4 | 976 | -47.55 | 19 | 32 | 1.54 | 0 |
| BPIW045A | 25/03/2021 11:42 | 66.7 | 44.8 | 0 | >>>> | >>>> | 66.7 | 976 | 0.28 | 9 | 1 | 1.49 | 0 |
| BPIW046A | 25/03/2021 11:46 | 22.8 | 15.2 | 13 | 49 | >>>> | 65.6 | 976 | 0.25 | 15 | 21 | 1.5 | 0 |
| BPIW0011 | 25/03/2021 11:48 | 53.3 | 37.6 | 2.8 | 6.3 | >>>> | 58.8 | 975 | -54.4 | 16 | 17 | 1.42 | 0 |
| BPIW0017 | 25/03/2021 11:54 | 60.5 | 37.8 | 2.2 | 0 | >>>> | 63.3 | 976 | -87.48 | 13 | 4 | 1.6 | 0 |
| BPIW0016 | 25/03/2021 11:56 | 53.5 | 33.7 | 1.2 | 11.6 | >>>> | 53.5 | 977 | -88.73 | 8 | 2 | 1.59 | 7.06 |
| BPIW0018 | 25/03/2021 11:58 | 68.7 | 37 | 0.3 | 0 | >>>> | 68.8 | 977 | -14.37 | 7 | 15 | 1.86 | 0 |
| BPIW0104 | 25/03/2021 12:02 | 55.3 | 33.5 | 1.8 | 9.4 | >>>> | 55.1 | 977 | -5.34 | 6 | 4 | 1.65 | 2.6 |
| BPIW0014 | 25/03/2021 12:06 | 65.5 | 39.3 | 1.2 | 0 | >>>> | 65.5 | 977 | -89.11 | 10 | 17 | 1.67 | 0 |
| BPIW0106 | 25/03/2021 12:08 | 51.3 | 36.3 | 0 | 12.4 | >>>> | 51.3 | 976 | -84.04 | 7 | 6 | 1.41 | 12.4 |
| BPIW0047 | 25/03/2021 12:25 | 52.9 | 36.5 | 0.2 | 10.4 | >>>> | 53 | 976 | -85.13 | 7 | 1 | 1.45 | 9.64 |
| BPIW0043 | 25/03/2021 12:29 | 46.8 | 33.5 | 0 | 19.7 | >>>> | 46.9 | 976 | -15.6 | 6 | 3 | 1.4 | 19.7 |
| BPIW0209 | 25/03/2021 13:53 | 64.1 | 30.8 | 0 | 5.1 | >>>> | 64.2 | 975 | -80.26 | 8 | 0 | 2.08 | 5.1 |
| BPIW0208 | 25/03/2021 13:56 | 51.8 | 33.4 | 0 | 14.8 | >>>> | 51.8 | 976 | -81.39 | 8 | 1 | 1.55 | 14.8 |
| BPIW0216 | 25/03/2021 13:58 | 46.2 | 31.3 | 1.4 | 21.1 | >>>> | 46.2 | 976 | -55.62 | 6 | 0 | 1.48 | 15.81 |
| BPIW0207 | 25/03/2021 14:00 | 4.4 | 2.7 | 21.5 | 71.4 | 88 |  | 976 | 0.13 | 3 | 0 | 1.63 | 0 |
| BPIW0207 | 25/03/2021 14:01 | 12.3 | 7.6 | 18.4 | 61.7 | >>>> | 12 | 976 | 0.12 | 3 | 0 | 1.62 | 0 |
| BPIW0215 | 25/03/2021 14:03 | 9.9 | 5.4 | 19.2 | 65.5 | >>>> | 9.8 | 977 | -64.9 | 4 | 0 | 1.83 | 0 |
| BPIW0202 | 25/03/2021 14:05 | 36.7 | 24.6 | 0 | 38.7 | >>>> | 36.7 | 976 | -61.13 | 7 | 0 | 1.49 | 38.7 |
| BPIW0203 | 25/03/2021 14:07 | 45.9 | 26 | 0.1 | 28 | >>>> | 45.9 | 976 | -66.98 | 6 | 1 | 1.77 | 27.62 |

Inlet Into Gas Utilisation Compound

|  | CH4 (%) | CO2 (%) | O2 (%) | Generator flow (M3/H) | Inlet flow (M3/H) | Inlet Vacuum (mbar) |
| --- | --- | --- | --- | --- | --- | --- |
| min | -24.90 | -24.90 | -6.20 | -0.63 | -2.30 | -944.92 |
| max | 66.32 | 37.50 | 20.80 | 12266.67 | 7136.33 | 33.83 |
| average | 49.16 | 31.46 | 1.36 | 424.09 | 470.71 | -64.25 |
| Total flow (metres cubed) |  |  |  | 832903.00 | 924469.08 |  |

Appendix E

Dust

| Customer Sample Ref. |  |  | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- |
| Sample Date/Time |  |  | 15/03/2020 00:00:00 | 15/03/2020 00:00:00 | 15/03/2020 00:00:00 |
| Sample Matrix |  |  | Dust | Dust | Dust |
| Date In | FIELD | dd/mm/yy | 15/03/21 | 15/03/21 | 15/03/21 |
| Date Out | FIELD | dd/mm/yy | 11/02/21 | 11/02/21 | 11/02/21 |
| Number of Days Exposed | FIELD |  | 32 | 32 | 32 |
| Mass of Undissolved Solids | 74 | mg | 0.0813 | 0.237 | 0.0202 |
| Deposited Dust, Total, Calc. | 74 | mg/m2/day | 0.06 | 0.19 | 0.02 |
| Frisbee Diameter | FIELD | mm | 227 | 227 | 227 |

Appendix F

Meteorological Data

January

February

March