The BAT AELs and monitoring requirements in this document have been taken from the BATc document; and are all that could apply for the treatment type- Treatment of water-based liquid waste.

As BAT 3, 7 and 20 and question 7 of the Reg 61(1) notice were not responded to with the relevant information they have all been included within the permit, with improvement conditions to obtain the required information.

Once the improvement conditions have been completed, we may be able to agree that some of these do not need to be complied with.

BAT 7. BAT is to monitor emissions to water with at least the frequency given below, and in accordance with EN standards. If EN standards are not available, BAT is to use ISO, national or other international standards that ensure the provision of data of an equivalent scientific quality.

Treatment of water-based liquid waste

| Waste treatment process | Parameter | Limit (including unit) | Monitoring frequency (1)(2) | Monitoring standard or method | Monitoring associated with |
| --- | --- | --- | --- | --- | --- |
| Treatment of water-based liquid waste | Adsorbable organically bound halogens (AOX)(3)(4) |  | Once every day | EN ISO 9562 | BAT 20 |
| Treatment of water-based liquid waste | Benzene, toluene, ethylbenzene, xylene (BTEX)(3)(4) |  | Once every month | EN ISO 15680 | BAT 20 |
| Treatment of water-based liquid waste | Chemical oxygen demand (COD)(5)(6) |  | Once every day | No EN standard available | BAT 20 |
| Treatment of water-based liquid waste | Free cyanide (CN-)(3)(4) |  | Once every day | Various EN standards available (i.e. EN ISO 14403-1 and -2) | BAT 20 |
| Treatment of water-based liquid waste | Hydrocarbon oil index (HOI)(4) |  | Once every day | EN ISO 9377-2 | BAT 20 |
| Treatment of water-based liquid waste | Arsenic (As), Cadmium (Cd), Chromium (Cr), Copper (Cu), Nickel (Ni), Lead (Pb), Zinc (Zn)(3)(4) |  | Once every day | Various EN standards available (e.g. EN ISO 11885, EN ISO 17294-2, EN ISO 15586) | BAT 20 |
| Treatment of water-based liquid waste | Manganese (Mn)(3)(4) |  | Once every day | Various EN standards available (e.g. EN ISO 11885, EN ISO 17294-2, EN ISO 15586) | BAT 20 |
| Treatment of water-based liquid waste | Hexavalent chromium (Cr(VI))(3)(4) |  | Once every day | Various EN standards available (i.e. EN ISO 10304-3, EN ISO 23913) | BAT 20 |
| Treatment of water-based liquid waste | Mercury (Hg)(3)(4) |  | Once every day | Various EN standards available (i.e. EN ISO 17852, EN ISO 12846) | BAT 20 |
| All waste treatments | PFOA(3) |  | Once every six months | No EN standard available | BAT 20 |
| All waste treatments | PFOS(3) |  | Once every six months | No EN standard available | BAT 20 |
| Treatment of water-based liquid waste | Phenol index(6) |  | Once every day | EN ISO 14402 | BAT 20 |
| Treatment of water-based liquid waste | Total nitrogen (Total N)(6) |  | Once every day | EN 12260, EN ISO 11905-1 | BAT 20 |
| Treatment of water-based liquid waste | Total organic carbon (TOC)(5)(6) |  | Once every day | EN 1484 | BAT 20 |
| Treatment of water-based liquid waste | Total phosphorus (Total P)(6) |  | Once every day | Various EN standards available (i.e. EN ISO 15681-1 and -2, EN ISO 6878, EN ISO 11885) | BAT 20 |
| Treatment of water-based liquid waste | Total suspended solids (TSS)(6) |  | Once every day | EN 872 | BAT 20 |

(1) Monitoring frequencies may be reduced if the emission levels are proven to be sufficiently stable.

(2) In the case of batch discharge less frequent than the minimum monitoring frequency, monitoring is carried out once per batch.

(3) The monitoring only applies when the substance concerned is identified as relevant in the waste water inventory mentioned in BAT 3.

(4) In the case of an indirect discharge to a receiving water body, the monitoring frequency may be reduced if the downstream waste water treatment plant abates the pollutants concerned.

(5) Either TOC or COD is monitored. TOC is the preferred option, because its monitoring does not rely on the use of very toxic compounds.

(6) The monitoring applies only in the case of a direct discharge to a receiving water body.

BAT 20. In order to reduce emissions to water, BAT is to treat waste water using an appropriate combination of the techniques given below.

Table 6.2 BAT-associated emission levels (BAT-AELs) for indirect discharges to a receiving water body

Treatment of water-based liquid waste

| Waste treatment process | Parameter |  | Limit (including unit) (1)(2) | Monitoring frequency | Monitoring standard or method | Monitoring associated with |
| --- | --- | --- | --- | --- | --- | --- |
| Treatment of water-based liquid waste | Hydrocarbon oil index (HOI) |  | 10 mg/l |  |  | BAT 7 |
|  | Free cyanide (CN-) (3) |  | 0.1 mg/l |  |  | BAT 7 |
|  | Adsorbable organically bound halogens (AOX) (3) |  | 1 mg/l |  |  | BAT 7 |
|  | Metals and metalloids (3) | Arsenic (expressed as As) | 0.1 mg/l |  |  | BAT 7 |
|  |  | Cadmium (expressed as Cd) | 0.1 mg/l |  |  | BAT 7 |
|  |  | Chromium (expressed as Cr) | 0.3 mg/l |  |  | BAT 7 |
|  |  | Hexavalent chromium (expressed as Cr(VI)) | 0.1 mg/l |  |  | BAT 7 |
|  |  | Copper (expressed as Cu) | 0.5 mg/l |  |  | BAT 7 |
|  |  | Lead (expressed as Pb) | 0.3 mg/l |  |  | BAT 7 |
|  |  | Nickel (expressed as Ni) | 1 mg/l |  |  | BAT 7 |
|  |  | Mercury (expressed as Hg) | 10 μg/l |  |  | BAT 7 |
|  |  | Zinc (expressed as Zn) | 2 mg/l |  |  | BAT 7 |

(1) The averaging periods are defined in the General considerations.

(2) The BAT-AELs may not apply if the downstream waste water treatment plant abates the pollutants concerned, provided this does not lead to a higher level of pollution in the environment.

(3) The BAT-AELs only apply when the substance concerned is identified as relevant in the waste water inventory mentioned in BAT 3.

(4) The upper end of the range is 0,3 mg/l for mechanical treatment in shredders of metal waste.

(5) The upper end of the range is 2 mg/l for mechanical treatment in shredders of metal waste.