Natural Resources Wales permitting decisions

# Normal Variation and Consolidation – CWM Environmental Limited

We have decided to issue the variation for Nantycaws HCI Waste Transfer & Treatment Facility in Llanddarog Road, Nantycaws, Carmarthen, SA32 8BG operated by CWM Environmental Limited.

The permit number is EPR/ EP3698FL.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- Explains how the application has been determined

- Provides a record of the decision-making process

- Shows how all relevant factors have been taken into account

- Justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise, we have accepted the applicant’s proposals.

Structure of this document

- Assessment of Nantycaws HCI Waste Transfer & Treatment Facility against the published BAT conclusions for Waste Treatment.

- Annex 1 – Decision Checklist regarding relevant BAT Conclusions

Assessment of Nantycaws HCI Waste Transfer & Treatment Facility against the published BAT conclusions for Waste Treatment

1. Our decision

We have issued a variation, which will allow CWM Environmental Limited to operate the installation, subject to the conditions in the varied permit.

The variation does three things:

- it consolidates the original permit to reflect changes made through earlier variations;

- it brings the permit into line with our modern regulatory template; and

- it varies the permit where appropriate to reflect the outcome of our statutory review and incorporate Best Available Techniques (BAT) and Associated Emission Limit Values (BAT-AELs).

We consider that, in reaching this decision, we have taken into account all relevant considerations and legal requirements and that the permit will continue to ensure that a high level of protection is provided for the environment and human health.

The original waste management licence EAWML 34207, issued on the 2nd September 2003, which was varied to an EPR permit in 2012 ((EP3698FL) ensured that the installation, employed Best Available Techniques (BAT) and ensured a high level of protection for human health and the environment. We have altered the permit as a result of the statutory review, and we are confident that the new requirements will deliver a superior level of protection to that which was previously achieved. Where a site is not currently compliant with BAT, Improvement Conditions have been included to bring the site up standard by 17th August 2022.

2. The legal framework

The consolidated variation notice will be issued under Regulation 20 of the EPR. The environmental permitting regime is a legal vehicle which delivers most of the relevant legal requirements for activities falling within its scope. In particular, the regulated facility is:

- an installation as described by the IED;

- subject to aspects of other relevant legislation which also have to be addressed.

We consider that, in issuing the consolidated variation notice, it will ensure that the operation of the installation complies with all relevant legal requirements and that a high level of protection will be delivered for the environment and human health.

We explain how we have addressed specific statutory requirements more fully in the rest of this document.

3. How we reached our decision

Requesting information to demonstrate compliance with BAT Conclusion techniques

We issued a Notice under Regulation 61(1) of the Environmental Permitting (England and Wales) Regulations 2016 on 5th April 2019 requiring the operator to provide information to demonstrate how the operation of their installation currently meets, or will subsequently meet, the revised standards described in the relevant BAT Conclusions document.

- Describes the techniques that will be implemented before 17th August 2022, which will then ensure that operations meet the revised standard, or

- Justifies why standards will not be met by 17th August 2022, and confirmation of the date when the operation of those processes will cease within the installation or an explanation of why the revised BAT standard is not applicable to those processes, or

- Justifies why an alternative technique will achieve the same level of environmental protection equivalent to the revised standard described in the BAT Conclusions.

- Where their permitted activity involves the use, production or release of a hazardous substance, as defined in Article 3(18) of the Industrial Emissions Directive, CWM Environmental Limited were required to carry out a risk assessment considering the possibility of soil and groundwater contamination at the permitted installation with such substances. Where risk of such contamination is established prepare a baseline report containing information necessary to determine the state of soil and groundwater contamination so as to make a quantified comparison with the state upon definite cessation of the activity. CWM Environmental Limited have reviewed their Environmental Risk Assessment and concluded that there are no risks of soil or groundwater contamination.

- Where their permitted activity involves the use, production, storage or release of a priority hazardous substances and any other relevant substances, as defined by the Water Framework Directive, the CWM Environmental Limited were required to carry out a risk screening assessment considering the presence of priority hazardous substances at the permitted installation. Where a risk of these substances is established the operator is to sample the effluent and screen for the priority hazardous substances. If these substances are found to be present in the effluent stream, then assessment using the H1 tool and potential detailed modelling will be required to demonstrate that the effluent discharge will not have a significant impact to the receiving water. There are no waste water discharges from the site (direct or indirect).

Where the operator proposed that they were not intending to meet a BAT standard, that also included a BAT Associated Emission Level (BAT-AEL) described in the BAT Conclusions Document, the Regulation 61 Notice requested that the operator make a formal request for derogation from compliance with that AEL (as provisioned by Article 15(4) of IED). In this circumstance, the Notice identified that any such request for derogation must be supported and justified by sufficient technical and commercial information that would enable us to determine acceptability of the derogation request.

The Regulation 61 Notice response from the operator was received on the 30th September 2019 and further information on 20th February 2020.

We considered that the response contained sufficient information for us to commence determination of the permit review. The operator made no claim for commercial confidentiality. We have not received any information in relation to the Regulation 61 Notice response that appears to be confidential in relation to any part.

# 4. Key issues/Regulation 61 response

# BAT Conclusions for the Waste Treatment were published as a Commission Implementing Decision ((EU 2018/1447) in the Official Journal of the EU on 17th  August 2018.  There are 53 BAT Conclusions. This annex provides a record of decisions made in relation to each relevant BAT Conclusion applicable to the installation. This should be read in conjunction with the permit/variation notice issued.

A basic response was received from CWM Environmental Limited. Where the operator has concluded that they have achieved BAT, and we are in agreement, no further information/justification has been sought by Natural Resources Wales. Where insufficient information has been provided, improvement conditions have been set.

5. Changes we have made

Improvement Conditions

Based on the information provided in the Regulation 61 response, we consider that we need to set improvement conditions. These conditions are set out below. We are using these conditions to require the operator to provide Natural Resources Wales with details that need to be established or confirmed during operations. The improvement conditions ensure compliance by 2022.

| Reference | Requirement | Date |
| --- | --- | --- |
| IC2 | The operator shall submit an updated written Fire prevention and mitigation plan to Natural Resources Wales. The Fire prevention and mitigation plan must be produced in line with the standards set out in Fire prevention and mitigation plan guidance – Waste. | 3 months after permit issue or otherwise agreed in writing with Natural Resources Wales |
| IC3 | BAT 1 – EMS The operator must submit an updated BAT conclusions document to Natural Resources Wales including the following: - The operator shall submit evidence of ISO 14001:2015 certification to Natural Resources Wales. - The operator is required to update the Environmental Management System to include the following procedures and to submit them to Natural Resources Wales: - Management commitment - Planning and implementing procedures and targets in conjunction with financial planning and investment - Communication - Employee involvement - Independent (where practicable) internal or external EMS auditing - Senior management review of the EMS - Following the development of cleaner technologies - Regular sectoral benchmarking - Waste stream management in line with BAT 2 - Inventory of waste gas stream in line with BAT 3 - Residues management plan In accordance with BAT 1 of the Waste Treatment BAT Conclusions | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC4 | BAT 2– Waste acceptance, tracking and management The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) in order to evidence compliance with BAT 2 (of the Waste Treatment BAT Conclusions): - Waste characterisation and pre-acceptance procedures (BAT 2 a)) - Waste acceptance procedures (BAT 2 b)) - Implementation of a waste tracking system and inventory (BAT 2 c) - Output quality management system (BAT 2 d)) Sorting of incoming waste (BAT 2 e)) | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC5 | BAT 3 – Inventory of waste gas streams The operator shall submit to Natural Resources Wales information in order to evidence compliance with BAT 3 (of the Waste Treatment BAT Conclusions): - Performance of waste gas treatment i.e. biofilter (BAT 3 i)) - Information on the characteristics of the waste gas stream (BAT 3 (iii)) | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC6 | BAT 4 – Storage of waste The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on reducing environmental risk associated with waste storage to evidence compliance with BAT 4 (of the Waste Treatment BAT Conclusions): a) Adequate storage capacity in particular how the quantity of waste stored is monitored against capacity (BAT 4 b)) | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC7 | BAT 5 – Handling and transfer of waste The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on reducing environmental risk associated with handling and transfer of waste to evidence compliance with BAT 5 (of the Waste Treatment BAT Conclusions): - Procedure to ensure handling and transfer of wastes are duly documented, validated prior to execution and verified after execution Procedure to ensure measures are taken to prevent, detect and mitigate spills (existing spill procedure does not provide information on how to prevent and detect spills) | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC8 | BAT 33 – Selection and suitability of waste input The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on how odour emissions are reduced and overall environmental performance is improved, by selecting the waste input to ensure its suitability for biological treatment in line with BAT 33 (of the Waste Treatment BAT Conclusions). | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC9 | BAT 35 – Emissions to water and water usage The operator must provide information to Natural Resources Wales to show compliance with BAT 35 c) detailing how the operator optimises the moisture content of the waste in order to minimises leachate generation. | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC10 | BAT 36 – Emissions to air monitoring and control of key waste and process parameters The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on how monitoring and/or controlling the following key waste and process parameters is implemented on site to reduce emissions to air and improve overall environmental performance in line with BAT 36 (of the Waste Treatment BAT Conclusions): - waste input characteristics e.g. C to N ratio, particle size windrow porosity, height and width | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC11 | BAT 37 – Diffuse emissions to air The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on how the following technique is implemented to reduce diffuse emissions to air of dust, odour and bioaerosols from open-air treatment steps: adaptation of operations to meteorological conditions (BAT 37 b) (of the Waste Treatment BAT Conclusions)) in particular with reference to the direction of outdoor windrows in relation to prevailing wind direction | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC12 | The Operator shall submit a written report to Natural Resources Wales for approval detailing how the site will implement the latest version of ‘Technical Guidance Note M9 Environmental monitoring of bioaerosols at regulated facilities dated July 2018 Version 2’. This assessment should specifically consider channelled emissions to air from the biofilter and the potential for bioaerosol releases. If your biofilter (open and closed biofilters) is within 250 metres of a sensitive receptor you are required to undertake and submit a site specific bioaerosol risk assessment and any monitoring must be in line with ‘M9: environmental monitoring of bioaerosols at regulated facilities’. | 17 February 2022 or otherwise agreed in writing with Natural Resources Wales |

As per NRW’s the Fire Prevention and Mitigation Guidance, if you have an environmental permit that authorises you to store combustible waste, or you have an existing permit with a permit condition requiring you to have a Fire Prevention Mitigation Plan (FPMP), you must adhere to this guidance.

Our guidance does not apply to hazardous waste storage therefore this bit of guidance cannot be used. A hazardous waste sites EMS should have a section for waste storage and fire prevention techniques and reference the relevant guidance for this. This could be the Health and Safety Executive guidance. The operator should be stating how it will store waste and prevent fires.

Emissions to Water

As part of our delivery of the Water Framework Directive requirements, we need to identify and assess the impact for all discharges to surface waters and/or sewer from the site for priority hazardous substances and any other relevant substances. The emissions monitoring for these substances should be carried out using the methods and standards described in the M18 guidance on “Monitoring of discharges to water and sewer”.

With reference to the risk assessment guidance on the gov.uk website entitled “Surface water pollution risk assessment for your environmental permit“ (accessible via: https://www.gov.uk/guidance/surface-water-pollution-risk-assessment-for-your-environmental-permit) the company has not carried out the following assessments:

•	Screening tests for priority hazardous pollutants and any other relevant priority hazardous substances.

•	For any substance which is not screened out by the screening tests further modelling, as described in the risk assessment guidance “Surface water pollution risk assessment for your environmental permit”.

The site does not have any direct discharges of process water/waste water to surface waters nor indirect discharges to sewer. Any process water/waste water is collected and taken off-site for treatment elsewhere. There are discharges of clean surface water run-off to site which are already noted on the permit and which have therefore not been assessed further as part of this variation.

- The specific BAT Conclusions on water emissions were also reviewed based on the Waste Treatment BREF and no changes are made to the ELV’s for emissions to water taking into account BAT Conclusion 20 from the BREF.

There are no emissions to water associated with the permitted activity.

Emissions to Air

There were changes to the ELV’s for emissions to air taking into account BAT conclusions from the Waste Treatment BREF.

The tables below outline the parameters and limits set to implement the BAT-AELs set out in:

Effective until 16th August 2022

| Release point | Parameter | Limit/ BAT AEL | Effective to |
| --- | --- | --- | --- |
| Biofilter | None | No limit | 16th August 2022 |

Effective from 17th August 2022

| Release point | Parameter | Limit/ BAT AEL | Effective from |
| --- | --- | --- | --- |
| Biofilter | H2S | No limit | 17th August 2022 |
|  | NH3 | 20 mg/Nm3 | 17th August 2022 |

Where BAT associated emission levels are identified (BAT-AELs), limits may be prescribed at the top end of the range unless the proximity of sensitive receptors requires a tighter limit or if tighter limits are previously on the permit, this ensures no backsliding of emission limits.

Emissions to Air – Article 15(4) Derogations

No derogations

# 6. Conclusion

We consider that the installation already employed what used to be BAT. The revised BREF and its BAT-AELs provide the opportunity to consider further environmental improvements.

Coupled with the consolidation and modernisation of the permit, we believe this variation provides a sound basis for ongoing regulation of the installation. The Sector Review provides the opportunity to consolidate and modernise the permit.

We believe this variation provides a sound basis for ongoing regulation of the installation and we are satisfied that the operator is currently achieving or will be achieving all relevant BAT by 17th August 2022.

We believe that we have ensured compliance with all relevant legal requirements in carrying out this review and making our determination on the variation.

Annex 1: Decision checklist regarding relevant BAT Conclusions

BAT Conclusions for the Waste Treatment Bref were published as a Commission Implementing Decision ((EU 2018/1147) in the Official Journal of the EU on 17th August 2018.  There are 53 BAT Conclusions. This checklist provides a record of decisions made in relation to each relevant BAT Conclusion applicable to the installation.  This annex should be read in conjunction with the consolidated variation notice.

All BAT Conclusions arising are listed by number in order below;

| ### BATc number |  | ### Summary of BAT Conclusion requirement |  | Status/comment One of the following: Not Applicable, Currently Compliant, Compliant in the future (within 4 years of publication of BAT conclusions), Not Compliant |
| --- | --- | --- | --- | --- |
| OVERALL ENVIRONMENTAL PERFORMANCE |  |  |  |  |
| 1 | Environment Management System (EMS) – ALL of the following: |  |  |  |
|  | I. | Management commitment |  | Compliant in future The operator has provided insufficient information to assess this requirement. ISO14001 requires a section on leadership and commitment. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). An improvement condition has been set. |
|  | II. | Environmental policy development including CI of performance |  | Currently compliant ISO14001 requires an environmental policy. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). The operators environmental policy is contained in their 2018 EMS. |
|  | III. | Planning and implementing procedures & targets in conjunction with financial planning & investment |  | Compliant in future The operator has provided insufficient information to assess this requirement. ISO14001 requires a section on planning actions including consideration of its financial requirements. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). An improvement condition has been set. |
|  | IV. | Implementation of procedures |  |  |
|  |  | (a) Structure & responsibility |  | Currently compliant Section 14 of the EMS 2018 and section 15.2 of the Odour Management Plan (OMP) 2018 cover roles and responsibilities. ISO14001 requires a section on 'Organisational roles, responsibilities and authorities'. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  |  | (b) Recruitment, training, awareness & competence |  | Currently compliant The EMS 2018 has a section on training and competence. ISO14001 requires a section on 'Resources, competence and awareness'. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  |  | (c) Communication |  | Compliant in future The operator has provided insufficient information to assess this requirement. ISO14001 requires a section on ‘communication'. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). No specific section in EMS 2018 on communication. An improvement condition has been set. |
|  |  | (d) Employee involvement |  | Compliant in future The operator has provided insufficient information to assess this requirement. The Environmental Policy notes a commitment to employee involvement through training and environmental awareness but there is no further detail / specific section in the EMS 2018 on employee involvement. An improvement condition has been set. |
|  |  | (e) Documentation |  | Currently compliant Section 16 of the EMS 2018 covers this requirement. ISO14001 requires a section on ‘control of documented information'. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  |  | (f) Effective process control |  | Currently compliant There are various controls and procedures for operations noted in the EMS 2018 to cover this requirement. ISO14001 requires a section on ‘operational planning and control’. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  |  | (g) Maintenance programmes |  | Currently compliant Section 10 in the EMS 2018 covers this requirement. |
|  |  | (h) Emergency preparedness & response |  | Currently compliant Emergency procedures and contacts in Accident Management Plan (AMP), Fire Prevention and Management Plan (FPMP), and Fire Emergency Plan. ISO14001 requires a section on ‘emergency preparedness’. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  |  | (i) Safeguarding compliance with environmental legislation |  | Currently compliant Sections 16 and 17 in EMS 2018 detail legal register, maintaining compliance and review of legal register. ISO14001 requires a section on ‘evaluation of compliance’. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  | V. | Checking performance and taking corrective action |  |  |
|  |  | (a) Monitoring & measurement |  | Currently compliant Appendix 8 of EMS 2018 covers procedures for monitoring. ISO14001 requires a section on ‘monitoring and measurement’. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  |  | (b) Corrective and preventive action |  | Currently compliant Section 11.4 of the EMS 2018 covers non-conformances, investigation of them and taking steps to ensure that there is no re-occurrence. ISO14001 requires a section on ‘non conformity and corrective action’. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  |  | (c) Maintenance of records |  | Currently compliant Section 16 of the EMS 2018 covers maintenance of records. ISO14001 requires a section on ‘creating and updating' and 'control of documented information'. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  |  | (d) Independent (where practicable) internal or external EMS auditing |  | Compliant in future The operator has provided insufficient information to assess this requirement. ISO14001 requires a section on 'internal audit' and external audit will be required to maintain ISO 14001 certification. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). |
|  | VI. | Senior management review of EMS |  | Compliant in future Section 17 of EMS relates to review of the EMS but does not specifically mention senior management involvement (or who is responsible for the review). The odour management plan (OMP) section 15.2 notes that that plan will be reviewed by senior management. ISO14001 requires a section on ‘management review'. A copy of the up to date ISO14001 certification is required to ensure the operator is still operating under a certified Environmental Management System (EMS). An improvement conditions has been set. |
|  | VII. | Following development of cleaner technologies |  | Compliant in future The operator has provided insufficient information to assess this requirement. The ‘energy and resources management system’ does not cover this requirement as indicated. An improvement condition has been set. |
|  | VIII. | Whole life cycle considerations when designing a new plant i.e. impacts from eventual decommissioning and throughout its operating life |  | Currently compliant Section 12 of EMS 2018 notes there is a site closure plan in place. |
|  | IX. | Regular sectoral bench marking |  | Compliant in future The operator has provided insufficient information to assess this requirement. The EMS 2018 does not note annual reviews are done as indicated by the operator. An improvement condition has been set. |
|  | X. | Waste stream management (BAT 2) |  | Compliant in future The operator has provided insufficient information to assess this requirement. Further information is provided in BAT 2 section of this table. |
|  | XI. | Inventory of waste water & waste gas streams (BAT 3) |  | Compliant in future The operator has provided insufficient information to assess this requirement. Further information is provided in BAT 3 section of this table. |
|  | XII. | Residues Management Plan – S6.5 |  | Compliant in future The operator has provided insufficient information to assess this requirement. There is no Residues Management Plan included in the EMS 2018. An improvement condition has been set. |
|  | XIII. | Accident Management Plan – S6.5 |  | Currently compliant An accident management plan is in place. |
|  | XIV. | Odour Management Plan (BAT 12) |  | Currently compliant An odour management plan is in place. |
|  | XV. | Noise & Vibration Management Plan (BAT 17) |  | Currently compliant A noise risk assessment and management plan is in place although this is a very small section (part 7) of the environmental risk assessment. The noise risk assessment and management plan indicates the risk from noise is low. |
| 2 | Improving overall environmental performance – ALL of the following: |  |  |  |
|  | a. |  | Set up and implement waste characterisation & pre-acceptance procedures | Compliant in the future The operator has provided insufficient information to assess this requirement. Pre-acceptance procedures are not contained in the EMS or OMP as indicated and the PAS100 and PAS111 Quality Management Systems (QMS) for the site have not been provided nor have the Feedstock Acceptance Criteria, Feedstock Supply Agreements, Feedstock Risk Assessment or New Feedstock Risk Assessment procedures. An improvement condition has been set. |
|  | b. |  | Set up and implement waste acceptance procedures | Compliant in the future The operator has referred to a ‘Compost Quality Management, PAS100’ procedure which has not been provided for review. The EMS 2018 contains an acceptance criteria scoring sheet and 'waste acceptance and rejection' procedure in Appendix 8 and the OMP indicates that where required feedstocks may be subjected to scientific evaluation. 'V004 Supporting Info Non-Technical summary & Flow Diagram v2' notes storage capacities for different permitted activities (not split between waste streams). However the procedures reviewed are not adequate to meet BAT. An improvement condition has been set. |
|  | c. |  | Set up and implement a waste tracking system & inventory | Compliant in the future The operator has provided insufficient information to assess this requirement. The EMS 2018 references the use of weighbridge tickets and log but no detail is provided. The ‘Compost Quality Management, PAS100’ document has not been provided. An improvement condition has been set. |
|  | d. |  | Set up and implement an output quality management system | Compliant in the future The operator has provided insufficient information to assess this requirement. The ‘Compost Quality Management, PAS100’ document has not been provided. The list of QMS documents listed in the EMS 2018 note quality tests on treated material but no detail is provided on the output quality management system. An improvement condition has been set. |
|  | e. |  | Ensure waste segregation | Currently Compliant The ‘Compost Quality Management, PAS100’ document has not been provided. The EMS 2018 section 2.3.2 notes food waste is source segregated. The EMS 2018 Appendix 8 'waste acceptance and rejection' procedure notes any incompatible wastes temporarily stored in the quarantine area will be kept physically separate and the 'waste control' procedure notes all wastes received will be kept separate from other wastes until they have been confirmed for acceptance and all storage containers will be labelled. The AMP notes separate storage of combustible wastes. |
|  | f. |  | Ensure waste compatibility prior to mixing or blending | Not applicable This is not relevant as the permitted activity is compositing and feedstock is green / food waste. |
|  | g. |  | Sort solid incoming waste – S6.4 | Compliant in the future The ‘Compost Quality Management, PAS100’ document has not been provided. The EMS 2018 or OMP does not contain procedures for sorting wastes other than a note within 'acceptance criteria' to say plastics / contaminants shall be removed. An improvement condition has been set. |
| 3 | Establish and maintain a waste water and waste gas inventory as part of the EMS - ALL of the following: |  |  |  |
|  | Information on characteristics of waste and waste treatment processes |  |  |  |
|  | (i)(a) |  | simplified process flow sheets showing emission sources | Currently compliant There is a waste flow diagram in 'V004 Supporting Info Non-Technical summary & Flow Diagram v2' but this does not note emissions points. However the EMS 2018 shows the IVC facility and yard area noting the location of biofilter (i.e. emission point). |
|  | (i)(b) |  | Process-integrated and waste water/waste gas treatment descriptions including performance | Compliant in the future The 'V004 Supporting Info Non-Technical summary & Flow Diagram v2' notes use of ventilation and filtration system from IVC building, and open top biofilter (7.8 other emissions section). This document also notes (7.7 exports section) how waste water is collected and disposed of. There is no information on treatment (i.e. biofilter) performance though. An improvement condition has been set. |
|  | Information on characteristics of waste water streams |  |  |  |
|  | (ii)(a) |  | Mean and variability of: |  |
|  |  |  | Flow | Not applicable There are no discharges of waste water. The effluent from the IVC building is noted in 'V004 Supporting Info Non-Technical summary & Flow Diagram v2' section 7.7 as being collected in a sealed system and stored in a steel tank within bunded containment prior to emptying where it's taken to an off-site treatment facility. Surface water from the north west of the IVC yard and from the composting pad is directed to the composting storage lagoon. The site Drainage Plan shows this. The lagoon is noted in the OMP 2018 as being fully sealed and lined and CAR_NRW0033447 notes liquid effluent from the lagoon is deployed to land under a mobile plant permit. |
|  |  |  | pH | As above |
|  |  |  | Temperature | As above |
|  |  |  | Conductivity | As above |
|  | (ii)(b) |  | Mean concentration, load and variability of: |  |
|  |  |  | Total suspended solids | As above |
|  |  |  | COD/TOC | As above |
|  |  |  | Nitrogen species | As above |
|  |  |  | Phosphorous | As above |
|  |  |  | Metals | As above |
|  |  |  | Priority substances/micropollutants | As above |
|  |  |  | Any other relevant compounds | As above |
|  | (ii)(c) |  | Bioeliminability data (see BAT 52): |  |
|  |  |  | BOD | As above |
|  |  |  | BOD to COD ratio | As above |
|  |  |  | Zahn-Wellens test | As above |
|  |  |  | Biological inhibition potential | As above |
|  | Information on characteristics of waste gas streams |  |  |  |
|  | (iii)(a) |  | Mean and variability of: |  |
|  |  |  | Flow | Compliant in future The site has an open top biofilter filtering air from the IVC building which is considered a channelled emission to air. Whilst the operator has indicated the IVC is not being used at present, this activity has not been surrendered from the environmental permit. This waste gas stream will need to be characterised and monitored in the future. An improvement condition has been set. |
|  |  |  | temperature | Compliant in future As above |
|  | (iii)(b) |  | Mean concentration, load and variability of relevant substances: |  |
|  |  |  | Organic compounds | Compliant in future As above |
|  |  |  | POPs e.g. PCBs | Compliant in future As above |
|  |  |  | Any other relevant compounds | Compliant in future As above |
|  | (iii)(c) |  | Flammability | Compliant in future As above |
|  |  |  | Lower and Higher Explosive Limits | Compliant in future As above |
|  |  |  | Reactivity | Compliant in future As above |
|  | (iii)(d) |  | Presence of other substances that may affect the gas treatment system or plant safety: |  |
|  |  |  | O2 | Compliant in future As above |
|  |  |  | N2 | Compliant in future As above |
|  |  |  | Water vapour | Compliant in future As above |
|  |  |  | Dust | Compliant in future As above |
| 4 | Reducing environmental risk associated with waste storage – ALL of the following: |  |  |  |
|  | a. |  | Optimised storage location | Not applicable There are nearby residential dwellings (OMP 2018 indicates 150m away) but primarily the site is located within agricultural surroundings. BAT indicates this requirement is generally applicable to new sites though which this site is not. |
|  | b. |  | Adequate storage capacity | Compliant in future Waste storage capacity is noted in 'V004 Supporting Info Non-Technical summary & Flow Diagram v2' for different permitted activities (but not split between waste streams). Maximum residence times of waste are established (EMS 2018 Appendix 8 procedures). It is not clear if the quantity of waste stored is regularly monitored against capacities. An improvement condition has been set. |
|  | c. |  | Safe storage operation | Currently compliant EMS 2018 Appendix 8 'waste acceptance and rejection' procedure notes any incompatible wastes temporarily stored in the quarantine area will be kept physically separate. EMS 2018 Appendix 8 'waste control' procedure notes all wastes received will be kept separate from other wastes until they have been confirmed for acceptance and all storage containers will be labelled. AMP 2018 notes separate storage of incompatible materials, combustible wastes and ignition sources. |
|  | d. |  | Separate area for storage & handling of packaged hazardous waste | Not applicable Hazardous waste is not accepted or stored as part of the S5.4 A1(b)(i) composting activities; only as part of the HCI waste transfer and treatment site which is a separate (waste operation) activity in a separate area of the site. |
| 5 | Set up and implement procedures to reduce the environmental risk associated with handling and transfer of waste - include following elements: |  |  |  |
|  | Carried out by competent staff |  |  | Currently compliant EMS section 14 covers staff training and competence and NRW have current WAMITAB certificates (dated May 2018 so continuing competency is due soon). |
|  | Duly documented, validated and verified |  |  | Compliant in future The operator has provided insufficient information to assess this requirement. There are no specific procedures in the AMP 2018 for duly documenting the handling and transfer of waste as indicated by the operator. An improvement condition has been set. |
|  | Spill prevention, detection and mitigation measures |  |  | Compliant in future A very basic spill procedure is in place and the AMP 2018 notes ‘high risk spillage points’ but very little on prevention and detection. An improvement condition has been set. |
|  | Take precautions when mixing or blending wastes |  |  | Currently compliant The 'V004 Supporting Info Non-Technical summary & Flow Diagram v2' states “green waste from other sources is to be brought to the IVC facility reception area and this is mixed with domestic food waste and prepared for composting within the Stage 1 and Stage 2 vessels". As it is indicated that this is undertaken within the IVC vessels it is considered adequate to meet BAT. |
|  | Procedures are risk-based and consider likelihood of accidents, incidents and their environmental impact |  |  |  |
| MONITORING |  |  |  |  |
| 6 | Relevant emissions to water: monitor key process parameters at key locations |  |  |  |
|  | Key process parameters |  |  |  |
|  | Waste water flow |  |  | Not applicable No waste water is discharged to water course or sewer. |
|  | pH |  |  | As above |
|  | Temperature |  |  | As above |
|  | Conductivity |  |  | As above |
|  | BOD |  |  | As above |
|  | Other process parameters |  |  | As above |
|  | Key monitoring locations |  |  |  |
|  | Pre-treatment inlet and/or outlet |  |  | Not applicable |
|  | Final treatment inlet |  |  | Not applicable |
|  | Discharge point (to the environment) |  |  | Not applicable |
|  | Other location |  |  | Not applicable |
| 7 | Monitoring emissions to water (refer to table) |  |  | Not applicable No waste water is discharged to water course or sewer. |
|  | Monitoring parameters depend on waste treatment process(es) involved |  |  |  |
| 8 | Monitoring emissions to air (refer to table) |  |  | Compliant in future The operator is required to start monitoring H2S and NH3 (although there are properties approximately 200m away the existing OMP indicates there is a low risk of odour nuisance to nearby receptors and the operator has indicated there is no odour nuisance) in line with BAT 8 from the IVC building biofilter. The permit monitoring requirements have been updated. |
|  | Monitoring parameters depend on waste treatment process(es) involved |  |  |  |
| 9 | Monitoring diffuse emissions of organic compounds to air from processes involving solvents. Use one or a combination of the following: |  |  |  |
|  | a |  | Measurement – S6.2 descriptions | Not applicable No processes involving solvents undertaken. |
|  | b |  | Emissions factor calculation | As above |
|  | c |  | Mass balance calculation | As above |
| 10 | Periodically monitor odour emissions where nuisance is expected and/or has been substantiated (monitoring frequency is outlined in BAT 12) |  |  |  |
|  | Use EN standards e.g. 13725 or 16841 |  |  | Not applicable The applicability of BAT 10 is restricted to cases where an odour nuisance at sensitive receptors is expected and/or has been substantiated. The operator has indicated there is no odour nuisance and therefore monitoring in line with H4 Guidance is adequate. |
|  | Use equivalent methods e.g. ISO / national / international monitoring standards |  |  | Currently compliant The operator has indicated that there is no odour nuisance. Daily sniff testing is undertaken as per the OMP 2018 which is sufficient if no odour issues are expected/substantiated. |
| 11 | Annual monitoring for: |  |  |  |
|  | - Water, energy and raw materials |  |  | Compliant in future The permit has been updated to include annual monitoring. |
|  | - Generation of residues and waste water |  |  | Compliant in future The permit has been updated to include annual monitoring. |
| EMISSIONS TO AIR |  |  |  |  |
| 12 | Set up, implement and review an Odour Management Plan (as part of the site EMS) where nuisance is expected and/or has been substantiated. Include ALL of the following: |  |  |  |
|  | Protocol containing actions and timelines |  |  | Currently compliant The odour monitoring, management and control matrix within the OMP 2018 lists a number of actions / checks. Daily sniff tests are also included in the OMP 2018 together with a schedule of biofilter checks and frequency provided. |
|  | Protocol for conducting odour monitoring (BAT 10) |  |  | Currently compliant Sniff testing is conducted which is sufficient where there is no odour problem/nuisance. If an odour problem arises the OMP 2018 will need to be updated to include monitoring in line with EN standards e.g. 13725 or 16841. |
|  | Protocol for response to odour incidents/complaints |  |  | Currently compliant The OMP 2018 contains a protocol for responding to odour complaints. |
|  | Odour prevention and reduction programme |  |  | Currently compliant Odour monitoring, management and control matrix lists a number of actions / checks / controls to prevent/reduce odours . |
| 13 | Techniques to prevent, or where not practicable reduce odour emissions. Use one or a combination of the following: |  |  |  |
|  | a. |  | Minimising residence times (open systems only) | Currently compliant OMP 2018 notes that solid waste is contained in the reception building benefitting from the extraction system. If required, waste is given priority for loading to the processing system. Any quarantined waste is enclosed/covered and stored only for a maximum of 24 hrs. The EMS 2018 notes in 'control and monitoring of odorous emissions' section that malodorous wastes will be dealt with or removed from site within 24 hours of its discovery unless otherwise agreed in writing with the Environment Agency. |
|  | b. |  | Use chemical treatment (N/A if desired output is hampered) | Not applicable No chemical treatments. |
|  | c. |  | Optimising aerobic treatment – see examples. Refer to BAT 36 for wastes other than water-based liquid waste. | Currently compliant The temperature and moisture of windrows and IVC is monitored in line with permit requirements to ensure treatment is optimised. Also EMS 2018 section 2.3.2. notes a 50:50 ratio of green waste and food waste is used to balance structure and porosity, and high energy and high temperature; and also that aerobic activity is closely controlled by introducing air into the vessel ventilation system / fans recirculating air from the top of the vessel back in underneath the material promoting uniform temperatures. Seasonal storage of green waste is also noted to help maintain bulk and porosity when green waste stock is limited which maintains carbon ratio in mix. OMP 2018 notes the composting pad system utilises mechanical windrow turning for maximised aeration effect. |
| 14 | Techniques to prevent, or where not practicable reduce diffuse emissions to air, in particular of dust, organic compounds and odour. Use one or a combination of the following: |  |  |  |
|  | a. |  | Minimising potential diffuse emission sources – see examples | The operator has indicated that emissions sources have been minimised through using a simple process. However, several diffuse emissions sources are noted outside: open windrows, wood shredding/chipping activities and stock piling and storage of some waste. As BAT 14 requires ‘one or a combination of’ techniques a – h, BAT is still considered met through other techniques employed. |
|  | b. |  | Select and use high-integrity equipment – see examples | Currently compliant The operator has indicated that reliable, robust equipment is used. |
|  | c. |  | Corrosion prevention – see examples | The operator has indicated that corrosion risks are minimised but has not provided any further information. |
|  | d. |  | Containment, collection and treatment of diffuse emissions – see examples | Currently compliant Containment, collection and treatment is undertaken in the IVC building using air extraction and filter through a biofilter. |
|  | e. |  | Dampening (with water or fog) | Currently compliant Dampening with water to supress dust is used where required. |
|  | f. |  | Maintenance – see examples | Currently compliant Maintenance of equipment and plant (section 10.2 of EMS 2018) is undertaken. |
|  | g. |  | Cleaning of waste treatment and storage areas – see examples | Currently compliant Cleaning of treatment and storage areas is undertaken in line with cleaning plans. The OMP 2018 also notes cleaning activities in the odour monitoring, management and control matrix. |
|  | h. |  | Leak Detection And Repair (LDAR) programme for organics – S6.2 | Not used. As BAT 14 requires ‘one or a combination of’ techniques a – h, BAT is still considered met through other techniques employed. |
| 15 | Use flaring only for safety reasons or non-routine operating conditions (OTNOC). Use both of the following: |  |  |  |
|  | a. |  | Correct plant design – see examples | Not applicable No flare on site. |
|  | b. |  | Plant management including gas system balancing and advanced process control | Not applicable No flare on site. |
| 16 | Reduce emissions to air when flaring is unavoidable. Use both of the following: |  |  |  |
|  | a. |  | Correct design of flaring devices – see examples | Not applicable No flare on site. |
|  | b. |  | Monitoring and recording as part of flare management – see examples | Not applicable No flare on site. |
| NOISE AND VIBRATIONS |  |  |  |  |
| 17 | Set up, implement, and regularly review a Noise and Vibration Management Plan (as part of the EMS) where nuisance is expected and/or has been substantiated. Include ALL of the following: |  |  |  |
|  | I. |  | Protocol with actions and timelines | Not applicable A very basic Noise Management Plan (NMP) is contained within the 'IED Risk Assessment' document in the EMS. This contains basic actions but no timelines. As the NMP section in the IED Risk Assessment indicates low risk from noise and there have been no noise related compliance issues this BAT conclusion is deemed not applicable. |
|  | II. |  | Noise and vibration monitoring plan/protocol | As above |
|  | III. |  | Noise & vibration complaint response plan/protocol | As above |
|  | IV. |  | Noise and vibration reduction programme | As above |
| 18 | Techniques to prevent, or where not practicable reduce noise and vibration emissions. Use one or a combination of the following: |  |  |  |
|  | a. |  | Appropriate location of equipment and buildings | Wood chipping area is very close to a dwelling (Ty-hen) and the chipper is used externally so this technique is not used. |
|  | b. |  | Operational measures – see examples | Currently compliant The operator employs a maintenance plan (see BAT 1) and the NMP notes most equipment is only operated during the day time (except fans for the IVC and the backup generator if needed). BAT 1 also ensures staff are adequately trained in site ops. |
|  | c. |  | Low-noise equipment – see examples | Currently compliant The NMP notes that machines and equipment are assessed before being used at site, to ensure that the noise emission levels from the machine comply with appropriate standards. The NMP also states fixed plant or equipment will be checked for noise emission data and appropriate measures taken to ensure it can operate within the NMP criteria (45dBA at 10m from source reducing with distance). |
|  | d. |  | Noise & vibration control equipment – see examples | Currently compliant NMP states machines and equipment will only be operated when all the appropriate noise mufflers, baffles, acoustic panels and dampening is properly fitted and in operational condition. |
|  | e. |  | Noise attenuation – see examples | Not used. |
| EMISSIONS TO WATER |  |  |  |  |
| 19 | Optimise water consumption, reduce waste water generation and prevent or where not practicable reduce emissions to soil and water. Use one or a combination of the following: |  |  |  |
|  | a. |  | Water management – see examples | Currently compliant The Energy and Resource Management plan (ERMP) states that surface water is harvested and used for dampening or wetting of the material in process. The Water Resource Plan notes harvested surface water and roof-rainwater is substituted for mains with grey water used for dust dampening and external processes. |
|  | b. |  | Water recirculation | Not applicable Activities do not produce / use sufficient water. |
|  | c. |  | Impermeable surface | Currently compliant Impermeable surface is implemented on site. |
|  | d. |  | Reduce likelihood and impact of tank/vessel overflows and failures – see examples | The operator has indicated that overflow risks have been considered but has not provided any further information. As BAT 19 requires ‘one or a combination of’ techniques a – i, BAT is still considered met through other techniques employed. |
|  | e. |  | Roofing of waste storage and treatment areas | The operator has indicated that the use of roofing has been considered. The IVCs are in a building but the open windrows are located outside as is some waste storage and wood shredding/chipping. As BAT 19 requires ‘one or a combination of’ techniques a – i, BAT is still considered met through other techniques employed. |
|  | f. |  | Segregation of water streams (being mindful of existing plant constraints) | Currently compliant Water streams are segregated (Site Drainage Plan and Non-Technical Summary diagram). |
|  | g. |  | Adequate drainage infrastructure | Currently compliant Adequate drainage infrastructure is in place as per technique f) and NRW site inspections noting sealed drainage. |
|  | h. |  | Design and maintenance provisions to allow risk-based leak detection and repair. Minimise use of underground components. | The operator has indicated that there is minimised use of sealed tanks below ground. It is not clear whether leak detection and repair monitoring is undertaken or whether underground tanks/sumps are adequately contained. As BAT 19 requires ‘one or a combination of’ techniques a – i, BAT is still considered met through other techniques employed. |
|  | i. |  | Appropriate buffer storage capacity (being mindful of existing plant constraints) | Currently compliant The operator has stated that infrastructure has been time-tested and it has been confirmed to be of adequate capacity however no information has been provided to evidence this. The Fire Management Plan (FPMP) indicates that the site provides the capability to contain firewater from fire-fighting in the effluent storage tanks. |
| 20 | Treat waste water using a combination of: |  |  |  |
|  | Preliminary, primary and general treatment |  |  |  |
|  | a. |  | Equalisation | Not applicable Waste water is not treated on site; it is taken off-site for treatment elsewhere. |
|  | b. |  | Neutralisation | As above |
|  | c. |  | Physical separation | As above |
|  | Physico-chemical treatment |  |  |  |
|  | d. |  | Adsorption | Not applicable Waste water is not treated on site; it is taken off-site for treatment elsewhere. |
|  | e. |  | Distillation/rectification | As above |
|  | f. |  | Precipitation | As above |
|  | g. |  | Chemical oxidation | As above |
|  | h. |  | Chemical reduction | As above |
|  | i. |  | Evaporation | As above |
|  | j. |  | Ion exchange | As above |
|  | k. |  | Stripping | As above |
|  | Biological treatment |  |  |  |
|  | l. |  | Activated sludge process | Not applicable Waste water is not treated on site; it is taken off-site for treatment elsewhere. |
|  | m. |  | Membrane bioreactor | As above |
|  | Nitrogen removal |  |  |  |
|  | n. |  | Nitrification/denitrification (where biological treatment used) | Not applicable Waste water is not treated on site; it is taken off-site for treatment elsewhere. |
|  | Solids removal |  |  |  |
|  | o. |  | Coagulation and flocculation | Not applicable Waste water is not treated on site; it is taken off-site for treatment elsewhere. |
|  | p. |  | Sedimentation | As above |
|  | q. |  | Filtration (sand, micro, ultra) | As above |
|  | r. |  | Flotation | As above |
|  | BAT-AELs for DIRECT discharges to a receiving waterbody (mg/l) |  |  |  |
|  | Table 6.1 and its supporting notes. Monitoring requirements are outlined in BAT 7 |  |  |  |
|  | TOC |  | 10.0-60 | Not applicable No waste water is discharged from site (directly to watercourse or indirectly to sewer). |
|  |  |  | 10-100 for water-based liquid waste |  |
|  | COD (TOC is preferred) |  | 30-180 | As above |
|  |  |  | 30-300 for water-based liquid waste |  |
|  | Suspended solids |  | 5.0-60 | As above |
|  | HOI |  | 0.5-10 applying to specific waste treatments | As above |
|  | Total N |  | 1-25 for biological treatment and waste oil re-refining | As above |
|  |  |  | 10-60 for water-based liquid waste |  |
|  | Total P |  | 0.3-2 for biological treatment | As above |
|  |  |  | 1-3 for water-based liquid waste |  |
|  | Phenol |  | 0.05-0.2 for waste oil re-refining and physio-chemical treatment of waste with CV | As above |
|  |  |  | 0.05-0.3 for water-based liquid waste |  |
|  | Free CN- |  | 0.02-0.1 for water-based liquid waste | As above |
|  | AOX |  | 0.2-1 for water-based liquid waste | As above |
|  | Metals & Metalloids – specific waste treatments as listed in Table 6.1 |  |  |  |
|  | As |  | 0.01-0.05 | As above |
|  | Cd |  | 0.01-0.05 | As above |
|  | Cr |  | 0.01-0.15 | As above |
|  | Cu |  | 0.05-0.5 | As above |
|  | Pb |  | 0.05-0.1 | As above |
|  | Ni |  | 0.05-0.5 | As above |
|  | Hg |  | 0.5-5 | As above |
|  | Zn |  | 0.1-1 | As above |
|  | Metals & Metalloids – treatment of water-based liquid waste |  |  |  |
|  | As |  | 0.01-0.1 | As above |
|  | Cd |  | 0.01-0.1 | As above |
|  | Cr |  | 0.01-0.3 | As above |
|  | Hexavalent Cr [Cr(VI] |  | 0.01-0.1 | As above |
|  | Cu |  | 0.05-0.5 | As above |
|  | Pb |  | 0.05-0.3 | As above |
|  | Ni |  | 0.05-1 | As above |
|  | Hg |  | 1.0-10 | As above |
|  | Zn |  | 0.1-2 | As above |
|  | BAT-AELs for INDIRECT discharges to a receiving waterbody (mg/l) |  |  |  |
|  | Table 6.2 and its supporting notes. Monitoring requirements are outlined in BAT 7 |  |  |  |
|  | HOI |  | 0.5-10 applying to specific waste treatments | Not applicable No waste water is discharged from site (directly to watercourse or indirectly to sewer). |
|  | Free CN- |  | 0.02-0.1 for water-based liquid waste | As above |
|  | AOX |  | 0.2-1 for water-based liquid waste | As above |
|  | Metals & Metalloids – specific waste treatments as listed in Table 6.2 |  |  |  |
|  | As |  | 0.01-0.05 | As above |
|  | Cd |  | 0.01-0.05 | As above |
|  | Cr |  | 0.01-0.15 | As above |
|  | Cu |  | 0.05-0.5 | As above |
|  | Pb |  | 0.05-0.1 | As above |
|  | Ni |  | 0.05-0.5 | As above |
|  | Hg |  | 0.5-5 | As above |
|  | Zn |  | 0.1-1 | As above |
|  | Metals & Metalloids – treatment of water-based liquid waste |  |  |  |
|  | As |  | 0.01-0.1 | As above |
|  | Cd |  | 0.01-0.1 | As above |
|  | Cr |  | 0.01-0.3 | As above |
|  | Hexavalent Cr [Cr(VI] |  | 0.01-0.1 | As above |
|  | Cu |  | 0.05-0.5 | As above |
|  | Pb |  | 0.05-0.3 | As above |
|  | Ni |  | 0.05-1 | As above |
|  | Hg |  | 1.0-10 | As above |
|  | Zn |  | 0.1-2 | As above |
| EMISSIONS FROM ACCIDENTS AND INCIDENTS |  |  |  |  |
| 21 | Techniques to prevent or limit the environmental consequences of accidents and incidents, as part of the Accident Management Plan. Use ALL of the following: |  |  |  |
|  | a. |  | Protection measures – see examples | Currently compliant The EMS 2018 notes the use of a mechanical windrow turner for open windrows and also temperature checks of the process. The FPMP states buildings and equipment are kept secure; fire-fighting facilities are available in the form of water supplies and small extinguishers; automatic detection of plant/ equipment failures integrated with sensors and systems linked to alarm systems and telemetric alarms to the manager’s telephone. The EMS 2018 notes the use of security measures and a security system. |
|  | b. |  | Management of incidental or accidental emissions | Currently compliant Containment of firewater and use of portable bunds and drain covers are noted in the FPMP. A spill procedure is in place as part of the EMS. |
|  | c. |  | Incident/accident registration and assessment system – see examples | Currently compliant Section 11 of the EMS 2018 includes info on recording incidents/accidents and reviewing reasons for their occurrence and taking actions to prevent re-occurrence. |
| MATERIAL EFFICIENCY |  |  |  |  |
| 22 | Use materials efficiently by substituting materials with waste e.g. waste acids/alkalis for pH adjustment, fly ashes for binders |  |  | Not applicable Minimal raw materials are used. The nature of composting is using waste to create a beneficial product. |
| ENERGY EFFICIENCY |  |  |  |  |
| 23 | Use energy efficiently by using both of the following techniques: |  |  |  |
|  | a. |  | Energy efficiency plan | Currently compliant An Energy Efficiency Plan (EEP) is in place including general targets however nothing included to indicate the targets will be reviewed. This is however a condition of the current permit i.e. review every 4 years. |
|  | b. |  | Energy balance record | Currently compliant Energy used per tonne input by activity listed in EEP. |
| REUSE OF PACKAGING |  |  |  |  |
| 24 | Maximise the reuse of packaging as part of a Residues Management Plan (see BAT 1 XII.) |  |  | Currently compliant The operator has indicated that minimal packaging is used. The Energy and Resource Management plan indicates pallets are reused. There is minimal opportunity for reusing packaging on this site. |
| MECHANICAL TREATMENT OF WASTE (GENERAL BAT) |  |  |  |  |
| 25 | Reduce emissions to air of dust, particulate-bound metals, PCDD/F and dioxin-like PCBs by applying BAT 14d AND using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Cyclone – see S6.1 | Not applicable Any mechanical treatment associated with installation activities is combined with biological treatment. |
|  | b. |  | Fabric filter – see S6.1 | As above |
|  | c. |  | Wet scrubbing – see S6.1 | As above |
|  | d. |  | Water injection into the shredder | As above |
|  | BAT-AEL for channelled dust emissions to air from the mechanical treatment of waste (mg/Nm3) |  |  |  |
|  | Table 6.3 and its supporting note. Monitoring requirements are outlined in BAT 8 |  |  |  |
|  | Dust |  | 2.0-5.0 | Not applicable Any mechanical treatment associated with installation activities is combined with biological treatment. |
| MECHANICAL TREATMENT OF METAL WASTE BY SHREDDING |  |  |  |  |
| 26 | Improve overall environmental performance and prevent emissions due to accidents and incidents. Use BAT 14g AND ALL of the following techniques: |  |  |  |
|  | (a) |  | Detailed inspection procedure for baled waste before shredding | Not applicable Any mechanical treatment associated with installation activities is combined with biological treatment and does not include shredding metal. |
|  | (b) |  | Remove dangerous items from waste inputs and dispose of them in a safe manner | As above |
|  | (c) |  | Treatment of containers accompanied by a declaration of cleanliness | As above |
| 27 | Prevent deflagrations and reduce emissions from deflagrations. Use technique a. AND ONE OR BOTH of techniques b. and c. |  |  |  |
|  | a. |  | Deflagration management plan with reduction programme, incident review and response protocol | Not applicable Any mechanical treatment associated with installation activities is combined with biological treatment and does not include shredding metal. |
|  | b. |  | Pressure relief dampers | As above |
|  | c. |  | Pre-shredding (device) | As above |
| 28 | Use energy efficiently by keeping the shredder feed stable |  |  | Not applicable Any mechanical treatment associated with installation activities is combined with biological treatment and does not include shredding metal. |
| MECHANICAL TREATMENT OF WEEE CONTAINING VFCS AND/OR VHCS |  |  |  |  |
| 29 | Techniques to prevent, or where not practicable reduce emissions of organic compounds to air. Apply BAT 14d AND BAT14h AND technique a. AND ONE OR BOTH of techniques b. and c. |  |  |  |
|  | a. |  | Optimised removal and capture of refrigerants and oils | Not applicable The operator does not undertake mechanical treatment of WEEE. |
|  | b. |  | Cryogenic condensation | As above |
|  | c. |  | Adsorption | As above |
|  | BAT-AELs for channelled TVOC and CFC emissions to air from treatment of WEEE containing VFCs and/or VHCs (mg/Nm3) |  |  |  |
|  | Table 6.4. Monitoring requirements are outlined in BAT 8 |  |  |  |
|  | TVOC |  | 3.0-15 | Not applicable The operator does not undertake mechanical treatment of WEEE. |
|  | CFCs |  | 0.5-10 | As above |
| 30 | Prevent emissions due to explosions when treating WEEE containing VFCs and/or VHCs. Use EITHER of the following techniques: |  |  |  |
|  | a. |  | Inert atmosphere e.g. N2 | Not applicable The operator does not undertake mechanical treatment of WEEE. |
|  | b. |  | Forced ventilation | As above |
| MECHANICAL TREATMENT OF WASTE WITH CALORIFIC VALUE |  |  |  |  |
| 31 | Reduce emissions to air of organic compounds by applying BAT 14d AND using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Adsorption – see S6.1 | Not applicable The operator does not undertake mechanical treatment of waste with calorific value. |
|  | b. |  | Biofilter – see S6.1 | As above |
|  | c. |  | Thermal oxidation – see S6.1 | As above |
|  | d. |  | Wet scrubbing – see S6.1 | As above |
|  | BAT-AEL for channelled TVOC emissions to air from the mechanical treatment of waste with calorific value (mg/Nm3) |  |  |  |
|  | Table 6.5 and its supporting note. Monitoring requirements are outlined in BAT 8 |  |  |  |
|  | TVOC |  | 10.0-30.0 | Not applicable The operator does not undertake mechanical treatment of waste with calorific value. |
| MECHANICAL TREATMENT OF WEEE CONTAINING MERCURY |  |  |  |  |
| 32 | Reduce mercury emissions to air by collecting them at source, sending them to abatement and carrying out adequate monitoring. This includes ALL of the following: |  |  |  |
|  | Equipment is enclosed, under negative pressure and connected to a LEV system |  |  | Not applicable The operator does not undertake mechanical treatment of WEEE. |
|  | Waste gas treated using dedusting techniques – see examples – followed by adsorption on activated carbon |  |  | As above |
|  | Monitoring of waste gas treatment efficiency |  |  | As above |
|  | Mercury levels measured at least weekly within treatment and storage areas |  |  | As above |
|  | BAT-AEL for channelled mercury (Hg) emissions to air from the mechanical treatment of WEEE containing mercury (µg/Nm3) |  |  |  |
|  | Table 6.6. Monitoring requirements are outlined in BAT 8 |  |  |  |
|  | Hg |  | 2.0-7.0 | Not applicable The operator does not undertake mechanical treatment of WEEE. |
| BIOLOGICAL TREATMENT OF WASTE (GENERAL BAT) |  |  |  |  |
| 33 | Reduce odour emissions and improve overall environmental performance by selecting the waste input (to ensure its suitability for biological treatment). See also BAT 2 |  |  | Compliant in future The operator has not provided a response and BAT 2 is not adequately met at present. An improvement condition has been set. |
| 34 | Reduce emissions to air of dust, organic compounds and odorous compounds (including H2S & NH3) by using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Adsorption – see S6.1 | Not used. As BAT 34 requires ‘one or a combination of’ techniques a – e, BAT is still considered met through other techniques employed. |
|  | b. |  | Biofilter – see S6.1 | Currently compliant An open topped biofilter is used for air extracted from the reception building. The OMP 2018 notes the biofilter is ‘based upon industry standards and meets the EA design requirements for air contact time (45 seconds)’. Further design detail is contained in the OMP 2018. |
|  | c. |  | Fabric filter – see S6.1. | Not used. As BAT 34 requires ‘one or a combination of’ techniques a – e, BAT is still considered met through other techniques employed. |
|  | d. |  | Thermal oxidation – see S6.1 | Not used. As BAT 34 requires ‘one or a combination of’ techniques a – e, BAT is still considered met through other techniques employed. |
|  | e. |  | Wet scrubbing – see S6.1 | Not used. As BAT 34 requires ‘one or a combination of’ techniques a – e, BAT is still considered met through other techniques employed. |
|  | BAT-AEL for channelled NH3, odour, dust and TVOC emissions to air from the biological treatment of waste (mg/Nm3) (ouE/m3) |  |  |  |
|  | Table 6.7 and its supporting notes. Monitoring requirements are outlined in BAT 8 |  |  |  |
|  | NH3 |  | 0.3-20 | Compliant in future Monitoring has been added to the permit. |
|  | Odour |  | 200-1000 | Not applicable Only applicable where BAT 3 indicates odour is relevant and/or where odour can potentially impact sensitive receptors. The existing OMP 2018 states “with benefit of containment, exhaust air bio filtration, suppressed emission and dispersal there is a low risk that the site will lead to odour nuisance at the nearest receptors’. |
|  | Dust |  | 2.0-5.0 | Not applicable Only applicable to mechanical biological treatment. |
|  | TVOC |  | 5.0-40 | Not applicable Only applicable to mechanical biological treatment. |
| 35 | Reduce the generation of waste water and reduce water usage by using ALL of the following: |  |  |  |
|  | a. |  | Segregation of water streams (see also BAT 19f) | Currently compliant Water streams are segregated (Site Drainage Plan and Non-Technical Summary diagram). |
|  | b. |  | Water recirculation | Not applicable The Non-Tech summary document indicates water is not recirculated on site. Activities do not produce / use sufficient water. |
|  | c. |  | Minimisation of the generation of leachate | Compliant in future The operator has indicated that effluent generation is minimised but has not provided detail as to how the operator optimises the moisture content of the waste in order to minimise the generation of leachate. An improvement condition has been set. |
| BIOLOGICAL TREATMENT OF WASTE: AEROBIC METHODS |  |  |  |  |
| 36 | Reduce emissions to air and improve overall environmental performance by monitoring and/or controlling key waste and process parameters. Include following elements: |  |  |  |
|  | Waste input characteristics e.g. C to N ratio, particle size |  |  | Compliant in future The operator has indicated this is controlled within the quality composting process but has not provided any further information. An improvement condition has been set. |
|  | Temperature and moisture content within windrows (Moisture monitoring not needed for enclosed processes where H&S issues have been identified) |  |  | Currently compliant The OMP 2018 states ‘computerised and bespoke PLC systems are utilised for automated control of process ventilation, airflows, temperature, air quality and moisture. The handling systems are based on techniques that do not aggressively agitate or aerate the material during external movement’. Temperature and moisture are also part of the process monitoring requirements in the current permit. |
|  | Aeration of the windrow |  |  | Currently compliant Use of mechanical windrow turning machine noted in IED Non Technical Summary document. The same document also notes the introduction of air into the vessel ventilation system / fans maintaining air movement by recirculating air from the top of the vessel back in underneath the material. OMP 2018 states that control systems enable modulation of the air flow and the oxygen level can be maintained at target throughout each stage of the In-Vessel process. |
|  | Windrow porosity, height and width |  |  | Compliant in future The operator has indicated that these are controlled within the quality composting process but has provided no further information. The EMS 2018 notes that green waste creates porosity and some is saved back during Autumn for use through winter but there is no detail on how porosity is monitored / controlled nor height and width of windrows. Improvement condition set. |
| 37 | Reduce diffuse emissions to air of dust, odour and bioaerosols from open-air treatment steps. Use ONE OR BOTH of the following techniques: |  |  |  |
|  | a. |  | Use of semi-permeable membrane covers | The operator has stated that this technique is not used. |
|  | b. |  | Adaptation of operations to the meteorological conditions | Compliant in future The OMP indicates operation of the shredder can be planned to suit weather conditions and that daily weather checks are undertaken to ensure contingency arrangements can be put in place prior to any problem occurring on site. If site is closed deliveries are diverted to an alternative suitably authorised site. The EMS 2018 also indicates specific incident management training is given to staff on adverse weather conditions and the 'IED Bios Risk Assessment' notes external operations are undertaken with due consideration for the wind direction and weather. However, the direction of outdoor windrows in relation to prevailing wind direction has not been considered and an improvement condition has been set. |
| BIOLOGICAL TREATMENT OF WASTE: ANAEROBIC METHODS |  |  |  |  |
| 38 | Reduce emissions to air and improve overall environmental performance by monitoring and/or controlling key waste and process parameters. Include following elements: |  |  |  |
|  | Implement a manual and/or automatic monitoring system to: |  |  |  |
|  | Ensure a stable digester operation |  |  | Not applicable The operator does not undertake anaerobic treatment of waste. |
|  | Minimise operational difficulties and associated odour emissions |  |  | As above |
|  | Provide sufficient early warning of system failures |  |  | As above |
|  | Windrow porosity, height and width |  |  | As above |
|  | Monitoring and/or control of key waste and process parameters – examples below: |  |  |  |
|  | pH and alkalinity of the digester feed |  |  | Not applicable The operator does not undertake anaerobic treatment of waste. |
|  | Digester operating temperature |  |  | As above |
|  | Hydraulic and organic loading rates of the digester feed |  |  | As above |
|  | Volatile fatty acids and NH3 concentrations within digester & digestate |  |  | As above |
|  | Biogas quantity, composition (e.g. H2S) and pressure |  |  | As above |
|  | Liquid and foam levels in the digester |  |  | As above |
| MECHANICAL BIOLOGICAL TREATMENT (MBT) OF WASTE |  |  |  |  |
| 39 | Reduce emissions to air. Generally applicable to new plants, existing plants may have layout constraints. Use BOTH of the following techniques: |  |  |  |
|  | a. |  | Segregation of the waste gas streams (refer to inventory described in BAT 3) | Not applicable The operator does not undertake mechanical biological treatment of waste. |
|  | b. |  | Recirculation of waste gas. Waste gas treatment is described in BAT 34 and recirculation in BAT 35. | As above |
| PHYSICO-CHEMICAL TREATMENT OF SOLID AND/OR PASTY WASTE |  |  |  |  |
| 40 | Improve overall environmental performance by monitoring the waste input as part of the waste pre-acceptance and acceptance procedures. See also BAT 2. |  |  |  |
|  | Monitoring the waste input |  |  |  |
|  | Content of organics, oxidising agents, metals, salts, odorous compounds |  |  | Not applicable The operator does not undertake physico-chemical treatment of solid and/or pasty waste. |
|  | H2 formation potential upon mixing of flue-gas treatment residues/ashes with water |  |  | As above |
| 41 | Reduce emissions to air of dust, organic compounds and NH3 by applying BAT 14d AND using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Adsorption – see S6.1 | Not applicable The operator does not undertake physico-chemical treatment of solid and/or pasty waste. |
|  | b. |  | Biofilter – see S6.1 | As above |
|  | c. |  | Fabric filter – see S6.1. | As above |
|  | d. |  | Wet scrubbing – see S6.1 | As above |
|  | BAT-AEL for channelled NH3, odour, dust and TVOC emissions to air from the physico-chemical treatment of solid and/or pasty waste (mg/Nm3) |  |  |  |
|  | Table 6.8. Monitoring requirements are outlined in BAT 8 |  |  |  |
|  | Dust |  | 2.0-5.0 | Not applicable The operator does not undertake physico-chemical treatment of solid and/or pasty waste. |
| RE-REFINING OF WASTE OIL |  |  |  |  |
| 42 | Improve overall environmental performance by monitoring the waste input as part of the waste pre-acceptance and acceptance procedures. See also BAT 2. |  |  |  |
|  | Monitoring the waste input |  |  |  |
|  | Chlorinated compounds e.g. solvents or PCBs |  |  | Not applicable The operator does not undertake re-refining of waste oil. |
| 43 | Reduce quantity of waste sent for disposal by using ONE OR BOTH of the following techniques: |  |  |  |
|  | a. |  | Material recovery e.g. organic residues in asphalt products | Not applicable The operator does not undertake re-refining of waste oil. |
|  | b. |  | Energy recovery | As above |
| 44 | Reduce emissions to air of organic compounds by applying BAT 14d AND using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Adsorption – see S6.1 | Not applicable The operator does not undertake re-refining of waste oil. |
|  | b. |  | Thermal oxidation – see S6.1 | As above |
|  | c. |  | Wet scrubbing – see S6.1 | As above |
|  | The BAT-AEL for TVOC emissions to air set in Section 4.5 (below) applies. |  |  |  |
|  | Monitoring requirements are outlined in BAT 8 |  |  |  |
| PHYSICO-CHEMICAL TREATMENT OF WASTE WITH CALORIFIC VALUE |  |  |  |  |
| 45 | Reduce emissions to air of organic compounds by applying BAT 14d AND using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Adsorption – see S6.1 | Not applicable The operator does not undertake physico-chemical treatment of wastes with calorific value. |
|  | b. |  | Cryogenic condensation – see S6.1 | As above |
|  | c. |  | Thermal oxidation – see S6.1 | As above |
|  | d. |  | Wet scrubbing – see S6.1 | As above |
|  | The BAT-AEL for TVOC emissions to air set in Section 4.5 (below) applies. |  |  |  |
|  | Monitoring requirements are outlined in BAT 8 |  |  |  |
| REGENERATION OF SPENT SOLVENTS |  |  |  |  |
| 46 | Improve overall environmental performance by using ONE OR BOTH of the following techniques: |  |  |  |
|  | a. |  | Material recovery (by evaporation from distillation residues) | Not applicable The operator does not undertake regeneration of spent solvents. |
|  | b. |  | Energy recovery e.g. using distillation residues | As above |
| 47 | Reduce emissions to air of organic compounds by applying BAT 14d AND using a combination of the following techniques: |  |  |  |
|  | a. |  | Recirculation of process off-gases in a steam boiler. Avoid generating PCBs and/or PCDD/Fs | Not applicable The operator does not undertake regeneration of spent solvents. |
|  | b. |  | Adsorption – see S6.1 | As above |
|  | c. |  | Thermal oxidation – see S6.1. Avoid generating PCBs and/or PCDD/Fs | As above |
|  | d. |  | Condensation or cryogenic condensation | As above |
|  | e. |  | Wet scrubbing – see S6.1 | As above |
|  | The BAT-AEL for TVOC emissions to air set in Section 4.5 (below) applies. |  |  |  |
|  | Monitoring requirements are outlined in BAT 8 |  |  |  |
| BAT-AEL FOR EMISSIONS OF ORGANIC COMPOUNDS TO AIR – SECTION 4.5 |  |  |  |  |
| (RE-REFINING OF WASTE OIL) |  |  |  |  |
| (PHYSICO-CHEMICAL TREATMENT OF WASTE WITH CV) |  |  |  |  |
| (REGENERATION OF SPENT SOLVENTS) |  |  |  |  |
|  | BAT-AEL for channelled TVOC emissions to air from the re-refining of waste oil, physico-chemical treatment of waste with calorific value and regeneration of spent solvents (mg/Nm3) |  |  |  |
|  | Table 6.9 and its supporting note. Monitoring requirements are outlined in BAT 8 |  |  |  |
|  | TVOC |  | 5.0-30 | Not applicable The operator does not undertake re-refining of waste oil, physico-chemical treatment of wastes with calorific value or regeneration of spent solvents. |
| THERMAL TREATMENT OF SPENT ACTIVATED CARBON, WASTE CATALYSTS AND EXCAVATED CONTAMINATED SOIL |  |  |  |  |
| 48 | Improve overall environmental performance by using ALL of the following techniques: |  |  |  |
|  | a. |  | Heat recovery from the furnace off-gas e.g. for preheating combustion air or steam generation | Not applicable The operator does not undertake thermal treatment of spent activated carbon, waste catalysts and/or excavated contaminated soil. |
|  | b. |  | Indirectly fired furnace i.e. avoids contact between the furnace contents and the burner flue-gases. Note applicability constraints. | As above |
|  | c. |  | Process-integrated techniques to reduce emissions to air – see examples | As above |
| 49 | Reduce emissions to air of HCl, HF, dust and organic compounds by applying BAT 14d AND using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Cyclone – see S6.1 | Not applicable The operator does not undertake thermal treatment of spent activated carbon, waste catalysts and/or excavated contaminated soil. |
|  | b. |  | Electrostatic precipitator (ESP) – see S6.1 | As above |
|  | c. |  | Fabric filter – see S6.1 | As above |
|  | d. |  | Wet scrubbing – see S6.1 | As above |
|  | e. |  | Adsorption – see S6.1 | As above |
|  | f. |  | Condensation – see S6.1 | As above |
|  | g. |  | Thermal oxidation – see S6.1 | As above |
|  | Note supporting text for BAT 49g (thermal oxidation) |  |  |  |
|  | Monitoring requirements are outlined in BAT 8. No BAT-AELs have been set for this BATc. |  |  |  |
| WATER WASHING OF EXCAVATED CONTAMINATED SOIL |  |  |  |  |
| 50 | Reduce emissions to air of dust and organic compounds from the storage, handling and washing steps by applying BAT 14d AND using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Adsorption – see S6.1 | Not applicable The operator does not undertake water washing of excavated contaminated soil. |
|  | b. |  | Fabric filter – see S6.1 | As above |
|  | c. |  | Wet scrubbing – see S6.1 | As above |
|  | Monitoring requirements are outlined in BAT 8. No BAT-AELs have been set for this BATc. |  |  |  |
| Decontamination of equipment containing PCBs |  |  |  |  |
| 51 | Reduce emissions to air of PCBs and organic compounds and improve overall environmental performance by using ALL of the following techniques: |  |  |  |
|  | a. |  | Coating of the storage and treatment areas – see examples | Not applicable The operator does not undertake decontamination of equipment containing PCBs. |
|  | b. |  | Implementation of staff access rules to prevent dispersion of contamination – see examples | As above |
|  | c. |  | Optimised equipment cleaning and drainage – see examples | As above |
|  | d. |  | Control and monitoring of emission to air – see examples | As above |
|  | e. |  | Disposal of waste treatment residues – see examples | As above |
|  | f. |  | Recovery of solvent when solvent washing is used | As above |
|  | Monitoring requirements are outlined in BAT 8. No BAT-AELs have been set for this BATc. |  |  |  |
| TREATMENT OF WATER-BASED LIQUID WASTE |  |  |  |  |
| 52 | Improve overall environmental performance by monitoring the waste input as part of the waste pre-acceptance and acceptance procedures. See also BAT 2. |  |  |  |
|  | Monitoring the waste input |  |  |  |
|  | Bioeliminability e.g. BOD, BOD-COD ratio, Zahn-Wellens test, biological inhibition potential |  |  | Not applicable The operator does not undertake treatment of water-based liquid waste. |
|  | Feasibility of emulsion breaking e.g. lab testing |  |  | As above |
| 53 | Reduce emissions to air of HCl, NH3 and organic compounds by applying BAT 14d AND using one or a combination of the following techniques: |  |  |  |
|  | a. |  | Adsorption – see S6.1 | Not applicable The operator does not undertake treatment of water-based liquid waste. |
|  | b. |  | Biofilter – see S6.1 | As above |
|  | c. |  | Thermal oxidation – see S6.1. | As above |
|  | d. |  | Wet scrubbing – see S6.1 | As above |
|  | BAT-AELs for channelled HCl and TVOC emissions to air from the treatment of water-based liquid waste (mg/Nm3) |  |  |  |
|  | Table 6.10 and its supporting notes. Monitoring requirements are outlined in BAT 8 |  |  |  |
|  | HCl |  | 1.0-5.0 | Not applicable The operator does not undertake treatment of water-based liquid waste. |
|  | TVOC |  | 3.0-20 | As above |

Annex 2: Consultation on the draft decision where an Article 15(4) derogation has been applied

# Not applicable