Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

CWM Environmental Limited

Nantycaws HCI Waste Transfer & Treatment Facility

Llanddarog Road

Nantycaws

Carmarthen

SA32 8BG

Permit number

EPR/EP3698FL

Nantycaws HCI Transfer & Treatment Facility

Permit number EPR/EP3698FL

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.

This permit is for the recovery and disposal of a non-hazardous and hazardous household, commercial and industrial waste transfer and treatment facility. The HCI waste transfer with treatment station includes enclosed vessel composting and open windrow composting activities where the recovery of non-hazardous biodegradable wastes must be carried out under aerobic conditions. It also permits the recovery of wastes for the production of soil, soil substitutes and aggregates, and a paint blending and stripping facility. The total throughput is 100,000 tonnes a year.

Any wastes controlled by the Animal By-products Regulations (ABPR) must be handled and treated in accordance with any requirements imposed by those Regulations.

The waste types permitted for the enclosed vessel composting activity and part of the open windrow composting activity are subject to PAS 100 and the Environment Agency’s Quality Protocol Appendix B list in order to meet the End-of-Waste (EoW) criteria for compost.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| ### EAWML 34207 determined | 02/09/03 | Permit issued to CWM Environmental Limited. |
| Variation determined EPR/EP3698FL/V002 (previously EAWML 34207) | 05/10/04 | Varied to amend Tables 1.1 and 1.2 |
| ### Application EPR/EP3698FL/V003 (previously EAWML 34207) | ### Duly Made 17/08/12 | Application to include composting and additional waste codes resulting in the re-classification of the site to a hazardous and non-hazardous HCI WTS with treatment. It also includes an extension to the site boundary. |
| ### Variation determined EPR/EP3698FL/V003 | ### 08/11/12 | Varied and consolidated permit issued in modern condition format. Also, to include composting and additional waste codes resulting in the re-classification of the site to a hazardous and non-hazardous HCI WTS with treatment. It also includes an extension to the site boundary. |
| ### Variation determined EPR/EP3698FL/V004 | ### 06/07/15 | Regular led variation to vary waste permit to an installation permit following the implementation of the Industrial Emissions Directive. Varied and consolidated permit issued in modern condition format. |
| Regulation 61(1) Notice sent to the Operator | 05/04/19 | Issue of a Notice under Regulation 61(1) of the EPR. Natural Resources Wales initiated review and variation to vary the permit following the publication of the revised Best Available Techniques (BAT) Reference Document (BREF) for Waste Treatment. |
| Regulation 61(1) Notice response | 30/09/19 | Response received from the operator. |
| Additional information received | 16/01/20 | Response received from the operator. |
| NRW initiated Variation determined EPR/EP3698FL/V005 | 08/07/21 | Varied and consolidated permit issued in modern condition format. |
| Variation application EPR/EP3698FL/V006 | Duly Made 17/02/21 | Operator led variation to add paint blending and stripping operations to the permit, and to consolidate the permit. |
| Variation issued EPR/EP3698FL | 15/07/21 | Varied and consolidated permit issued. |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/EP3698FL

The Natural Resources Body for Wales (“Natural Resources Wales”) authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016

CWM Environmental Limited (“the operator”)

whose registered office is

Head Office

Nantycaws Recycling Centre

Llanddarog Road

Carmarthen

SA32 8BG

company registration number 02640102

to operate an installation and waste operation at

Nantycaws HCI Waste Transfer Station

Nantycaws

Carmarthen

SA32 8BG

to the extent authorised by and subject to the conditions of this permit.

| Name | Date |
| --- | --- |
| Huw Davies | 15/07/2021 |

Authorised on behalf of Natural Resources Wales

Conditions

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

### The operator shall comply with the requirements of an approved competence scheme.

## 1.2	Energy efficiency

### 1.2.1	For the following activities referenced in schedule 1, table S1.1 (A1 to A2) the operator shall:

(a)	take appropriate measures to ensure that energy is used efficiently in the activities;

(b)	review and record at least every four years whether there are suitable opportunities to improve the energy efficiency of the activities; and

(c)	take any further appropriate measures identified by a review.

## 1.3	Efficient use of raw materials

### 1.3.1	For the following activities referenced in schedule 1, table S1.1 (A1 to A2) the operator shall:

(a)	take appropriate measures to ensure that raw materials and water are used efficiently in the activities;

(b)	maintain records of raw materials and water used in the activities;

(c)	review and record at least every four years whether there are suitable alternative materials that could reduce environmental impact or opportunities to improve the efficiency of raw material and water use; and

(d)	take any further appropriate measures identified by a review.

## Avoidance, recovery and disposal of wastes produced by the activities

### The operator shall take appropriate measures to ensure that:

### the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied to the generation of  waste by the activities; and

### any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

### The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

### 2.1.2	For the following activities referenced in schedule 1, table S1.1 (A1 to A2) waste authorised by this permit shall be clearly distinguished from any other waste on the site.

## 2.2	The site

2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

## 2.3	Operating techniques

#### 2.3.1	(a) 	The activities shall, subject to the conditions of this permit, be operated using the

#### techniques and in the manner described in the documentation specified in 	schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan , and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

### 2.3.2	Waste shall only be accepted if:

### (a)	it is of a type and quantity listed in schedule 2 table(s) S2.1, S2.2, S2.3, S2.4, S2.5 and S2.6; and

### (b)	it conforms to the description in the documentation supplied by the producer and holder.

### 2.3.3	The operator shall ensure that where waste produced by the activities is sent to a relevant waste operation, that operation is provided with the following information, prior to the receipt of the waste:

### (a)	the nature of the process producing the waste;

### (b)	the composition of the waste;

### (c)	the handling requirements of the waste;

### (d)	the hazardous property associated with the waste, if applicable; and

### (e)	the waste code of the waste.

### 2.3.4	The operator shall ensure that where waste produced by the activities is sent to a landfill site, it meets the waste acceptance criteria for that landfill.

2.4	Technical requirements

Hazardous waste storage and treatment

### 2.4.1	Hazardous waste shall not be mixed, either with a different category of hazardous waste or with other waste, substances or materials, unless it is authorised by schedule 1 table S1.1 and appropriate measures are taken.

## 2.5	Improvement programme

### 2.5.1 	The operator shall complete the improvements specified in schedule 1 table S1.3 by the date specified in that table unless otherwise agreed in writing by Natural Resources Wales.

### 2.5.2	Except in the case of an improvement which consists only of a submission to Natural Resources Wales, the operator shall notify Natural Resources Wales within 14 days of completion of each improvement.

# 3	Emissions and monitoring

## 3.1	Emissions to water, air or land

### 3.1.1	There shall be no point source emissions to water, air or land except from the sources and emission points listed in schedule 3 tables S3.1, S3.2,S3.3 and S3.6.

### 3.1.2	The limits given in schedule 3 shall not be exceeded.

    - Periodic monitoring shall be carried out at least once every 5 years for groundwater and 10 years for soil, unless such monitoring is based on a systematic appraisal of the risk of contamination.

## Emissions of substances not controlled by emission limits

### Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

3.2.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

#### (b)	implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3.2.3	All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

## 3.3	Odour

#### 3.3.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

3.3.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to odour, submit to Natural Resources Wales for approval within the period specified, an odour management plan which identifies and minimises the risks of pollution from odour;

#### (b)	implement the approved odour management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.4	Noise and vibration

#### 3.4.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

3.4.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to noise and vibration, submit to Natural Resources Wales for approval within the period specified, a noise and vibration management plan which identifies and minimises the risks of pollution from noise and vibration;

#### (b)	implement the approved noise and vibration management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.5	Monitoring

### 3.5.1	The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

#### point source emissions specified in tables S3.1, S3.2 and S3.3;

#### surface water specified in table S3.4;

#### (c)	process monitoring specified in table S3.5;

#### (d)	bioaerosol monitoring specified in table S3.6.

3.5.2	The operator shall maintain records of all monitoring required by this permit including records of the taking and analysis of samples, instrument measurements (periodic and continual), calibrations, examinations, tests and surveys and any assessment or evaluation made on the basis of such data.

3.5.3	Monitoring equipment, techniques, personnel and organisations employed for the emissions monitoring programme and the environmental or other monitoring specified in condition 3.5.1 shall have either MCERTS certification or MCERTS accreditation (as appropriate), where available, unless otherwise agreed in writing by Natural Resources Wales.

3.5.4	Permanent means of access shall be provided to enable sampling/monitoring to be carried   out in relation to the emission points specified in schedule 3 tables S3.1, S3.2 ,S3.3, S3.4, S3.5 and S3.6 unless otherwise agreed in writing by Natural Resources Wales.

## 3.6	Pests

3.6.1 	The activities shall not give rise to the presence of pests which are likely to cause pollution, hazard or annoyance outside the boundary of the site. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved pests management plan, have been taken to prevent or where that is not practicable, to minimise the presence of pests on the site.

3.6.2 	The operator shall:

(a)           if notified by Natural Resources Wales, submit to Natural Resources Wales for approval within the period specified, a pests management plan which identifies and minimises risks of pollution, hazard or annoyance  from pests;

## (b)           implement the pests management plan, from the date of approval, unless   otherwise agreed in writing by Natural resources Wales

3.7    Fire

3.7.1 	The operator shall manage and operate the activities in accordance with a written fire prevention plan using the current, relevant fire prevention and mitigation plan guidance.

3.7.2     The operator shall:

(a)         if notified by Natural Resources Wales that the activities could cause a fire risk, submit to Natural Resources Wales a fire prevention mitigation plan which identifies and minimises the risks of fire;

(b)        Operate the activity in accordance with the fire prevention mitigation plan, from the date of submission, unless otherwise agreed in writing by Natural Resources Wales.

# 4	Information

## 4.1	Records

### All records required to be made by this permit shall:

### be legible;

### be made as soon as reasonably practicable;

### if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

### be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- off-site environmental effects; and

- matters which affect the condition of the land and groundwater.

4.1.2	The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

## 4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

### 4.2.2	For the following activities referenced in schedule 1, table S1.1 (A1 to A2) a report or reports on the performance of the activities over the previous year shall be submitted to Natural Resources Wales by 31 January (or other date agreed in writing by Natural Resources Wales) each year.  The report(s) shall include as a minimum:

#### (a)	a review of the results of the monitoring and assessment carried out in accordance with the permit including an interpretive review of that data;

#### (b)	the annual production /treatment data set out in schedule 4 table S4.2; and

#### (c)	the performance parameters set out in schedule 4 table S4.3 using the forms specified in table S4.4 of that schedule.

### 4.2.3	Within 28 days of the end of the reporting period the operator shall, unless otherwise agreed in writing by Natural Resources Wales, submit reports of the monitoring and assessment carried out in accordance with the conditions of this permit, as follows:

- in respect of the parameters and emission points specified in schedule 4 table S4.1;

- for the reporting periods specified in schedule 4 table S4.1 and using the forms specified in schedule 4 table S4.4 ; and

- giving the information from such results and assessments as may be required by the forms specified in those tables.

4.2.4	The operator shall, unless notice under this condition has been served within the preceding four years, submit to Natural Resources Wales, within six months of receipt of a written notice, a report assessing whether there are other appropriate measures that could be taken to prevent, or where that is not practicable, to minimise pollution.

### 4.2.5	Within 1 month of the end of each quarter, the operator shall submit to Natural Resources Wales using the form made available for the purpose, the information specified on the form relating to the site and the waste accepted and removed from it during the previous quarter.

### 4.2.6	The operator shall submit to Natural Resources Wales a bi-annual report of the efficiency of the biofilter in the first year of compost operations. This shall include but not be limited to, the assessment of the efficiency to reduce odours, the summary of maintenance and any recommissioning planned or conducted, assessment of back pressure, venting and cracking. Thereafter the operator shall submit the report within one month of the end of each year, unless otherwise agreed in writing by Natural Resources Wales.

## 4.3	Notifications

4.3.1	For the following activities referenced in schedule 1, table S1.1 (A1 to A2 inclusive)

(a)	In the event that the operation of the activities gives rise to an incident or accident which significantly affects or may significantly affect the environment, the operator must immediately—

(i)	inform Natural Resources Wales,

(ii)	take the measures necessary to limit the environmental consequences of such an incident or accident, and

(iii)	take the measures necessary to prevent further possible incidents or accidents;

(b)	in the event of a breach of any permit condition the operator must immediately—

(i)	inform Natural Resources Wales, and

(ii)	take the measures necessary to ensure that compliance is restored within the shortest possible time;

(c)	in the event of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment, the operator must immediately suspend the operation of the activities or the relevant part of it until compliance with the permit conditions has been restored.

### 4.3.2	Any information provided under condition 4.3.1 (a)(i), or 4.3.1 (b)(i) where the information relates to the breach of a limit specified in the permit, shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that  schedule.

### 4.3.3	For the following activities referenced in schedule 1, table S1.1 (A3-A5) Natural Resources Wales shall be notified without delay following the detection of:

### any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

### the breach of a limit specified in the permit; or

### any significant adverse environmental effects.

### 4.3.4	Any information provided under condition 4.3.3 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

### 4.3.5	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.6	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

#### any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

#### any change in the operator’s name or address; and

#### any steps taken with a view to the dissolution of the operator.

#### In any other case:

#### the death of any of the named operators (where the operator consists of more than one named individual);

#### any change in the operator’s name(s) or address(es); and

#### any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.7	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

### Natural Resources Wales shall be notified at least 14 days before making the change; and

### the notification shall contain a description of the proposed change in operation.

### 4.3.8	Natural Resources Wales shall be given at least 14 days notice before implementation of any part of the site closure plan.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “immediately”, in which case it may be provided by telephone.

# Schedule 1 - Operations

| Table S1.1 activities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Activity reference | Activity listed in Schedule 1 of the EP Regulations | Description of specified activity and WFD Annex I and II operations | Limits of specified activity and waste types |  |
| A1 Open Windrow Composting – Compost ‘Pad’ | S5.4 A1(b)(i) Recovery or a mix of recovery and disposal of non-hazardous waste with a capacity exceeding 75 tonnes per day involving biological treatment. | R3: Recycling/reclamation of organic substances which are not used as solvents R13: Storage of wastes pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced) | Treatment and composting including stabilisation and sanitisation, shredding and maturation of wastes listed in table S2.2 to produce compost. The storage, physical treatment and composting of wastes under anaerobic conditions shall be prevented. Physical treatment and composting of wastes including sanitisation and stabilisation shall take place only on an impermeable surface with a sealed drainage system. Storage of green waste shall take place on hard standing with a sealed drainage system. Secure storage of wastes listed in table S2.2. |  |
| A2 Closed Vessel Composting | S5.4 A1(b) (i) Recovery or a mix of recovery and disposal of non-hazardous waste with a capacity exceeding 75 tonnes per day involving biological treatment | R3: Recycling/reclamation of organic substances which are not used as solvents R13: Storage of wastes pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced) | The storage, physical treatment and composting of wastes under anaerobic conditions shall be prevented, or where that is not practicable, minimised. The storage, physical treatment and composting of wastes shall take place on an impermeable surface with sealed drainage. Physical treatment and composting of the types of wastes listed in table S2.5. The treatment capacity of any plant shall not exceed 10 tonnes per day of animal waste. Each composting batch shall undergo an identifiable sanitisation and stabilisation stage. The sanitisation of wastes shall take place in a closed system incorporating a |  |
| A2 Closed Vessel Composting continued |  |  | bio-filter. The Stabilisation phase is not restricted to the closed vessel and may take place upon an external pad as per activity A1. The bio-filter shall be specifically designed to minimise the release of odour, bioaerosols and micro-organisms and be fit for purpose. Secure storage of wastes listed in table S2.5 |  |
| Activity reference | Description of activities for waste operations |  |  | Limits of activities |
| A3 HCI Transfer & Treatment Facility | R3: Recycling/reclamation of organic substances which are not used as solvents R4: Recycling/reclamation of metals and metal compounds R5: Recycling/reclamation of other inorganic materials R13: Storage of wastes pending the operation numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced D9: Physico-chemical treatment not specified elsewhere in Annex I which results in final compounds or mixtures which are discarded by means of any of the operations numbered D1 to D12 (e.g. evaporation, drying, calcination,etc.) D14: Repackaging prior to submission to any of the operations numbered D1 to D13 D15: Storage pending any of the operations numbered D1 to D14 (excluding temporary storage, pending collection, on the site where it is produced) |  | Treatment consisting of: - sorting - separation - shredding - compaction - dewatering of waste into different components for disposal (no more than 50 tonnes per day) or recovery. There shall be no treatment of hazardous waste. Treatment and storage of wastes shall take place on an impermeable surface with sealed drainage. No more than a total of 50 tonnes of shredded waste vehicle tyres (waste code 19 12 04) shall be stored at site. ### The maximum quantity of hazardous waste that can be stored at the site in total for recovery or disposal, shall not exceed 50 tonnes at any one time. Secure storage of wastes listed in table S2.1. |  |
| A4 Soil, soil substitutes & aggregates | R3: Recycling/reclamation of organic substances which are not used as solvents R5: Recycling/reclamation of other inorganic materials R13: Storage of wastes pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced) |  | Treatment consisting of sorting, separation, screening, shredding and blending of wastes for recovery as a soil, soil substitute or aggregate of wastes listed in table S2.3. The storage and treatment of wastes shall take place on hard standing with a sealed drainage system. |  |
| A5 Street Sweepings | R3: Recycling/reclamation of organic substances which are not used as solvents R5: Recycling/reclamation of other inorganic compounds R13: Storage of waste pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced) D9: Physico-chemical treatment not specified elsewhere in Annex I which results in final compounds or mixtures which are discarded by means of any of the operations numbered D1 to D8 and D10 to D12. D15: Storage pending any of the operations numbered D1 to D14 (excluding temporary storage, pending collection, on the site where it is produced) |  | Treatment consisting only of mechanical or biological processes for the disposal or recovery of this waste type. Physical treatment of 20 03 03 street-cleaning residues as per waste list S2.4. Wastes shall be stored and treated on an impermeable surface with a separate sealed drainage system. |  |  |
| A6 Paint blending and stripping facility | R3: Recycling/reclamation of organic substances which are not used as solvents (including composting and other biological transformation processes) R4: Recycling/reclamation of metals and metal compounds R5: Recycling/reclamation of other inorganic materials D9: Physico-chemical treatment not specified elsewhere in this Annex which results in final compounds or mixtures which are discarded by means of any of the operations numbered D1 to D12 (e.g. evaporation, drying, calcination, etc.) D14 Repackaging prior to submission to any of the operations numbered D1 to D13 R13: Storage of waste pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where the waste is produced) D15: Storage pending any of the operations numbered D1 to D14 (excluding temporary storage, pending collection, on the site where the waste is produced) |  | All hazardous and non-hazardous waste must be stored and treated within a building on an impermeable surface with sealed drainage. Treatment operations shall be limited to manual and/or mechanical: - blending; - mixing; - stripping (shot-blasting); and - repackaging of permitted waste for the purpose of recovery and/or disposal. ### The maximum quantity of hazardous waste that can be stored at the site in total for recovery or disposal, shall not exceed 50 tonnes at any one time. The maximum quantity of hazardous waste that can be treated for recovery or a mix of recovery and disposal in total at the site shall not exceed 10 tonnes per day. Hazardous waste must be stored in containers or other appropriate receptacles. |  |  |
| A6 Paint blending and stripping facility continued |  |  | Hazardous waste must be treated within a designated area, within a suitably adapted container or within an enclosed environment, with relevant environmental controls in place. The stripping treatment must be carried out within a fully enclosed, sealed booth unit. Waste types as specified in Table S2.6. |  |  |

| Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| Description | Parts | Date Received |
| How to comply with your environmental permit | All | N/A |
| The composting industry code of practice (AfOR) | All | N/A |
| Industry guide for prevention and control of odours at bio waste processing facilities (AfOR) | All | N/A |
| BSI PAS 100 Compost Specification | All | N/A |
| BSI PAS 111 Recycled Wood Quality Standard | All | N/A |
| BS 3882:2007 Top soils Standard | All | N/A |
| Quality Protocol for Compost – WRAP/EA | All | N/A |
| Health & Safety at bio waste processing sites - AfOR | All | N/A |
| Standardised protocol for the monitoring of bio aerosols at open composting facilities - AfOR | All | N/A |
| Environment Management System (EMS) | Appendix 7. Waste Acceptance Criteria Appendix 8. Procedures Appendix 9. Accident Management Plan | 06/08/12 |
| General Environmental Risk Assessment for the CWM Environmental Ltd CA Site and Composting Facility – Sections: | 2.1 Adapted Generic risk assessment Waste Operation: Non-hazardous and Hazardous Household Waste Amenity Site. | 06/08/12 |
|  | 2.2 Adapted Generic risk assessment Waste Operation: Composting in Closed Systems (in-vessel composting) vessels (including animal by-products) | 06/08/12 |
|  | 2.3 Adapted Generic risk assessment Waste Operation: External Windrow Composting | 06/08/12 |
| Odour and Emissions Management Plan | All | 06/08/12 |
| Site Specific Bioaerosol Risk Assessment | All | 06/08/12 |
| Response to regulation 61(1) Notice – request for information dated 05/04/2019 |  | 30/09/19 |
| Additional information for regulation 61(1) Notice |  | 16/01/20 |
| Fire prevention and mitigation plan guidance – Waste management | All relevant sections | N/A |
| Fire prevention and mitigation plan | All relevant sections | In line with IC 2 |
| BAT conclusions for Waste Treatment, under Directive 2010/75/EU of the European Parliament and of the Council | All relevant sections | N/A |
| Application (V006) | Environmental Permit Variation Application Supporting Statement Document reference: CWM1013_04 - NYC HWRC WTS EPVA SS Rev 1(Final) Sections: - 3.2.1 and 3.2.2- Site Operations - 3.4.3- Hazardous Waste Storage Capacity - 3.5.6 and 3.5.7 Fugitive Emissions to Air - 3.5.9 and 3.5.10 Fugitive Emissions to Surface and groundwater - 3.7.1 Site and Equipment Maintenance Plan - 3.8.1 Contingency Plans 3.9.3 Accident Prevention and Management Plan | 04/02/21 |
|  | Environmental Permit Variation Application – Amenity Risk Assessment. Document reference: App ARA1 (Final Rev 1). Sections: all | 17/02/21 |

| Table S1.3 Improvement programme requirements |  |  |
| --- | --- | --- |
| Reference | Requirement | Date |
| IC1 | Make updates to the Environment Management System (EMS) to accurately reflect the re-classification of the site to a hazardous and non-hazardous HCI Transfer & Treatment Facility including associated activities. | COMPLETED |
| IC2 | The operator shall submit an updated written Fire prevention and mitigation plan to Natural Resources Wales. The Fire prevention and mitigation plan must be produced in line with the standards set out in Fire prevention and mitigation plan guidance – Waste. | 3 months after permit issue or otherwise agreed in writing with Natural Resources Wales |
| IC3 | BAT 1 – EMS The operator must submit an updated BAT conclusions document to Natural Resources Wales including the following: - The operator shall submit evidence of ISO 14001:2015 certification to Natural Resources Wales. - The operator is required to update the Environmental Management System to include the following procedures and to submit them to Natural Resources Wales: - Management commitment - Planning and implementing procedures and targets in conjunction with financial planning and investment - Communication - Employee involvement - Independent (where practicable) internal or external EMS auditing - Senior management review of the EMS - Following the development of cleaner technologies - Regular sectoral benchmarking - Waste stream management in line with BAT 2 - Inventory of waste gas stream in line with BAT 3 - Residues management plan In accordance with BAT 1 of the Waste Treatment BAT Conclusions | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC4 | BAT 2– Waste acceptance, tracking and management The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) in order to evidence compliance with BAT 2 (of the Waste Treatment BAT Conclusions): - Waste characterisation and pre-acceptance procedures (BAT 2 a)) - Waste acceptance procedures (BAT 2 b)) - Implementation of a waste tracking system and inventory (BAT 2 c) - Output quality management system (BAT 2 d)) - Sorting of incoming waste (BAT 2 e)) | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC5 | BAT 3 – Inventory of waste gas streams In order to facilitate the reduction of emissions to water and air, BAT is to establish and to maintain an inventory of waste water and waste gas streams, as part of the environmental management system (see BAT 1), that incorporates all of the following features: - information about the characteristics of the waste to be treated and the waste treatment processes; - information about the characteristics of the waste water streams; and - information about the characteristics of the waste gas streams. - This assessment should specifically consider channelled emissions to air from the biofilter and the potential for bioaerosol releases. If bioaerosols are identified the operator will be required to monitor in line with Technical Guidance Note (Monitoring) M9 - Environmental monitoring of bioaerosols at regulated facilities. | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC6 | BAT 4 – Storage of waste The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on reducing environmental risk associated with waste storage to evidence compliance with BAT 4 (of the Waste Treatment BAT Conclusions): a) Adequate storage capacity in particular how the quantity of waste stored is monitored against capacity (BAT 4 b)) | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC7 | BAT 5 – Handling and transfer of waste The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on reducing environmental risk associated with handling and transfer of waste to evidence compliance with BAT 5 (of the Waste Treatment BAT Conclusions): - Procedure to ensure handling and transfer of wastes are duly documented, validated prior to execution and verified after execution - Procedure to ensure measures are taken to prevent, detect and mitigate spills (existing spill procedure does not provide information on how to prevent and detect spills) | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC8 | BAT 33 – Selection and suitability of waste input The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on how odour emissions are reduced and overall environmental performance is improved, by selecting the waste input to ensure its suitability for biological treatment in line with BAT 33 (of the Waste Treatment BAT Conclusions). | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC9 | BAT 35 – Emissions to water and water usage The operator must provide information to Natural Resources Wales to show compliance with BAT 35 c) detailing how the operator optimises the moisture content of the waste in order to minimises leachate generation. | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC10 | BAT 36 – Emissions to air monitoring and control of key waste and process parameters The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on how monitoring and/or controlling the following key waste and process parameters is implemented on site to reduce emissions to air and improve overall environmental performance in line with BAT 36 (of the Waste Treatment BAT Conclusions): - waste input characteristics e.g. C to N ratio, particle size - windrow porosity, height and width | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC11 | BAT 37 – Diffuse emissions to air The operator shall submit to Natural Resources Wales written procedure(s) (including updates to existing procedures) on how the following technique is implemented to reduce diffuse emissions to air of dust, odour and bioaerosols from open-air treatment steps: - adaptation of operations to meteorological conditions (BAT 37 b) (of the Waste Treatment BAT Conclusions)) in particular with reference to the direction of outdoor windrows in relation to prevailing wind direction | 17th February 2022 or otherwise agreed in writing with Natural Resources Wales |
| IC12 | The Operator shall submit a written report to Natural Resources Wales for approval detailing how the site will implement the latest version of ‘Technical Guidance Note M9 Environmental monitoring of bioaerosols at regulated facilities dated July 2018 Version 2’. This assessment should specifically consider channelled emissions to air from the biofilter and the potential for bioaerosol releases. If your biofilter (open and closed biofilters) is within 250 metres of a sensitive receptor you are required to undertake and submit a site specific bioaerosol risk assessment and any monitoring must be in line with ‘M9: environmental monitoring of bioaerosols at regulated facilities’. | 17 February 2022 or otherwise agreed in writing with Natural Resources Wales |

Schedule 2 - Waste types, raw materials and fuels

| Table S2.1 Waste types and quantities – HCI Waste Transfer Station |  |
| --- | --- |
| Maximum Quantities The total quantity of waste accepted at the site shall not exceed 100,000 tonnes a year. |  |
| Exclusions Wastes having any of the following characteristics shall not be accepted: - Consisting solely or mainly of dusts, powders or loose fibres - Malodorous wastes |  |
| Waste Code | Description |
| 01 | WASTES RESULTING FROM EXPLORATION, MINING, QUARRYING AND PHYSICAL AND CHEMICAL TREATMENT OF MINERALS |
| 01 01 | wastes from mineral excavation |
| 01 01 01 | wastes from mineral metalliferous excavation |
| 01 01 02 | wastes from mineral non-metalliferous excavation |
| 01 04 | wastes from physical and chemical processing of non-metalliferous minerals |
| 01 04 08 | waste gravel and crushed rocks other than those mentioned in 01 04 07 |
| 01 04 09 | waste sand and clays |
| 02 | WASTES FROM AGRICULTURE, HORTICULTURE, AQUACULTURE, FORESTRY, HUNTING AND FISHING, FOOD PREPARATION AND PROCESSING |
| 02 01 | wastes from agriculture, horticulture, aquaculture, forestry, hunting and fishing |
| 02 01 01 | sludges from washing and cleaning |
| 02 01 03 | plant-tissue waste |
| 02 01 07 | wastes from forestry |
| 02 02 | wastes from the preparation and processing of meat, fish and other foods of animal origin |
| 02 02 02 | animal tissue waste (shells for soils only) |
| 02 04 | wastes from sugar processing |
| 02 04 01 | soil from cleaning and washing beet |
| 02 04 02 | off-specification calcium carbonate |
| 03 | WASTES FROM WOOD PROCESSING AND THE PRODUCTION OF PANELS AND FURNITURE, PULP, PAPER AND CARDBOARD |
| 03 01 | wastes from wood processing and the production of panels and furniture |
| 03 01 01 | waste bark and cork |
| 03 01 05 | sawdust, shavings, cuttings, wood, particle board and veneer other than those mentioned in 03 01 04 |
| 03 03 | wastes from pulp, paper and cardboard production and processing |
| 03 03 01 | waste bark and wood |
| 03 03 05 | de-inking sludges from paper recycling |
| 03 03 07 | mechanically separated rejects from pulping of waste paper and cardboard |
| 03 03 08 | wastes from sorting of paper and cardboard destined for recycling |
| 03 03 09 | lime mud waste |
| 03 03 10 | fibre rejects, fibre-, filter- and coating sludges from mechanical separation |
| 03 03 11 | sludges from on-site effluent treatment other than those mentioned in 03 03 10 |
| 04 | WASTES FROM THE LEATHER, FUR AND TEXTILE INDUSTRIES |
| 04 02 | wastes from the textile industry |
| 04 02 10 | organic matter from natural products (for example grease, wax) |
| 06 | WASTES FROM INORGANIC CHEMICAL PROCESSES |
| 06 09 | Wastes from the MFSU of phosphorous chemicals and phosphorous chemical processes |
| 06 09 02 | phosphorous slag |
| 06 09 04 | calcium-based reaction wastes other than those mentioned in 06 09 03 |
| 10 | WASTES FROM THERMAL PROCESSES |
| 10 01 | waste from power stations and other combustion plants |
| 10 01 01 | bottom ash and slag only |
| 10 01 02 | coal fly ash only |
| 10 01 05 | calcium-based reaction wastes from flue-gas desulphurisation in solid form |
| 10 01 07 | calcium-based reaction wastes from flue-gas desulphurisation in sludge form |
| 10 01 15 | bottom ash, slag and boiler dust from co-incineration other than those mentioned in 10 01 14 |
| 10 11 | wastes from manufacture of glass and glass products |
| 10 11 12 | clean glass other than those mentioned in 10 11 11 |
| 10 12 | wastes from manufacture of ceramic goods, brick, stiles and construction products |
| 10 12 08 | waste ceramics, bricks, tiles and construction products (after thermal processing) |
| 10 13 | wastes from manufacture of cement, lime and plaster products and articles and products made from them |
| 10 13 14 | waste concrete only |
| 13 | OIL WASTES AND WASTES OF LIQUID FUELS (except edible oils and those in chapters 05, 12 and 19) |
| 13 02 | waste engine, gear and lubricating oils |
| 13 02 05* | mineral based non-chlorinated engine, gear and lubricating oils |
| 13 02 06* | synthetic engine, gear and lubricating oils |
| 13 02 07* | readily biodegradable engine, gear and lubricating oils |
| 13 02 08* | other engine, gear and lubricating oils (from householders only) |
| 15 | WASTE PACKAGING; ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | packaging (including separately collected municipal packaging waste) |
| 15 01 01 | paper and cardboard packaging |
| 15 01 02 | plastic packaging |
| 15 01 03 | wooden packaging |
| 15 01 04 | metallic packaging |
| 15 01 05 | composite packaging (only biodegradable organic packaging) |
| 15 01 06 | mixed packaging |
| 15 01 07 | glass packaging |
| 15 01 09 | textile packaging |
| 15 01 10* | packaging containing residues of or contaminated by hazardous substances |
| 15 01 11* | metallic packaging containing a dangerous solid porous matrix (for example asbestos), including empty pressure containers |
| 15 02 | absorbents, filter materials, wiping cloths and protective clothing |
| 15 02 02* | absorbents, filter materials (including oil filters not otherwise specified), wiping cloths, protective clothing contaminated by dangerous substances |
| 15 02 03 | absorbents, filter materials, wiping cloths and protective clothing other than those mentioned in 15 02 02 |
| 16 | WASTES NOT OTHERWISE SPECIFIED IN THE LIST |
| 16 02 | wastes from electrical and electronic equipment |
| 16 02 12* | discarded equipment containing free asbestos |
| 16 02 14 | discarded equipment other than those mentioned in 16 02 09 to 16 02 13 |
| 16 05 | gases in pressure containers and discarded chemicals |
| 16 05 04* | gases in pressure containers (including halons) containing dangerous substances |
| 16 05 05 | gases in pressure containers other than those mentioned in 16 05 04 |
| 16 06 | batteries and accumulators |
| 16 06 01* | lead batteries |
| 16 06 02* | ni-Cd batteries |
| 16 06 03* | mercury-containing batteries |
| 16 06 04 | alkaline batteries (except 16 06 03) |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 01 | concrete, bricks, tiles and ceramics |
| 17 01 01 | concrete |
| 17 01 02 | bricks |
| 17 01 03 | tiles and ceramics |
| 17 01 07 | mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 06 |
| 17 02 | Wood, glass and plastic |
| 17 02 01 | wood |
| 17 02 02 | glass |
| 17 02 03 | plastic |
| 17 03 | bituminous mixtures, coal tar and tarred products |
| 17 03 01* | bituminous mixtures containing coal tar |
| 17 03 02 | bituminous mixtures other than those mentioned in 17 03 01 |
| 17 03 03* | coal tar and tarred products |
| 17 04 | metals (including their alloys) |
| 17 04 01 | copper, bronze, brass |
| 17 04 02 | aluminium |
| 17 04 05 | iron and steel |
| 17 04 07 | mixed metals |
| 17 05 | soil (including excavated soil from contaminated sites), stones and dredging spoil |
| 17 05 04 | soil and stones other than those mentioned in 17 05 03 |
| 17 05 06 | dredging spoil other than those mentioned in 17 05 05 |
| 17 05 08 | track ballast other than those mentioned in 17 05 07 |
| 17 06 | insulation materials and asbestos-containing construction materials |
| 17 06 01* | insulation materials containing asbestos |
| 17 06 04 | insulation materials other than those mentioned in 17 06 01 and 17 06 03 |
| 17 06 05* | construction materials containing asbestos |
| 17 08 | gypsum-based construction material |
| 17 08 02 | gypsum-based construction materials other than those mentioned in 17 08 01 |
| 17 09 | other construction and demolition wastes |
| 17 09 03* | other construction and demolition wastes (including mixed wastes) containing dangerous substances |
| 17 09 04 | mixed construction and demolition wastes other than those mentioned in 17 09 01, 17 09 02 and 17 09 03 |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION/INDUSTRIAL USE |
| 19 05 | wastes from the aerobic treatment of solid wastes |
| 19 05 01 | non-composted fraction of municipal and similar wastes |
| 19 05 02 | non-composted fraction of animal and vegetable wastes |
| 19 05 03 | off-specification compost |
| 19 06 | wastes from anaerobic treatment of waste |
| 19 06 03 | liquor from anaerobic treatment of municipal waste |
| 19 06 04 | digestate from anaerobic treatment of municipal waste |
| 19 06 05 | liquor from anaerobic treatment of animal and vegetable waste |
| 19 06 06 | digestate from anaerobic treatment of animal and vegetable waste |
| 19 08 | wastes from waste water treatment plants not otherwise specified |
| 19 08 02 | washed sewage grit (waste from desanding) free from sewage contamination only |
| 19 08 05 | sludges from treatment of urban waste water |
| 19 08 12 | sludges from biological treatment of industrial waste water other than those mentioned in 19 08 11 |
| 19 08 14 | sludges from other treatment of industrial waste water other than those mentioned in 19 08 13 |
| 19 08 99 | wastes not otherwise specified (stone filter media) |
| 19 09 | wastes from the preparation of water intended for human consumption or water for industrial use |
| 19 09 01 | solid waste from primary filtration and screenings |
| 19 09 02 | sludges from water clarification |
| 19 09 03 | sludges from decarbonation |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 01 | paper and cardboard |
| 19 12 05 | clean glass only |
| 19 12 07 | wood other than that mentioned in 19 12 06 |
| 19 12 09 | minerals (for example sand, stones) |
| 19 12 12 | other wastes (including mixtures of materials) from mechanical treatment of wastes other than those mentioned in 19 12 11 |
| 19 13 | wastes from soil and groundwater remediation |
| 19 13 02 | solid wastes from soil remediation other than those mentioned in 19 13 01 |
| 19 13 04 | sludges from soil remediation other than those mentioned in 19 13 03 |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | separately collected fractions (except 15 01) |
| 20 01 01 | paper and cardboard |
| 20 01 02 | glass |
| 20 01 08 | biodegradable kitchen and canteen waste |
| 20 01 10 | clothes |
| 20 01 11 | textiles |
| 20 01 13* | solvents |
| 20 01 14* | acids |
| 20 01 15* | alkalines |
| 20 01 17* | photochemicals |
| 20 01 19* | pesticides |
| 20 01 21* | fluorescent tubes and other mercury-containing waste |
| 20 01 23* | discarded equipment containing chlorofluorocarbons |
| 20 01 25 | edible oil and fat |
| 20 01 26* | oil and fat other than those mentioned in 20 01 25 |
| 20 01 27* | paint, inks, adhesives and resins containing dangerous substances |
| 20 01 28 | paint, inks, adhesives and resins other than those mentioned in 20 01 27 |
| 20 01 29* | detergents containing dangerous substances |
| 20 01 30 | detergents other than those mentioned in 20 01 29 |
| 20 01 33* | batteries and accumulators included in 16 06 01, 16 06 02 or 16 06 03 and unsorted batteries and accumulators containing these batteries |
| 20 01 34 | batteries and accumulators other than those mentioned in 20 01 33 |
| 20 01 35* | discarded electrical and electronic equipment other than those mentioned in 20 01 21 and 20 01 23 containing hazardous components |
| 20 01 36 | discarded electrical and electronic equipment other than those mentioned in 20 01 21 and 20 01 23 containing hazardous components |
| 20 01 37* | wood containing dangerous substances |
| 20 01 38 | wood other than that mentioned in 20 01 37 (where no non-biodegradable coating or preserving substance present) |
| 20 01 39 | plastics (only if independently certified to European standard EN 14995) |
| 20 01 40 | metals |
| 20 01 41 | wastes from chimney sweeping |
| 20 02 | garden and park wastes (including cemetery waste) |
| 20 02 01 | biodegradable waste |
| 20 02 02 | soil and stones |
| 20 02 03 | other non-biodegradable wastes |
| 20 03 | other municipal wastes |
| 20 03 01 | mixed municipal waste |
| 20 03 02 | waste from markets |
| 20 03 03 | street-cleaning residues |
| 20 03 07 | bulky waste |

| Table S2.2 Waste types and quantities – Open windrow composting ‘PAD’ |  |
| --- | --- |
| Maximum Quantities The total quantity of waste accepted at the site shall not exceed 100,000 tonnes a year. |  |
| Exclusions Wastes having any of the following characteristics shall not be accepted: - Consisting solely or mainly of dusts, powders or loose fibres - Hazardous wastes |  |
| Waste Code | Description |
| 02 | WASTES FROM AGRICULTURE, HORTICULTURE, AQUACULTURE, FORESTRY, HUNTING AND FISHING, FOOD PREPARATION AND PROCESSING |
| 02 01 | wastes from agriculture, horticulture, aquaculture, forestry, hunting and fishing |
| 02 01 03 | plant-tissue waste |
| 02 01 04 | waste plastics (BIODEGRADABLE packaging only) |
| 02 01 06 | animal faeces, urine and manure (including spoiled straw), effluent, collected separately and treated off-site |
| 02 01 07 | wastes from forestry – only if plant material |
| 02 01 99 | wastes not otherwise specified (mushroom compost only) |
| 02 02 | wastes from the preparation and processing of meat, fish and other foods of animal origin |
| 02 02 99 | horse manure, farmyard manure and bedding |
| 02 03 | wastes from fruit, vegetables, cereals, edible oils, cocoa, coffee, tea and tobacco preparation and processing; conserve production; yeast and yeast extract production, molasses preparation and fermentation |
| 02 03 04 | biodegradable materials unsuitable for consumption or processing (biodegradable only) |
| 02 04 | Wastes from sugar processing |
| 02 04 01 | soil from cleaning and washing beet |
| 02 04 02 | off-specification calcium carbonate (for blending at the end of the composting process only) |
| 02 06 | wastes from the baking and confectionery industry |
| 02 06 01 | materials unsuitable for consumption or processing |
| 02 07 | wastes from the production of alcoholic and non-alcoholic beverages (except coffee, tea and cocoa) |
| 02 07 01 | wastes from washing, cleaning and mechanical reduction of raw materials |
| 02 07 02 | wastes from spirits distillation |
| 02 07 04 | materials unsuitable for consumption or processing |
| 02 07 99 | wastes not otherwise specified (malt husks, malt sprouts, yeast and yeast-like residues only) |
| 03 | WASTES FROM WOOD PROCESSING AND THE PRODUCTION OF PANELS AND FURNITURE, PULP, PAPER AND CARDBOARD |
| 03 01 | wastes from wood processing and the production of panels and furniture |
| 03 01 01 | waste bark and cork |
| 03 01 05 | sawdust, shavings, wood, particle board and veneer other than those containing dangerous substances other than 03 01 04. |
| 03 03 | wastes from pulp, paper and cardboard production and processing |
| 03 03 01 | waste bark and wood |
| 03 03 05 | de-inking sludge from paper recycling |
| 03 03 07 | mechanically separated rejects from pulping of waste paper and cardboard |
| 03 03 09 | lime mud waste (for blending at the end of the composting process only) |
| 03 03 10 | fibre rejects only (not containing hazardous substances) |
| 04 | WASTES FROM THE LEATHER, FUR AND TEXTILE INDUSTRIES |
| 04 02 | wastes from the leather and fur industry |
| 04 02 10 | organic matter from natural products (un-dyed and untreated, for example grease, wax) |
| 15 | WASTE PACKAGING; ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | packaging (including separately collected municipal packaging waste) |
| 15 01 01 | paper and cardboard packaging (not containing non-biodegradable veneers or plastics) |
| 15 01 03 | wooden packaging (untreated) |
| 15 01 05 | composite packaging (only biodegradable organic packaging) |
| 15 01 09 | textile packaging (only entirely natural fibres) |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 02 | wood, glass and plastic |
| 17 02 01 | wood (only entirely natural fibres) |
| 17 05 | soil (including excavated soil from contaminated sites), stones and dredging spoil |
| 17 05 06 | dewatered dredging spoil and plant tissue waste from inland waters, not containing Japanese Knotweed and not containing dangerous substances' |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND THE PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION AND WATER FOR INDUSTRIAL USE |
| 19 05 | wastes from aerobic treatment of solid wastes |
| 19 05 03 | off-specification compost |
| 19 06 | wastes from anaerobic treatment of wastes |
| 19 06 04 | digestate from anaerobic treatment of municipal wastes |
| 19 08 | Wastes from waste water treatment plants not otherwise specified |
| 19 08 05 | sludges from treatment of urban waste water |
| 19 08 12 | sludges from biological treatment of industrial waste water other than those mentioned in 19 08 11 |
| 19 08 14 | sludges from other treatment of industrial waste water other than those mentioned in 19 08 13 |
| 19 09 | wastes from the preparation of water intended for human consumption or water for industrial use |
| 19 09 01 | solid waste from primary filtration and screenings |
| 19 09 02 | sludges from water clarification (organic waste only) |
| 19 09 03 | sludges from decarbonation (organic waste only) |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 01 | paper and cardboard not containing non-biodegradable veneers or plastics |
| 19 12 07 | wood other than that mentioned in 19 12 06 |
| 19 12 12 | other wastes (including mixtures of materials) from mechanical treatment of wastes other than those mentioned in 19 12 11 |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | Separately collected fractions (except 15 01) |
| 20 01 01 | paper and cardboard |
| 20 01 38 | wood other than that mentioned in 20 01 37 |
| 20 02 | garden and park wastes (including cemetery waste) |
| 20 02 01 | biodegradable waste (comprising wood and plant tissue only) |
| 20 02 02 | soil and stones (for blending at the end of the composting process) |
| 20 03 | other municipal wastes |
| 20 03 02 | waste from markets (biodegradable only) |

| Table S2.3 Permitted waste types and quantities for the production of soil, soil substitutes and aggregates |  |
| --- | --- |
| Exclusions Wastes having any of the following characteristics shall not be accepted: - Consisting solely or mainly of dusts, powders or loose fibres - Hazardous wastes |  |
| Waste code | Description |
| 01 | WASTES RESULTING FROM EXPLORATION, MINING, QUARRYING AND PHYSICAL AND CHEMICAL TREATMENT OF MINERALS |
| 01 04 | wastes from physical and chemical processing of non-metalliferous minerals |
| 01 04 08 | waste gravel and crushed rocks other than those mentioned in 01 04 07 |
| 01 04 09 | waste sand and clays |
| 02 | WASTES FROM AGRICULTURE, HORTICULTURE, AQUACULTURE, FORESTRY, HUNTING AND FISHING, FOOD PREPARATION AND PROCESSING |
| 02 02 | wastes from the preparation and processing of meat, fish and other foods of animal origin |
| 02 02 02 | animal tissue waste (shells for soils only) |
| 03 | WASTES FROM WOOD PROCESSING AND THE PRODUCTION OF PANELS AND FURNITURE, PULP, PAPER AND CARDBOARD |
| 03 01 | wastes from wood processing and the production of panels and furniture |
| 03 01 01 | waste bark and cork |
| 03 03 | wastes from pulp, paper and cardboard production and processing |
| 03 03 01 | waste bark and wood |
| 10 | WASTES FROM THERMAL PROCESSES |
| 10 01 | waste from power stations and other combustion plants |
| 10 01 01 | bottom ash and slag only |
| 10 01 02 | coal fly ash only |
| 10 01 05 | calcium-based reaction wastes from flue-gas desulphurisation in solid form |
| 10 01 07 | calcium-based reaction wastes from flue-gas desulphurisation in sludge form |
| 10 01 15 | bottom ash, slag and boiler dust from co-incineration other than those mentioned in 10 01 14 |
| 10 11 | wastes from manufacture of glass and glass products |
| 10 11 12 | clean glass other than those mentioned in 10 11 11 |
| 10 12 | wastes from manufacture of ceramic goods, brick, stiles and construction products |
| 10 12 08 | waste ceramics, bricks, tiles and construction products(after thermal processing) |
| 10 13 | wastes from manufacture of cement, lime and plaster products and articles and products made from them |
| 10 13 14 | waste concrete only |
| 15 | WASTE PACKAGING; ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | packaging (including separately collected municipal packaging waste) |
| 15 01 07 | glass packaging |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 01 | concrete, bricks, tiles and ceramics |
| 17 01 01 | concrete |
| 17 01 02 | bricks |
| 17 01 03 | tiles and ceramics |
| 17 01 07 | mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 06 |
| 17 02 | wood, glass and plastic |
| 17 02 02 | glass |
| 17 03 | bituminous mixtures, coal tar and tarred products |
| 17 03 02 | bituminous mixtures other than those mentioned in 17 03 01 |
| 17 05 | soil (including excavated soil from contaminated sites), stones and dredging spoil |
| 17 05 04 | soil and stones other than those mentioned in 17 05 03 |
| 17 05 06 | dredging spoil other than those mentioned in 17 05 05 |
| 17 05 08 | track ballast other than those mentioned in 17 05 07 |
| 17 06 | insulation materials and asbestos-containing construction materials |
| 17 06 04 | insulation materials other than those mentioned in 17 06 01 and 17 06 03 |
| 17 08 | gypsum-based construction material |
| 17 08 02 | gypsum-based construction materials other than those mentioned in 17 08 01 |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION/INDUSTRIAL USE |
| 19 05 | wastes from the aerobic treatment of solid wastes |
| 19 05 03 | off-specification compost |
| 19 08 | wastes from waste water treatment plants not otherwise specified |
| 19 08 02 | washed sewage grit (waste from desanding) free from sewage contamination only |
| 19 08 99 | wastes not otherwise specified (stone filter media) |
| 19 09 | wastes from the preparation of water intended for human consumption or water for industrial use |
| 19 09 02 | sludges from water clarification |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 05 | clean glass only |
| 19 12 09 | minerals (for example sand, stones) |
| 19 12 12 | other wastes (including mixtures of materials) from mechanical treatment of wastes other than those mentioned in 19 12 11 |
| 19 13 | wastes from soil and groundwater remediation |
| 19 13 02 | solid wastes from soil remediation other than those mentioned in 19 13 01 |
| 19 13 04 | sludges from soil remediation other than those mentioned in 19 13 03 |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | separately collected fractions (except 15 01) |
| 20 01 02 | glass |
| 20 02 | garden and park wastes (including cemetery waste) |
| 20 02 02 | soil and stones |

| Table S2.4 Permitted waste types and quantities for the processing street-cleaning residues |  |
| --- | --- |
| Exclusions Wastes having any of the following characteristics shall not be accepted: - Consisting solely or mainly of dusts, powders or loose fibres - Hazardous wastes |  |
| Waste code | Description |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 03 | other municipal wastes |
| 20 03 03 | street-cleaning residues |

| Table S2.5 Waste types and quantities for closed vessel composting |  |
| --- | --- |
| Maximum Quantities The total quantity of waste accepted at the site shall be less than 100,000 tonnes a year. |  |
| Exclusions Wastes having any of the following characteristics shall not be accepted: - Consisting solely or mainly of dusts, powders or loose fibres - Hazardous wastes |  |
| Waste Code | Description |
| 02 | WASTES FROM AGRICULTURE, HORTICULTURE, AQUACULTURE, FORESTRY, HUNTING AND FISHING, FOOD PREPARATION AND PROCESSING |
| 02 01 | wastes from agriculture, horticulture, aquaculture, forestry, hunting and fishing |
| 02 01 01 | sludges from washing and cleaning (food preparation and processing waste, food washing only) |
| 02 01 02 | animal tissue waste |
| 02 01 03 | plant-tissue waste |
| 02 01 06 | animal faeces, urine and manure (including spoiled straw) |
| 02 01 07 | wastes from forestry (comprising wood and plant tissue) |
| 02 01 99 | wastes not otherwise specified (mushroom compost only) |
| 02 02 | wastes from the preparation and processing of meat, fish and other foods of animal origin |
| 02 02 01 | sludges from washing and cleaning (process water and food washing waste only) |
| 02 02 02 | animal tissue waste |
| 02 02 03 | materials unsuitable for consumption or processing |
| 02 02 99 | wastes not otherwise specified (horse manure, farmyard manure and bedding only) |
| 02 03 | wastes from fruit, vegetables, cereals, edible oils, cocoa, coffee, tea and tobacco preparation and processing; conserve production; yeast and yeast extract production, molasses preparation and fermentation |
| 02 03 01 | sludges from washing, cleaning, peeling, centrifuging and separation |
| 02 03 04 | biodegradable materials unsuitable for consumption or processing (other than those containing dangerous substances) |
| 02 03 05 | sludges from on-site effluent treatment |
| 02 05 | wastes from the dairy products industry |
| 02 05 01 | biodegradable materials unsuitable for consumption or processing (other than those containing dangerous substances) |
| 02 06 | wastes from the baking and confectionery industry |
| 02 06 01 | biodegradable materials unsuitable for consumption or processing (other than those containing dangerous substances) |
| 02 07 | wastes from the production of alcoholic and non-alcoholic beverages (except coffee, tea and cocoa) |
| 02 07 01 | spent grains or hops. |
| 02 07 02 | whisky filter sheets or cloths. |
| 02 07 04 | material unsuitable for consumption or processing |
| 02 07 99 | Wastes not otherwise specified (malt husks, malt sprouts, spent grains, sludges from the production process, yeast and yeast-like residues only) |
| 03 | WASTES FROM WOOD PROCESSING AND THE PRODUCTION OF PANELS AND FURNITURE, PULP, PAPER AND CARDBOARD |
| 03 01 | wastes from wood processing and the production of panels and furniture |
| 03 01 01 | waste bark and cork |
| 03 01 05 | Sawdust, shavings, wood, particle board and veneer other than those containing dangerous substances other than 03 01 04. Untreated wood only. NO VENEERS OR PRESERVATIVES |
| 03 03 | wastes from pulp, paper and cardboard production and processing |
| 03 03 01 | waste bark and wood |
| 03 03 10 | fibre rejects (not containing hazardous substances) |
| 04 | WASTES FROM THE LEATHER FUR AND TEXTILE INDUSTRIES |
| 04 02 | Waste from the textile industry |
| 04 02 10 | organic matter from natural products (from example grease, wax) |
| 15 | WASTE PACKAGING; ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | packaging (including separately collected municipal packaging waste) |
| 15 01 01 | paper and cardboard packaging not containing veneers or plastic coated. |
| 15 01 03 | wooden packaging (untreated) |
| 15 01 05 | composite packaging (only biodegradable organic packaging) |
| 15 01 09 | textile packaging (only entirely natural fibres) |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 02 | wood, glass and plastic |
| 17 02 01 | wood (untreated) |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION/INDUSTRIAL USE |
| 19 05 | wastes from the aerobic treatment of solid wastes |
| 19 05 03 | off-specification compost (only if the compost is derived from input types allowed by the QP requirements) |
| 19 06 | waste from the anaerobic treatment of waste |
| 19 06 03 | liquor from anaerobic treatment of municipal waste |
| 19 06 04 | digestate from anaerobic treatment of municipal waste |
| 19 06 05 | liquor from anaerobic treatment of animal and vegetable waste |
| 19 06 06 | digestate from anaerobic treatment of animal and vegetable waste |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 01 | paper and cardboard |
| 19 12 07 | wood other than that mentioned in 19 12 06 |
| 19 12 12 | other wastes (including mixtures of materials) from mechanical treatment of wastes other than those mentioned in 19 12 11 |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | separately collected fractions (except 15 01) |
| 20 01 01 | paper and cardboard |
| 20 01 08 | biodegradable kitchen and canteen waste |
| 20 01 25 | edible oil and fat |
| 20 01 38 | wood other than that mentioned in 20 01 37 (where no non-biodegradable coating or preserving substance present) |
| 20 02 | garden and park wastes (including cemetery waste) |
| 20 02 01 | biodegradable waste (comprising wood and plant tissue) |
| 20 03 | other municipal wastes |
| 20 03 01 | mixed municipal waste (must be separately collected biodegradable wastes allowed under the Quality Protocol for compost only) |
| 20 03 02 | biodegradable waste from markets |

| Table S2.6 Waste types and quantities for paint blending and stripping facility |  |
| --- | --- |
| Maximum quantity The maximum quantity of waste to be accepted on site shall not exceed 100,000 tonnes per year. |  |
| Exclusions Notwithstanding the waste types set out in this table, wastes having any of the following characteristics shall not be accepted: - consisting solely or mainly of dusts, powders or loose fibres - sludges - odorous or odour producing |  |
| Waste code | Description |
| 08 | WASTES FROM THE MANUFACTURE, FORMULATION, SUPPLY AND USE (MFSU) OF COATINGS (PAINTS, VARNISHES AND VITREOUS ENAMELS), ADHESIVES, SEALANTS AND PRINTING INKS |
| 08 01 | wastes from MFSU and removal of paint and varnish |
| 08 01 11* | waste paint and varnish containing organic solvents or other hazardous substances |
| 08 01 12 | waste paint and varnish other than those mentioned in 08 01 11 |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | separately collected fractions (except 15 01) |
| 20 01 27* | paint, inks, adhesives and resins containing hazardous substances |
| 20 01 28 | paint, inks, adhesives and resins other than those mentioned in 20 01 27 |
| 20 01 37* | wood containing hazardous substances (consisting of painted wooden furniture or doors only) |
| 20 01 38 | wood other than that mentioned in 20 01 37 (consisting of painted wooden furniture or doors only) |
| 20 01 40 | metals (consisting of painted metal furniture or doors only) |

Schedule 3(a) – Emissions and monitoring until 16th August 2022

| Table S3.1a Point source emissions to air – emission limits and monitoring requirements up to 16th August 2022 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. & location | Source | Parameter | Limit (including unit) | Reference period | Monitoring frequency | Monitoring standard or method |  |
| A1 east side of the IVC building as shown on Figure 3 The IVC Facility and Yard Area in the Odour & Emissions Management Plan July 2018 | Biofilter | None | No Limits | - | - | - |

| Table S3.2 Point Source emissions to water (other than sewer) and land – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. & location | Parameter | Source | Limit (incl. unit) | Reference Period | Monitoring frequency | Monitoring standard or method |
| Rainwater effluent lagoon located on the external pad area | No parameters set | Surface water from external pad area | None specified | Every inspection | Daily | Visual inspection |
| Interceptor East | No parameters set | Surface water from external yard | None specified | Every inspection | As required | Visual inspection |
| Interceptor West | No parameters set | Surface water from CA site yard | None specified | Every inspection | As required | Visual inspection |

| Table S3.3 Point source emissions to sewer, effluent treatment plant or other transfers off-site– emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. & location | Parameter | Source | Limit (incl. Unit) | Reference period | Monitoring frequency | Monitoring standard or method |
| IVC Effluent tanks | No parameters set | Site effluent holding tanks and bunded area | None specified | Every inspection | As required | Visual inspection |
| HCI WTS bunded waste oil tank area | No parameters set | Holding tank & overspill | None specified | Every inspection | As required | Visual inspection |

| Table S3.4 Surface water monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location or description of point of measurement | Parameter | Monitoring frequency | Monitoring standard or method | Other specifications |  |
| The watercourse adjacent to the site which flows south before joining the Afon-y-Bantwen | Any visible oil and grease | Weekly or as agreed in writing with Natural Resources Wales | Visual inspections |  | - |  |

| Table S 3.5 Process monitoring requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Monitoring point | Substance or parameter | Monitoring frequency | Monitoring method | Other specifications |
| Internal for each enclosed composting vessel | Temperature | Continuous | Thermocouple probe | None |
|  | Moisture | As required | Moisture meter or touch test | None |
| Internal for each windrow and for any sample of waste or compost | Temperature | As required | Thermocouple probe | Monitoring shall be carried out to ensure aerobic conditions are maintained and to prevent waste becoming anaerobic |
|  | Moisture | As required | Moisture meter or moisture touch test |  |
| Biofilter | Temperature, moisture, thatching and compaction | As required | Thermocouple probe and moisture meter or touch test | Biofilters should be regularly checked and maintained to ensure appropriate temperature and moisture content. |

| ### Table S3.6 Bioaerosol monitoring requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ### Location or description of point of measurement | ### Parameter | ### Bioaerosol threshold limits CFU m-3 | ### Monitoring frequency | ### Monitoring standard or method | ### Other specifications |
| ### Bioaerosols shall be monitored: ### - upwind of the site and ### - downwind of the site; ### in accordance with” M9 – Technical Guidance Note (Monitoring) – Environmental monitoring of bioaerosols at regulated facilities, 2018 Version 2”. | ### Total bacteria | ### 1000 | ### Quarterly or other frequency as agreed in writing with Natural Resources Wales. | ### In accordance with ”M9 – Technical Guidance Note (Monitoring) – Environmental monitoring of bioaerosols at regulated facilities, 2018 Version 2” together with the Environment Agency’s “Guidance on the evaluation of bioaerosol risk assessments for composting facilities | ### As described M9 Technical Guidance Note, including all the additional data requirements specified therein. |
|  | ### Aspergillus Fumigatus | ### 500 |  |  |  |

Schedule 3(b) – Emissions and monitoring from 17th August 2022

| Table S3.1 Point source emissions to air – emission limits and monitoring requirements from 17th August 2022 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. & location | Source | Parameter | Limit (including unit) | Reference period | Monitoring frequency | Monitoring standard or method |  |
| A1 east side of the IVC building as shown on Figure 3 The IVC Facility and Yard Area in the Odour & Emissions Management Plan July 2018 in Schedule 7 | Biofilter | Hydrogen Sulphide Ammonia | No limit 20 mg/Nm3 |  | Once every six months [Note1] Once every six months [Note 1] | No EN Standard available |

Note 1: Monitoring requirements for the biofilter are only applicable when the IVC reception building is operational in each six monthly period running from January to June as period 1 and July to December as period 2.

| Table S3.2 Point Source emissions to water (other than sewer) and land – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. & location | Parameter | Source | Limit (incl. unit) | Reference Period | Monitoring frequency | Monitoring standard or method |
| Rainwater effluent lagoon located on the external pad area | No parameters set | Surface water from external pad area | None specified | Every inspection | Daily | Visual inspection |
| Interceptor East | No parameters set | Surface water from external yard | None specified | Every inspection | As required | Visual inspection |
| Interceptor West | No parameters set | Surface water from CA site yard | None specified | Every inspection | As required | Visual inspection |

| Table S3.3 Point source emissions to sewer, effluent treatment plant or other transfers off-site– emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. & location | Parameter | Source | Limit (incl. Unit) | Reference period | Monitoring frequency | Monitoring standard or method |
| IVC Effluent tanks | No parameters set | Site effluent holding tanks and bunded area | None specified | Every inspection | As required | Visual inspection |
| HCI WTS bunded waste oil tank area | No parameters set | Holding tank & overspill | None specified | Every inspection | As required | Visual inspection |

| Table S3.4 Surface water monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location or description of point of measurement | Parameter | Monitoring frequency | Monitoring standard or method | Other specifications |  |
| The watercourse adjacent to the site which flows south before joining the Afon-y-Bantwen | Any visible oil and grease | Weekly or as agreed in writing with Natural Resources Wales | Visual inspections |  | - |  |

| Table S 3.5 Process monitoring requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Monitoring point | Substance or parameter | Monitoring frequency | Monitoring method | Other specifications |
| Internal for each enclosed composting vessel | Temperature | Continuous | Thermocouple probe | None |
|  | Moisture | As required | Moisture meter or touch test | None |
| Internal for each windrow and for any sample of waste or compost | Temperature | As required | Thermocouple probe | Monitoring shall be carried out to ensure aerobic conditions are maintained and to prevent waste becoming anaerobic |
|  | Moisture | As required | Moisture meter or moisture touch test |  |
| Biofilter | Temperature, moisture, thatching and compaction | As required | Thermocouple probe and moisture meter or touch test | Biofilters should be regularly checked and maintained to ensure appropriate temperature and moisture content. |

| ### Table S3.6 Bioaerosol monitoring requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ### Location or description of point of measurement | ### Parameter | ### Bioaerosol threshold limits CFU m-3 | ### Monitoring frequency | ### Monitoring standard or method | ### Other specifications |
| ### Bioaerosols shall be monitored: ### - upwind of the site and ### - downwind of the site; ### in accordance with” M9 – Technical Guidance Note (Monitoring) – Environmental monitoring of bioaerosols at regulated facilities, 2018 Version 2”. | ### Total bacteria | ### 1000 | ### Quarterly | ### In accordance with ”M9 – Technical Guidance Note (Monitoring) – Environmental monitoring of bioaerosols at regulated facilities. | ### As described M9 Technical Guidance Note, including all the additional data requirements specified therein. |
|  | ### Aspergillus Fumigatus | ### 500 |  |  |  |

Schedule 4 (a) – Reporting effective until 16 August 2022 Schedule 4 - Reporting

Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

| Table S4.1 Reporting of monitoring data |  |  |  |
| --- | --- | --- | --- |
| Parameter | Emission or monitoring point/reference | Reporting period | Period begins |
| Emissions to air Parameters as required by condition 3.5.1. | A1 | Every 6 months | 1 January, 1 July |
| Surface water monitoring Parameters as required by condition 3.5.1 | The watercourse adjacent to the site which flows south before joining the Afon-y-Bantwen | Every 3 months | 1 January, 1 April, 1 July, 1 October |
| Bioaerosol emissions as required by condition 3.5.1 | Upwind of the site and downwind of the site | Quarterly or as agreed in writing by the Natural Resources Wales | 1 January, 1 April, 1 July, 1 October |

| Table S4.2: Annual production/treatment |  |
| --- | --- |
| Parameter | Units |
| Compost | tonnes |
| Total amount of waste treated | tonnes |

| Table S4.3 Performance parameters |  |  |
| --- | --- | --- |
| Parameter | Frequency of assessment | Units |
| Water usage | Annually | tonnes |
| Energy usage | Annually | MWh |
| Total raw material used | Annually | tonnes |
| Generation of waste water | Annually | m3 |
| Generation of residue | Annually | tonnes |

| Table S4.4 Reporting forms |  |  |
| --- | --- | --- |
| Media/parameter | Reporting format | Date of form |
| Air | Form air 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Water and Land | Form water 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Water usage | Form water usage 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Energy usage | Form energy 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Other performance indicators | Form performance 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Waste Subject to Condition 4.2.5 | Waste tonnage return form from the Natural Resources Wales website or other form as agreed in writing by Natural Resources Wales | N/A |

| ### Table S4.3 Reporting forms |  |
| --- | --- |
| ### Media/parameter | ### Reporting format |
| ### Bioaerosol monitoring | ### As specified in the” M9 – Technical Guidance Note (Monitoring) – Environmental monitoring of bioaerosols at regulated facilities, 2018 Version 2”. or other form as agreed in writing by Natural Resources Wales |

Schedule 4 (b) – Reporting effective from 17 August 2022

Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

| Table S4.1 Reporting of monitoring data |  |  |  |
| --- | --- | --- | --- |
| Parameter | Emission or monitoring point/reference | Reporting period | Period begins |
| Emissions to air Parameters as required by condition 3.5.1. | A1 | Every 6 months | 1 January, 1 July |
| Surface water monitoring Parameters as required by condition 3.5.1 | The watercourse adjacent to the site which flows south before joining the Afon-y-Bantwen | Every 3 months | 1 January, 1 April, 1 July, 1 October |
| Bioaerosol emissions as required by condition 3.5.1 | Upwind of the site and downwind of the site | Quarterly or as agreed in writing by the Natural Resources Wales | 1 January, 1 April, 1 July, 1 October |

| Table S4.2: Annual production/treatment |  |
| --- | --- |
| Parameter | Units |
| Compost | tonnes |
| Total amount of waste treated | tonnes |

| Table S4.3 Performance parameters |  |  |
| --- | --- | --- |
| Parameter | Frequency of assessment | Units |
| Water usage | Annually | tonnes |
| Energy usage | Annually | MWh |
| Total raw material used | Annually | tonnes |
| Generation of waste water | Annually | m3 |
| Generation of residue | Annually | tonnes |

| Table S4.4 Reporting forms |  |  |
| --- | --- | --- |
| Media/parameter | Reporting format | Date of form |
| Air | Form air 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Water and Land | Form water 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Water usage | Form water usage 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Energy usage | Form energy 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Other performance indicators | Form performance 1 or other form as agreed in writing by Natural Resources Wales | 08/07/21 |
| Waste Subject to Condition 4.2.5 | Waste tonnage return form from the Natural Resources Wales website or other form as agreed in writing by Natural Resources Wales | N/A |

| ### Table S4.3 Reporting forms |  |
| --- | --- |
| ### Media/parameter | ### Reporting format |
| ### Bioaerosol monitoring | ### As specified in the” M9 – Technical Guidance Note (Monitoring) – Environmental monitoring of bioaerosols at regulated facilities, 2018 Version 2”. or other form as agreed in writing by Natural Resources Wales |

# Schedule 5 - Notification

These pages outline the information that the operator must provide.

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A

| Permit Number |  |
| --- | --- |
| Name of operator |  |
| Location of Facility |  |
| Time and date of the detection |  |

| (a) Notification requirements for any activity that gives rise to an incident or accident which significantly affects or may significantly affect the environment Or any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution; |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Date and time of the event |  |
| Reference or description of the location of the event |  |
| Description of where any release into the environment took place |  |
| Substances(s) potentially released |  |
| Best estimate of the quantity or rate of release of substances |  |
| Measures taken, or intended to be taken, to stop any emission |  |
| Description of the failure or accident. |  |

| (b) Notification requirements for the breach of a permit condition |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Emission point reference/ source |  |
| Parameter(s) |  |
| Limit |  |
| Measured value and uncertainty |  |
| Date and time of monitoring |  |
| Measures taken, or intended to be taken, to stop the emission |  |

| Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| Parameter | Notification period |

| (c) In the event of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment: |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Description of where the effect on the environment was detected |  |
| Substances(s) detected |  |
| Concentrations of substances detected |  |
| Date of monitoring/sampling |  |

Part B - to be submitted as soon as practicable

| Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| Name* |  |
| --- | --- |
| Post |  |
| Signature |  |
| Date |  |

* authorised to sign on behalf of the operator

# Schedule 6 - Interpretation

“accident” means an accident that may result in pollution.

“Annex I” means Annex I to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“Annex II” means Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

“building” means a construction that has the objective of providing sheltering cover and minimising emissions of noise, particulate matter, odour and litter.

“compost” means solid particulate material that is the result of composting, which has been sanitised and stabilised, and which confers beneficial effects when added to soil, used as a component of growing media or used in another way in conjunction with plants.

“composting” means the biological decomposition of organic materials, under conditions that are predominantly aerobic and that allow the development of thermophilic temperatures as a result of biologically produced heat.

“disposal” or “D” means any of the operations provided for in Annex I to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“emissions to land” includes emissions to groundwater.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission or background concentration limit.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“hazardous property” has the meaning in Annex lll of the Waste Framework Directive

“hazardous waste” has the meaning given in the Hazardous Waste (Wales) Regulations 2005 (as amended)

“hazardous substance” means a substance classified as hazardous as a consequence of fulfilling the criteria laid down in parts 2 to 5 of Annex I to Regulation (EC) No 1272/2008

“Industrial Emissions Directive” means DIRECTIVE 2010/75/EU OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 24 November 2010 on industrial emissions

“maturation” means a stage when by agitating and turning the compost it no longer results in reheating and the monitored temperature falls to ambient without the compost being too dry or anaerobic. Phytotoxins that are formed during the 'active' composting phase are metabolised by micro-organisms, which will result in the final material not being harmful to plants. This usually coincides with drop in pH toward neutral, and the conversion of ammonia into nitrates and recolonisation of beneficial micro-organisms. The maturation phase may need active management by turning to prevent the material becoming anaerobic.

“nearest sensitive receptors” means the nearest place to the composting operations where people are likely to be for prolonged or frequent periods. This term would therefore apply to dwellings (including any associated gardens) and to workplaces where workers would frequently be present. It does not apply to the operators of composting facilities or their staff while carrying out the composting operation as their health is covered by Health and Safety legislation.

“Pests” means Birds, Vermin and Insects.

“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“recovery” or “R” means any of the operations provided for in Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“Waste code” means the six digit code referable to a type of waste in accordance with the list of wastes established by Commission Decision 2000/532/EC as amended from time to time (the ‘List of Wastes Decision’) and in relation to hazardous waste, includes the asterisk.

“Waste Framework Directive” or “WFD” means Waste Framework Directive 2008/98/EC of the European Parliament and of the Council on waste.

“Waste Treatment BAT Conclusions” means the BAT Conclusions for the Waste Treatment sector published as a Commission Implementing Decision EU 2018/1447 in the Official Journal of the EU on 17 August 2018

“year” means calendar year ending 31 December.

Where a minimum limit is set for any emission parameter, for example pH, reference to exceeding the limit shall mean that the parameter shall not be less than that limit.

Unless otherwise stated, any references in this permit to concentrations of substances in emissions into air means:

- in relation to emissions from combustion processes, the concentration in dry air at a temperature of 273K, at a pressure of 101.3 kPa and with an oxygen content of 3% dry for liquid and gaseous fuels, 6% dry for solid fuels; and/or

- in relation to emissions from non-combustion sources, the concentration at a temperature of 273K and at a pressure of 101.3 kPa, with no correction for water vapour content.

Schedule 7 - Site plan

© Crown Copyright and database right 2020. Ordnance Survey licence number 100019741.

END OF PERMIT