Part A

| Permit Number | PP3139GB |
| --- | --- |
| Name of Operator | Enovert North Limited |
| Location of Installation | Bangor Road Johnstown Wrexham LL14 6ET |
| Time and date of the detection | When data entered onto database |
| b) Notification requirements for breach of a limit |  |
| To be notified within 24 hours of detection unless otherwise specified |  |
| Emission point reference/source | Leachate Head Limit: >2m Sample Point; Date; Leachate Head (m). HAF LMP2AR; 30/06/2021; 29.74. HAF LMP4b; 30/06/2021; 12.44. HAF LC4; 30/06/2021; 1.82 “inc” offset. HAF LMP1AR; 30/06/2021; 0.90 “inc” offset. HAF LC3; 30/06/2021; 1.67 “inc” offset. |
| Parameter(s) |  |
| Limit |  |
| Measured value and uncertainty |  |
| Date and time of monitoring |  |
| Measures taken, or intended to be taken, to stop the emission | Leachate Levels will continue to be monitored in accordance with Table S3.1 in Environmental Permit PP3139GB. |

Part B

| Any more accurate information on the matters for notification under Part A. | In June 1653 m3 of leachate was tankered off site for treatment. The levels in LMP2AR are considered to be perched due to the design of the well casing with perforations at different levels allowing leachate to enter the chamber at various heights. The pump in LMP2AR had to be removed to allow the well to be raised and waste to be tipped around it. When the dip was taken the pump in LMP2AR had been removed to allow the well to be raised and waste to be tipped around it. As stated in the previous S5’s (HAF288 & HAF290) the manual dips in LMP4b are thought to be due to perched levels. The level has increased by 0.17m since May. The transducer readings in LC3 and LC4 are between the pump set points of 0.2 and 2mtr. Transducer readings have been provided for LMP1AR, LC4 and LC3, where access is unsafe for the monitoring technician. |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment or harm which has been or may be caused by the emission |  |
| The dates of any unauthorised emissions from the installation in the preceding 24 months. | Please see previous notifications. |
| Name* | Elizabeth Forrest |
| Post | Environmental Data Analyst |
| Date | 12.07.2021 |

* authorised to sign on behalf of Enovert North Ltd