| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-014655 |
| Permit Reference Number: | EPR/UP3996ED |
| Permit Holder: | Miss Lorraine Rowlands, Mr Darren Lee Parry, Mr Owen Richard Parry |
| Site Address: | Plas Newydd Home Farm, Llanedwen Road, Llanedwen, LL61 6DQ |
| Grid Reference: | Farm location: SH 51362 69105 Fields: 1 SH 51803 69398 2 SH 51379 69481 3 SH 50913 69474 4 SH 50824 69679 5 SH 50824 69903 6 SH 50956 69796 Storage: None |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk Environment team: TimAmgylcheddMonacArfon@cyfoethnaturiolcymru.gov.uk NLO: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk |
| Date due: | 22/07/2021 |
| Are fields on Anglesey? If yes, then send application. | Yes: ajxht@anglesey.gov.uk pem@anglesey.gov.uk |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 43 |
| Fee on MyNRW | Y | Amount: £1018.00 Date cleared on MyNRW: 07/07/2021 Transaction number:2090932 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | N | Not supplied or required at time of application |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | Y | Resubmission for permit-PAN-014245 |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y | Miss Lorraine Rowlands |
| ## Has the nominated competent person details been provided? | Y | As above |
| ## Has the address and grid reference of the main farm address been given? | Y | SH 51362 69105 |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | 53/023/0070 verified |
| ## Has all of parcels of land table been completed? | Y | Fields 1-6 |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 30.39 |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has consent been given to carry out the activity? | Y |  |
| Has all of the previous land treatment table been completed? | Y | PAN-008833-expired 01/03/2021 |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | Y |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N | No evidence supplied or required at time of application |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | n/a |  |
| Has all of the waste types table been completed? | Y | 02 01 99 |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Google |
| Has all of waste storage table been completed? | n/a |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | n/a |  |
| Has checklist been completed? | Y |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y | Letter saved to authorisation folder in Duly Making in Waste wip |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | n/a |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no spreading within 10 meters) (buffer zones needed) | Y | Afon Braint adjacent to 5, 10m buffer zone on map |
| groundwater source protection zones (permit condition: no spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | n/a |  |
| any spring(or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | Menai Strait and Conwy Bay 243m from 1 |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument x 4 - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y some not all | PS- European Water Vole very close to NW corner of 5; Menai Strait migratory route for Atlantic salmon, eel, lampern and May fish; PH- Purple moor grass and rush pastures close to 6; Lowland dry acid grassland and Lowland calcareous grassland close to 4, 5 ; SAMs : Bryn-Celli-Ddu Standing Stone 173m from 5; Bryn-yr-Hen-Bobl Burial Chamber 209m from 1; Plas Newydd Burial Chambers 190m from 1; Bryn-Celli-Ddu Burial Chamber 140m from 5; AW: Ancient Semi Natural Woodland adjacent to SW corner of 2; Restored Ancient Woodland Site adjacent to1, 2, 3; Plantation on Ancient Woodland Site close to 3; SINC : Coed yr Ardd adjacent to 1, close to 2; Coed Odyn Calch, Coed Bryncelli Ddu within 500m but not close . |
| What is the slope of the fields? Justification + mitigation given in ABS for slope but no buffer zones included for gradient. Steepest part of 1 borders Coed yr Ardd SINC and should have a non-spread buffer zone | 0-7 degrees (transparent)- fields 2,3,5,6 7 -12 degrees (yellow)- southern edge of field 1, small strip at eastern edge of 4 12 degrees plus (red)- southern edge of field 1 Updated map with non-spread buffer zones and SSRA provided 13/08 |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y | Mar 21 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | N |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y | Sept.20 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y | BASIS-R/FE/2596 |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y | Needs more info Clarified and confirmed in updated ABS 13/08 |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y | Clarified and confirmed in updated ABS 13/08 |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y | Contradiction within ABS- Clarified and confirmed in updated ABS 13/08 |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | Y |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y | Confirmed by e-mail of 13/08 + clarification |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. Menai Strait and Conwy Bay SAC is 243m away at closest point of spreading area, distance, low volume of waste to be deployed and appropriate operating procedures should prevent any risk of impact. |  |

Comments

Consultation responses: - Unable to update with “consultation closed” after the deadline has passed because form checked out by officer who then went on leave. - Comments provided in e-mail instead but just repetition of those made for previous deployments* and reminders to follow COGAP etc. - *Buffer zone for Coronation Meadow already included in application and confirmation given that will follow permit conditions and COGAP.

MyMap and location assessment: Saved in projects under PAN-014655 and copies saved in DMS

Protection of habitats and ecosystems No evidence provided/required at this time so no assessment made

Agricultural benefit calculations: Differ from our own-to be queried in RFI- Clarified and confirmed in updated ABS 13/08

Request for information: RFI to provide buffer zone for field 1 at border with Coed yr Ardd SINC-updated map and SSRA; Confirm application rates; Clarify calculations Confirm benefit-insufficient info given RFI sent 12/08/2021 deadline for response 26/08/21 Response, updated map with buffer zones re gradient, updated ABS and SSRA received 13/08. Application rate confirmed as total of 19mᶾ per ha-all queries addressed, RFI fulfilled.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 18/08/2021 |
| ## Effective date | 18/08/2021 |
| ## Permitting Officer Date | Judith Green 18/08/2021 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Anna Palmer 18/08/2021 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 945 |
| Sign off | 30 |
| Issuing | 60 |
| Total | 1095 |